3GPP TSG RAN WG1 Meeting #93	R1-1806889

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	7.1.6.3

Source:	Huawei, HiSilicon

Title:	Discussion on PHR for SUL

Document for:	Discussion and decision

Introduction

In RAN1 #90bis [1] meeting and RAN2#99bis [2] meetings, the followings were agreed for a serving cell configured with SUL:

Agreement: - SRS related RRC parameters are independently configured for SRS on the SUL carrier and SRS on the non-SUL UL carrier in the SUL band combination - SRS can be configured on the SUL carrier and non-SUL UL carrier, irrespective of the carrier configuration for PUSCH and PUCCH

Agreement: Agreements for SUL operation in connected mode: - When SUL is configured there are 2 ULs configured for one DL of the same cell. (FFS how much configuration is provided for the 2 ULs) - At any point in time, each serving cell has at most one PUSCH for transmission Clarification of agreements - In any slot, one PUSCH is used for transmission for a single serving cell (i.e. associated to a single DL). This excludes simultaneous transmission on 2 PUSCH within a single slot but does not restrict switching between the two PUSCH based on L1 /MAC/RRC signalling options. …

PHR is to report the power difference between the configured maximum output power and the predicted transmission power of PUSCH, PUCCH or SRS, so that the BS can make full use of the radio resources with respect to the UE transmission power. But there are still some issues related to serving cells configured with both SUL and non-SUL.

Discussion

PHR for SUL

According to the previous agreements in RAN1 and RAN2, for a serving cell configured with both SUL and non-SUL carriers, the PUSCH on two UL carriers will be scheduled in a slot-based TDM manner. In addition, both PUSCH and SRS can be scheduled on SUL and non-SUL carrier dynamically. As a typical use case where SRS on non-SUL carrier providing channel information for DL MIMO transmission by the channel reciprocity of unpaired band, PUSCH and SRS may be scheduled on different UL carriers (SUL and non-SUL) in different symbols within a same slot, as shown in Figure 1.

The SUL and non-SUL carriers are in different bands, e.g. 3.5GHz for non-SUL and 1.8GHz for SUL respectively, thus the transmission on SUL and non-SUL may use different RF chains including different PAs, as well as probably different beams. As a result, the power control parameters can be quite different between SUL and non-SUL and two independent power controls have been introduced for two UL carriers within one serving cell according to Section 7 of TS 38.213[3]. Obviously, the power headrooms of two UL carriers for one UE can be different for the following reasons,

- Independent power control loop for each UL;

- Independent parameters of power control for each UL, e.g. Pcmax,f,c and P0;

- Quite different fading characteristics of channel propagation between two ULs due to quite different channel frequencies, e.g. 3.5 GHz v.s. 1.8 GHz;

- Different maximum output power reduction (MPR) for each UL , e.g. due to different UL transmission waveforms, DFT-s-OFDM or CP-OFDM [4];

- Different additional maximum output power reduction (A-MPR) for each UL [4].

However, the current PH reporting per cell mechanism is not able to report PH for the two UL carriers in the same cell, simply because only single PH-entry PHR MAC CE can be used for non-CA case like a cell configured with SUL. Therefore, separate PHRs, at least separate PHRs for PUSCH and SRS on different UL carriers within the same cell should be supported.

3.5GHz

TDD

1.8GHz

SUL

PDCCH
PDSCH

PDCCH
PDSCH

PDCCH
PDSCH

PDCCH
PDSCH

PUSCH
PUCCH

SRS

PUSCH
PUCCH

SRS

PUSCH
PUCCH

SRS

PUSCH
PUCCH

SRS

PUSCH
PUCCH

SRS

SRS

Figure 1 Example of a typical use case for PUSCH and SRS transmission

Proposal 1: Power headroom should be reported per UL within a serving cell at least when the SRS and PUSCH are scheduled on different UL carriers.

Based on the above discussion that PH reporting should be per UL carrier within a cell, the PHR MAC CE format should also be changed accordingly.

As to the MAC CE format for reporting PHR for SUL and non-SUL, one option is to define a new MAC CE format with two entries for stacking the UL carriers in a certain cell based on a pre-specified ordering, e.g. SUL first and non-SUL next, etc. Another option is to reuse the MAC CE for CA and define a mapping from entries with the same serving cell index to normal UL or SUL.

Proposal 2: The following two options can be considered as the MAC CE format for reporting PHRs for SUL and non-SUL:

Option 1: New MAC CE with 2 entries, one for SUL and the other one for non-SUL

Option 2: Reuse MAC CE for CA and define a mapping from entries with the same serving cell index to non-SUL and SUL

Conclusions

In this contribution, the PHR for serving cells configured with SUL and non- SUL is discussed. We have the following proposals:

Proposal 1: Power headroom should be reported per UL within a serving cell at least when the SRS and PUSCH are scheduled on different UL carriers.

Proposal 2: The following two options can be considered as the MAC CE format for reporting PHRs for SUL and non-SUL:

Option 1: New MAC CE with 2 entries, one for SUL and the other one for non-SUL

Option 2: Reuse MAC CE for CA and define a mapping from entries with the same serving cell index to non-SUL and SUL

- References

RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #90bis, Prague, CZ, 9th – 13th, October 2017.

RAN2 Chairman’s note, 3GPP TSG RAN WG2 Meeting #99bis Prague, Czech Republic, 9th – 13th October 2017

3GPP TS 38.213, NR; Physical layer procedures for control, v15.1.0.

3GPP TS 38.101-1, NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, v15.1.0.