3GPP TSG RAN WG1 Meeting #93         　　	                              R1-1807294

Busan, Korea, May 21st – May 25, 2018

Source:	Mitsubishi Electric

Title:	Discussions on intra-UE multiplexing between URLLC and eMBB

Agenda Item:	7.2.2 Others

Document for: 	Discussion/Decision

Introduction

In RAN1 #92b, the following conclusion was made to postpone discussion on inter-UE UL multiplexing for Rel. 16 [1]. It was also proposed to continue discussion on multiplexing between eMBB and URLLC for the intra UE scenario, aiming for possible specification for Rel. 15. In this contribution, we make proposals on approaches to multiplex eMBB and URLLC data.

Conclusion: - There is no concensus in Rel-15 to support handling inter-UE UL multiplexing of transmission with different reliability requirements (including the potential need for UL UE pre-emption)

Handling URLLC and eMBB PUSCH for intra UE

Intra UE grant based transmission

The following proposal was made in R1-1805629 in RAN1#92b [2].

Proposal: For a UE supporting URLLC, if the UE receives a dynamic grant scheduling PUSCH overlapping in time with another PUSCH scheduled by an earlier received grant, the UE follows the later received grant and the previously scheduled PUSCH is dropped

In the above proposal, it is suggested to follow the later grant for PUSCH transmission in case earlier and later grant correspond to eMBB and URLLC PUSCH, respectively. The intention is to prioritize URLLC PUSCH transmission in case eMBB and URLLC PUSCH overlap in time domain.

In the above proposal, timing of receiving the second grant should be clarified. If the second grant is received before transmission of PUSCH scheduled by the earlier grant, the transmission can be canceled. On the other hand, if the second grant is received during the transmission of PUSCH scheduled by the earlier grant, the transmission of PUSCH can be stopped.

Hence, it is possible to receive the second grant before and after eMBB PUSCH transmission, whether to drop the ongoing transmission should also be considered in the proposal. Thus, the following proposal is made.

Proposal 1:

For a UE supporting URLLC, if the UE receives a dynamic grant scheduling PUSCH which postpones transmission of PUSCH scheduled by an earlier received grant, the UE follows the later received grant and the previously scheduled PUSCH is dropped or transmission of the previously scheduled PUSCH is stopped

Switching BWP between URLLC and eMBB

In NR, BWP can be switched for URLLC transmission. In this case, different numerology (i.e., wider subcarrier spacing) can be used for URLLC transmission to realize low latency transmission. Hence, separate BWP from eMBB’s BWP can be scheduled for URLLC transmission. In addition, to protect URLLC data from interferences, UE may switch BWP for URLLC transmission.

As an example, as shown in Figure 1, URLLC PUSCH transmission in different BWP is scheduled during eMBB PUSCH transmission. More detailed operations for DL and UL are illustrated in Figure 2. Based on this example, we make the following proposal.

Proposal 2:

Study how active BWP can be switched in the middle of eMBB transmission to transmit URLLC data in a different BWP

Regarding the above proposal, at least the following two issues need to be investigated

- Signaling mechanism (how to switch BWP in the middle of eMBB transmission)

- Treatment of eMBB in the currently active BWP

  - Drop eMBB, an example of the operation is shown in Figure 2.

  - Resume eMBB transmission after the end of URLLC data transmission, i.e., BWP needs to be switched back to the original BWP for this operation

Figure 1 Switching BWP in the middle of eMBB transmission

Figure 2 An example of cancellation of eMBB transmission and BWP switching

Grant free based transmission

In the grant-free based transmission, GF (grant free) resource locations can be specified by the higher layer for URLLC transmission. GF resource for URLLC transmission can be used by eMBB. However, in case of transmission of URLLC in the GF resource, URLLC is prioritized.

Conclusion

In this contribution we make the following proposals:

Proposal 1:

For a UE supporting URLLC, if the UE receives a dynamic grant scheduling PUSCH which postpones transmission of PUSCH scheduled by an earlier received grant, the UE follows the later received grant and the previously scheduled PUSCH is dropped or transmission of PUSCH is stopped

Proposal 2:

Study how active BWP can be switched in the middle of eMBB transmission to transmit URLLC data in a different BWP

- References

- [1]	Draft Minutes Report of 3GPP TSG RAN WG1 #92b, Sanya, China, Apr. 2018.

- [2]	R1-1805629, “Summary of handling UL multiplexing of transmission with different reliability requirements”, vivo, RAN1#92bis, April 2018.