3GPP TSG RAN WG1 Meeting #93																		R1-1807498

Busan, Korea, May 21st – 25th, 2018

Agenda item:	6.2.6.5

Source:	Samsung

Title:	Summary on Flexible PDSCH/PUSCH starting PRB

Document for:	Discussion and Decision

# 1. Introduction

In RAN1 meeting #92, the following agreements were made, as following:

Agreement

For a BL/CE UE, higher layer signaling enable/disable

- the more flexible starting PDSCH PRB

- the more flexible starting PUSCH PRB

This makes no assumption about signaling details

Agreement

- For a BL/CE UE configured with flexible starting PDSCH PRB and max 1.4 MHz PDSCH channel bandwidth,

  - PDSCH frequency hopping is supported.

    - Details of frequency hopping is FFS

- For a BL/CE UE configured with flexible starting PUSCH PRB and max 1.4 MHz PUSCH channel bandwidth,

  - PUSCH frequency hopping is supported.

    - Details of frequency hopping is FFS

In RAN1 meeting #92bis, the following agreements were made, as following:

Agreement

At least for BL/CE UE configured without other Rel-14/15 features, DCI size after padding is not increased in order to support flexible starting PRB.

Note: For PUSCH and PDSCH, the solution may or may not be the same

Note: The solution may or may not be the same for FDD and TDD

This contribution summarizes solutions from contributions for flexible PDSCH/PUDSCH starting PRB.

# 2. Flexible starting PRB for PDSCH

# 2.1 For CE mode A

Solution #2a: Use all or some of the unused 11 states in 5 LSB bits of ‘Resource block assignment’ field in DCI format 6-1A to extend RA. FFS between:

- Alt 1: Definition of one common set of RIV values 21-31 to be used for all system bandwidths and narrowbands

  - Support by:  Ericsson, Samsung, ½ Sony, ½ Nokia, ½ NSB

- Alt 2: Define the usage of the RIV values 21-31 specifically for system bandwidths and narrowbands

  - Support Huawei, HiSilicon, ZTE,  Qualcomm

Solution #2b: Use up to 2^⌈log_2 ⌊N_RB^DL/6⌋⌉ ∙11 unused states in the ‘Resource block assignment’ field in DCI format 6-1A to extend RA

  - Support by:  Sharp, ½ Sony, ½ Nokia, ½ NSB

Proposal #1: For PDSCH in CE mode A, use all or some of the unused states in DCI format 6-1A to extend RA.

# 2.2 For CE mode B

Solution #3: Introduce 1 bit in RRC to enable the shift of narrowband to align with RBG. The shift of narrowband depends on the system bandwidth and the allocated narrowband.

- Support by:  Ericsson, Nokia, NSB, Sony, Samsung, Qualcomm

Solution #4:  Introduce 1 bit in DCI to indicate shift of narrowband to align with RBG. The PRB shift definition depends on the system bandwidth and the allocated NB.  Not align with agreement

- Support by:  Huawei, HiSilicon

Proposal #2: For PDSCH in CE mode B, introduce 1 bit in RRC to enable the shift of narrowband to align with RBG. The shift of narrowband depends on the system bandwidth and the allocated narrowband.

# 3. Flexible starting PRB for PUSCH

# 3.1 For CE mode A

Solution #1: Use resource allocation type 2 with starting PRB within the full system BW.

- Support by:  Ericsson (FDD), Qualcomm

Solution #2: Use all or some of the unused 11 states in DCI format 6-0A to extend RA.

- Support by:  Ericsson (TDD), Huawei, HiSilicon, Nokia, NSB, ZTE

Solution #3: Introduce 1 bit in RRC to indicate shift of narrowband to align with RBG. The PRB shift definition depends on the system bandwidth and the allocated NB.

- Support by:  Sony, Samsung

Proposal #3: For PUSCH in CE mode A, use DCI to provide more flexible resource allocation for PUSCH. FFS on use RA type 2 or use all or some of the unused 11 states in DCI format 6-0A to extend RA.

# 3.2 For CE mode B

Solution #3: Use RRC to indicate shift of narrowband or resource allocation to align with RBG. FFS between:

- Alt1: 1 bit to enable the narrowband shift. The shift of narrowband depends on the system bandwidth and the allocated narrowband.

  - Support by:  Nokia, NSB, Sony, Samsung, Qualcomm

- Alt 2: 1 bit to configure the shift the 2-RB start from the 4th RB in the NB for RIV=7

  - Support by:  Qualcomm

- Alt 3: Adjust the starting PRB of the resource allocation for 2 PRBs by {1, 2, 3} PRBs towards higher PRB index.

  - Support by:  Ericsson

Solution #4:  Introduce 1 bit in DCI to indicate shift of narrowband to align with RBG. The PRB shift definition depends on the system bandwidth and the allocated NB. Not align with agreement

- Support by:  Huawei, HiSilicon

Proposal #4: For PUSCH in CE mode B, use RRC to indicate shift of narrowband or resource allocation to align with RBG.  Further discuss online to down select among the above solutions.

# 4. Others

Proposal #5: Flexible starting PUSCH PRB for BL/CE UE configured with 5 MHz max PUSCH channel bandwidth is not considered further in Rel-15.

# 5. Reference

| R1-1805853 | Flexible PDSCH/PUSCH starting PRB for MTC | Ericsson |
| --- | --- | --- |
| R1-1805982 | On more flexible starting PRB for PDSCH/PUSCH resource allocation | Huawei, HiSilicon |
| R1-1806176 | Support for flexible PDSCH/PUSCH starting PRB | Nokia, Nokia Shanghai Bell |
| R1-1806189 | Flexible starting PRB of PUSCH/PDSCH for MTC | ZTE |
| R1-1806562 | On flexible PDSCH/PUSCH PRB allocation in efeMTC | Sony |
| R1-1806686 | Discussion on Flexible PDSCH/PUSCH starting PRB | Samsung |
| R1-1806938 | On more flexible PDSCH/PUSCH resource allocation | Sharp |
| R1-1807106 | Flexible allocation for PDSCH and PUSCH | Qualcomm Incorporated |