3GPP TSG-RAN WG1 Meeting #93	R1-1807590

Busan, Korea, May 21st – 25th, 2018

Source: 	WI rapporteur (Ericsson)

Title:	RAN1 agreements for Rel-15 Further NB-IoT enhancements

Agenda Item:	6.2.7

Document for:	Information

# Introduction

This contribution lists RAN1 agreements made for Rel-15 Further NB-IoT enhancements (WI code NB_IOTenh2, WID in RP-172063) until after the RAN1#91 in Nov 2017.

Since the purpose of this contribution is to serve as input to RAN1 CR drafting, this contribution does not capture observations, simulation assumptions and other informative notes, only agreements, working assumptions, and conclusions with potential RAN1 specification impacts. Some agreements have been combined or edited for better readability. Agreements, working assumptions and FFSs that have been made obsolete by later agreements are usually not included. Note that this document is just an interpretation of the RAN1 agreements, i.e. this document does in no way override the official RAN1 minutes.

# Higher layer parameters (36.331)

A list of higher layer parameters is provide in R1-1807573.

# Latency and power consumption reduction

## Power consumption reduction for paging and connected-mode DRX

RAN1#88bis agreements: - Techniques to be evaluated: - Wake-up signal/channel (either relying or not relying on DL synchronization) - Go-to-sleep signal/channel (either relying or not relying on DL synchronization) - Compact DCI - Dynamic USS periodicity - The use of the technique can be semi-statically enabled/disabled by the network - Simulation assumption on signal for power consumption reduction is agreed in R1-1706777. RAN1#89 agreements: - A physical signal/channel indicating whether the UE needs to decode subsequent physical channel(s) is introduced, at least for idle mode paging. Candidates for the signal/channel are: - Wake-up signal or DTX - Go-to-sleep signal or DTX - Wake-up signal with no DTX - Downlink control information - FFS whether synchronization to the camped-on cell is assumed for detecting/decoding WUS/GTS, depending on the (e)DRX cycle length - Design details are FFS - Connected mode DRX is FFS - The impact of the physical signal/channel, on Idle mode physical layer paging performance (missed paging detection and paging reception latency) should be studied and reported with the physical signal/channel design. - The current paging mechanism is used as the baseline for evaluation. RAN1#90 agreements: - For idle mode, - The power saving signal in a cell supports being applied to FFS between: - All the UEs associated to a PO in the cell - A group of more than one of the UEs associated to a PO in the cell - Both (a) and (b) - How many POs the power saving signal applies to from the UE perspective is FFS between - A single PO only - One or more than one PO (details are FFS) - How many POs the power saving signal applies to from the eNB perspective is FFS between - A single PO only - One or more than one PO (details are FFS) - The power saving signal applicable to a UE is sent on the same paging carrier as the associated subsequent physical channel(s) Others: Email discussion to agree simulation assumptions for NB-IoT and eMTC power consumption evaluation of saving signal/channel. Agreed simulation assumptions are given in R1-1714993 for NB-IoT. RAN1#90bis agreements: - The UE is configured with a transmission duration of WUS by higher layers - The WUS signal may be decoded with or without relying on prior synchronization - Whether the UE needs to acquire (further) synchronization using NPSS/NSSS to decode the NPDCCH following the WUS is FFS - The power saving of using existing synchronization signal to achieve sync and using WUS for synchronization should be compared. - For RAN#78 timeline, RAN4 can assume the UE is synchronized prior to the WUS - After RAN#78, RAN1 and RAN4 will conduct additional work to allow the assumption that the UE is not synchronized prior to the WUS (with the same WUS signal design) for 144 dB MCL; and will study to allow the assumption that the UE is not synchronized prior to the WUS (with the same WUS signal design) for 154, 164 dB MCL - Status of work on ‘Relaxed monitoring for cell reselection’ in RAN2/4 should be considered - RAN1 assumes that introduction of WUS does not alter PO/PF definition - At least in a UE’s DRX cycle: - WUS supports at least being applied to all the UEs monitoring WUS associated to a PO in an NB-IoT carrier; - FFS: eNB can configure WUS being applied to a group of more than one of the UEs associated to a PO in an NB-IoT carrier. - At least in a UE’s DRX cycle, one WUS informs UE whether to monitor the PO in a single DRX cycle - WUS signal is at least cell-specific; - FFS scrambling of WUS including time varying scrambling - Long ZC sequence based signal is considered as the starting point for WUS signal: - FFS: whether the sequence can span over multiple subframes - FFS: whether accumulated multiplication is applied between sub-sequences from the long ZC sequence to reduce the impact of frequency error; - FFS: Support transmit diversity for NB-IoT WUS - FFS: NSSS like signal is used as the wake-up signal RAN1#90bis Working assumptions: - At least in a UE’s DRX cycle, how the UE knows the WUS time location, is: - A WUS has a time location which is configurable with respect to the associated PO(s) location(s) RAN1#91 agreements: - The [maximum] duration of WUS is configured in SIB per NB-IoT carrier as one value from a list. FFS if the list: - depends on Rmax and if so the number of lists specified - is a single list for all Rmax - Note: the Rmax refers to the one configured for paging - The non-zero gap from the end of the configured [maximum] WUS duration to the associated PO is configurable - FFS the minimum duration - FFS the configuration is explicit or implicitly derived - The [maximum] duration of WUS is configured per NB-IoT carrier - FFS: WUS actual transmission duration can be shorter than the configured maximum duration of WUS. - Alt 1: The actual WUS duration is transmitted aligning to the start of the configured maximum duration of WUS. - Alt 2: The actual WUS duration is transmitted aligning to the end of the configured maximum duration of WUS. - There is a non-zero gap from the end of configured [maximum] WUS duration to the associated PO - FFS: exact value of non-zero-gap - FFS if it is fixed in spec or configurable explicitly, or known implicitly from other configured parameters - The network can enable or disable use of the WUS - How UE acquires information on WUS enabling/disabling is up to RAN2 decision - UE can assume all the REs for transmission of WUS in a given subframe use the same antenna port. - The UE shall not assume the transmission of WUS in more than X consecutive subframes use same antenna port. - FFS: value of X - WUS sequence is based on ZC-sequence - When designing WUS sequence, negative impact on legacy NSSS detection should be avoided. RAN1#91 Working assumptions: - WUS sequence is a sequence mapping within one subframe as a basic unit and repeated/extended for multiple subframes to support larger coverage (WA confirmed in RAN1#92). - Prioritize to minimize impact on UE synchronization performance. RAN1#92 agreements: - The maximum duration of WUS is configured in SIB per NB-IoT carrier as one value from a list. - WUS actual transmission duration can be shorter than the configured maximum duration of WUS. - The list used for configuring maximum duration of WUS at least depends on Rmax associated type 1 CSS, and FFS the number and exact values of the scaling factors between maximum duration of WUS and Rmax associated type 1 CSS - There is a non-zero gap from the end of configured maximum WUS duration to the associated PO - FFS: exact value of non-zero-gap - FFS if it is fixed in spec or configurable explicitly, or known implicitly from other configured parameters - The maximum duration of WUS is configured in SIB per NB-IoT carrier as one value from a list. FFS if the list: - depends on Rmax and if so the number of lists specified - is a single list for all Rmax Note: the Rmax refers to the one configured for paging - The non-zero gap from the end of the configured maximum WUS duration to the associated PO is configurable - FFS the minimum duration - FFS the configuration is explicit or implicitly derived - In eDRX, from the UE perspective, the default UE configuration is a one-to-one mapping between WUS and PO. - In eDRX, from the UE perspective, an optional UE configuration is a 1-to-N mapping between WUS and PO. Note: The WUS design and configuration for eDRX must allow the network to reach a UE within a PTW. - Confirm the working assumption that the WUS sequence is a sequence mapping within one subframe as a basic unit and repeated/extended for multiple subframes to support larger coverage. - WUS conveys the cell ID; - FFS: UE group ID - FFS how/whether to handle the case of false alarm resulting from detecting WUS corresponding to different POs/UE groups (if introduced) RAN1#92bis agreements: - At least WUS/DTX is supported at least for paging for RRC_IDLE UEs. - Confirm the following working assumption - WUS transmission relative to associated PO of subgroup of UEs is aligned to the start of the configured maximum duration of WUS. - Note: the above applies to at least the case where the gap is large enough for scheduling UE - Note: the above does not imply that subgroup of UEs is introduced and that subgroup is TDM - 3 bits are used to indicate the scaling factors between maximum duration of WUS and Rmax associated with type 1 CSS. - The gap between the end of configured maximum WUS duration and the first associated PO is equal or larger than the minimum value which is implicitly or explicitly configured and is an absolute number of subfames - There are at least 10 valid subframes between the end of configured maximum WUS duration and the first associated PO. - FFS: whether to define UE capability for wake-up time - WUS is postponed in subframes that are not NB-IoT DL subframes and is not transmitted [FFS dropped/postponed] in the subframes that carries SIs other than SIB1. - Note: “Postpone” means the corresponding subframes are not counted as configured maximum WUS transmission duration and actual WUS transmission duration. - Note: This does not imply that the minimum gap between the end of actual WUS duration and the first associated PO is reduced - WUS can be time-varying from subframe to subframe. - Besides information on cell ID, WUS signal is designed based on the following information: - UE group ID (if introduced) - time information of the starting subframe of the WUS or PO (Paging Occasion) - FFS: (part of) the SFN information - When generating power saving signal in a subframe where NRS is assumed to be transmitted, the power saving signal in the subframes shall be generated and mapped in the same way as in other valid subframes where NRS is not assumed to be transmitted; - WUS is punctured in RE-level by NRSs. - WUS signal in a subframe is as follows, where LZC = 131 for inband mode (FFS for SA and GB modes) - FFS RE-level cover codes/RE-level scrambling sequence c(m) using - Hadamard codes - Gold sequences - M sequences - FFS phase shift - FFS detailed design for time-varying - 11 symbols for inband mode and 14 symbols for SA and GB modes - Strive toward as much as possible commonality between SA/GB and inband RAN1#93 agreements: The following working assumption in RAN1#92 has been automatically confirmed on NB-IoT WUS: - For UE operating WUS, UE is allowed to relax RRM measurements, at least for low mobility UEs, to once every N DRX cycles, where N is FFS - The RRM measurement relaxation is enabled/disabled by the network. - WUS can still be enabled by the network when the RRM measurement relaxation is disabled. - Note: This does not imply any change in the random access procedure / power control / CE level selection, nor relaxations in the requirements related to the random access procedure. - WUS provides sync of up to the timing/frequency offset resulting from not synchronizing for N DRX cycles - FFS for eDRX case - FFS whether N depends on the length of PTW. - For 164dB MCL, there is a WUS configuration that enables sync for at least a value of N>1 for at least the smallest DRX cycle. - FFS for other DRX cycles and values of N - FFS whether N is fixed, configurable, or depends on the DRX cycle on eMTC WUS/sync: - For eMTC, a new periodic synchronization signal is introduced. - The new periodic sync signal is configurable (including OFF/ON configuration) - FFS on the functionality/information provided by the synchronization signal, including - whether the additional synchronization signal can provide WUS-related information for a subset or a group of POs - In case the additional synchronization signal provides WUS-related information, FFS whether there is an additional WUS/DTX signal, which may be separately configured. - In case the additional synchronization signal does not provides WUS-related information, there is an additional WUS/DTX signal - System information change notification - FFS on location of the sync signal - The new synchronization signal can be used also for non-WUS purposes (e.g. by UEs that need to synchronize after exiting PSM state) - FFS whether the “new sync signal” can reuse the NB-IoT WUS sequence or a different sequence.. For UE capability - The UE is not required to monitor the actual WUS duration other than power of 2 values from 1ms to the maximum WUS duration. - The UE reports one capability for "minimum gap between WUS and associated PO" for eDRX, selected among {Y1, Y2, …, YN}. - There is no UE capability reporting for “minimum gap between WUS and associated PO” for DRX (i.e, the minimum gap is fixed in specification). - {Y1, Y2, …, YN}={40ms, 240ms, 1s, 2s} for eDRX. - The “minimum gap between WUS and associated PO” is fixed as 40ms for DRX. - Gold sequence is used as the RE-level cover codes/RE-level scrambling sequence c(m), and phase shift θ=0. - For Guard Band and Standalone modes, the WUS signal in a subframe is generated as follows: - The WUS signal in the last 11 OFDM symbols are exactly the same as that under inband mode - Copy 3 OFDM symbol from the 8th, 9th, and 10th OFDM symbols to the first 3 OFDM symbols of the subframe - The UE can assume the transmission of WUS in at least one subframe use same antenna port. Note: the meaning of “at least” is clarified during the CR phase. - UE shall not assume that the WUS is transmitted on the same antenna port as any of the downlink reference signals or synchronization signals. - c(m) is generated the same as the RE-level scrambling of Rel-14 NPDCCH/NPDSCH; - c(m) shall be initialized at the start of WUS with cinit_WUS=(NIDNcell+1)10nf_start_PO+ns_start_PO2mod 2048+129+NIDNcell For eNB configuration - There is no explicit signaling of WUS TX diversity configuration. If only one NRS port is configured by the eNB, the UE may assume the transmission of all WUS subframes is using the same antenna port; otherwise, the UE can assume the transmission of WUS in at least one subframe is using the same antenna port. - Support WUS with synchronization of up to the timing/frequency offset resulting from not synchronizing due to the RRM measurement relaxation of N DRX cycles, where N is configured from the list of {1, 2, 4, 8} with the condition that N DRX cycles <= 10.24s. Configuration of N is cell specific. - The scaling factor between maximum duration of WUS and Rmax associated with type 1 CSS is selected from {1/128,1/64,1/32,1/16,1/8,1/4,1/2}. - Define candidate values for actual WUS duration for UE blind detection. - Higher-layer configured WUS_offset is the non-zero gap from the end of the configured maximum WUS duration to the associated PO, as a number of absolute subframes. - WUS is dropped in subframes that carry SIB1-NB and other SIs. - WUS is postponed in subframes that are not NB-IoT DL subframes and does not carry SIB1-NB. - Note: this reverses the previous agreement of “WUS is postponed in subframes that are not NB-IoT DL subframes and is not transmitted [FFS dropped/postponed] in the subframes that carries SIs other than SIB1.”. - Note: “Postpone” means the corresponding subframes are not counted as configured maximum WUS transmission duration and actual WUS transmission duration. - Note: This does not imply that the minimum gap between the end of actual WUS duration and the first associated PO is reduced - WUS is punctured in CRS REs for in-band deployment - Sub-grouping is not supported in Rel-15. - For DRX, a gap duration is configured with a value range of {40ms, 80ms, 160ms, 240ms}. - For eDRX, a short gap is configured with a value range of {40ms, 80ms, 160ms, 240ms} and another longer gap can be optionally configured with a value range of {1s, 2s}. - The short gap can be same or larger than the gap configured for DRX. - For eDRX, if a short gap (G1) and a longer gap (G2) are both configured, and the configured gap (G1 for Case 1 and G2 for Case2) is the same as “minimum gap between WUS and associated PO” reported by UE UE reported one capability of “minimum gap between WUS and associated PO”; UE expects the WUS with G1; UE expects the WUS with G2. Case 1: {40ms or 240ms}; Y; N. Case 2: {1s or 2s}; N; Y. - If the configured gap (G1 for Case 1 and G2 for Case2) is other than the UE reported capability of “minimum gap between WUS and associated PO”, - Case A: if the configured gap is larger than the UE reported capability of “minimum gap between WUS and associated PO”, UE monitors WUS with the configured larger gap - UE could wait until the end of the configured gap to detect paging. - Case B: if the configured gap is smaller than the UE reported capability of “minimum gap between WUS and associated PO”, UE does not monitor WUS with the configured long gap for eDRX and uses short gap to monitor WUS with the configured short gap.

## Reduced system acquisition time

RAN1#88bis agreements: - For reduced system acquisition time for NB-IoT, at least the following candidates can be considered - Enhancement(s) to NPSS/NSSS - Enhancement(s) to MIB-NB - SIB1-NB accumulation across multiple SIB1-NB TTIs (with or without specification impact) - New mechanism allowing to skip SIB1-NB and/or SI messages and/or MIB-NB reading - Additional SIB1-NB is transmitted on other subframes in addition to the existing SIB1-NB transmission - Use of physical signal/channel in agenda item 7.2.7.1.1 (if introduced) - FFS on other SIBx-NB - Details of all solutions are FFS - Solutions need to be backwards compatible and take care of impacts to Rel-13/Rel-14 networks RAN1#89 agreements: - Additional transmissions of NPSS/NSSS in subframes other than those used in Rel-13 for in-band, guard-band and standalone are not considered in Rel-15 for an anchor carrier - FFS: NPSS/NSSS transmission using unused 3-OFDM symbols in existing subframe#5,9 on anchor carrier at least for stand-alone and guard band modes RAN1#90 agreements: - For enhancement of MIB-NB acquisition time, the following candidates are considered: - Options 1: Using the unused 3-OFDM symbols in subframe#0 on the anchor carrier in standalone and guard-band operation modes - Options 2: Introducing additional subframe(s) for NPBCH transmission, FFS if there is a possible restriction to non-anchor carriers - Option 3: Enhanced MIB-NB decoding techniques, e.g. combining the NPBCH across several MIB-NB TTIs. - At least the following issues are FFS regarding whether to introduce support for the network to optionally transmit additional repetitions(s) of SIB1-NB, in subframe(s) other than R13 existing SIB1-NB transmission subframes: - The position of the subframe(s) for additional Rel-13 SIB1-NB repetitions - The scrambling on the new additional repetitions of SIB1 - The pattern of mapping to resource elements for the additional repetition(s) (e.g., order of coded bits-to-subframe allocation) - If a SIB1-NB with additional subframe(s) can be transmitted on a non-anchor - How the UE determines if the additional subframe(s) are being transmitted - The number of additional subframe(s) - How to handle correct understanding of NPDCCH/NPDSCH transmission for legacy NB-IoT UEs. RAN1#90bis agreements: - SIB1-NB can be additionally transmitted in subframe(s) other than Rel.13 existing SIB1-NB transmission subframes on the anchor-carrier. - Additional SIB1-NBs are transmitted on subframe #3 - The periodicity of additional SIB1-NB transmissions is 20 ms and in the same radio frame as legacy transmission - FFS the supported number(s) of additional transmissions of SIB1-NB - There is no signalling of the number of additional SIB1-NB transmissions - The TBS, coding, and modulation of additional SIB1-NB repetitions are the same as the existing ones for Rel.13 SIB1-NB - FFS scrambling - FFS: The sequence of coded bits-to-subframe allocation of additional SIB1-NB transmission can be interleaved compared to the existing SIB1-NB transmission - When additional SIB1-NBs are transmitted, the subframe(s) carrying additional SIB1-NB(s) can be declared as invalid downlink subframe by downlinkBitmap - Rel.15 UEs interpret invalid downlink subframes whose indices are corresponding to additional SIB1-NBs transmissions but not carrying additional SIB1-NB (and NSSS) as valid downlink subframes - Additional SIB1-NB transmission can be configured by eNB, and the presence of additional SIB1-NB can be indicated by one of unused bits in MIB-NB - FFS if additional SIB1-NB transmissions are also supported on non-anchor carriers RAN1#91 agreements: - SIB1-NB transmissions in non-anchor carrier are not further considered in Rel-15 for NB-IoT FDD. - Update the agreement from the RAN1#90bis meeting as follows - When additional SIB1-NBs are transmitted, the subframe(s) carrying additional SIB1-NB(s) can be declared as invalid downlink subframe by downlinkBitmap - Rel.15 UEs interpret invalid downlink subframes whose indices are corresponding to additional SIB1-NBs transmissions but not carrying additional SIB1-NB as valid downlink subframes only when the UE attempts to decode DCI format N0/N1 scrambled by C-RNTI in UE-specific search space or receive NPDSCH scheduled by DCI format N1 scrambled by C-RNTI in UE-specific search space. - For the repetition number 4 and 8, the total number of subframes for additional SIB1-NB transmission will be down selected between the following alternatives - (Alt.1) no additional SIB1-NB transmission - (Alt.2) half as many as that of the legacy SIB1-NB transmissions - (Alt.3) the same as that of the legacy SIB1-NB transmissions - For the repetition number 16, the total number of subframes for additional SIB1-NB transmission will be down selected between the following alternatives - (Alt.1) the same as that of the legacy SIB1-NB transmissions - (Alt.2) depends on code rate (e.g., TBS, #CRS/NRS ports, operation mode) of SIB1-NB - When the code rate of SIB1-NB is equal to or larger than X, additional SIB1-NB subframes are transmitted on every other subframe #3 - Otherwise, additional SIB1-NB subframes are transmitted on every 4th subframe #3 - If the total number of subframes for additional SIB1-NB transmission is less than that of the legacy SIB1-NB transmissions, the starting radio frame index of additional SIB1-NB transmission depends on Cell ID and the number of SIB1-NB repetition scheduled by MIB-NB Note that this is not relevant to the case of 4 and 8 repetitions and this does not imply any selection among Alt. 1, 2, 3 in the above agreement - The sequence of coded bits-to-subframe allocation of additional SIB1-NB transmission will be down selected between the following alternatives - (Alt.1) The additional SIB1-NB uses the same coded bits-to-subframes mapping as the legacy SIB1-NB - (Alt.2) reuse coded bits generated for existing SIB1-NB transmission while coded bits-to-subframe allocation is circularly shifted as much as 8 radio frames compared to the existing SIB1-NB transmission - (Alt.3) the coded bits that are mapped to subframe #3 used for additional SIB1-NB transmissions are generated by continuing reading from the virtual circular buffer - Scrambling sequence will be generated and applied to subframes for additional SIB1-NB following one of the alternatives below - (Alt.1) The additional SIB1-NB reuses the bit-level scrambling mechanism of legacy SIB1-NB, and uses the same symbol-level scrambling mechanism as NPBCH by replacing the initialization equation with cinit=nRNTI∙215+(NIDNcell+1)(nfmod 59+1) - (Alt.2) The scrambling sequence generator for additional SIB1-NB transmission is initialized with cinit=(nRNTI-1)∙215+(NIDNcell+1)(nfmod 61+1) - (Alt.3) The scrambling sequence used for the new SIB1-NB subframe is generated based on advancing the Gold sequence generators used for generating the scrambling sequence for SIB1-NB in subframe #4 in the same frame by 2560 shifts - (Alt.4) Keep the same scrambling sequence as the legacy one if the additional SIB1-NB does not use the same coded bits-to-subframes mapping as the legacy SIB1-NB RAN1#92 agreements: - For additional SIB1-NB - For the repetition number 4 and 8, the following alternative is downselected for the total number of subframes for additional SIB1-NB transmission - no additional SIB1-NB transmission - For the repetition number 16, the following alternative is downselected for the total number of subframes for additional SIB1-NB transmission - the same as that of the legacy SIB1-NB transmissions - The following alternative is downselected for the sequence of coded bits-to-subframe allocation of additional SIB1-NB transmission - the coded bits that are mapped to subframe #3 used for additional SIB1-NB transmissions are generated by continuing reading from the virtual circular buffer - The following alternative is downselected for the scrambling sequence applied to subframes for additional SIB1-NB - Keep the same scrambling sequence as the legacy one - For NPBCH - Do not introduce new NPBCH repetitions - For MIB skipping - “From RAN1 point of view, SI acquisition latency reduction after SI modification indication is expected if the UE can skip MIB-NB decoding, e.g., by using Direct indication information field in the DCI format N2 scrambled by P-RNTI.” RAN1#92bis: No agreements were made in this meeting.

## Data transmission during the random-access procedure

RAN1#89 agreements: - From RAN1 perspective, it is beneficial to support early data transmission for NB-IoT UEs with any coverage. RAN1#90bis agreements: - From RAN1 point of view, it is feasible to support early UL data transmission in Msg3 from an NB-IoT UE using some TBS value(s) from the TBS range specified for NB-IoT in Rel-13 with a maximum total TBS of 1000 bits. - FFS if and how there will also be a larger supported maximum total TBS - The detailed value(s) should consider the payload size of early data packets from RAN2. - From RAN1 perspective, the physical layer design will assume eNB is not required to always provide a grant of a larger TBS for Msg3 and can decide to just provide a grant for 88 bits instead RAN1#91 agreements: - The number of MCS/TBS/RU states that can be used for EDT will be chosen from - Limited MCS/TBS/RU states - Alt. 0: 5 unused MCS/TBS/RU states and 0 bit in SIB - Alt. 1: As many as supported by using 1 spare bit from RAR and 0 bit in SIB - Alt. 2: As many as supported by using 2 spare bits from RAR and 0 bit in SIB - Alt. 3: As many as supported by using 2 bits in SIB and 0 spare bit in RAR - Alt. 4: As many as supported by using maximum TBS value in SIB and 0 spare bit in RAR - Alt. 5: 1 spare bit in RAR used for new/modified UL grant and 0 bit in SIB - From RAN1’s point of view, - Uplink subcarrier spacing field, subcarrier indication field, scheduling delay field and Msg3 repetition number field in RAR UL Grant for uplink EDT in Msg3 do not need to be changed according to current RAN2 agreements. - The above applies to above Alts. 1-4 RAN1#92 agreements: - The maximum TBS broadcasted in system information are selected from 8 values which are taken from the Rel-13 NPUSCH tables. - The up to 4 possible TBS which is smaller than or equal to the maximum broadcast TBS values for the UE to choose among are FFS. FFS: How the UE obtains the up to 4 possible values. - Support NW enabling the use of TBS smaller than the maximum configured. FFS details. RAN1#92bis agreements: - The 8 maximum TBS is {1000, 936, 808, 680, 584, 504, 408, 328}. - Only values of NRU and values of TBS in legacy Rel-13 NPUSCH table can be used for EDT - For each of the 8 maximum TBS values - For each of the Rel-13 reserved ‘MCS index’ states there is one number of RUs - The number of repetitions changes depending on the actual TBS, potentially including numbers of repetitions which are not included in legacy NPUSCH repetition numbers (FFS how) - Potential new numbers of repetitions may include multiple of 4 or 8 - The 8 maximum TBS values are: 1000, 936, 808, 680, 584, 504, 408, 328 bits - For 1000 bits max TBS: I_RU = 3, 4, 5, 6, 7 - For 936 bits max TBS: I_RU = 3, 4, 5, 6, 7 - For 808 bits max TBS: I_RU = 3, 4, 5, 6, 7 - For 680 bits max TBS: I_RU = 2, 3, 4, 6, 7 - For 584 bits max TBS: I_RU = 2, 3, 4, 5, 6 - For 504 bits max TBS: I_RU = 2, 3 4, 5, 6 - For 408 bits max TBS: I_RU = 2, 3, 4, 5, 6 - For 328 bits max TBS: I_RU = 2, 3, 4, 5, 6 - The 3 legacy MCS indices are used for fallback to non-EDT transmission; - The use of TBS smaller than the maximum configured is configured per CE level in SIB. - Per cell, in the below, Ti < Ti+1, eNB can configure that the UE chooses from: - When there are 4 permitted actual transmitted TBS {T1, T2, T3, T4} - T2 or T4 - T1, or T2, or T3, or T4 - When there are 3 permitted actual transmitted TBS {T1, T2, T3} - T2 or T3 - T1 or T2 or T3 - When there are 2 permitted actual transmitted TBS {T1, T2} - T1 or T2 - Ti with the maximum value of i is the maximum TBS in SIB. - Up to 4 TBS values are defined based on only the maximum broadcast TBS 328; 408; 504; 584; 680; 808; 936; 1000. T1; 328; 328; 328; 328; 328; 328; 328; 328. T2; 408; 408; 408; 456; 504; 504; 536. T3; 504; 504; 584; 680; 712; 776. T4; 584; 680; 808; 936; 1000. RAN1#92bis conclusion - Further study aspects related to Msg. 3 retransmission until RAN1#93 meeting RAN1#93 agreements: - The repetition number of actual TBS（Ti）for initial transmission of Msg3 is determined by scaling the repetition number indicated in the corresponding RAR as following: - The repetition number indicated in the corresponding RAR is set based on the broadcast maximum TBS; - The scaling factor is the ratio of the actual TBS(Ti) and the broadcast maximum TBS for Msg3; - The repetition number of the actual TBS is rounded up to the nearest higher integer number of L after the scaling; - For EDT Msg3 transmission/retransmission using single-tone, pi/4 QPSK is used always. - The parameter L for cyclic repetition is identical for all 4 actual TBS candidates. - When the repetition number for the maximum TBS is equal or larger than 8, L = 4. - When the repetition number for the maximum TBS is smaller than 8, L=1. - For a retransmission of msg3 carrying early data transmission, the UE uses the same TBS as previous transmission of Msg3 scheduled by RAR. - Note: this does not exclude possibility that DCI can indicate fallback to transmit legacy Msg3; - For retransmission, if the UE selects an actual TBS smaller than the maximum TBS, the number of repetitions is down-scaled in the same way as for the initial transmission; - DCI format for scheduling retransmission of Msg3 can be used to indicate UE to transmit a legacy Msg3 to fallback to non-EDT Msg3 transmission when the indicated TBS is 88 bits; - Note: it is up to spec. editor which state to choose for EDT retransmission - Note: Whether to introduce higher layer support for this mechanism is up to RAN2. - The timing for monitoring DCI is based on the number of Msg3 NPUSCH repetitions indicated in RAR/DCI

## Semi-persistent scheduling (SPS)

RAN1#90 agreements: - If SPS is supported in NB-IoT, at least the following physical layer aspects need to be further studied, considering the objective to reduce UE power consumption: - DCI format(s), size(s), and purpose(s) - Reduction of NPDCCH monitoring occasions - Retransmission scheme(s) for UL and DL. - Activation/release mechanism(s) - Issues between SPS and dynamic scheduling - What baseline should be used to compare SPS to RAN1#92bis: No agreements were made in this meeting. RAN1#93 agreements: - UL SPS for BSR with skipUplink is introduced for RRC connected mode - Periodicity of UL SPS resource, SPS C-RNTI are configured via RRC signaling - SPS activation/deactivation is indicated by DCI scrambled by SPS C-RNTI on UE seach space - The TBS is 16 bits. - Define separate UE capability signaling for UL SPS for BSR as optional - The fields and their sizes in the DCI scrambled by SPS C-RNTI for UL BSR activation/release are the same as DCI format N0 DCI for Semi-Persistent Scheduling Activation NPDCCH Validation DCI format N0. Redundancy version; set to '0'. HARQ process number (if 2 HARQ processes are configured); set to '0'. Modulation and coding scheme; set to ‘0000'. Resource assignment; Set to ‘000’. Flag for format N0/format N1 differentiation; Set to ‘0’. Semi-Persistent Scheduling Release NPDCCH Validation DCI format N0. Redundancy version; set to '0'. Repetition number; set to '000'. Subcarrier indication; Set to all '1's. Modulation and coding scheme; Set to ‘1111’. HARQ process number (if 2 HARQ processes are configured); set to '0'. Flag for format N0/format N1 differentiation; Set to ‘0’.

## Physical layer scheduling request

RAN1#88bis agreements: - SR should only be used when an NB-IoT UE is in uplink sync in RRC connected mode. - TA estimation should not be a design target of SR signal. - Sending SR with HARQ ACK/NACK can serve as the baseline case for UE with DL data - Further designs to be considered for dedicated SR signal design are: - Based on NPRACH signal; - Based on NPUSCH format 2: - Non-coherent detection based format is not precluded - Collision handling for dedicated SR is FFS - Design criteria for physical layer SR: - Power consumption reduction - Latency reduction - Impact on legacy NB-IoT scheduling and resources - Traffic models used and SR resource configurations should be reported together with evaluations. RAN1#89 agreements: - Piggybacked SR with HARQ-ACK is chosen between the following options, with evaluations encouraged at RAN1#90: - Option 1: QPSK-based constellation - Option 3: Cover code/Orthogonal sequence on ACK/NACK data symbols and/or DM-RS symbols RAN1#92 Working assumptions Confirmed in RAN1#93: Physical-layer SR with and without HARQ-ACK transmission is supported. - When SR is transmitted with HARQ-ACK: - Option 3 is adopted; - SR on/off is carried by two orthogonal length-16 cover codes on ACK/NACK data symbols. - [1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1] is used to signal SR OFF. - [1 -1 1 -1 1 -1 1 -1 1 -1 1 -1 1 -1 1 -1] is used to signal SR ON. - When SR is transmitted without HARQ-ACK - FFS whether it is transmitted in NPUSCH resources or reserved NPRACH resources. - FFS whether BSR is conveyed on SR without HARQ-ACK Note that companies are encouraged to provide evaluation results based on the agreed criterion, which are considered when confirming the WA. RAN1#92bis agreements: - If the working assumption is confirmed and when SR is transmitted without HARQ-ACK, it is transmitted in NPRACH resource using NPRACH based signal. - If the working assumption is confirmed, UE transmits the physical layer SR at the first opportunity unless there is a collision with another physical layer transmission/reception: - Collision between SR without HARQ-ACK resource and NPDSCH - SR is not transmitted, and remains pending - FFS: Collision with NPDCCH search space RAN1#93 agreements: - Confirm the WA on physical layer SR - Physical layer SR without HARQ-ACK does not convey BSR information in Rel-15; - Physical layer SR without HARQ-ACK is by presence/absence of a NPRACH preamble of preamble format 0/1/2; - For physical layer SR without HARQ-ACK, Non-contention-based RACH resource is used for SR transmission. - The configuration is UE-specific as follows which is similar as that in NPDCCH order: - Carrier indication, value range is {0, 1, …, 15}; - Subcarrier indication, value range is {0, 1, … , 47} for NPRACH format 0/1 and the value range is {0, 1, …, 143} for NPRACH format 2; - Note: Only the subcarriers within non-contention based RACH resource can be indicated. - Repetition number, value range reuses Table 16.3.2-1 in TS 36.213, i.e. {0, 1, 2, 3}; - NPRACH format 0/1/2 indication is used to indicate which NPRACH resource is used.

# TDD

## DL aspect

RAN1#90 agreements: - We will position NPSS and NSSS and NPBCH in subframes from among the set: {0, 4, 5, 8, 9} – FFS which precise subframes. - If NPSS and NSSS are the same as FDD: - The combination of {NPSS in subframe #5 and NSSS in subframe #9} is a precluded option. - Subframes 0 and 5 will certainly be used - The design shall be decodable within the same signal processing effort as the design used for FDD - RAN1 intends to prefer NPSS designs for TDD with the smallest practicable impact to FDD NB-IoT UEs’ initial cell acquisition - The transmission of SIB1-NB is FFS between: - Always on the same NB-IoT carrier as NPSS/NSSS - Always on a different NB-IoT carrier as NPSS/NSSS - Can be on a different NB-IoT carrier as NPSS/NSSS - Other SIBs than SIB1-NB can be transmitted on non-anchor carrier RAN1#90bis agreements: - TDD UL:DL configuration 0 is not supported in TDD NB-IoT in Rel-15. - MIB-NB is transmitted on the same NB-IoT carrier as NPSS/NSSS. - The single NB-IoT carrier for all the other SIBs than SIB1-NB, when not the anchor carrier, is: - In a PRB indicated by SIB1-NB with exact signaling design left to RAN2 including whether to signal anything in case these other SIBs are on the anchor carrier - It is supported that SIB1-NB is transmitted only on the anchor carrier - In at least subframe #0 in odd frames - It is supported that SIB1-NB can be transmitted on non-anchor carrier, FFS details - It is necessary to consider SFN wraparound as part of FFS - Periodicity of SIB1-NB in TDD is the same as FDD (i.e. 2560ms) - One transport block of SIB1-NB is transmitted over 8 SIB1-NB subframes (i.e. same as FDD) - For NPSS, NSSS and NPBCH transmission in TDD: - NPSS is transmitted on subframe #5 in every radio frame - NSSS is transmitted on subframe #0 in every even-numbered radio frame - NPBCH is in subframe 9 in every radio frame on the same carrier as NPSS/NSSS. - Confirm the working assumptions from RAN1#90, i.e. NPSS uses the lower 11 subcarriers in one subframe and the same cover code for TDD as FDD. - The NPSS and NSSS sequences for TDD are the same as FDD. - TDD and FDD NB-IoT are distinguished by the relative location of NPSS and NSSS. RAN1#90bis working assumptions: Confirmed in RAN1#93 - TDD UL:DL configuration 6 is not supported in TDD NB-IoT in Rel-15. RAN1#91 agreements: - It is not supported that SIB1-NB is transmitted on both anchor and non-anchor carrier. - At least for 16 repetitions for SIB1-NB transmission, - Whether SIB1-NB transmitted on anchor carrier or non-anchor one is indicated by MIB-NB. - When SIB1-NB is transmitted on non-anchor carrier, at least subframe #0 is used. - FFS: SIB1-NB can be transmitted on anchor carrier other than subframe #0 - FFS: The frequency position of non-anchor carrier is indicated by [0, 1, or 2] bits in MIB-NB for in-band scenario. - FFS: case for guard-band and stand-alone scenarios - FFS: Cases for 4 and 8 repetitions - The radio frame is determined by table-1 RAN1#92 agreements: - On an anchor carrier with 16 SIB1-NB repetitions, SIB1-NB can be transmitted on either of subframe #0 or subframe #4. FFS how the subframe is known to UE. - The following pairings of anchor and non-anchor SIB1-NB transmissions are supported: - In-band anchor + in-band non-anchor - Guard-band anchor + guard-band non-anchor, FFS which guard-band combinations the signaling will permit - Guard-band anchor + in-band non-anchor, at least for differentPCI (FFS for samePCI) - Standalone anchor + standalone non-anchor - For NPDSCH - Transmission in DwPTS is supported for in-band when the number of OFDM symbols in DwPTS is greater than 3. - FFS if all DwPTS configurations are supported for NPDCCH - Transmission in DwPTS is supported for guard-band and standalone for all DwPTS configurations. - FFS if all numbers of repetitions of NPDCCH are supported - FFS if all aggregation levels of NPDCCH are supported - FFS on NRS mapping - For starting radio frame number for 16 SIB1-NB repetitions, support the following - When SF#0 is used for 16 SIB1-NB repetitions on anchor carrier, Number of NPDSCH repetitions; PCID; Starting radio frame number for NB-SIB1 repetitions. 16; All PCIDS; SFN mod 256 = 1. - When SF#4 is used for 16 SIB1-NB repetitions on anchor carrier, Number of NPDSCH repetitions; PCID; Starting radio frame number for NB-SIB1 repetitions. 16; PCID mod 2 = 0; SFN mod 256 = 0. PCID mod 2 = 1; SFN mod 256 = 1. RAN1#92 working assumptions: - For NPDCCH - Transmission in DwPTS is supported for in-band when the number of OFDM symbols in DwPTS is greater than 3. - FFS if all DwPTS configurations are supported for NPDCCH - Transmission in DwPTS is supported for guard-band and standalone for all DwPTS configurations. - FFS if all numbers of repetitions of NPDCCH are supported - FFS if all aggregation levels of NPDCCH are supported - FFS on NRS mapping - For 8 repetitions for SIB1-NB transmission, SIB1-NB can be transmitted on non-anchor carrier. RAN1#92bis agreements: - For SIB1-NB transmission in TDD on an anchor carrier, the subframe index and TBS are indicated by schedulingInfoSIB1 as follows: Value of schedulingInfoSIB1; Number of NPDSCH repetitions; Subframe; TBS. 0; 4; 0; 208. 1; 8; 0; 208. 2; 16; 0; 208. 3; 4; 0; 328. 4; 8; 0; 328. 5; 16; 0; 328. 6; 4; 0; 440. 7; 8; 0; 440. 8; 16; 0; 440. 9; 4; 0; 680. 10; 8; 0; 680. 11; 16; 0; 680. 12; 16; 4; 208. 13; 16; 4; 328. 14; 16; 4; 440. 15; 16; 4; 680. - For 16 and 8 repetitions of SIB1-NB on a non-anchor carrier, the starting radio frame is as follows: Number of SIB1-NB repetitions; PCID; Starting radio frame number for SIB1-NB repetitions (nf mod 256). 16; PCID mod 2 = 0; 0. PCID mod 2 = 1; 1. 8; PCID mod 2 = 0; 0. PCID mod 2 = 1; 16. - When SIB-NB is transmitted on a non-anchor carrier, two subframes, i.e., subframe #0 and #5 in the same radio frame, are used for SIB1-NB transmission in every other radio frame - One transport block of SIB1-NB is transmitted over 16 SIB1-NB subframes - 16 SIB1-NB subframes carry encoded bits of SIB1-NB continuing reading them from circular buffer. - For non-anchor carrier transmission of SIB1-NB - MIB-NB has 3 bits to indicate the number of repetitions/TBS - MIB-NB has 1 bit to indicate the frequency domain location of SIB1-NB non-anchor carrier - `For In-band anchor + in-band non-anchor, either the adjacent lower PRB relative to anchor PRB, or the adjacent higher PRB relative to anchor PRB - FFS guard-band combinations - For standalone+standalone, to indicate one of the 2 adjacent carriers relative to anchor carrier. FFS signaling of frequency offset of the carrier. - For normal subframes, NRS resource mapping is the same as FDD. - For special subframe configurations #3, #4 and #8, NRS is mapped to the 3rd and 4th symbols of each slot - For special subframe configurations #9 and #10, NRS is mapped to the 3rd and 4th symbols of the first slot. - For special subframe configurations #1, #2, #6 and #7, NRS is mapped to - The 6th and 7th symbols of the first slot - For special subframe configuration #0 and #5, NRS is not transmitted RAN1#93 agreements: - For guard-band anchor, in-band non-anchor for SIB1-NB transmission can be samePCI mode. - When SIB1-NB is transmitted on a non-anchor carrier and the anchor-carrier is guard-band mode, MIB-NB indicates: - One of the following possible locations for the non-anchor NB-IoT carrier for SIB1-NB transmission is indicated in MIB-NB: - The NB-IoT carrier adjacent to anchor carrier in the same guard-band and to the outer side of the guard-band, i.e. the side away from the LTE carrier - The NB-IoT carrier in the opposite guard-band side and closest to the edge of the LTE carrier - The in-band PRB (samePCI=true) at the edge of the LTE carrier and on the same side as the anchor carrier. - For 5 MHz and 15 MHz LTE system bandwidth, the offset between the NB-IoT carriers is 45 kHz. - The in-band PRB (samePCI=false) at the edge of the LTE carrier and on the same side as the anchor carrier. - For 5 MHz and 15 MHz LTE system bandwidth, the offset between the NB-IoT carriers is 45 kHz. - If non-anchor PRB is in the opposite guard-band side (symmetric guard-band) or in-band samePCI, the LTE bandwidth between 5 and 15MHz or between 10 and 20MHz - For in-band samePCI=false case, the number of LTE CRS ports. - For standalone anchors, the frequency offset between anchor and SIB1-NB non-anchor carrier is up to RAN4. - For an anchor carrier, before obtaining the operation mode information (MIB-NB), the UE may assume NRSs are transmitted in SF #9 and SF #0 not containing NSSS. - On an anchor carrier for SIB1-NB, before the UE obtains SIB1-NB, the UE may assume NRSs are transmitted in SF#0 not containing NSSS, SF #9, and SF #4 if SF #4 is configured for SIB1-NB transmission. After the UE obtains SIB1-NB, the UE may assume NRS are also transmitted in any other valid DL subframe, including special subframe in which NRS is supported - On a non-anchor carrier for SIB1-NB, the UE may assume NRS are transmitted in SF #0 and SF #5 before the UE obtains SIB1-NB. - Special subframes are not used for NPRS transmission in TDD NB-IoT. - In normal DL subframes that are used for NPRS transmission, the NPRS generation is the same as NB-IoT FDD, including any revised NPRS introduced in FDD Rel-14. - The Part A NPRS configuration in TDD NB-IoT is the same as FDD, except the bitmap length: - 8 bits for 10 ms, 32 bits for 40 ms, assuming subframe #1 and #2 are always not used for NPRS. - For TDD NB-IoT, Part B OTDOA configuration is: - Extend the value of to {40, 80, 160, 320, 640, 1280, 2560} subframes - Extend the value of to {160, 320, 640, 1280, 2560} ms - is the same as FDD - For NPDSCH without repetition, rate matching is used for RE mapping in the special subframe. - For NPDSCH with repetition/cyclic repetition, - The resource mapping in special subframe follows the regular full DL subframe, with the transmission on remaining symbols that are not part of DwPTS being punctured. - NPDSCH is punctured by NRS REs - Valid special subframes are counted for number of NPDSCH repetitions - For NPDCCH candidates without repetition, the rate matching per NPDCCH in a special subframe is done according to the available REs within DwPTS in each transmission subframe - For NPDCCH candidates with repetitions, the rate matching in special subframe follows the regular full DL subframe, with the transmission on remaining symbols that are not part of DwPTS being punctured. - For NPDCCH USS and Type 2 CSS, - the modified value of G is {4, 8, 16, 32, 48, 64, 96, 128} - the value set of for FDD is reused in TDD NB-IoT. - Confirm the working assumption at the 3GPP RAN1#92 meeting as For NPDCCH: - Transmission in DwPTS is supported for in-band when the number of OFDM symbols in DwPTS is greater than 3. - All DwPTS configurations are supported for NPDCCH - Transmission in DwPTS is supported for guard-band and standalone for all DwPTS configurations. - The NPDCCH/NPDSCH starting position in special subframe is in TDD LTE where l’DataStart is eturaControlRegionSize provided by system information - dl-GapPeriodicity value range is doubled in TDD i.e. {128, 256, 512, 1024} subframes. - dl-GapThreshold and the dl-GapDurationCoeff are the same as FDD.

## UL aspect

RAN1#90 agreements: - NPRACH for TDD supports single-tone with frequency hopping - Multi-tone NPRACH formats can also be considered - FFS details of frequency hopping - One symbol group is defined by one CP, and N symbols. - FFS the value(s) of N - FFS CP durations, symbol duration - A preamble is defined by P symbol groups - FFS: Guard time usage - Repetition of NPRACH preamble is supported - The cell radius target for TDD NPRACH is FFS RAN1#90bis agreements: - NPUSCH transmissions with 15 kHz subcarrier spacing are supported in all supported UL/DL configurations for NB-IoT TDD. - 3.75 kHz is also supported, in UL/DL configurations #1, [#3], #4, with the same definition of NB-slot and resource unit as FDD - FFS if there any need for any other subcarrier spacing and/or slot structure for TDD - This needs to take into consideration the co-existence of NPRACH and NPUSCH. - TDD NPRACH supports at least 3.75KHz subcarrier spacing single-tone with frequency hopping. - FFS if also 5 kHz subcarrier spacing is supported e.g. for UL-DL configuration #2 - NPRACH formats using G symbol groups with back-to-back transmission followed by a guard time (FFS guard time duration) are supported for 1, 2, and 3 contiguous uplink subframes - An NPRACH format is associated with one value of N (the number of symbols per symbol group) and CP duration - G is FFS, and G≥2 - P (number of symbol groups in a preamble) is even. - For the G symbols groups that are transmitted back-to-back with 3.75 kHz subcarrier spacing, 3.75 kHz and 22.5 kHz hopping distances are supported. - FFS the details of the hopping pattern - FFS the hopping distance and pattern if 5 kHz subcarrier spacing is supported - For the hopping between discontinuous transmissions within one preamble - FFS hopping distance and hopping pattern - Cell specific pseudo-random hopping is used between NPRACH preamble repetitions - FFS details RAN1#91 agreements: For NPUSCH in NB-IoT TDD: - For the NPUSCH transmissions with 15 kHz subcarrier spacing, both Multi-tone (3, 6, 12 subcarriers with RU lengths 4ms, 2ms, 1ms) and single tone (with RU length 8ms) transmissions are supported as in FDD NB-IoT for all the supported TDD configurations in TDD NB-IoT other than the TDD configuration #3 and [#6]. The number of supported subcarriers, and RU lengths for TDD configuration #3 and [#6] is FSS (The number of subcarriers and RU lengths as defined in FDD NB-IoT are not precluded). RAN1#92 agreements: - NPRACH for TDD NB-IoT uses only 3.75 kHz as subcarrier spacing. - NPRACH symbol duration is 266.67us. - Define the following NPRACH formats Format; Description; G; P; N; CP length; Nominal cell size. 0; Two symbol groups followed by a guard time fit into 1 UL subframe; 2; 4; 1; 4778 Ts (~155.5us); ~23.3km. 1; Two symbol groups followed by a guard time fit into 2 UL subframes; 2; 4; 2; 8192 Ts (~266.7us); ~40.0km. 2; Two symbol groups followed by a guard time fit into 3 UL subframes; 2; 4; 4; 8192 Ts (~266.7us); ~40.0km. 0-a; Three symbol groups followed by a guard time fit into 1 UL subframe; 3; 6; 1; 1536 Ts (~49.95us); ~7.5km. 1-a; Three symbol groups followed by a guard time fit into 2 UL subframes; 3; 6; 2; 3072 Ts (~99.9us); ~15.0km. - FFS how and which format to use for each TDD UL/DL configuration - For the table above referring to NPRACH format for TDD NB-IoT, a guard time will be created naturally to fit into one (format 0,0-a), two (format 1, 1-a) or three (format 2) UL subframes. - FFS the supported number of repetitions for each format RAN1#92bis agreements: Mapping table for TDD UL/DL configuration and TDD NPRACH formats is list in Table X - Transmit Format 0 and Format 0-a in all NPRACH subframes, and aligned with subframe boundary Table X Mapping table for UL/DL configuration and NPRACH format UL/DL Configuration index; Format 0; Format 1; Format 2; Format 0-a; Format 1-a. 1; √; √; √; √. 2; √; √. [3]; [√]; [√]; [√]. 4; √; √; √; √. 5; √; √. - For G=2, P=4 (format 0, 1, 2) - when the repetition number =1 Tone index of the first and third symbol groups in the preamble repetition unit is chosen by [SFN and cell specific pseudo-random sequence (FFS details)]. The initial tone index to hopping pattern mapping is Index of the tone used by the 1st symbol group; Deterministic hopping length for the 2nd within a repetition unit. 0, 2, 4, 6, 8, 10; +3.75kHz. 1, 3, 5, 7, 9, 11; -3.75kHz. Index of the tone used by the 3rd symbol group; Deterministic hopping length for the 4th within a repetition unit. 0, 1, 2, 3, 4, 5; +22.5kHz. 6, 7, 8, 9, 10, 11; -22.5kHz. Table 16 - when repetition number >=2 The initial tone index to hopping pattern mapping follows table above. Tone index of the first symbol groups in the odd preamble repetition unit is chosen by [SFN and cell specific pseudo-random sequence (FFS details)] . Aiming at cancelling phase errors, for an odd preamble that has been transmitted with a given tone index for its first and third symbol groups, the candidate tone index for the first and third symbol groups in the even preamble is chosen by [SFN and ]cell specific pseudo-random sequence and will be confined to one of the tone indexes in the opposite half of the bandwidth as follows: Odd Preamble Repetition Unit; Even Preamble Repetition Unit. Index of the tone used by the 1st symbol group; Candidate Indexes for the tone to be used by the 1st symbol group. 0, 2, 4, 6, 8, 10; 1, 3, 5, 7, 9, 11. 1, 3, 5, 7, 9, 11; 0, 2, 4, 6, 8, 10. Index of the tone used by the 3rd symbol group; Candidate Indexes for the tone to be used by the 3rd symbol group. 0, 1, 2, 3, 4, 5; 6, 7, 8, 9, 10, 11. 6, 7, 8, 9, 10, 11; 0, 1, 2, 3, 4, 5. Table 17 - For G=3, P=6 (format 0-a, format 1-a) - Tone index of the first and fourth symbol groups in the preamble repetition unit is chosen by [SFN and cell specific pseudo-random sequence (FFS details)]. The initial tone index of the first and fourth symbol groups to hopping pattern mapping is - Table 18 Hopping patterns for 2nd and 3rd symbol group within a repetition unit Index of the tone used by the 1st symbol group; Hopping pattern for the 2nd and 3rd symbol group within a repetition unit. 0, 2, 4,6, 8, 10; +3.75kHz，-3.75kHz. 1, 3, 5,7, 9, 11; -3.75kHz，+3.75kHz. - Table 19 Hopping patterns for 5th and 6th symbol group within a repetition unit Index of the tone used by the 4th symbol group; Hopping pattern for the 5th and 6th symbol group within a repetition unit. 0, 1, 2, 3, 4, 5; +22.5kHz，-22.5kHz. 6, 7, 8,9, 10, 11; -22.5kHz，+22.5kHz，. - For NPUSCH transmission with 15 kHz subcarrier spacing, the number of subcarriers supported in TDD UL/DL configuration #3 is 1, 3, 6 and 12. - RU length reuses FDD - For TDD NB-IoT with 15 kHz subcarrier spacing, the resource unit of NPUSCH format 2 is single tone with RU length 2 ms. - The previous agreement above refers to NPUSCH format 1 - 3.75kHz subcarrier spacing is not supported for TDD UL/DL configuration #3. RAN1#93 agreements: - For TDD configuration 1 and 4, for NPUSCH format 1 and format 2 with 3.75 kHz subcarrier spacing, if one of the two consecutive UL subframes is configured invalid, postpone the NB-slot. - DMRS location in FDD NB-IoT is reused for TDD NB-IoT. - For multi-tone RU(s), DMRS sequence in FDD NB-IoT is reused for TDD NB-IoT. - For single-tone RU(s), DMRS sequence in FDD NB-IoT is reused for TDD NB-IoT. - NPRACH Format 0, 2, 0-a are supported for TDD UL/DL configuration 3. - For TDD NPRACH, the proposed configuration parameters are as following: - numRepetitionsPerPreambleAttempt, the value range is {1, 2, 4, 8, 16, 32, 64, 128, 256, 512, 1024} - nprach-Periodicity( ), the value range is {80, 160, 320, 640, 1280, 2560, 5120, 10240}ms - The value range of nprach-StartTime ,, is 10, 20, 40, 80, 160, 320, 640, 1280, 2560, 5120}ms - TDD NPRACH transmission shall start at the first valid UL subframe that is time units after the start of a radio frame fulfilling . - G symbol groups of NPRACH are dropped when there are not enough consecutive valid UL subframes to transmit G symbol groups back-to-back. - For TDD NPRACH, the cell specific pseudo random hopping pattern is initialized with c_init=N_ID^Ncell. - For NPRACH formats 0, 1 and 2, the followings are applied to random hopping: - The tone index of 1st symbol group of 1st preamble repetition is randomly selected by MAC layer. - The tone index of 1st symbol group and 3rd symbol group of each preamble repetition, except for the tone index of 1st symbol group of 1st preamble repetition, is selected according to the cell specific Pseudo random sequence. - Frequency hopping shall be used within the subcarriers. - For NPRACH format 0-a, 1-a, the followings are applied to random hopping: - The tone index of 1st symbol group of 1st preamble repetition is randomly selected by MAC layer. - The tone index of 1st symbol group and 4th symbol group of each preamble repetition, except for the tone index of 1st symbol group of 1st preamble repetition, is selected according to the cell specific Pseudo random sequence. - Frequency hopping shall be used within the subcarriers. - The value range of ack-NACK-NumRepetitions and ack-NACK-NumRepetitions-Msg4 in TDD NB-IoT is same as FDD NB-IoT. - 2-bit signaling indicates offset between UL PRB center with respect to DL PRB center to be either 7.5kHz, -7.5kHz, or 0 kHz

## Common aspect

RAN1#90 agreements: - MCL target of 164 dB at an ‘application layer’ data rate of 160 bps is targeted for at least one UL:DL configuration (FFS which one or more than one). - NOTE: The at least one UL:DL configuration may or may not be different for UL MCL target than DL MCL target - For evaluations, the FDD numbers of repetitions for physical channels are assumed - FFS the noise figure (eNB and UE) which will be assumed - The 2.6 GHz TDD band is prioritized for evaluations - This does not imply that 164 dB MCL or ‘application layer’ data rate targets will be relaxed - Targets of latency, and capacity may be relaxed for TDD NB-IoT. - For DL: subcarrier spacing, CP length, symbol length, subframe length, and radio frame length are the same in TDD as FDD. - At least NPSS, NSSS are transmitted on the same NB-IoT carrier. - Non-anchor carriers at least for unicast, paging and RACH are supported in NB-IoT TDD. No agreements were made in RAN1#90bis meeting. No agreements were made in RAN1#91 meeting. RAN1#92 agreements: - UL/DL configuration and the special subframe configuration are indicated via SIB1-NB. - For standalone mode, at least the same UL/DL configurations as TDD NB-IoT in-band/guard-band are supported. FFS new UL/DL configurations in standalone. - Confirm the following working assumption as agreement. - TDD NB-IoT will support all LTE special subframe configurations - FFS CRS-less special subframe configuration 10 is supported - For in-band - UpPTS is not used for NPUSCH and NPRACH - For standalone and guard-band - In the LTE special subframe configurations, UpPTS behaviour is the same as in-band - For standalone - FFS if to introduce new special subframe configurations comprising ‘DwPTS+GP’ and ‘GP+UpPTS’, and FFS the use of DwPTS/UpPTS in them - Supporting two HARQ processes is an optional UE capability in NB-IoT TDD system. - A 2-HARQ capable UE configured with 2 HARQ processes can be scheduled to transmit in UL subframes that occur during a DL reception, and receive in DL subframes that occur during a UL transmission. - Dynamic indication of scheduling delay in DCI is used for TDD NB-IoT. - FFS: definition of DL/UL scheduling delay - Higher layers signal one bitmap containing to indicate whether the DL/UL/special subframes are valid or not. - The length of the bitmap applies to - For guard-band: 10 ms - For standalone: 10 ms - FFS: other values if any for co-existence purpose - For in-band: At least 10 ms and 40 ms are supported; FFS if also an 80 ms length is supported for coexistence with dynamic TDD. - The maximum UL and DL TBS for Cat. NB1 and Cat. NB2 are kept the same as Rel-13/Rel-14 (e)NB-IoT FDD systems RAN1#92 Conclusion: Revisit the working assumption about TDD UL/DL configuration 6 once the TDD design as a whole is more advanced. RAN1#92bis agreements: - DwPTS can be used for NB-IoT transmission in CRS-less special subframe configuration #10 in TDD NB-IoT. - FFS on how to use the blank REs corresponding to CRSs - Downlink scheduling delay is defined by 4 physical subframes + k0, and k0 is based on valid downlink subframes. The scheduling delay values in FDD NB-IoT are reused. - FFS on whether valid special subframes which include DwPTS can be counted as a part of the scheduling delay k0 - Email discussion [92b-LTE-12] to decide the details on DL/UL interlacing until 5/4. RAN1#93 agreements: - No new UL/DL configurations for standalone operation mode are supported in TDD NB-IoT - Confirm the working assumption to not support UL/DL configuration #6 in Rel-15. - Do not map transmissions to DwPTS and UpPTS in special subframe configuration 10 in Rel-15. - For downlink scheduling delay, valid special subframes which include DwPTS to which NPDSCH can be mapped are counted as a part of the scheduling delay k0. - Uplink scheduling delays for NPUSCH format 1 for 3.75kHz and 15kHz, are 8 absolute subframes + k0 valid uplink subframes, where a set of k0 is {0, 8, 16, 32} - Uplink scheduling delays for NPUSCH format 2 for 3.75kHz and 15kHz, are 12 absolute subframes + k0 valid uplink subframes, where k0 is the same as described in Tables of 16.4.2-1 and 16.4.2-2 respectively in TS36.213, subtracting 12 from the existing values - For TDD configuration 1 and 4, for NPUSCH format 1 and format 2 with 3.75 kHz subcarrier spacing, if the indicated starting subframe is the 2nd of the two contiguous UL subframes, postpone the NB-slot. - Single-HARQ capable UE does not support UL/DL interlaced transmission and reception - The 1 ms minimum gap from end of NPUSCH format 1 for one HARQ process to NPDCCH for the other HARQ process is not specified for TDD NB-IoT. - For UE configured with 2 HARQ processes, the UE is not required to monitor NPDCCH 2ms before the start of the earliest NPDSCH until the end of the latest NPUSCH format 2 - For UE configured with 2 HARQ processes, the UE is not required to monitor NPDCCH 2ms before the start of the first scheduled NPUSCH until the end of the latest NPUSCH format 1 - For UE configured with 2 HARQ processes, an explicit guard time is not defined for the minimum gap between NPDCCH/NPDSCH reception and NPUSCH transmission - For UE configured with 2 HARQ processes the minimum gap between NPUSCH transmission and NPDCCH/NPDSCH reception is as follows: - In the case of in-band operation mode, an explicit guard time is not defined - For 15 kHz SCS, in the case of guard-band and standalone operation modes, UE is allowed to skip at least part of the first OFDM symbol in a downlink subframe right after an uplink subframe where the UE performs uplink transmission. - Clarify which part in CR phase - For UE configured with 1 HARQ process, minimum timing relationships between channels in TDD NB-IoT are the same as FDD NB-IoT - The number of soft channel bits for Cat. NB1 and Cat. NB2 is the same as Rel-13/Rel-14 (e)NB-IoT FDD systems. - The Rel-14 FDD interference randomization based on RE level rotation is applied to downlink channels for both anchor and non-anchor carrier in TDD NB-IoT.

# Narrowband measurement accuracy improvements

RAN1#88bis agreements: - At least NSSS is used additionally to NRS for in-band, guard-band, standalone. - FFS details of related signalling - Additional candidates can be NPSS, NPBCH, NPRS, PRS (in-band), CRS (in-band), physical signal that can be efficiently decoded or detected prior to decoding NPDCCH/NPDSCH (if standardized) and the performance improvements should be evaluated until RAN1#89 - Among the additional candidates, companies are recommended to prioritise CRS (in-band) - FFS if the existing signal(s)/channel(s) used additionally to NRS can be different in the serving cell to a neighbour cell - FFS for RRC_connected mode where at least the NPDCCH could be helpful for narrowband measurement accuracy improvement - FFS details of how UE is informed of differing EPRE between signals RAN1#89 agreements: - For connected mode in the serving cell and idle mode in camped cell and neighbour cells, it is feasible from RAN1 point of view to use NSSS on an anchor carrier for RRM measurement - RAN4 are requested to consider if NSSS is a suitable transmission - How to use, i.e. if and/or under what circumstances to combine NRS with NSSS depend on RAN4 study - Send an LS containing the above to RAN4, including the agreement from RAN1#88bis that “At least NSSS is used additionally to NRS for in-band, guard-band, standalone” - Continue to study - CRS for in-band operation mode - NPBCH, NPDCCH, NPDSCH on an anchor carrier in the serving cell RAN1#89 Working assumption, confirmation depending on RAN4 feedback: - For NSSS-aided measurement accuracy enhancement at least in idle mode, the NSSS-NRS EPRE ratios of serving cell and neighbour cells are signalled to UE by higher layers. - FFS on details of sets of power offset (e.g., {-3, …, 0, …, 3} in dB} - FFS on power offset information per-neighbor cell or common for all neighbor cells or common for serving cell and neighbor cells. - The possibility to use NSSS is indicated by higher layers. - Detailed signaling design would be left to RAN2. RAN1#90 Working assumption, confirmation depending on RAN4 feedback: - The ratio of NSSS EPRE to NRS EPRE is configured from {-3, 0, 3, spare} dB. - Send LS to RAN4 (cc RAN2) to inform the above, and request feedback on the above working assumption. RAN1#92 agreements: - The combination of NRS with NSSS for RRM measurement accuracy improvement is not considered further in RAN1. - NPDCCH, NPDSCH are not considered as candidates in addition to NSSS, nor in combination with NSSS, to improve the measurement accuracy for serving cell and neighboring cells. - FFS changes to the NRSSI definitions based on RAN1/4 agreements. RAN1#92 Working assumption, confirmation depending on RAN4 feedback - It is feasible from RAN1 point of view to use NPBCH in addition to NRS for RRM measurement, to be confirmed by RAN4 - In setting new RAN4 requirements (if any), RAN4 does not assume UE regenerates NPBCH and this is up to UE implementation - Ask RAN4 feedback on the combination of NPBCH with NRS RAN1#92bis agreements: - The eNB signals the number of consecutive NSSS occasions, with possible values of 2, 4, [8], that use different precoders for NSSS transmission, and the UE may take such information into account when using NSSS for measurement. If eNB does not signal this, the UE makes no assumptions of the antenna port(s), i.e., “the UE shall not assume that the transmissions of the narrowband secondary synchronization signal in a given subframe use the same antenna port, or ports, as the narrowband secondary synchronization signal in any other subframe”. The signaling does not mandate how, when, and over how many subframes the UE should measure the NSSS, and the measurement accuracy should not assume the UE measures consecutive NSSS occasions. RAN1#93 agreements: • Based on the RAN4 confirmation, confirm the working assumptions on use of NSSS for RRM •The NSSS-NRS EPRE ratio is signaled per neighbor cell •nsss-NumOccasionDifferentPrecoder = {2, 4, 8} •NPBCH for RRM is not specified until or unless it is confirmed by RAN4

# NPRACH false alarm and cell range enhancements

RAN1#89 agreements: - To support of cell range of at least 100 km, FFS between: - Cat 1: Rel-13 NPRACH - Cat 2: Sharing the same NPRACH resources as Rel-13 NPRACH formats, with symbol or symbol-group level scrambling; maintaining feasibility of FFT processing and orthogonality of preambles on different tones - CP length FFS between same as or longer than Rel-13 formats - Cat 3: New NPRACH numerology with CP length FFS between same as or longer than Rel-13 formats Option A: 1.25 kHz subcarrier spacing with minimum hop distance of 1.25 kHz Option B: 3.75 kHz subcarrier spacing with minimum hop distance 1.25 kHz Option C: 3.75 kHz subcarrier spacing with minimum hop distance 3.75 kHz with new hopping pattern - Combinations of Category 2 and Category 3 solutions are not precluded - For reduction of NPRACH false alarm probability, FFS between: - Alt 1: Sharing the same NPRACH resources as Rel-13 NPRACH formats, with symbol or symbol-group level scrambling; maintaining feasibility of FFT processing and orthogonality of preambles on different tones - Alt 2: A frequency shift of k*0.75 kHz is applied to all NPRACH signal in a Cell. - FFS k=[-2, -1, 0, 1, 2] or [-2, -1 1 2]. - FFS if to apply a phase rotation of m*pi/2 with m=0,1,2,3 is applied to the 4th symbol group of each repetition. - Signaling of the above frequency shift and phase rotation is FFS. - Alt 3: 3.75 kHz subcarrier spacing with minimum hop distance 3.75 kHz with new hopping pattern - Combinations of the above alternatives are not precluded RAN1#90 Conclusion - Regarding NPRACH false alarm rate reduction and cell range extension, companies are encouraged to explain the potential impacts of the agreed categories/alternatives on implementations. RAN1#92 agreements: - For NPRACH range enhancements, - New NPRACH numerology with 1.25 kHz subcarrier spacing with minimum hop distance of 1.25 kHz. - Only 800 us CP length is supported RAN1#92bis agreements: Confirm the following working assumption made in RAN1#92 - Sharing the same NPRACH resources as Rel-13 NPRACH formats, with symbol or symbol-group level scrambling; maintaining feasibility of FFT processing and orthogonality of preambles on different tones. - Down-select the following alternatives at RAN1#92bis: - Symbol level scrambling - Symbol group scrambling - Configurable NPRACH bandwidth is integer multiple of 45 kHz. - For Rel-15 UEs, in case NPUSCH transmission collides with any configured NPRACH resources in the cell, the NPUSCH transmission is postponed. - For one NPRACH symbol group for subcarrier spacing of 1.25 kHz, one NPRACH symbol group consists of CP+3 Symbols - FFS the number of symbol groups per NPRACH preamble repetition. - For frequency hopping within one NPRACH preamble, down-select the following alternatives at RAN1#93 meeting - Alt 1: 2-level -/+ 1.25 kHz -/+ 22.5 kHz - Alt. 2: 3-level -/+ 1.25 kHz -/+ 3.75 kHz 22.5 kHz - Specification supports that the new NPRACH format resources can be configured to overlap or not overlap with the NPRACH resources of Rel-13/Rel-14. RAN1#93 agreements: - Number of symbol groups per NPRACH preamble repetition is 6 symbol groups per NPRACH preamble repetition. - Frequency hopping within one NPRACH preamble is 3-level -/+ 1.25 kHz -/+ 3.75 kHz 22.5 kHz - The parameters and their values for the type2-CSS configurations of RAR corresponding to the Rel-15 NPRACH resources remain the same as in legacy NPRACH. - Reuse the Rel-13 values for nprach-StartTime, numRepetitionsPerPreambleAttemp, npdcch-CarrierIndex, for Rel-15 NPRACH format 2 resources configurations - Values for NPRACH periodicity are 40, 80, 160, 320, 640, 1280, 2560, 5120 ms Values for nprach-SubcarrierOffset and nprach-NumCBRA-StartSubcarriers are - nprach-SubcarrierOffset: {0, 36, 72, 108, 6, 54, 102, 42, 78, 90, 12, 24, 48, 84, 60, 18} subcarriers. - nprach-NumCBRA-StartSubcarriers: 4 bits - It is up to RAN2 to choose the exact subcarrier numbers - The new NPRACH format has: - 6 symbol groups per preamble. - Symbol-group level frequency hopping within one preamble - 1.25 kHz hopping gap for 1st to 2nd symbol group and with opposite direction for 5th to 6th symbol group. - 3.75 kHz hopping gap for 2nd to 3rd symbol group and with opposite direction for 4th to 5th symbol group. - 22. 5 kHz hopping gap for 3rd to 4th symbol group. - nprach-NumSubcarriers: {n36, n72, n108, n144} - nprach-SubcarrierMSG3-RangeStart: same as legacy

# NB-IoT small cell support

RAN1#89 agreements: - The number of repetitions in the DL and UL for small cells is not increased - NOTE: This may imply a reduction in MCL for small cells compared to wide area base stations. - Inform RAN4 - FFS other aspects of small cells to identify possible RAN1 impacts. RAN1#92 agreements: - FFS the needs of UL power control enhancement in NB-IoT small cell. - RAN1 does not further study the following topics in NB-IoT small cell in Rel-15 - UL/DL decoupling - Cell range expansion techniques with NB-IoT small cell RAN1#92bis agreements: No agreements were made in this meeting.

# Others

# References

## Liaison statements

R1-1721281, “LS on RAN on power consumption reduction progress”, RAN1#91.

R1-1721241, “LS on wake-up signal”, RAN1#91.

R1-1721255, “Reply LS on early data transmission”, RAN1#91.

R1-1719305, “LS on Early Data Transmission”, from RAN2, RAN1#91.

R1-1719322, “Reply LS to R1-1715304 LS on minimum time for DL-to-UL and UL-to-DL switching on one NB-IoT carrier for TDD NB-IoT UEs” from RAN4, RAN1#91

R1-1719333, “Reply LS on measurement accuracy improvement”, from RAN4, RAN1#91.

R1-1719207, “LS on wake-up signal configuration and procedures for NB-IoT and BL/CE UEs in Rel-15”, RAN1#90bis.

R1-1719103, “LS on agreements on early data transmission during random access procedure for NB-IoT and BL/CE UEs in Rel-15”, RAN1#90bis.

R1-1715304, “LS on minimum time for DL-to-UL and UL-to-DL switching on one NB-IoT carrier for TDD NB-IoT UEs”, RAN1#90.

R1-1715300, “LS on narrowband measurement accuracy enhancement”, RAN1#90.

R1-1715301, “LS on TDD NB-IoT”, RAN1#90.

R1-1715304, “LS on minimum time for DL-to-UL and UL-to-DL switching on one NB-IoT carrier for TDD NB-IoT UE”, RAN1#90.

R1-1709781, “LS on narrowband measurement accuracy enhancement”, RAN1#89.

R1-1709780, “LS on NB-IoT small cell”, RAN1#89.

R1-1803150, “LS on wake-up signal”, RAN1#92

R1-1803519, “LS on System acquisition time reduction for Rel-15 NB-IoT and eMTC”, RAN1#92

R1-1803151, “LS on Narrowband measurement accuracy improvements”, RAN1#92

R1-1801306, “Reply LS to RAN1 on wake-up signal”, RAN1#92

R1-1803573, “Reply LS to RAN1 on wake-up signal”, RAN1#92bis

R1-1803598, “LS on NSSS based NRSRP measurement definition”, RAN1#92bis

R1-1805434, “LS narrowband measurement accuracy improvements”, RAN1#92bis

R1-1807572, “LS on NPBCH for RRM measurements”, RAN1#93

R1-1807858, “LS on L1 parameter list of further enhancements of NB-IoT”, RAN1#93