3GPP TSG RAN WG1 Meeting #93                           R1-1807613

Busan, Korea, May 21st – May 25th, 2018

Source: 	General Motors

Title:  	V2X NR Evaluation Methodology – Vehicle UE Antenna Radiation Patterns

Agenda Item:	7.5.2

Document for:	Discussion and decision

# Introduction

In our last contribution [1] we have shown a more realistic radiation patterns of vehicle antennas at several locations. These patterns differ significantly from an isotropic antenna and thus it was agreed in Sanya meeting that we would provide the radiation patterns in four locations with compliance to [2] radiation tables. This contribution uses a 3D radiation patterns interpolation algorithm [3] as a weighted summation of the vertical & horizontal cuts.

# Radiation patterns of Rooftop & Bumpers Vehicle Antennas

In this section, we present the analytical fit for the radiation pattern simulated in [1] using HFSS. As agreed the chosen locations of vehicle antennas are: Front bumper, Front rooftop, Rear rooftop and Rear bumper.

## Simulation Setup

For each antenna location, we analyzed the simulated 3D radiation pattern to find the best fit. The fitting was done on several horizontal & vertical cuts (2D fitting) separately and then was combined with weighted functions to get the interpolated 3D pattern. We chose a sum of sine & cosine as our fitting functions for the horizontal cut and sum of sine for fitting the vertical cuts. We used a 3D radiation pattern interpolation algorithms [3] that makes use of four points on the sphere closest to an arbitrary point of interest P(φ ,θ).

Each table outlines the vertical/horizontal cuts estimated as a sum of sines terms with their corresponding coefficients. Chapter 3 describes UE antenna radiation pattern below 6GHz and chapter 4 above 6GHz.

NOTE: All radiation patterns as in dB and normalized to maximum directivity i.e G(φ ,θ)<=0.

We used the following coordinates: φ indicate the azimuth angle and θ the zenith

Figure 1: Coordinate system

# UE Antenna Radiation Patterns Below 6GHz

## Front Bumper

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=14aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 27.855; 0.9281; -1.3062. 2; 31.967; 2.7336; 1.4674. 3; 23.665; 3.0514; -1.8167. 4; 1.8441; 7.7354; -0.6126. AvdB"θ,ϕ=π=k=14aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 146.47; 1.095; -1.6043. 2; 131.87; 1.192; 1.4566. 3; 5.2383; 3.9996; -0.8389. 4; 1.9429; 9.0248; 1.6705. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] K; ak; bk. 0; -20.793; -. 1; -3.4026; 0.0721. 2; 1.3366; 0.1213. 3; 2.1764; -0.0595. 4; 0.5881; 0.0102. 5; -2.4068; 0.0675. 6; 1.7561; -0.1513. 7; 0.7899; -0.0133. 8; -1.0144; 0.0177. |
| 3D radiation power pattern (dB) | Weighted summing algorithm [3] Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2 |
| Maximum directional gain of an antenna element GE,max | 13dBi |

Figure 2: Front bumper 3D radiation pattern simulated and fitted

Figure 3: Front bumper azimuth radiation pattern at θ=90 degrees

Figure 4: Front bumper elevation radiation pattern at φ=0,180 degrees

## Front Rooftop

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=16aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 27.156; 0.8354; -0.9866. 2; 10.637; 1.2651; 1.9323. 3; 2.1018; 3.7081; -1.6452. 4; 0.9267; 12.057; -2.9006. 5; 0.8573; 8.5269; 3.5787. 6; 0.7456; 15.27; -2.9591. AvdB"θ,ϕ=π=k=14aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 18.648; 1.0206; -0.9611. 2; 11.099; 2.2755; 1.7808. 3; 213.68; 3.7367; -1.6448. 4; 208.11; 3.7558; 1.492. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] k; ak; bk. 0; -11.719; -. 1; -2.4504; 0.0053. 2; -0.9825; -0.0508. 3; -0.0419; -0.0225. 4; 0.2384; 0.0065. 5; 0.2406; -0.0023. 6; 0.1532; 0.006. 7; 0.1285; 0.0062. 8; 0.1407; 0.0038. |
| 3D radiation power pattern (dB) | Weighted summing algorithm [3] Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2 |
| Maximum directional gain of an antenna element GE,max | 3dBi |

Figure 5: Front rooftop 3D radiation pattern simulated and fitted

Figure 6: Front rooftop azimuth radiation pattern at θ=90 degrees

Figure 7: Front rooftop elevation radiation pattern at φ=0 degrees

## Rear Rooftop

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=15aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 21.785; 0.7541; -2.5975. 2; 122; 2.0393; -0.3421. 3; 4.5185; 4.4006; 2.4657. 4; 25.257; 3.2885; -0.6558. 5; 125.16; 2.3309; 2.7194. AvdB"θ,ϕ=π=k=14aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 89.841; 0.5245; -0.2707. 2; 64.942; 0.6058; 2.9391. 3; 2.4779; 4.1726; -1.4849. 4; 1.097; 7.9882; -1.7998. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] k; ak; bk. 0; -12.399; -. 1; -0.6788; -0.0034. 2; -1.025; -0.0364. 3; -0.0755; -0.0404. 4; 0.4216; 0.014. 5; -0.6139; -0.0053. 6; 0.2297; 0.0063. 7; -0.4418; -0.0038. 8; 0.0055; -0.0141. |
| 3D radiation power pattern (dB) | Weighted summing algorithm [3] Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2 |
| Maximum directional gain of an antenna element GE,max | 3dBi |

Figure 8: Rear rooftop 3D radiation pattern simulated and fitted

Figure 9: Rear rooftop azimuth radiation pattern at θ=90 degrees

Figure 10: Rear rooftop elevation radiation pattern at φ=0,180 degrees

## Rear Bumper

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=16aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 28.64; 0.6822; -1.6819. 2; 17.941; 1.0841; 0.8466. 3; 10.034; 4.5559; -2.317. 4; 9.9931; 4.9505; 0.8248. 5; 103.05; 9.2847; 0.5153. 6; 102.49; 9.3113; -2.6065. AvdB"θ,ϕ=π=k=18aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 29.045; 0.2614; -0.7172. 2; 7.9836; 2.8649; 2.965. 3; 5.1881; 3.3343; -0.1509. 4; 0.4384; 7.4621; 1.8565. 5; 1.0833; 47.882; 2.0727. 6; 1.131; 35.828; -0.901. 7; 1.0114; 33.576; -0.1873. 8; 1.0014; 39.562; -1.0305. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] k; ak; bk. 0; -17.964; -. 1; 4.1952; 0.0073. 2; -0.3746; -0.0202. 3; 1.2325; 0.005. 4; 0.2862; -0.0764. 5; -0.2796; 0.0252. 6; -0.063; -0.0114. 7; -1.0205; 0.0298. 8; -0.2964; 0.0095. |
| 3D radiation power pattern (dB) | Weighted summing algorithm [3] Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2 |
| Maximum directional gain of an antenna element GE,max | 11dBi |

Figure 11: Rear bumper 3D radiation pattern simulated and fitted

Figure 12: Rear bumper azimuth radiation pattern at θ=90 degrees

Figure 13: Rear bumper elevation radiation pattern at φ=0 degrees

# UE Antenna Radiation Patterns Above 6GHz

## Front Bumper

Parameter; Values. Vertical cut of the radiation power pattern (dB); AvdB"θ,ϕ=0°=k=13aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 103.4; 1.2756; -1.4588. 2; 93.054; 1.4402; 1.6919. 3; 1.2433; 16.391; -0.2701. AvdB"θ,ϕ=π=k=13aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 97.485; 1.2777; -1.5428. 2; 88.914; 1.4458; 1.5626. 3; 1.6315; 6.6824; -1.4228.. Horizontal cut of the radiation power pattern (dB); AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] k; ak; bk. 0; -14.712; -. 1; -0.814; -0.0126. 2; 4.5025; -0.4615. 3; 0.6572; -0.0404. 4; 0.3252; 0.5985. 5; -2.0187; 0.0372. 6; -1.6555; 0.2589. 7; -0.843; 0.6858. 8; -0.2954; -0.2211.. 3D radiation power pattern (dB); Weighted summing algorithm [3] Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2. Maximum directional gain of an antenna element GE,max; 16.6dBi.

Figure 2: Front bumper 3D radiation pattern simulated and fitted

Figure 3: Front bumper azimuth radiation pattern at θ=90 degrees

Figure 4: Front bumper elevation radiation pattern at φ=0,180 degrees

## Front Rooftop

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=16aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 53.19; 1.0466; -0.9824. 2; 171.45; 2.3513; 2.2593. 3; 154.83; 2.4609; -0.8176. 4; 2.166; 6.2024; 0.5122. 5; 1.2277; 9.599; 0.5604. 6; 1.1363; 14.311; -0.4448. AvdB"θ,ϕ=π=k=18aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 34.284; 1.0869; -1.1043. 2; 36.721; 2.2823; 1.4488. 3; 4.2902; 5.2423; 1.1692. 4; 1.7681; 7.7706; -3.641. 5; 14.094; 2.7398; -1.5731. 6; 1.2267; 10.465; 2.0934. 7; 1.0796; 14.285; 2.8759. 8; 0.8846; 31.578; -1.1871. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] k; ak; bk. 0; -16.085; -. 1; -7.903; 0.1101. 2; -0.2972; -1.4639. 3; 0.4083; -0.0866. 4; 0.6706; 1.0598. 5; -0.9804; -2.5714. 6; -0.9085; 0.1326. 7; 0.1433; -0.1372. 8; -0.1282; -0.2697. |
| 3D radiation power pattern (dB) | Weighted summing algorithm [3] Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2 |
| Maximum directional gain of an antenna element GE,max | 11.5dBi |

Figure 5: Front rooftop 3D radiation pattern simulated and fitted

Figure 6: Front rooftop azimuth radiation pattern at θ=90 degrees

Figure 7: Front rooftop elevation radiation pattern at φ=0 degrees

## Rear Rooftop

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=17aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 55.839; 1.0927; -1.1273. 2; 95.402; 2.0245; 2.1133. 3; 2.9533; 6.0111; 1.5482. 4; 1.5696; 12.462; 3.2683. 5; 1.2073; 10.016; 2.2276. 6; 70.244; 2.1361; -0.8181. 7; 1.4093; 13.127; 0.6659. AvdB"θ,ϕ=π=k=17aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 42.818; 0.9167; -0.9757. 2; 18.065; 1.895; 1.5513. 3; 8.8444; 3.6653; -0.1862. 4; 9.5301; 7.6263; 0.1674. 5; 15.018; 9.0456; 0.2995. 6; 20.098; 8.565; -2.8308. 7; 1.5077; 12.14; -1.3725. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] k; ak; bk. 0; -18.471; -. 1; 3.6106; -0.1824. 2; -1.4888; 1.3614. 3; 0.5772; 0.5331. 4; 1.7125; 1.3205. 5; -0.0573; -2.5904. 6; 0.0628; -1.0024. 7; -1.0526; -1.2366. 8; 0.0433; -0.1443. |
| 3D radiation power pattern (dB) | Weighted summing algorithm [3] Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2 |
| Maximum directional gain of an antenna element GE,max | 12.5dBi |

Figure 8: Rear rooftop 3D radiation pattern simulated and fitted

Figure 9: Rear rooftop azimuth radiation pattern at θ=90 degrees

Figure 10: Rear rooftop elevation radiation pattern at φ=0,180 degrees

## Rear Bumper

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=14aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 87.458; 0.0103; -2.9955. 2; 4.1908; 2.1185; 1.8618. 3; 1.0935; 127.65; 2.3653. 4; 1.2773; 16.837; 1.1214. AvdB"θ,ϕ=π=k=16aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 17.631; 0.823; -1.5005. 2; 12.155; 1.9599; 1.2643. 3; 15.226; 5.6698; 2.2669. 4; 15.495; 5.7995; -0.9411. 5; 0.3371; 10.294; -3.8515. 6; 0.5965; 11.41; 0.4383. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] k; ak; bk. 0; -12.742; -. 1; -1.2007; 0.2451. 2; 4.1916; -0.5651. 3; 0.1901; 0.453. 4; 0.4235; 0.6884. 5; -1.3848; -0.4145. 6; -0.2381; 0.2875. 7; 0.4488; 0.5441. 8; 0.1606; -0.2555. |
| 3D radiation power pattern (dB) | Weighted summing algorithm [3] Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2 |
| Maximum directional gain of an antenna element GE,max | 13.6dBi |

Figure 11: Rear bumper 3D radiation pattern simulated and fitted

Figure 12: Rear bumper azimuth radiation pattern at θ=90 degrees

Figure 13: Rear bumper elevation radiation pattern at φ=0 degrees

# References

[1] R1-1805259, “Suggestion for the incorporation of realistic vehicle antenna patterns”. Sanya, China, April 16th – 20th 2018.

[2] 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz (Release 14)”.

[3] F. Gil, A. R. Claro, J. M. Ferreira, C. Pardelinha, and L. M. Correia, “A

3-D extrapolation model for base station antenna’s radiation patterns,”

in Proc. IEEE VTC-Fall, Amsterdam, The Netherlands, Sep. 1999, pp.

1341–1345.

# Summary

Observation 1:

The radiation patterns in this contribution are significantly different from the previously proposed vehicle UE type antennas. In order to capture the real effects of the wireless vehicular communications these antennas should be used in the technical evaluation simulations.