3GPP TSG RAN WG1 Meeting #93	R1-1807759

Busan, Korea, May 21 – 25, 2018

Agenda Item:	7.8.1

Source:	Huawei

Title:	Summary on offline discussion on mMTC evaluation method and parameters for IMT-2020 self-evaluation

Document for:	Discussion and decision

Introduction

In RAN1#93 meeting, the self evaluation study is further progressed and the evaluation method and parameters for mMTC connection density are discussed. This document provides the summary of the discussion and the resulted proposals for non-full buffer system level simulation.

Proposal

The following proposals apply to non-full buffer system level simulation for connection density.

Procedure consideration

For NB-IoT and eMTC evaluation of connection density, the early data transmission procedure and the RRC Resume procedure can be employed. Other procedures supported by NB-IoT and eMTC can also be considered.

The procedure illustrated in Table 1 can be used as reference.

Table 1 Early data transmission procedure and RRC Resume procedure of NB-IoT and eMTC

| Early data transmission |  |  |  | RRC Resume |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UE |  | BS |  | UE |  | BS |
|  | Step1: Sync (NPSS, NSSS) + PBCH (MIB) |  |  |  | Step1: Sync (NPSS, NSSS) + PBCH |  |
|  | Step 2: PRACH Msg1 |  |  |  | Step2: PRACH Msg1 |  |
|  | Step 3: Random Access Response (Msg2) (NPDCCH + RAR; including UL grant) |  |  |  | Step3: Random Access Response (Msg 2) (PDCCH + RAR; including UL grant) |  |
|  | Step 4: UL data transmission |  |  |  | Step4: RRC Connection Resume Request (Msg 3) |  |
|  | Step 5: HARQ Ack |  |  |  | Step5: RRC Connection Resume (Msg 4) |  |
|  | Step 6: RRCEarlyDataComplete |  |  |  | Step 6: HARQ ACK |  |
|  | Step 7: HARQ Ack |  |  |  | Step 7: RRC Connection Resume Complete (Msg 5) |  |
|  |  |  |  |  | Step 8: RLC ACK Msg 5 |  |
|  |  |  |  |  | Step 9: HARQ ACK |  |
|  |  |  |  |  | Step 10: UL data transmission (NPUSCH) |  |
|  |  |  |  |  | Step 11: RLC ACK UL data transmission |  |
|  |  |  |  |  | Step 12: HARQ ACK |  |
|  |  |  |  |  | Step 13: RRC Connection Release |  |
|  |  |  |  |  | Step 14: HARQ ACK |  |
|  |  |  |  |  | Step 15: RLC ACK RRC Connection Release |  |

Simulation method

As a starting point, the simulation for evaluating connection density can be conducted per PRB (for NB-IoT) or per narrow band (for eMTC), which derives the connection density per PRB or per narrow band. The connection density for assumed operational bandwidth is then derived by multiplying connection density per PRB or per narrow band by the number of PRBs or narrow bands.

The system level simulation will evaluate each packet’s total delay tpacket. If tpacket > 10s, this packet is regarded as failed to be delivered to the destination receiver. The total delay consists of the delays from all the steps in the evaluated procedure,

where tUL_data is the UL data transmission time duration, and ti is the time delay for step i (excluding UL data transmission step) in the evaluated procedure.

Companies report the detailed models for delay of each step in the selected procedure. The models as in Appendix can be used as reference.

- Appendix

The detailed modeling of delay of Step 1~5 of early data transmission procedure is provided in [1].

- References

[1] R1-1805934, “Considerations and evaluation results for IMT-2020 for mMTC connection density”, Huawei, HiSilicon, May 2018.