3GPP TSG RAN WG1 Meeting #93                                                         	                      R1-1807779

Busan, Korea, May 21st - 25th, 2018

Source:	ZTE

Title:	Offline summary for AI 7.4.2 Receivers for NOMA

Agenda Item:	7.4.2

Document for: 	Discussion and Decision

# Introduction

This document summarizes key issues relevant to AI 7.4.2 receivers for NOMA.

# Receivers for NOMA

### 2.1 Typical receiver types

The following receivers are discussed for NOMA schemes.

RAN1#92bis

Agreements:

Adopt Figure 1 as the general block diagram of multi-user receiver for UL data transmissions.

- The algorithms for the detector block (for data) can be e.g. MMSE, MF, ESE, MAP, MPA, EPA.

- The interference cancellation can be hard, soft, or hybrid, and can be implemented in serial, parallel, or hybrid.

  - Note: the IC block may consist of an input of the received signal for some types of IC implementations

- The interference cancellation block may or may not be used.

  - Note: if not used, an input of interference estimation to the decoder may be required for some cases.

- The input to interference cancellation may come directly from the Detector for some cases

Figure 1 A high-level block diagram of multi-user receiver

Based on the current agreements, there could be too many different combinations for the advanced receiver, i.e. MMSE/MF/ESE/MAP/MPA/EPA + Hard/Soft/Hybrid IC. In order to facilitate the discussion on the further performance evaluations of NOMA, typical receiver types shoud be identified and the details would be beneficial to evaluate the receiver complexity. Different types of receivers are mentioned in the contributions submitted to RAN1#93, where several typical types can be found as follows:

- MMSE + Hard IC, where the interference cancellation can be implemented in serial, parallel or hybrid [1][4][5][6][7][8][9][10][12][13]

- ESE + Soft IC, where the initial detector of ESE could be based on MF or MMSE, and the soft iteration can be implemented in serial, parallel or hybrid. [1][2][8][11][12][13]

- MAP + Hybrid IC, where the MAP can be simplified by MPA or EPA, and both hard IC and soft IC can be implemented to further reduce the complexity [1][2][5][8][9][12].

Any comments?

| Company | View |
| --- | --- |

### 2.2 Receiver complexity

Several contribitions address the receiver complexity issues and provide quantitative analysis for the detection and decoding of the data signal [1][5][9][12][13].

Based on the agreed general structure of advanced receiver, it is reasonable that the computational complexity should include the detector complexity, decoder complexity, and the number of inner/outer iterations. Therefore, we propose to have a template so that more companies can provide their receiver implementations and the complexity values for each of the processing blocks.

Proposal 1:

- Companies should provide the receiver computational complexity and latency accompany the performance evaluation, at least:

  - Detector complexity

  - Decoding complexity

  - Number of iteration(s)

  - Other receiver optimization, if any

  - Complexity for the preamble/DMRS detection is FFS.

  - FFS how to calculate latency

  - FFS which simulation cases to be selected for evaluation

Table 1 Template for the report of receiver complexity and latency

| Compenent | Number of adding operation | Number of multiplication operation |
| --- | --- | --- |
| Detector algorithm |  |  |
| Detector complexity (Counting the number of iterations within detector) |  |  |
| Decoder algorithm |  |  |
| Decoder complexity (Counting the number of iteration within decoder) |  |  |
| Number of outer iterations e.g. de/re-coding operations |  |  |
| Other enhancements |  |  |
| Total complexity |  |  |
| Overall complexity |  |  |
| Overall latency |  |  |

Any comments?

| Company | View |
| --- | --- |

# References

- R1-1805841	Multi-user receivers for NOMA,	ZTE

- R1-1805908	Discussion on the design of NoMA receiver,	Huawei, HiSilicon

- R1-1806080	Receiver design considerations for NOMA,	vivo

- R1-1806242	Receiver design for NOMA,	Ericsson

- R1-1806306	Discussion on NOMA receivers,	CATT

- R1-1806532	Receiver structure for NOMA,	Intel Corporation

- R1-1806636	Considerations on the receiver types and NoMA schemes,	LG Electronics

- R1-1806752	Considerations on receiver types for NoMA schemes,	Samsung

- R1-1806931	Receiver considerations for UL NOMA,	Nokia, Nokia Shanghai Bell

- R1-1806980	Receiver techniques for NOMA,	AT&T

- R1-1807022	IDMA Receiver Performance Evaluation ,	InterDigital, Inc.

- R1-1807074	Receiver candidates for uplink NOMA transmission,	NTT DOCOMO, INC.

- R1-1807378	Receivers for NOMA,	Qualcomm Incorporated