3GPP TSG RAN WG1 Meeting #94	R1-1808071

Gothenburg, Sweden, August 20 – 24, 2018

Agenda Item:	7.1

Source:	Huawei, HiSilicon

Title:	Corrections for TR38.811 Chapter 6 Non-Terrestrial Networks channel models

Document for:	Discussion and decision

# Introduction

Version 1.0.0 of TR38.811 was approved in the RAN plenary meeting in La Jolla in June 2018. Some fast fading parameters require adjustments based on latest simulation results.

This contribution proposes updated values to Urban and Rural fast fading parameters (Appendix A).

# References

RP-181393	TR 38.811 v1.0.0 on Study on NR to support non-terrestrial networks, 3GPP RAN#80, La Jolla, June 2018.

# Appendix A. Text Proposal

--- start of text proposal ---

Table 6.7-6 Part-3b: Channel model parameters for Urban Scenario (LOS) at S band.

| Scenarios |  | Urban LOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -7.97 | -8.12 | -8.21 | -8.31 | -8.37 | -8.39 | -8.38 | -8.35 | -8.34 |
|  | lgDS | 1 | 0.83 | 0.68 | 0.48 | 0.38 | 0.24 | 0.18 | 0.13 | 0.09 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | -2.6 | -2.48 | -2.44 | -2.6 | -2.71 | -2.76 | -2.78 | -2.65 | -2.27 |
|  | lgASD | 0.79 | 0.8 | 0.91 | 1.02 | 1.17 | 1.17 | 1.2 | 1.45 | 1.85 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | 0.18 | 0.42 | 0.41 | 0.18 | -0.07 | -0.43 | -0.64 | -0.91 | -0.54 |
|  | lgASA | 0.74 | 0.9 | 1.3 | 1.69 | 2.04 | 2.54 | 2.47 | 2.69 | 1.66 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | -0.63 | -0.15 | 0.54 | 0.35 | 0.27 | 0.26 | -0.12 | -0.21 | -0.07 |
|  | lgZSA | 2.61.62 | 3.31 | 1.1 | 1.59 | 1.62 | 0.97 | 1.99 | 1.82 | 1.43 |
| ZOD spread (ZSD) lgZSA=log10(ZSD/1) | lgZSD | -2.54 | -2.67 | -2.03 | -2.28 | -2.48 | -2.56 | -2.96 | -3.08 | -3 |
|  | lgZSD | 2.621.02 | 2.96 | 0.86 | 1.19 | 1.4 | 0.85 | 1.61 | 1.49 | 1.09 |
| Shadow fading (SF) [dB] | SF | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| K-factor (K) [dB] | K | 31.83 | 18.78 | 10.49 | 7.46 | 6.52 | 5.47 | 4.54 | 4.03 | 3.68 |
|  | K | 13.84 | 13.78 | 10.42 | 8.01 | 8.27 | 7.26 | 5.53 | 4.49 | 3.14 |
| Cross-Correlations | ASD vs DS | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
|  | ASA vs DS | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 |
|  | ASA vs SF | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 |
|  | ASD vs SF | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 |
|  | DS vs SF | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 |
|  | ASD vs ASA | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASD vs  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASA vs  | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 |
|  | DS vs  | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 |
|  | SF vs  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Cross-Correlations | ZSD vs SF | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSA vs SF | -0.8 | -0.8 | -0.8 | -0.8 | -0.8 | -0.8 | -0.8 | -0.8 | -0.8 |
|  | ZSD vs K | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSA vs K | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSD vs DS | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 |
|  | ZSA vs DS | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSD vs ASD | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
|  | ZSA vs ASD | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSD vs ASA | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 |
|  | ZSA vs ASA | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
|  | ZSD vs ZSA | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Delay scaling parameter r |  | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| XPR [dB] | XPR | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
|  | XPR | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Number of clusters |  | 4 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Number of rays per cluster |  | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Cluster DS () in [ns] |  | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 |
| Cluster ASD () in [deg] |  | 0.09 | 0.09 | 0.12 | 0.16 | 0.2 | 0.28 | 0.44 | 0.9 | 2.87 |
| Cluster ASA () in [deg] |  | 12.55 | 12.76 | 14.36 | 16.42 | 17.13 | 19.01 | 19.31 | 22.39 | 27.8 |
| Cluster ZSA () in [deg] |  | 1.25 | 3.23 | 4.39 | 5.72 | 6.17 | 7.36 | 7.3 | 7.7 | 9.25 |
| Per cluster shadowing std  [dB] |  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Correlation distance in the horizontal plane [m] | DS | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
|  | ASD | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 |
|  | ASA | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
|  | SF | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 |
|  |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
|  | ZSA | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
|  | ZSD | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: All large scale parameters are assumed to have no correlation between different floors. NOTE 4: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. NOTE 6: For UMa and frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent LSP values NOTE 7: For UMi and frequencies below 2 GHz, use fc = 2 when determining the values of the frequency-dependent LSP values NOTE 8: For satellite (e.g.GEO/LEO), the departure angle spreads are zeros, i.e. µlgASD and µlgZSD are –∞, and corresponding standard deviations are zeros. |  |  |  |  |  |  |  |  |  |  |

Table 6.7-6 Part-3b: Channel model parameters for Urban Scenario (LOS) at Ka band.

| Scenarios |  | Urban LOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -8.52 | -8.59 | -8.51 | -8.49 | -8.48 | -8.44 | -8.4 | -8.37 | -8.35 |
|  | lgDS | 0.921.02 | 0.79 | 0.65 | 0.48 | 0.46 | 0.34 | 0.27 | 0.19 | 0.14 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | -3.18 | -3.05 | -2.98 | -3.11 | -3.19 | -3.25 | -3.33 | -3.22 | -2.83 |
|  | lgASD | 0.79 | 0.87 | 1.04 | 1.06 | 1.12 | 1.14 | 1.25 | 1.35 | 1.62 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | -0.4 | -0.15 | -0.181.33 | -0.31 | -0.58 | -0.9 | -1.162.04 | -1.48 | -1.14 |
|  | lgASA | 0.77 | 0.97 | 1.58 | 1.69 | 2.13 | 2.51 | 2.47 | 2.61 | 1.7 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | -0.67 | -0.34 | 0.07 | -0.08 | -0.211.52 | -0.251.65 | -0.61 | -0.79 | -0.58 |
|  | lgZSA | 2.22 | 3.04 | 1.33 | 1.45 | 1.620.52 | 1.06 | 1.88 | 1.87 | 1.19 |
| ZOD spread (ZSD) lgZSA=log10(ZSD/1) | lgZSD | -2.61 | -2.82 | -2.48 | -2.76 | -2.93 | -3.05 | -3.45 | -3.66 | -3.56 |
|  | lgZSD | 2.41 | 2.59 | 1.020.92 | 1.27 | 1.38 | 0.96 | 1.51 | 1.49 | 0.89 |
| Shadow fading (SF) [dB] | SF | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| K-factor (K) [dB] | K | 40.18 | 23.62 | 12.48 | 8.56 | 7.42 | 5.97 | 4.88 | 4.22 | 3.81 |
|  | K | 16.99 | 18.96 | 14.23 | 11.06 | 11.21 | 9.47 | 7.24 | 5.79 | 4.25 |
| Cross-Correlations | ASD vs DS | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
|  | ASA vs DS | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 |
|  | ASA vs SF | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 |
|  | ASD vs SF | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 |
|  | DS vs SF | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 |
|  | ASD vs ASA | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASD vs  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASA vs  | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 |
|  | DS vs  | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 |
|  | SF vs  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Cross-Correlations | ZSD vs SF | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSA vs SF | -0.8 | -0.8 | -0.8 | -0.8 | -0.8 | -0.8 | -0.8 | -0.8 | -0.8 |
|  | ZSD vs K | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSA vs K | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSD vs DS | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 |
|  | ZSA vs DS | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSD vs ASD | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
|  | ZSA vs ASD | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSD vs ASA | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 |
|  | ZSA vs ASA | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
|  | ZSD vs ZSA | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Delay scaling parameter r |  | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| XPR [dB] | XPR | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
|  | XPR | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Number of clusters |  | 4 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Number of rays per cluster |  | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Cluster DS () in [ns] |  | 1.6 | 1.6 | 1.6 | 1.6 | 1.6 | 1.6 | 1.6 | 1.6 | 1.6 |
| Cluster ASD () in [deg] |  | 0.09 | 0.09 | 0.11 | 0.15 | 0.18 | 0.27 | 0.42 | 0.86 | 2.55 |
| Cluster ASA () in [deg] |  | 11.78 | 11.6 | 13.05 | 14.56 | 15.35 | 16.97 | 17.96 | 20.68 | 25.08 |
| Cluster ZSA () in [deg] |  | 1.14 | 2.78 | 3.87 | 4.94 | 5.41 | 6.31 | 6.66 | 7.31 | 9.23 |
| Per cluster shadowing std  [dB] |  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Correlation distance in the horizontal plane [m] | DS | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
|  | ASD | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 |
|  | ASA | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
|  | SF | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 |
|  |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
|  | ZSA | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
|  | ZSD | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: All large scale parameters are assumed to have no correlation between different floors. NOTE 4: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. NOTE 6: For UMa and frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent LSP values NOTE 7: For UMi and frequencies below 2 GHz, use fc = 2 when determining the values of the frequency-dependent LSP values NOTE 8: For satellite (e.g.GEO/LEO), the departure angle spreads are zeros, i.e. µlgASD and µlgZSD are –∞, and corresponding standard deviations are zeros. |  |  |  |  |  |  |  |  |  |  |

Table 6.7-6 Part-4a: Channel model parameters for Urban Scenario (NLOS) at S band.

| Scenarios |  | Urban NLOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -7.21 | -7.63 | -7.75 | -7.97 | -7.99 | -8.01 | -8.09 | -7.97 | -8.17 |
|  | lgDS | 1.19 | 0.98 | 0.84 | 0.73 | 0.73 | 0.72 | 0.71 | 0.78 | 0.67 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | -1.55 | -1.61 | -1.73 | -1.95 | -1.94 | -1.88 | -2.1 | -1.8 | -1.77 |
|  | lgASD | 0.87 | 0.88 | 1.15 | 1.13 | 1.21 | 0.99 | 1.77 | 1.54 | 1.4 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | 0.17 | 0.32 | 0.52 | 0.61 | 0.68 | 0.64 | 0.58 | 0.71 | 0.49 |
|  | lgASA | 2.97 | 2.99 | 2.71 | 2.26 | 2.08 | 1.93 | 1.71 | 0.96 | 1.16 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | -0.97 | 0.49 | 1.03 | 1.12 | 1.3 | 1.32 | 1.35 | 1.31 | 1.5 |
|  | lgZSA | 2.35 | 2.11 | 1.29 | 1.45 | 1.07 | 1.2 | 1.1 | 1.35 | 0.56 |
| ZOD spread (ZSD) lgZSA=log10(ZSD/1) | lgZSD | -2.86 | -2.64 | -2.05 | -2.18 | -2.24 | -2.21 | -2.69 | -2.81 | -4.29 |
|  | lgZSD | 2.77 | 2.79 | 1.53 | 1.67 | 1.95 | 1.87 | 2.72 | 2.98 | 4.37 |
| Shadow fading (SF) [dB] | SF | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| K-factor (K) [dB] | K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Cross-Correlations | ASD vs DS | 0.72 | 0.59 | 0.76 | 0.66 | 0.82 | 0.81 | 0.85 | 0.77 | 0.84 |
|  | ASA vs DS | 0.47 | 0.41 | 0.34 | 0.35 | 0.24 | 0.28 | 0.26 | 0.27 | 0.33 |
|  | ASA vs SF | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASD vs SF | -0.6 | -0.6 | -0.6 | -0.6 | -0.6 | -0.6 | -0.6 | -0.6 | -0.6 |
|  | DS vs SF | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 |
|  | ASD vs ASA | 0.53 | 0.48 | 0.4 | 0.44 | 0.27 | 0.29 | 0.3 | 0.37 | 0.34 |
|  | ASD vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ASA vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | DS vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | SF vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Cross-Correlations | ZSD vs SF | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSA vs SF | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 |
|  | ZSD vs K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ZSA vs K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ZSD vs DS | 0.98 | 0.97 | 0.96 | 0.9 | 0.89 | 0.92 | 0.93 | 0.89 | 0.8 |
|  | ZSA vs DS | -0.01 | 0.06 | 0.06 | 0.06 | -0.05 | -0.19 | -0.18 | -0.39 | -0.34 |
|  | ZSD vs ASD | 0.72 | 0.6 | 0.75 | 0.69 | 0.7 | 0.71 | 0.79 | 0.77 | 0.92 |
|  | ZSA vs ASD | 0 | 0.08 | 0.11 | 0.12 | -0.04 | -0.13 | -0.17 | -0.35 | -0.3 |
|  | ZSD vs ASA | 0.47 | 0.43 | 0.38 | 0.41 | 0.34 | 0.33 | 0.28 | 0.3 | 0.42 |
|  | ZSA vs ASA | 0.04 | 0.15 | 0.18 | 0.15 | 0.15 | 0.16 | -0.03 | -0.26 | -0.55 |
|  | ZSD vs ZSA | -0.01 | 0.06 | 0.09 | 0.09 | -0.02 | -0.14 | -0.16 | -0.35 | -0.36 |
| Delay scaling parameter r |  | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 |
| XPR [dB] | XPR | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
|  | XPR | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Number of clusters |  | 3 | 3 | 3 | 3 | 3 | 3 | 2 | 2 | 2 |
| Number of rays per cluster |  | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Cluster DS () in [ns] |  | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 |
| Cluster ASD () in [deg] |  | 0.08 | 0.1 | 0.14 | 0.23 | 0.33 | 0.53 | 1 | 1.4 | 6.63 |
| Cluster ASA () in [deg] |  | 15.07 | 16.2 | 18.14 | 19.96 | 21.53 | 22.44 | 23.59 | 26.57 | 32.7 |
| Cluster ZSA () in [deg] |  | 1.66 | 4.71 | 7.33 | 9.82 | 11.52 | 11.75 | 10.93 | 12.19 | 16.68 |
| Per cluster shadowing std  [dB] |  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Correlation distance in the horizontal plane [m] | DS | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 |
|  | ASD | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  | ASA | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  | SF | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ZSA | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  | ZSD | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: All large scale parameters are assumed to have no correlation between different floors. NOTE 4: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. NOTE 6: For UMa and frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent LSP values NOTE 7: For UMi and frequencies below 2 GHz, use fc = 2 when determining the values of the frequency-dependent LSP values NOTE 8: For satellite (e.g.GEO/LEO), the departure angle spreads are zeros, i.e. µlgASD and µlgZSD are –∞, and corresponding standard deviations are zeros. |  |  |  |  |  |  |  |  |  |  |

Table 6.7-6 Part-4b: Channel model parameters for Urban Scenario (NLOS) at Ka band.

| Scenarios |  | Urban NLOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -7.24 | -7.7 | -7.82 | -8.04 | -8.08 | -8.1 | -8.16 | -8.03 | -8.33 |
|  | lgDS | 1.26 | 0.99 | 0.86 | 0.75 | 0.77 | 0.76 | 0.73 | 0.79 | 0.7 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | -1.58 | -1.67 | -1.84 | -2.02 | -2.06 | -1.99 | -2.19 | -1.88 | -2 |
|  | lgASD | 0.89 | 0.89 | 1.3 | 1.15 | 1.23 | 1.02 | 1.78 | 1.55 | 1.4 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | 0.13 | 0.19 | 0.44 | 0.48 | 0.56 | 0.55 | 0.48 | 0.53 | 0.32 |
|  | lgASA | 2.99 | 3.12 | 2.69 | 2.45 | 2.17 | 1.93 | 1.72 | 1.51 | 1.2 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | -1.13 | 0.49 | 0.95 | 1.15 | 1.14 | 1.13 | 1.16 | 1.28 | 1.42 |
|  | lgZSA | 2.66 | 2.03 | 1.54 | 1.02 | 1.61 | 1.84 | 1.81 | 1.35 | 0.6 |
| ZOD spread (ZSD) lgZSA=log10(ZSD/1) | lgZSD | -2.87 | -2.68 | -2.12 | -2.27 | -2.5 | -2.47 | -2.83 | -2.82 | -4.55 |
|  | lgZSD | 2.76 | 2.76 | 1.54 | 1.77 | 2.36 | 2.33 | 2.84 | 2.87 | 4.27 |
| Shadow fading (SF) [dB] | SF | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| K-factor (K) [dB] | K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Cross-Correlations | ASD vs DS | 0.74 | 0.6 | 0.75 | 0.65 | 0.72 | 0.78 | 0.85 | 0.8 | 0.86 |
|  | ASA vs DS | 0.47 | 0.43 | 0.35 | 0.35 | 0.23 | 0.24 | 0.27 | 0.25 | 0.51 |
|  | ASA vs SF | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASD vs SF | -0.6 | -0.6 | -0.6 | -0.6 | -0.6 | -0.6 | -0.6 | -0.6 | -0.6 |
|  | DS vs SF | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 |
|  | ASD vs ASA | 0.53 | 0.51 | 0.41 | 0.44 | 0.29 | 0.24 | 0.36 | 0.35 | 0.47 |
|  | ASD vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ASA vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | DS vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | SF vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Cross-Correlations | ZSD vs SF | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSA vs SF | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 |
|  | ZSD vs K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ZSA vs K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ZSD vs DS | 0.98 | 0.97 | 0.96 | 0.88 | 0.84 | 0.92 | 0.91 | 0.88 | 0.83 |
|  | ZSA vs DS | 0 | 0.06 | 0.08 | 0.1 | -0.04 | -0.17 | -0.16 | -0.32 | -0.37 |
|  | ZSD vs ASD | 0.72 | 0.61 | 0.73 | 0.67 | 0.56 | 0.68 | 0.79 | 0.75 | 0.96 |
|  | ZSA vs ASD | 0.01 | 0.08 | 0.12 | 0.16 | 0 | -0.11 | -0.16 | -0.31 | -0.28 |
|  | ZSD vs ASA | 0.47 | 0.45 | 0.38 | 0.41 | 0.32 | 0.29 | 0.29 | 0.29 | 0.46 |
|  | ZSA vs ASA | 0.05 | 0.15 | 0.18 | 0.18 | 0.21 | 0.24 | 0.02 | -0.16 | -0.43 |
|  | ZSD vs ZSA | 0 | 0.07 | 0.11 | 0.13 | -0.02 | -0.11 | -0.16 | -0.3 | -0.32 |
| Delay scaling parameter r |  | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 |
| XPR [dB] | XPR | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
|  | XPR | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Number of clusters |  | 3 | 3 | 3 | 3 | 3 | 3 | 2 | 2 | 2 |
| Number of rays per cluster |  | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Cluster DS () in [ns] |  | 1.6 | 1.6 | 1.6 | 1.6 | 1.6 | 1.6 | 1.6 | 1.6 | 1.6 |
| Cluster ASD () in [deg] |  | 0.08 | 0.1 | 0.14 | 0.22 | 0.31 | 0.49 | 0.97 | 1.52 | 5.36 |
| Cluster ASA () in [deg] |  | 14.72 | 14.62 | 16.4 | 17.86 | 19.74 | 19.73 | 20.5 | 26.16 | 25.83 |
| Cluster ZSA () in [deg] |  | 1.57 | 4.3 | 6.64 | 9.21 | 10.32 | 10.3 | 10.2 | 12.27 | 12.75 |
| Per cluster shadowing std  [dB] |  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Correlation distance in the horizontal plane [m] | DS | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 |
|  | ASD | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  | ASA | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  | SF | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ZSA | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  | ZSD | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: All large scale parameters are assumed to have no correlation between different floors. NOTE 4: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. NOTE 6: For UMa and frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent LSP values NOTE 7: For UMi and frequencies below 2 GHz, use fc = 2 when determining the values of the frequency-dependent LSP values NOTE 8: For satellite (e.g.GEO/LEO), the departure angle spreads are zeros, i.e. µlgASD and µlgZSD are –∞, and corresponding standard deviations are zeros. |  |  |  |  |  |  |  |  |  |  |

--- end of text proposal (to be continued) ---

--- start of text proposal ---

Table 6.7-6 Part-7a: Channel model parameters for Rural Scenario (LOS) at S band.

| Scenarios |  | Rural LOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -9.55 | -8.68 | -8.46 | -8.36 | -8.29 | -8.26 | -8.22 | -8.2 | -8.19 |
|  | lgDS | 0.66 | 0.44 | 0.28 | 0.19 | 0.14 | 0.1 | 0.1 | 0.05 | 0.06 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | -3.42 | -3 | -2.86 | -2.78 | -2.7 | -2.66 | -2.53 | -2.21 | -1.78 |
|  | lgASD | 0.89 | 0.63 | 0.52 | 0.45 | 0.42 | 0.41 | 0.42 | 0.5 | 0.91 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | -9.45 | -4.45 | -2.39 | -1.28 | -0.99 | -1.05 | -0.9 | -0.891.17 | -0.81 |
|  | lgASA | 7.83 | 6.86 | 5.14 | 3.44 | 2.59 | 2.42 | 1.78 | 1.65 | 1.26 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | -4.2 | -2.31 | -0.28 | -0.38 | -0.38 | -0.46 | -0.49 | -0.53 | -0.46 |
|  | lgZSA | 6.3 | 5.04 | 0.81 | 1.16 | 0.82 | 0.67 | 1 | 1.18 | 0.91 |
| ZOD spread (ZSD) lgZSA=log10(ZSD/1) | lgZSD | -6.03 | -4.31 | -2.57 | -2.59 | -2.59 | -2.65 | -2.69 | -2.65 | -2.65 |
|  | lgZSD | 5.19 | 4.18 | 0.61 | 0.79 | 0.65 | 0.52 | 0.78 | 1.01 | 0.71 |
| Shadow fading (SF) [dB] | SF | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| K-factor (K) [dB] | K | 24.72 | 12.31 | 8.05 | 6.21 | 5.04 | 4.42 | 3.92 | 3.65 | 3.59 |
|  | K | 5.07 | 5.75 | 5.46 | 5.23 | 3.95 | 3.75 | 2.56 | 1.77 | 1.77 |
| Cross-Correlations | ASD vs DS | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASA vs DS | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASA vs SF | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASD vs SF | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | DS vs SF | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 |
|  | ASD vs ASA | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASD vs  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASA vs  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | DS vs  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | SF vs  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Cross-Correlations | ZSD vs SF | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
|  | ZSA vs SF | -0.17 | -0.17 | -0.17 | -0.17 | -0.17 | -0.17 | -0.17 | -0.17 | -0.17 |
|  | ZSD vs K | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSA vs K | -0.02 | -0.02 | -0.02 | -0.02 | -0.02 | -0.02 | -0.02 | -0.02 | -0.02 |
|  | ZSD vs DS | -0.05 | -0.05 | -0.05 | -0.05 | -0.05 | -0.05 | -0.05 | -0.05 | -0.05 |
|  | ZSA vs DS | 0.27 | 0.27 | 0.27 | 0.27 | 0.27 | 0.27 | 0.27 | 0.27 | 0.27 |
|  | ZSD vs ASD | 0.73 | 0.73 | 0.73 | 0.73 | 0.73 | 0.73 | 0.73 | 0.73 | 0.73 |
|  | ZSA vs ASD | -0.14 | -0.14 | -0.14 | -0.14 | -0.14 | -0.14 | -0.14 | -0.14 | -0.14 |
|  | ZSD vs ASA | -0.20 | -0.20 | -0.20 | -0.20 | -0.20 | -0.20 | -0.20 | -0.20 | -0.20 |
|  | ZSA vs ASA | 0.24 | 0.24 | 0.24 | 0.24 | 0.24 | 0.24 | 0.24 | 0.24 | 0.24 |
|  | ZSD vs ZSA | -0.07 | -0.07 | -0.07 | -0.07 | -0.07 | -0.07 | -0.07 | -0.07 | -0.07 |
| Delay scaling parameter r |  | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 |
| XPR [dB] | XPR | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
|  | XPR | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Number of clusters |  | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Number of rays per cluster |  | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Cluster DS () in [ns] |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Cluster ASD () in [deg] |  | 0.39 | 0.31 | 0.29 | 0.37 | 0.61 | 0.9 | 1.43 | 2.87 | 5.48 |
| Cluster ASA () in [deg] |  | 10.81 | 8.09 | 13.7 | 20.05 | 24.51 | 26.35 | 31.84 | 36.62 | 36.77 |
| Cluster ZSA () in [deg] |  | 1.94 | 1.83 | 2.28 | 2.93 | 2.84 | 3.17 | 3.88 | 4.17 | 4.29 |
| Per cluster shadowing std  [dB] |  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Correlation distance in the horizontal plane [m] | DS | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  | ASD | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
|  | ASA | 35 | 35 | 35 | 35 | 35 | 35 | 35 | 35 | 35 |
|  | SF | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 |
|  |  | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 |
|  | ZSA | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
|  | ZSD | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: All large scale parameters are assumed to have no correlation between different floors. NOTE 4: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. NOTE 6: For UMa and frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent LSP values NOTE 7: For UMi and frequencies below 2 GHz, use fc = 2 when determining the values of the frequency-dependent LSP values NOTE 8: For satellite (e.g.GEO/LEO), the departure angle spreads are zeros, i.e. µlgASD and µlgZSD are –∞, and corresponding standard deviations are zeros. |  |  |  |  |  |  |  |  |  |  |

Table 6.7-6 Part-7b: Channel model parameters for Rural Scenario (LOS) at Ka band.

| Scenarios |  | Rural LOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -9.68 | -8.86 | -8.59 | -8.46 | -8.36 | -8.3 | -8.26 | -8.22 | -8.21 |
|  | lgDS | 0.46 | 0.29 | 0.18 | 0.19 | 0.14 | 0.15 | 0.13 | 0.03 | 0.07 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | -4.03 | -3.55 | -3.45 | -3.38 | -3.33 | -3.29 | -3.24 | -2.9 | -2.5 |
|  | lgASD | 0.91 | 0.7 | 0.55 | 0.52 | 0.46 | 0.43 | 0.46 | 0.44 | 0.82 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | -9.74 | -4.88 | -2.6 | -1.92 | -1.56 | -1.66 | -1.59 | -1.58 | -1.51 |
|  | lgASA | 7.52 | 6.67 | 4.63 | 3.45 | 2.44 | 2.38 | 1.67 | 1.44 | 1.13 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | -5.85 | -3.270.95 | -0.88 | -0.93 | -0.99 | -1.04 | -1.17 | -1.19 | -1.13 |
|  | lgZSA | 6.51 | 5.36 | 0.93 | 0.96 | 0.97 | 0.83 | 1.01 | 1.01 | 0.85 |
| ZOD spread (ZSD) lgZSA=log10(ZSD/1) | lgZSD | -7.45 | -5.25 | -3.16 | -3.15 | -3.2 | -3.27 | -3.42 | -3.36 | -3.35 |
|  | lgZSD | 5.3 | 4.42 | 0.68 | 0.73 | 0.77 | 0.61 | 0.74 | 0.79 | 0.65 |
| Shadow fading (SF) [dB] | SF | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| K-factor (K) [dB] | K | 25.43 | 12.72 | 8.40 | 6.52 | 5.24 | 4.57 | 4.02 | 3.70 | 3.62 |
|  | K | 7.04 | 7.47 | 7.18 | 6.88 | 5.28 | 4.92 | 3.40 | 2.22 | 2.28 |
| Cross-Correlations | ASD vs DS | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASA vs DS | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASA vs SF | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASD vs SF | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | DS vs SF | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 |
|  | ASD vs ASA | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASD vs  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASA vs  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | DS vs  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | SF vs  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Cross-Correlations | ZSD vs SF | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
|  | ZSA vs SF | -0.17 | -0.17 | -0.17 | -0.17 | -0.17 | -0.17 | -0.17 | -0.17 | -0.17 |
|  | ZSD vs K | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSA vs K | -0.02 | -0.02 | -0.02 | -0.02 | -0.02 | -0.02 | -0.02 | -0.02 | -0.02 |
|  | ZSD vs DS | -0.05 | -0.05 | -0.05 | -0.05 | -0.05 | -0.05 | -0.05 | -0.05 | -0.05 |
|  | ZSA vs DS | 0.27 | 0.27 | 0.27 | 0.27 | 0.27 | 0.27 | 0.27 | 0.27 | 0.27 |
|  | ZSD vs ASD | 0.73 | 0.73 | 0.73 | 0.73 | 0.73 | 0.73 | 0.73 | 0.73 | 0.73 |
|  | ZSA vs ASD | -0.14 | -0.14 | -0.14 | -0.14 | -0.14 | -0.14 | -0.14 | -0.14 | -0.14 |
|  | ZSD vs ASA | -0.20 | -0.20 | -0.20 | -0.20 | -0.20 | -0.20 | -0.20 | -0.20 | -0.20 |
|  | ZSA vs ASA | 0.24 | 0.24 | 0.24 | 0.24 | 0.24 | 0.24 | 0.24 | 0.24 | 0.24 |
|  | ZSD vs ZSA | -0.07 | -0.07 | -0.07 | -0.07 | -0.07 | -0.07 | -0.07 | -0.07 | -0.07 |
| Delay scaling parameter r |  | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 |
| XPR [dB] | XPR | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
|  | XPR | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Number of clusters |  | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Number of rays per cluster |  | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Cluster DS () in [ns] |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Cluster ASD () in [deg] |  | 0.36 | 0.3 | 0.25 | 0.35 | 0.53 | 0.88 | 1.39 | 2.7 | 4.97 |
| Cluster ASA () in [deg] |  | 4.63 | 6.83 | 12.91 | 18.9 | 22.44 | 25.69 | 27.95 | 31.45 | 28.01 |
| Cluster ZSA () in [deg] |  | 0.75 | 1.25 | 1.93 | 2.37 | 2.66 | 3.23 | 3.71 | 4.17 | 4.14 |
| Per cluster shadowing std  [dB] |  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Correlation distance in the horizontal plane [m] | DS | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  | ASD | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
|  | ASA | 35 | 35 | 35 | 35 | 35 | 35 | 35 | 35 | 35 |
|  | SF | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 | 37 |
|  |  | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 |
|  | ZSA | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
|  | ZSD | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: All large scale parameters are assumed to have no correlation between different floors. NOTE 4: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. NOTE 6: For UMa and frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent LSP values NOTE 7: For UMi and frequencies below 2 GHz, use fc = 2 when determining the values of the frequency-dependent LSP values NOTE 8: For satellite (e.g.GEO/LEO), the departure angle spreads are zeros, i.e. µlgASD and µlgZSD are –∞, and corresponding standard deviations are zeros. |  |  |  |  |  |  |  |  |  |  |

Table 6.7-6 Part-8a: Channel model parameters for Rural Scenario (NLOS) at S band.

| Scenarios |  | Rural NLOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -9.01 | -8.37 | -8.05 | -7.92 | -7.92 | -7.96 | -7.91 | -7.79 | -7.74 |
|  | lgDS | 1.59 | 0.95 | 0.92 | 0.92 | 0.87 | 0.87 | 0.82 | 0.86 | 0.81 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | -2.9 | -2.5 | -2.12 | -1.99 | -1.9 | -1.85 | -1.69 | -1.46 | -1.32 |
|  | lgASD | 1.34 | 1.18 | 1.08 | 1.06 | 1.05 | 1.06 | 1.14 | 1.16 | 1.3 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | -3.33 | -0.74 | 0.08 | 0.32 | 0.53 | 0.33 | 0.55 | 0.45 | 0.4 |
|  | lgASA | 6.22 | 4.22 | 3.02 | 2.45 | 1.63 | 2.08 | 1.58 | 2.01 | 2.19 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | -0.88 | -0.07 | 0.75 | 0.72 | 0.951.02 | 0.97 | 1.1 | 0.97 | 1.35 |
|  | lgZSA | 3.26 | 3.29 | 1.92 | 1.92 | 1.45 | 1.62 | 1.43 | 1.88 | 0.62 |
| ZOD spread (ZSD) lgZSA=log10(ZSD/1) | lgZSD | -4.92 | -4.06 | -2.33 | -2.24 | -2.24 | -2.22 | -2.19 | -2.41 | -2.45 |
|  | lgZSD | 3.96 | 4.07 | 1.7 | 2.01 | 2 | 1.82 | 1.66 | 2.58 | 2.52 |
| Shadow fading (SF) [dB] | SF | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| K-factor (K) [dB] | K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Cross-Correlations | ASD vs DS | 0.5 | 0.3 | 0.32 | 0.39 | 0.25 | 0.16 | 0.23 | 0.18 | 0.91 |
|  | ASA vs DS | 0.2 | 0.3 | 0.31 | 0.38 | 0.44 | 0.38 | 0.49 | 0.27 | -0.01 |
|  | ASA vs SF | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASD vs SF | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |
|  | DS vs SF | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 |
|  | ASD vs ASA | 0.57 | 0.14 | 0.08 | 0.25 | 0.17 | 0.15 | 0.23 | -0.04 | -0.05 |
|  | ASD vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ASA vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | DS vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | SF vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Cross-Correlations | ZSD vs SF | -0.04 | -0.04 | -0.04 | -0.04 | -0.04 | -0.04 | -0.04 | -0.04 | -0.04 |
|  | ZSA vs SF | -0.25 | -0.25 | -0.25 | -0.25 | -0.25 | -0.25 | -0.25 | -0.25 | -0.25 |
|  | ZSD vs K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ZSA vs K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ZSD vs DS | 0.75 | 0.79 | 0.74 | 0.9 | 0.93 | 0.93 | 0.94 | 0.74 | 0.61 |
|  | ZSA vs DS | 0.05 | 0.03 | 0 | -0.09 | -0.2 | -0.22 | -0.33 | -0.41 | -0.36 |
|  | ZSD vs ASD | 0.7 | 0.42 | 0.38 | 0.33 | 0.19 | 0.17 | 0.21 | 0.15 | 0.31 |
|  | ZSA vs ASD | 0.12 | -0.04 | -0.1 | -0.13 | -0.13 | -0.12 | -0.13 | -0.14 | -0.29 |
|  | ZSD vs ASA | 0.35 | 0.35 | 0.31 | 0.38 | 0.47 | 0.45 | 0.52 | 0.28 | 0.1 |
|  | ZSA vs ASA | 0.09 | 0.21 | 0.22 | 0.06 | -0.06 | -0.15 | -0.36 | -0.28 | -0.4 |
|  | ZSD vs ZSA | 0.15 | 0.07 | -0.06 | -0.13 | -0.21 | -0.22 | -0.31 | -0.33 | -0.27 |
| Delay scaling parameter r |  | 1.7 | 1.7 | 1.7 | 1.7 | 1.7 | 1.7 | 1.7 | 1.7 | 1.7 |
| XPR [dB] | XPR | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
|  | XPR | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Number of clusters |  | 3 | 3 | 2 | 2 | 2 | 2 | 2 | 1 | 2 |
| Number of rays per cluster |  | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Cluster DS () in [ns] |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Cluster ASD () in [deg] |  | 0.03 | 0.05 | 0.07 | 0.1 | 0.15 | 0.22 | 0.5 | 1.04 | 2.11 |
| Cluster ASA () in [deg] |  | 18.16 | 26.82 | 21.99 | 22.86 | 25.93 | 27.79 | 28.5 | 37.53 | 29.23 |
| Cluster ZSA () in [deg] |  | 2.32 | 7.34 | 8.28 | 8.76 | 9.68 | 9.94 | 8.9 | 13.74 | 12.16 |
| Per cluster shadowing std  [dB] |  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Correlation distance in the horizontal plane [m] | DS | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 |
|  | ASD | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
|  | ASA | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 |
|  | SF | 120 | 120 | 120 | 120 | 120 | 120 | 120 | 120 | 120 |
|  |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ZSA | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  | ZSD | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: All large scale parameters are assumed to have no correlation between different floors. NOTE 4: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. NOTE 6: For UMa and frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent LSP values NOTE 7: For UMi and frequencies below 2 GHz, use fc = 2 when determining the values of the frequency-dependent LSP values NOTE 8: For satellite (e.g.GEO/LEO), the departure angle spreads are zeros, i.e. µlgASD and µlgZSD are –∞, and corresponding standard deviations are zeros. |  |  |  |  |  |  |  |  |  |  |

Table 6.7-6 Part-8b: Channel model parameters for Rural Scenario (NLOS) at Ka band.

| Scenarios |  | Rural NLOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -9.13 | -8.39 | -8.1 | -7.96 | -7.99 | -8.05 | -8.01 | -8.05 | -7.91 |
|  | lgDS | 1.91 | 0.94 | 0.92 | 0.94 | 0.89 | 0.87 | 0.82 | 1.65 | 0.76 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | -2.9 | -2.53 | -2.16 | -2.04 | -1.99 | -1.95 | -1.81 | -1.56 | -1.53 |
|  | lgASD | 1.32 | 1.18 | 1.08 | 1.09 | 1.08 | 1.06 | 1.17 | 1.2 | 1.27 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | -3.4 | -0.51 | 0.06 | 0.2 | 0.4 | 0.32 | 0.46 | 0.33 | 0.24 |
|  | lgASA | 6.28 | 3.75 | 2.95 | 2.65 | 1.85 | 1.83 | 1.57 | 1.99 | 2.18 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | -1.19 | -0.11 | 0.72 | 0.69 | 0.84 | 0.99 | 0.95 | 0.92 | 1.29 |
|  | lgZSA | 3.81 | 3.33 | 1.93 | 1.91 | 1.7 | 1.27 | 1.86 | 1.84 | 0.59 |
| ZOD spread (ZSD) lgZSA=log10(ZSD/1) | lgZSD | -5.47 | -4.06 | -2.32 | -2.19 | -2.16 | -2.24 | -2.29 | -2.65 | -2.23 |
|  | lgZSD | 4.39 | 4.04 | 1.542.59 | 1.73 | 1.5 | 1.64 | 1.66 | 2.86 | 1.12 |
| Shadow fading (SF) [dB] | SF | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| K-factor (K) [dB] | K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Cross-Correlations | ASD vs DS | 0.51 | 0.36 | 0.29 | 0.41 | 0.27 | 0.19 | 0.44 | 0.2 | 0.9 |
|  | ASA vs DS | 0.22 | 0.33 | 0.32 | 0.41 | 0.47 | 0.37 | 0.34 | 0.28 | -0.02 |
|  | ASA vs SF | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ASD vs SF | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |
|  | DS vs SF | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 |
|  | ASD vs ASA | 0.57 | 0.15 | 0.09 | 0.24 | 0.11 | 0.16 | 0.26 | -0.03 | -0.03 |
|  | ASD vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ASA vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | DS vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | SF vs  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Cross-Correlations | ZSD vs SF | -0.04 | -0.04 | -0.04 | -0.04 | -0.04 | -0.04 | -0.04 | -0.04 | -0.04 |
|  | ZSA vs SF | -0.25 | -0.25 | -0.25 | -0.25 | -0.25 | -0.25 | -0.25 | -0.25 | -0.25 |
|  | ZSD vs K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ZSA vs K | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ZSD vs DS | 0.72 | 0.78 | 0.72 | 0.91 | 0.91 | 0.86 | 0.93 | 0.82 | 0.75 |
|  | ZSA vs DS | 0.05 | 0.02 | 0.04 | -0.06 | -0.16 | -0.17 | -0.31 | -0.35 | -0.29 |
|  | ZSD vs ASD | 0.71 | 0.42 | 0.4 | 0.45 | 0.15 | 0.2 | 0.32 | 0.11 | 0.44 |
|  | ZSA vs ASD | 0.1 | -0.04 | -0.07 | -0.11 | -0.12 | -0.1 | -0.13 | -0.14 | -0.31 |
|  | ZSD vs ASA | 0.4 | 0.35 | 0.33 | 0.4 | 0.47 | 0.46 | 0.35 | 0.3 | 0.08 |
|  | ZSA vs ASA | 0.1 | 0.21 | 0.22 | 0.1 | 0 | -0.11 | -0.25 | -0.18 | -0.21 |
|  | ZSD vs ZSA | 0.19 | 0.06 | -0.02 | -0.11 | -0.19 | -0.19 | -0.3 | -0.28 | -0.22 |
| Delay scaling parameter r |  | 1.7 | 1.7 | 1.7 | 1.7 | 1.7 | 1.7 | 1.7 | 1.7 | 1.7 |
| XPR [dB] | XPR | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
|  | XPR | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Number of clusters |  | 3 | 3 | 2 | 2 | 2 | 2 | 2 | 1 | 2 |
| Number of rays per cluster |  | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Cluster DS () in [ns] |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Cluster ASD () in [deg] |  | 0.03 | 0.05 | 0.07 | 0.09 | 0.16 | 0.22 | 0.51 | 0.89 | 1.68 |
| Cluster ASA () in [deg] |  | 18.21 | 24.08 | 22.06 | 21.4 | 24.26 | 24.15 | 25.99 | 36.07 | 24.51 |
| Cluster ZSA () in [deg] |  | 2.13 | 6.52 | 7.72 | 8.45 | 8.92 | 8.76 | 9 | 13.6 | 10.56 |
| Per cluster shadowing std  [dB] |  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Correlation distance in the horizontal plane [m] | DS | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 |
|  | ASD | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
|  | ASA | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 |
|  | SF | 120 | 120 | 120 | 120 | 120 | 120 | 120 | 120 | 120 |
|  |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | ZSA | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  | ZSD | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: All large scale parameters are assumed to have no correlation between different floors. NOTE 4: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. NOTE 6: For UMa and frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent LSP values NOTE 7: For UMi and frequencies below 2 GHz, use fc = 2 when determining the values of the frequency-dependent LSP values NOTE 8: For satellite (e.g.GEO/LEO), the departure angle spreads are zeros, i.e. µlgASD and µlgZSD are –∞, and corresponding standard deviations are zeros. |  |  |  |  |  |  |  |  |  |  |

--- end of text proposal ---