3GPP TSG RAN WG1 Meeting #94	R1-1808077

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:	7.2.7.2

Source:	Huawei, HiSilicon

Title:	Considerations and evaluation results for IMT-2020 for peak spectral efficiency and peak data rate

Document for:	Discussion and decision

Introduction

In this contribution, the consideration on self evaluation for peak spectral efficiency and peak data rate is provided.

In previous contribution [1], the calculation method on peak spectral efficiency and peak data rate were provided. In RAN1#91 meeting, the data rate calculation method was adopted in NR UE category discussion, as shown in [2]. In this contribution, the peak spectral efficiency and peak data rate evaluation for NR and LTE are considered. The evaluation assumption for NR is updated. In addition, the two frequency ranges, i.e., frequency range 1 (FR1) for sub 6 GHz, and frequency range 2 (FR2) for above 24 GHz, are taken into account for NR.

Peak spectral efficiency

According to Report ITU-R M.2410 (see [3]), the peak spectral efficiency is defined as in subsection 3.1. Based on the definition, the DL and UL peak spectral efficiency of NR is analysed.

NR

From the definition and [2] we can derive the generic formula for peak spectral efficiency for FDD and TDD for a specific component carrier (say j-th CC) as below.

(1)

wherein

- Rmax = 948/1024

- For the j-th CC,

  - is the maximum number of layers

  - is the maximum modulation order

  - is the scaling factor

      - The scaling factor can at least take the values 1 and 0.75.

      - is signalled per band and per band per band combination as per UE capability signalling

  - is the numerology (as defined in TS38.211)

  - is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed.

  - is the maximum PRB allocation in bandwidth  with numerology , as given in TR 38.817-01 section 4.5.1, where  is the UE supported maximum bandwidth in the given band or band combination.

  - is the overhead  calculated as the average ratio of the number of REs occupied by L1/L2 control, Synchronization Signal, PBCH and reference signals etc. with respect to the total number of REs in effective bandwidth  time product.

j is the normalized scalar considering the downlink/uplink ratio; for FDD j=1 for DL and UL; and for TDD and other duplexing j for DL and UL is calculated based on the frame structure.

DL peak spectral efficiency

Based on the formula provided in Eq. (1), the DL peak spectral efficiency for NR is given in this sub-section.

The DL peak spectral efficiency for NR FDD for different bandwidth and sub-carrier spacing (SCS) parameters is given in Table 2.1.1-1. The results are according to Eq. (1) and the detailed parameters as listed in Appendix. It is noted that if the channel bandwidth is less than the bandwidth of SSB, then SSB is not transmitted and the overhead of SSB is zero. If the channel bandwidth is less than TRS bandwidth given in Table A-1, the TRS bandwidth is assumed to be equal to the channel bandwidth. The above assumptions are also used for TDD peak spectral efficiency.

Table 2.1.1-1 NR FDD DL peak spectral efficiency (bit/s/Hz)

| SCS [kHz] |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Req. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 15 | 40.3 | 44.0 | 45.6 | 46.4 | 46.6 | 47.2 | 47.8 | 47.8 | - | - | - | - | 30 |
|  | 30 | 32.8 | 39.2 | 42.9 | 43.7 | 45.0 | 45.6 | 46.4 | 47.1 | 47.8 | 48.1 | 48.8 | 48.9 | 30 |
|  | 60 | - | 32.8 | 38.2 | 39.7 | 41.5 | 42.9 | 44.2 | 45.0 | 46.1 | 47.4 | 47.6 | 47.8 | 30 |

The DL peak spectral efficiency for NR TDD for different bandwidth and SCS parameters is shown in Table 2.1.1-2. The results are according to Eq. (1) and the detailed parameters as listed in Appendix. In this evaluation, the DL dominant frame structure “DDDSU” (DL:UL=4:1) is selected.

Table 2.1.1-2 NR TDD DL peak spectral efficiency (bit/s/Hz)
(Frame structure: DDDSU)

| SCS [kHz] |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Req. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 15 | 38.3 | 43.0 | 44.0 | 44.8 | 45.5 | 45.7 | 46.8 | 46.8 | - | - | - | - | - | - | 30 |
|  | 30 | 31.5 | 37.7 | 41.4 | 42.6 | 44.0 | 44.0 | 45.4 | 45.5 | 46.8 | 47.0 | 47.1 | 47.3 | - | - | 30 |
|  | 60 | - | 31.5 | 36.8 | 38.2 | 40.5 | 41.9 | 42.6 | 44.0 | 45.1 | 45.8 | 46.6 | 46.8 | - | - | 30 |
| FR2 | 60 | - | - | - | - | - | - | - | 32.7 | - | - | - | 33.5 | 33.9 | - | 30 |
|  | 120 | - | - | - | - | - | - | - | 30.6 | - | - | - | 33.1 | 33.5 | 33.9 | 30 |

UL peak spectral efficiency

Similarly, based on the formula provided in Eq. (1), the UL peak spectral efficiency for NR is given in this sub-section.

The UL peak spectral efficiency for NR FDD for different bandwidth and SCS parameters is shown in Table 2.1.2-1. The results are according to Eq. (1) and the detailed parameters as listed in Appendix.

Table 2.1.2-1 NR FDD UL peak spectral efficiency (bit/s/Hz)

| SCS [kHz] |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Req. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 15 | 22.9 | 23.8 | 24.1 | 24.3 | 24.4 | 24.4 | 24.7 | 24.7 | - | - | - | - | 15 |
|  | 30 | 20.1 | 22.0 | 23.2 | 23.4 | 23.8 | 23.8 | 24.3 | 24.4 | 24.7 | 24.8 | 24.9 | 25.0 | 15 |
|  | 60 | - | 20.1 | 22.0 | 22.0 | 22.7 | 23.2 | 23.6 | 24.1 | 24.4 | 24.8 | 24.9 | 25.0 | 15 |

The TDD UL peak spectral efficiency for NR TDD for different bandwidth and SCS parameters is shown in Table 2.1.2-2. In this evaluation, the DL dominant frame structure “DDDSU” is selected.

Table 2.1.2-2 NR TDD UL peak spectral efficiency (bit/s/Hz) for DL dominant frame structure

(Frame structure: DDDSU, DL:UL = 4:1)

| SCS [kHz] |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Req. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 15 | 20.9 | 21.7 | 22.0 | 22.2 | 22.2 | 22.3 | 22.6 | 22.6 | - | - | - | - | - | - | 15 |
|  | 30 | 18.6 | 20.3 | 21.7 | 21.8 | 22.3 | 22.3 | 22.7 | 22.8 | 23.1 | 23.2 | 23.3 | 23.4 | - | - | 15 |
|  | 60 | - | 18.8 | 20.5 | 20.5 | 21.2 | 21.7 | 21.8 | 22.5 | 22.8 | 23.2 | 23.3 | 23.4 | - | - | 15 |
| FR2 | 60 | - | - | - | - | - | - | - | 21.5 | - | - | - | 21.5 | 21.5 | - | 15 |
|  | 120 | - | - | - | - | - | - | - | 21.2 | - | - | - | 21.8 | 21.8 | 21.8 | 15 |

Observation 1: NR can fulfill DL and UL peak spectral efficiency requirements for all SCS and bandwidth, for both below and above 6GHz.

LTE

DL peak spectral efficiency

DL peak spectral efficiency for LTE are provided in Table 2.2.1-1 and Table 2.2.1-2 for FDD and TDD, respectively. It should be noted that the transport block size (TBS) table defined in TS 36.213 is used to calculate the DL and UL peak spectral efficiency.

Table 2.2.1-1 LTE FDD DL peak spectral efficiency (bit/s/Hz)

| Modulation order |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Req. |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | 256QAM | 39.16 | 39.16 | 39.98 | 39.17 | 30 |
|  | 1024QAM | 50.32 | 49.13 | 50.06 | 50.26 | 30 |

Table 2.2.1-2 LTE TDD DL peak spectral efficiency (bit/s/Hz) for DL dominant frame structure 
(Frame structure: DSUDD, DL:UL=4:1)

| Modulation order |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Req. |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | 256QAM | 37.83 | 38.08 | 38.71 | 38.24 | 30 |
|  | 1024QAM | 48.26 | 47.63 | 48.45 | 48.83 | 30 |

UL peak spectral efficiency

UL peak spectral efficiency for LTE are provided in Table 2.2.2-1 and Table 2.2.2-2 for FDD and TDD, respectively.

Table 2.2.2-1 LTE FDD UL peak spectral efficiency (bit/s/Hz)

| Modulation order |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Req. |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | 256QAM | 19.58 | 19.58 | 19.99 | 19.58 | 15 |

Table 2.2.2-2 LTE TDD UL peak spectral efficiency (bit/s/Hz) for DL dominant frame structure 
(Frame structure: DSUDD, DL:UL=4:1)

| Modulation order |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Req. |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | 256QAM | 17.13 | 17.13 | 17.49 | 17.13 | 15 |

Observation 2: LTE can fulfill DL and UL peak spectral efficiency requirements.

Peak data rate

According to Report ITU-R M.2410, the peak data rate is defined as in subsection 4.1. Based on the definition, the DL and UL peak data rate of NR and LTE is analyzed.

The DL/UL peak data rate for FDD and TDD can be calculated as below

(3)

where the scaling factor (j) is the value for DL or UL on band j, respectively.

It is noted that in the above equation, Wj is the effective bandwidth that takes into account the uplink/downlink ratio as defined in Report ITU-R M.2410.

NR

DL peak data rate

The DL peak data rate for NR is shown in Table 3.1.1-1.

Table 3.1.1-1 NR DL peak data rate

| Duplexing | SCS [kHz] |  | Per CC BW (MHz) | Peak data rate per CC (Gbit/s) | Number of CC | Total bandwidth (MHz) | Aggregated peak data rate (Gbit/s) | Req. (Gbit/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | FR1 | 15 | 50 | 2.39 | 9 | 450 | 21.51 | 20 |
|  |  | 30 | 100 | 4.89 | 5 | 500 | 24.45 |  |
|  |  | 60 | 100 | 4.78 | 5 | 500 | 23.9 |  |
| TDD (DDDSU， αDLj=0.7714) | FR1 | 15 | 50 | 1.805 | 12 | 600 | 21.66 |  |
|  |  | 30 | 100 | 3.649 | 6 | 600 | 21.89 |  |
|  |  | 60 | 100 | 3.61 | 6 | 600 | 21.66 |  |
|  | FR2 | 60 | 200 | 5.23 | 4 | 800 | 20.92 |  |
|  |  | 120 | 400 | 10.461 | 2 | 800 | 20.92 |  |

UL peak data rate

The UL peak data rate for NR is shown in Table 3.1.2-1. It is noted that in Rel-15 NR WI, NR-LTE spectrum sharing using a TDD band and a supplementary uplink (SUL) band is enabled to enhance the NR uplink coverage and capacity. For the peak data rate calculation in SUL band, the duplexing mode FDD is employed. For TDD band, the frame structure “DDDSU” is selected. In the evaluation, the data is assumed to be transmitted in the SUL band and the peak spectral efficiency in SUL band is assumed to be the same as that of UL FDD in NR. The UL FDD peak spectral efficiency for 15 kHz SCS and 50 MHz bandwidth in Table 2.1.2-2 is selected to calculate the peak data rate in SUL band. The UL peak data rate for TDD + SUL band is shown in Table 3.1.2-2.2.The UL peak data rate for NR is shown in Table 3.1.2-1. z SCFF f

Table 3.1.2-1 NR UL peak data rate

| Duplexing | SCS [kHz] |  | Per CC BW (MHz) | Peak data rate per CC (Gbit/s) | Number of CC | Total bandwidth (MHz) | Aggregated peak data rate (Gbit/s) | Req. (Gbit/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | FR1 | 15 | 50 | 1.235 | 9 | 450 | 11.12 | 10 |
|  |  | 30 | 100 | 2.5 | 4 | 400 | 10.0 |  |
|  |  | 60 | 100 | 2.5 | 4 | 400 | 10.0 |  |

Table 3.1.2-2 NR UL peak data rate for TDD band + SUL band

(TDD band frame structure: DDDSU)

| Duplexing | TDD band SCS [kHz] |  | SUL band SCS [kHz] | Per CC BW in SUL band (MHz) | Peak data rate per CC in SUL band (Gbit/s) | Number of CC in SUL band | Total bandwidth (MHz) | Aggregated peak data rate (Gbit/s) | Req. (Gbit/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDD (TDD band) + FDD (SUL band) | FR1 | 15 | 15 | 50 | 1.235 | 9 | 450 | 11.12 | 10 |
|  |  | 30 | 15 | 50 | 1.235 | 9 | 450 | 11.12 |  |
|  |  | 60 | 15 | 50 | 1.235 | 9 | 450 | 11.12 |  |
|  | FR2 | 60 | 15 | 50 | 1.235 | 9 | 450 | 11.12 |  |
|  |  | 120 | 15 | 50 | 1.235 | 9 | 450 | 11.12 |  |

Based on the above evaluation results, it is observed that,

Observation 3: NR can fulfill DL and UL peak data rate requirements for both below and above 6GHz with carrier aggregation.

Observation 4: The frequency range of below 6 GHz can achieve similar peak data rate performance to above 6GHz by using less bandwidth.

LTE

DL peak data rate

DL peak data rate for LTE are provided in Table 3.2.1-1 for FDD. The detailed assumptions are provided in Appendix-B.

Table 3.2.1-1 LTE DL peak data rate

| Duplexing | Modulation order | Per CC BW (MHz) | Peak data rate per CC (Gbit/s) | Number of CC | Total bandwidth (MHz) | Aggregated peak data rate (Gbit/s) | Req. (Gbit/s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 256 QAM | 20 | 0.783 | 26 | 520 | 20.35 | 20 |
|  | 1024 QAM | 20 | 1.005 | 20 | 400 | 20.1 |  |
| TDD (DSUDD, αDLj=0.7714) | 1024 QAM | 20 | 0.753 | 27 | 540 | 20.33 |  |

UL peak data rate

UL peak data rate for LTE are provided in Table 3.2.2-1 for FDD.

Table 3.2.2-1 LTE UL peak data rate

| Duplexing | Per CC BW (MHz) | Peak data rate per CC (Gbit/s) | Number of CC | Total bandwidth (MHz) | Aggregated peak data rate (Gbit/s) | Req. (Gbit/s) |
| --- | --- | --- | --- | --- | --- | --- |
| FDD | 20 | 0.392 | 26 | 520 | 10.19 | 10 |

Observation 5: LTE can fulfill DL and UL peak data rate requirements with carrier aggregation.

Conclusions

In this document, the peak spectral efficiency and peak data rate of NR for the two frequency ranges are evaluated. The following observations and proposal are made.

Observation 1: NR can fulfill DL and UL peak spectral efficiency requirements for all SCS and bandwidth, for both below and above 6GHz.

Observation 2: LTE can fulfill DL and UL peak spectral efficiency requirements.

Observation 3: NR can fulfill DL and UL peak data rate requirements for both below and above 6GHz with carrier aggregation.

Observation 4: The frequency range of below 6 GHz can achieve similar peak data rate performance to above 6GHz by using less bandwidth.

Observation 5: LTE can fulfill DL and UL peak data rate requirements with carrier aggregation.

Proposal: Capture the evaluation results in Section 2 and 3 into TR37.910.

- References

R1-1805931, “Considerations and evaluation results for IMT-2020 for peak spectral efficiency and peak data rate”, Huawei, HiSilicon, May 2018.

R1-1721732, “Reply to LS on NR UE Category”, RAN1, November 2017.

Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Appendix-A       Detailed evaluation assumptions for NR

Table A-1 Parameter assumptions of NR FDD DL peak spectral efficiency

| # Parameters | # Values | # Remarks |
| --- | --- | --- |
|  | 8 | NR supports up to 8 layers for a single user for DL |
|  | 8 | NR supports up to 256QAM for DL |
|  | 1 | The value of 1 is chosen as scaling factor for DL peak spectral efficiency evaluation. |
| Rmax | 948/1024 = 0.9258 | NR supports highest coding rate as Rmax=948/1024. |
|  | 0, 1, 2, 3 | SCS, i.e., - 15kHz SCS for =0 - 30kHz SCS for  =1 - 60kHz SCS for  =2 - 120kHz SCS for  =3 |
|  | For FR1: - 270 for 50MHz with 15kHz SCS - 273 for 100MHz with 30kHz SCS - 135 for 100MHz with 60kHz SCS | See Section 5.3.2 of TS38.104 v0.5.0 |
|  |  | SCS, i.e., - 15kHz SCS for =0 - 30kHz SCS for  =1 - 60kHz SCS for  =2 - 120kHz SCS for  =3 |
|  | For FR1: - 0.11 for 50MHz with 15kHz SCS - 0.10 for 100MHz with 30kHz SCS - 0.11 for 100MHz with 60kHz SCS | For FR1: - CORESET of 24 PRBs (4 CCE) in every slot - 12 RE/PRB/slot - TRS burst of 2 slots with periodicity of 20ms and occupies 52 PRBs - 6 RE/PRB/slot - DMRS: 16 RE/PRB/slot in every slot and PRB - CSI-RS: 8 CSI-RS ports with 8 RE/PRB/slot with periodicity of 20ms in every PRB - 2 SS/PBCH blocks per 20ms; one SS/PBCH block occupies 960REs = 4 OFDM symbols × 20 PRB × 12 REs/PRB NOTE1: if the channel bandwidth is less than the bandwidth of SS/PBCH block, then SS/PBCH block is not transmitted and the overhead of SS/PBCH block is zero. NOTE2: If the channel bandwidth is less than TRS bandwidth, the TRS bandwidth is assumed to be equal to the channel bandwidth. |
|  | See Section 2.2. | See Section 5.3.2 of TS38.104 |

Table A-2 Parameter assumptions of NR TDD DL peak spectral efficiency 
for DL dominant frame structure (DDDSU)

| # Parameters | # Values | # Remarks |
| --- | --- | --- |
|  | FR1: 8 FR2: 6 | NR supports up to 8 layers for a single user for DL in FR1 and 6 layers in FR2 when PTRS is transmitted. |
|  | 0.7714 | This value corresponds to DL:UL=4:1, where 3 DL slots, 1 S slot mixing DL/UL symbols, and 1 UL slot are configured in every 5 slots; S slot includes 11 DL symbols , one symbol for GP, and two UL symbols. |
|  | For FR1: - 0.13 for 50MHz with 15kHz SCS - 0.13 for 100MHz with 30kHz SCS - 0.13 for 100MHz with 60kHz SCS For FR2: - 0.14 for 200MHz with 60kHz SCS - 0.14 for 400MHz with 120kHz SCS | GP overhead is included in DL slot. For FR1: - CORESET of 24 PRBs (4 CCE) in every slot - 12 RE/PRB/slot - TRS burst of 2 slots with periodicity of 20ms and occupies 52 PRBs - 6 RE/PRB/slot - DMRS: 16 RE/PRB/slot in every slot and PRB - CSI-RS: 8 CSI-RS ports with 8 RE/PRB/slot with periodicity of 20ms in every PRB - 2 SS/PBCH blocks per 20ms; one SS/PBCH block occupies 960REs = 4 OFDM symbols × 20 PRB × 12 REs/PRB NOTE1: if the channel bandwidth is less than the bandwidth of SS/PBCH block, then SS/PBCH block is not transmitted and the overhead of SS/PBCH block is zero. NOTE2: If the channel bandwidth is less than TRS bandwidth, the TRS bandwidth is assumed to be equal to the channel bandwidth. For FR2: - CORESET of 24 PPRBs (4 CCE) in every slot - 12 RE/PRB/slot - TRS burst of 2 slots with periodicity of 10ms and occupies 52 PRBs - 6 RE/PRB/slot - DMRS: 12 RE/PRB/slot in every slot and PRB - PTRS: 2 ports PTRS with 25% frequency domain density and every symbol time domain density - CSI-RS: 8 CSI-RS ports with 8 RE/PRB/slot with periodicity of 10ms in every PRB - CSI-RS for BM: 1 CSI-RS port with 2 RE/PRB/slot with periodicity of 10ms in every PRB - 8 SS/PBCH blocks per 20ms; one SS/PBCH block occupies 960REs = 4 OFDM symbols × 20 PRB × 12 REs/PRB |
| NOTE: Other parameters are the same as NR FDD DL (Table A-1). |  |  |

Table A-3 Parameter assumptions of NR FDD UL peak spectral efficiency

| # Parameters | # Values | # Remarks |
| --- | --- | --- |
|  | 4 | NR supports up to 4 layers for a single user for UL |
|  | 8 | NR supports up to 256QAM for UL |
|  | 1 | The value of 1 is chosen as scaling factor for UL peak spectral efficiency evaluation. |
| Rmax | 948/1024 = 0.9258 | NR supports highest coding rate as Rmax=948/1024. |
|  | 0, 1, 2, 3 | SCS, i.e., - 15kHz SCS for =0 - 30kHz SCS for  =1 - 60kHz SCS for  =2 - 120kHz SCS for  =3 |
|  | For FR1: - 270 for 50MHz with 15kHz SCS - 273 for 100MHz with 30kHz SCS - 135 for 100MHz with 60kHz SCS For FR2: - 264 for 200MHz with 60kHz SCS - 264 for 400MHz with 120kHz SCS | See Section 5.3.2 of TS38.104 |
|  |  | SCS, i.e., - 15kHz SCS for =0 - 30kHz SCS for  =1 - 60kHz SCS for  =2 - 120kHz SCS for  =3 |
|  | For FR1: - 0.08 for 50MHz with 15kHz SCS - 0.08 for 100MHz with 30kHz SCS - 0.07 for 100MHz with 60kHz SCS | For FR1: - PUCCH: short PUCCH with 1 PRB and 1 symbol in every UL slot - DM-RS: 12 RE/PRB/slot - SRS: 1 symbol per slot with periodicity of 10ms |
|  | See Section 2.3. | See Section 5.3.2 of TS38.104 |

Table A-4 Parameter assumptions of NR TDD UL peak spectral efficiency 
for DL dominant DL/UL configuration (“DDDSU”)

| # Parameters | # Values | # Remarks |
| --- | --- | --- |
|  | 0.2286 | This value corresponds to DL:UL=4:1, where 3 DL slots, 1 S slot mixing DL/UL symbols, and 1 UL slot are configured in every 5 slots; S slot includes 11 DL symbols , one symbol for GP, and two UL symbols. |
|  | For FR1: - 0.16 for 50MHz with 15kHz SCS - 0.14 for 100MHz with 30kHz SCS - 0.13 for 100MHz with 60kHz SCS For FR2: - 0.18 for 200MHz with 60kHz SCS - 0.17 for 400MHz with 120kHz SCS | GP overhead is included in DL. For FR1: - PUCCH: short PUCCH with 1 PRB and 1 symbol in every UL slot - DM-RS: 12 RE/PRB/slot - SRS: 1 symbols per slot with periodicity of 10 ms For FR2: - PUCCH: short PUCCH with 1 PRB and 1 symbol in every UL slot - DM-RS: 12 RE/PRB/slot - SRS: 1 symbols per slot with periodicity of 5ms - PTRS: 2 ports PTRS with 25% frequency domain density) and every symbol time domain density |
| NOTE: Other parameters are the same as NR FDD UL (Table A-3). |  |  |

- Appendix-B       Detailed evaluation assumptions for LTE

Evaluation parameters for LTE DL peak spectral efficiency and peak data rate is shown in Table B-1. The notations can be found in equation (2.1.1-1) in Section 2.1.1.

Table B-1 LTE Parameters for FDD DL peak spectral efficiency and peak data rate evaluation

| # Parameters | # Values | # Remarks |
| --- | --- | --- |
| Max. number of layers | 8 |  |
| Highest modulation order | 8 10 | 256QAM 1024QAM |
| Scaling factor of modulation | 1 |  |
| Max. coding rate Rmax | According to Transport block size (TBS) table defined in TS36.213 |  |
|  | See Table 8.1.2-2 for specific component carrier bandwidth. | The maximum number of RBs for the specific component carrier bandwidth is used. |

Table B-2 LTE Parameters for TDD DL peak spectral efficiency and peak data rate evaluation for DL dominant frame structure (“DSUDD”)

| # Parameters | # Values | # Remarks |
| --- | --- | --- |
| Max. number of layers | 8 |  |
|  | 0.7714 | S subframe includes 11 DL symbols, one symbol for GP, and 2 UL symbols. |
| Highest modulation order | 8 10 | 256QAM 1024QAM |
| Scaling factor of modulation | 1 |  |
| Max. coding rate Rmax | According to Transport block size (TBS) table defined in TS36.213 |  |
|  | See Table 8.1.2-2 for specific component carrier bandwidth. | The maximum number of RBs for the specific component carrier bandwidth is used. |

Evaluation parameters for LTE UL peak spectral efficiency and peak data rate is shown in Table B-3. The notations can be found in equation (2.1.1-1) in Section 2.1.1.

Table B-3 LTE Parameters for UL peak spectral efficiency and peak data rate evaluation

| # Parameters | # Values | # Remarks |
| --- | --- | --- |
| Max. number of layers | 4 |  |
| Highest modulation order | 8 | 256QAM |
| Scaling factor of modulation | 1 |  |
| Max. coding rate Rmax | According to Transport block size (TBS) table defined in TS36.213 |  |
|  | See Table 8.1.2-2 for specific component carrier bandwidth. | The maximum number of RBs for the specific component carrier bandwidth is used. |