3GPP TSG RAN WG1 Meeting #94	R1-1808115

Gothenburg, Sweden, Aug 20-24, 2018

Agenda Item:	6.2.1.7

Source:	Huawei, HiSilicon

Title:	Coverage enhancement for Non-BL UE

Document for:	Discussion and decision

# Introduction

In RAN #80 meeting, the Rel-16 WI on additional MTC enhancements was approved [1], which includes the following objective:

Extreme coverage for non-BL UEs: - Specify CE mode A and B improvements for non-BL UEs from among the following list[RAN1, RAN2, RAN4] - Enhancements to idle mode mobility - UE demodulation performance requirements for 2 RX antennas and full duplex FDD - Dual layer DL reception - Feedback based on CSI-RS - ETWS/CMAS in connected mode

In this contribution, we provide our preliminary considerations on coverage enhancement for non-BL UEs.

# Aspects on coverage enhancement for non-BL UEs

In the following, we give our preliminary considerations on the coverage enhancement aspects for non-BL UEs considered in the WID which are more RAN1 related.

## UE demodulation performance requirements for 2 RX antennas

For BL UEs, in order to achieve low cost, a single receive RF chain is assumed. The coverage mechanism is common for BL UEs in CE and non-BL UEs in CE. Therefore, UE demodulation performance requirement in coverage enhancement is based on a single receive RF chain.

However, compared to two receive RF chains supported by non-BL UE, about 4dB coverage loss can be envisaged for a single receive RF chain, which will imply more repetitions especially in extreme channel condition. Since non-BL UEs definitely support 2 RX antennas, it is beneficial to support UE demodulation performance requirements for 2 RX antennas. With 2 RX antenna, the number of repetitions for DL reception can be significantly reduced, and resource utilization can be improved.

Proposal 1: UE demodulation performance requirements for 2 RX antennas is supported for non-BL UEs in coverage enhancement.

## Dual layer DL reception

In current specification, only one layer can be supported of UE in coverage enhancement mode. However, there may be no repetition or a small number of repetitions for UEs in CEMode A. For non-BL UE, it stronger capability on signal reception and processing, and it also has high requirement on data rate. For example, voice connection prefers to be maintained even if non-BL UE channel condition is gradually deteriorating.

For example, if one non-BL UE is in the CEMode A and receives DL without repetition, the non-BL UE can still be served in CEMode A with dual layer reception. Thus, the mode switch from CEMode A to normal coverage is avoided, and traffic continuity can be maintained. Once the channel condition of this non-BL UE is getting worst, the eNB can indicate to the UE to receive the PDSCH with repetition and single layer. The mode switch from normal coverage to CEMode is also avoided.

As a result, supporting dual layer DL reception is beneficial for non-BL UEs in coverage enhancement.

Proposal 2: Dual layer DL reception is supported for non-BL UEs in coverage enhancement.

## Feedback based on CSI-RS

Currently, for BL/CE UE’s CSI reporting, the CSI measurement is based on the narrowband of MPDCCH transmission. As a result, the real-time CSI measurement on full bandwidth is not possible. Even if MPDCCH frequency hopping is considered, only two or four possible MPDCCH narrowband locations can be measured if the initial narrowband of MPDCCH configured by higher layer signaling is not changed. As a result, the BL/CE UE cannot estimate the channel condition on any subband within system bandwidth dynamically.

For non-BL UEs, since it has the capability to receive the CSI-RS within the whole system bandwidth, it is possible for non-BL UEs to measure subband on the whole system bandwidth based on CSI-RS, and report the optimum subband to eNB. The eNB can schedule the DL transmission on the optimum subband in time, and the throughput could be improved correspondingly.

## ETWS/CMAS in connected mode

In LTE, ETWS/CMAS are notified by the Paging message for both RRC_IDLE and RRC_CONNECTED modes. However, BL/CE UEs are not allowed monitoring CSS for Paging in connected mode. In that case, BL/CE UEs in connected mode may miss the ETWS/CMAS notification and reception. An implementation method is that the eNB releases the UEs to RRC_IDLE mode using RRCConnectionRelease. Considering the latency requirement of ETWS/CMAS and the actual latency of hundreds of RRCConnectionRelease repetitions and MPDCCH repetitions for ETWS/CMAS notification, it may not be very efficient using the implementation method. Therefore, it would be beneficial that ETWS/CMAS in RRC_CONNECTED mode can be supported for non-BL UEs in coverage enhancement. Currently, the ETWS/CMAS can be dynamically indicated by DCI format 6-2, which is only monitored by idle UEs. Similarly, it can consider using DCI to indicate ETWS/CMAS for connected non-BL UEs.

Proposal 3: ETWS/CMAS reception for connected non-BL UEs in coverage enhancement can be indicated by DCI.

# Conclusions

In this contribution, preliminary considerations on coverage enhancement for non-BL UEs are given, and following proposals are provided:

Proposal 1: UE demodulation performance requirements for 2 RX antennas is supported for non-BL UEs in coverage enhancement.

Proposal 2: Dual layer DL reception is supported for non-BL UEs in coverage enhancement.

Proposal 3: ETWS/CMAS reception for connected non-BL UEs in coverage enhancement can be indicated by DCI.

# References

- RAN #80 meeting, La Jolla, USA, June 11-14, 2018