3GPP TSG RAN WG1 Meeting #94                                                             R1-1808154

Gothenburg, Sweden, August 20th - 24th, 2018

# Source: 	ZTE

# Title:	Preliminary system-level simulation results for NOMA

# Agenda Item:	7.2.1.4

# Document for:	Discussion/Decision

# Introduction

In RAN1 #92bis meeting and RAN1 #93 meeting, most of the aspects on performance metrics, traffic models and simulation parameters for system-level evaluations for NOMA were determined, and the agreement on system-level assumptions for calibration purpose were also achieved [1][2].

In this contribution, system level calibration results are provided, and preliminary system level evaluation results for NOMA are demonstrated and discussed.

# SLS calibration

The CDF of coupling loss and downlink geometry are shown in Figure 1 and Figure 2 respectively, which are derived based on the system-level assumptions for calibration in Table A1 in the Appendix, and some additional assumptions described in the following.

- Minimum distance between UE and gNB: 10m. For Case 1, 35m can be also used, the differences are minor.

- Building penetration loss: The building penetration model defined in Table 7.4.3-3 in TR 38.901 is used for frequencies below 6 GHz, which is in alignment with the agreement for NOMA evaluation [2].

- UE noise figure: Since below 6 GHz carrier frequency is assumed for all cases, the UE noise figure can be set to 9dB according to Table A.2.1-1 in TR 38.802.

- Handover margin: 0dB, which is usually used for calibration and evaluation.

Proposal 1: Consider the following additional assumptions for system level calibration:

- Minimum distance between UE and gNB: 10m.

- Building penetration loss: The building penetration model defined in Table 7.4.3-3 in TR 38.901 is used for frequencies below 6 GHz.

- UE noise figure: 9dB;

- Handover margin: 0dB;

Proposal 2: Capture the calibration results on coupling loss and downlink geometry into the TR 38.812, which are shown in Figure 1 and 2, respectively.

Figure 1 CDF of coupling loss			Figure 2 CDF of downlink geometry

# Preliminary SLS results

In the preliminary evaluation, the performance comparison between baseline scheme and NOMA in mMTC, URLLC and eMBB scenarios are conducted with corresponding assumptions shown in Table A2 in the Appendix. It should be noted that some parameters are not determined in existing agreement, which should be reported by companies, e.g. number of UEs per cell, power control, and HARQ/repetition, etc. The details of them are given in Table 1. In addition, some details of the baseline scheme and NOMA scheme can be also found in Table 1 and below.

As for the packet size for NOMA evaluations in eMBB scenario, it is still FFS. From our view [3], the packet size of 40~600 bytes Pareto distribution with shaping parameter alpha = 1.5 is basically in alignment with the packet size distributions for heavier background traffic and IM traffic that have been studied in “LTE_eDDA” [4], so we propose to confirm this packet size distribution for eMBB scenario.

Table 1 Assumptions reported for NOMA evaluation

| Parameters | mMTC | URLLC | eMBB |
| --- | --- | --- | --- |
| ISD + Carrier Frequency | 1732m + 700MHz for configured grant 500m + 700MHz for grant-free with random selection | 200m + 4GHz | 200m + 4GHz |
| Simulation bandwidth | 6 PRBs | 12 PRBs | 12 PRBs |
| Resource allocation | Baseline: 1 PRB per UE NOMA: 1 PRBs +4ms per UE, spreading codes of length 4 are used [5] | Baseline: 3 PRB per UE NOMA: 12 PRBs per UE, spreading codes of length 4 are used [5] | Baseline: 3 PRB per UE NOMA: 12 PRBs per UE, spreading codes of length 4 are used [5] |
| Packet size | 20~200 bytes Pareto distribution with shaping parameter alpha = 2.5, and 29 bytes protocol overhead | 60 bytes | 40~600 bytes Pareto distribution with shaping parameter alpha = 1.5, no protocol overhead |
| Segmentation | Yes | No | Yes |
| HARQ/repetition | For configured grant: Case 1: HARQ transmission(s) Case 2: Repetition Details can be found below For grant-free with random selection, no HARQ/repetition | Number of transmission(s) = 1 No HARQ/repetition | Number of transmission(s) = 1 No HARQ/repetition |
| ARQ retransmission | Not modelled, if one TB is not received correctly after HARQ/ repetition, the packet is dropped | Not modelled | Not modelled, if one TB is not received correctly after HARQ transmission, the packet is dropped |
| Number of UEs per cell | >=100 | 20 | >=100 |
| UE power control | Open loop power control, P0 = -100 dBm, alpha = 1 | Open loop power control, P0 = -90 dBm, alpha = 1 | Open loop power control, P0 = -95 dBm, alpha = 1 |
| Number of BS antennas | 2Rx | 4Rx | 4Rx |
| BS antenna downtilt | 92 | 102 | 102 |
| Channel estimation | Realistic | Realistic | Ideal, or realistic |
| BS receiver | Baseline: MMSE-IRC NOMA: MMSE-PIC | Baseline: MMSE-IRC or MMSE-PIC (max times of decoding for a UE = 2) NOMA: same as baseline | Baseline: MMSE-IRC NOMA: MMSE-PIC |

## mMTC (configured grant)

In mMTC configured grant scenario, there is no DMRS collision, and the following case is simulated:

- Case 1: Maximum number of HARQ transmission(s) = 8, the energy of spreading codes for NOMA is normalized to 1 for fair comparison.

Figure 3(a) shows the packet drop rate (PDR) performance of baseline and NOMA for Case 1. As shown in the figure, obvious performance gain can be observed for NOMA relative to the baseline. Distribution of maximal number of UEs per transmission resource and resource utilization for Case 1 are shown in Figure 3(b) and 3(c) respectively. It can be observed that the maximum number of UEs per transmission resource and resource utilization are increasing with the traffic load, and these metrics for NOMA are relatively higher than baseline because that multiple basic resources would be used after spreading and more users would use the same transmission resource. It should be noted that multiple times of HARQ transmissions also have impact on these metrics.

| Figure 3(a) Packet drop rate for Case 1 | Figure 3(b) Max number of UEs per Tx resource for Case 1 | Figure 3(c) Resource utilization for Case 1 |
| --- | --- | --- |

## mMTC (grant free with random selection)

In this section, we demonstrate some preliminary simulation results for grant-free with random selection in mMTC scenario, where transmission resource and DMRS are randomly selected by each UE, and DMRS collision exists.

Due to no HARQ transmission and combination is conducted for grant-free with random selection, and to avoid introducing repetitions, which can ensure the performance but would increase the simulation complexity, 500m inter-site distance is used in the simulation.

As for the baseline scheme, 24 orthogonal ports for the enhanced DMRS configuration is assumed without spreading, while for NOMA scheme, 64 orthogonal ports via configuration is assumed, and symbol spreading with spreading code of length 4 is used. The number of available spreading code is 64 which are one-to-one mapping with the 64 DMRS. The PHY abstraction of the receiver can be found in our companion contribution [6].

Figure 5(a) shows the PDR performance of baseline and NOMA. We can observe that grant-free NOMA with random selection has obvious gain. Maximum number of UEs per transmission resource and resource utilization are also shown in Figure 5(b) and 5(c) respectively. Similar to above, the maximum number of UEs per transmission resource and resource utilization are increasing with the traffic load, and these metrics for NOMA are relatively higher than baseline because that multiple basic resources would be used after spreading and more users would use the same transmission resource.

It should be noted that the user identification procedure may be a little ideal in the current simulation, which would be further studied in the future. And we think that grant-free transmission with random selection is worthy of research in NOMA study.

| Figure 5(a) Packet drop rate | Figure 5(b) Max number of UEs per Tx resource | Figure 5(c) Resource utilization |
| --- | --- | --- |

## URLLC

For URLLC, configured grant scenario is simulated and there is no DMRS collision. The transmission resource including time/frequency resource and DMRS are preconfigured. And for NOMA scheme, the energy of spreading codes is normalized to 1 for fair comparison.

Figure 6(a) shows the percentage of users satisfying reliability and latency requirements, from which we can observe that NOMA has significant performance gain compared to baseline, even with MMSE-IRC receiver. With MMSE-PIC receiver, the packet error rate can be further reduced, and the percentage of users satisfying requirements is increased for baseline, however, for NOMA scheme, due to the packet error rate is very low with MMSE-IRC receiver, the performance improvement on the percentage of users can not be obviously observed.

Maximum number of UEs per transmission resource and resource utilization are shown in Figure 6(b) and 6(c) respectively. Similar to mMTC scenario, the maximum number of UEs per transmission resource and resource utilization are increasing with the traffic load, and these metrics for NOMA are relatively higher than baseline because that multiple basic resources would be used after spreading and more users would use the same transmission resource.

| Figure 6(a) Packet drop rate | Figure 6(b) Max number of UEs per Tx resource | Figure 6(c) Resource utilization |
| --- | --- | --- |

## eMBB

For eMBB, the scenario of configured grant with no DMRS collision is also firstly simulated. For fair comparison, the energy of spreading codes for NOMA is normalized to 1.

Figure 7(a) shows the PDR performance of baseline and NOMA with ideal channel estimation (ICE) and realistic channel estimation (RCE). We can observe obvious performance gain for NOMA relative to baseline from this figure, with both ICE and RCE.

Maximum number of UEs per transmission resource and resource utilization are also shown in Figure 7(b) and 7(c) respectively. Similarly, the maximum number of UEs per transmission resource and resource utilization are increasing with the traffic load, and these metrics for NOMA are relatively higher than baseline because that multiple basic resources would be used after spreading and more users would use the same transmission resource.

| Figure 7(a) Packet drop rate | Figure 7(b) Max number of UEs per Tx resource | Figure 7(c) Resource utilization |
| --- | --- | --- |

Observation 1: Spreading based NOMA has obvious performance gain in mMTC, URLLC and eMBB scenarios.

Observation 2: The number of UEs per transmission resource and resource utilization are increasing with the traffic load, and multiple times of HARQ transmissions or repetitions have impact on these metrics.

Observation 3: The number of UEs per transmission resource and resource utilization for NOMA are relatively higher than that of baseline.

Proposal 3: Confirm the packet size for NOMA evaluations in eMBB scenario: 40~600 bytes Pareto distribution, with shaping parameter alpha = 1.5.

Proposal 4: Grant-free transmission with random selection of MA signatures should be studied in NOMA SI to demonstrate the NOMA gain.

# Conclusions

In this contribution, system level calibration results are provided, and preliminary system level evaluation results for NOMA are demonstrated and discussed. It should be noted that more cases with scheme or parameter adjustments should be studied and evaluated to fully understand the performance of NOMA, which would be done in the following meetings.

Based on this contribution, we make the following observations and proposals:

Observation 1: Spreading based NOMA has obvious performance gain in mMTC, URLLC and eMBB scenarios.

Observation 2: The number of UEs per transmission resource and resource utilization are increasing with the traffic load, and multiple times of HARQ transmissions or repetitions have impact on these metrics.

Observation 3: The number of UEs per transmission resource and resource utilization for NOMA are relatively higher than that of baseline.

Proposal 1: Consider the following additional assumptions for system level calibration:

- Minimum distance between UE and gNB: 10m.

- Building penetration loss: The building penetration model defined in Table 7.4.3-3 in TR 38.901 is used for frequencies below 6 GHz.

- UE noise figure: 9dB;

- Handover margin: 0dB;

Proposal 2: Capture the calibration results on coupling loss and downlink geometry into the TR 38.812, which are shown in Figure 1 and 2, respectively.

Proposal 3: Confirm the packet size for NOMA evaluations in eMBB scenario: 40~600 bytes Pareto distribution, with shaping parameter alpha = 1.5.

Proposal 4: Grant-free transmission with random selection of MA signatures should be studied in NOMA SI to demonstrate the NOMA gain.

# References

- 3GPP, Draft Report of 3GPP TSG RAN WG1 #92bis v0.1.0

- 3GPP, Draft minutes report RAN1#93 v020

- 3GPP, R1-1805843, Remaining evaluation methodology and assumptions for NOMA, ZTE, RAN1 #93

- 3GPP TR 36.822 V11.0.0, LTE Radio Access Network (RAN) enhancements for diverse data applications (eDDA), Release 11.

- 3GPP, R1-1808155, Link-level simulation results for mMTC scenario, RAN1#94

- 3GPP, R1-1808158, Physical layer abstraction for NOMA SLS, RAN1#94

# Appendix 1 System-level assumptions for calibration

Table A1 System-level assumptions for calibration purpose

| Parameters | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |  |  |
| Inter-BS distance | 1732m | 500m | 200m |
| Carrier frequency | 700MHz | 700MHz | 4GHz |
| Channel model | UMa in TR 38.901 |  |  |
| UE Tx power | Max 23 dBm |  |  |
| BS Tx power | Max 46 dBm |  |  |
| BS antenna configurations | 2 ports: (M, N, P, Mg, Ng) = (10, 1, 2, 1, 1), +-45 Polarization; dH = dV = 0.8λ; |  |  |
| BS antenna downtilt | 92 | 98 | 102 |
| BS antenna height | 25m |  |  |
| BS antenna element gain + connector loss | 8 dBi, including 3dB cable loss |  |  |
| BS receiver noise figure | 5dB |  |  |
| UE antenna configuration | 1 |  |  |
| UE antenna height | Follow the modelling of TR 38.901 |  |  |
| UE antenna gain | 0dBi |  |  |
| UE distribution | Follow the evaluation assumptions |  |  |
| UE power control | Open loop PC, P0 = [-90] dBm, alpha = 1. |  |  |
| HARQ/repetition | 1 |  |  |
| UE attachment | Refer to 36.873 |  |  |

# Appendix 2 System-level assumptions for NOMA

Table A2 System-level assumptions for NOMA evaluation

| Parameters | mMTC | URLLC | eMBB |
| --- | --- | --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |  |  |
| Inter-BS distance | 1732m | 200m for 4GHz 500m for 700MHz | 200m |
| Carrier frequency | 700MHz | 4GHz or 700MHz | 4GHz |
| Simulation bandwidth | 6 PRBs as starting point | 12 PRBs | 12 PRBs |
| Number of UEs per cell | Companies report |  |  |
| Channel model | UMa in TR 38.901; The building penetration model defined in Table 7.4.3-3 in TR 38.901 is used for SLS with frequencies below 6 GHz. |  |  |
| UE Tx power | Max 23 dBm |  |  |
| BS antenna configurations | 2 Rx or 4 Rx for 700MHz; 2 ports: (M, N, P, Mg, Ng) = (10, 1, 2, 1, 1), 2 TXRU; 4 ports: (M, N, P, Mg, Ng) = (10, 2, 2, 1, 1), 4 TXRU; dH = dV = 0.5λ; BS antenna downtilt: companies to report, FFS a single value 4 Rx or 16 Rx for 4GHz; 4 ports: (M, N, P, Mg, Ng) = (10, 2, 2, 1, 1), 4 TXRU; 16 ports: (M, N, P, Mg, Ng) = (10, 8, 2, 1, 1), 16 TXRU; dH = 0.5λ, dV = 0.8λ; BS antenna downtilt: companies to report, FFS a single value |  |  |
| BS antenna height | 25m |  |  |
| BS antenna element gain + connector loss | 8 dBi, including 3dB cable loss |  |  |
| BS receiver noise figure | 5dB |  |  |
| UE antenna configuration | 1Tx as starting point |  |  |
| UE antenna height | Follow the modelling of TR 38.901 |  |  |
| UE antenna gain | 0dBi as starting point |  |  |
| UE distribution | For mMTC: 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); Users dropped uniformly in entire cell For URLLC: 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); Users dropped uniformly in entire cell; Note: Other option(s) not precluded, e.g., 500m ISD, 80% of users are outdoors (3km/h), 20% of users are indoor (3km/h). For eMBB: 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); Users dropped uniformly in entire cell |  |  |
| UE power control | Open loop PC for mMTC. Companies report the PC mechanisms used for eMBB and URLLC. |  |  |
| HARQ/repetition | Companies report (including HARQ mechanisms). |  |  |
| Channel estimation | Realistic |  |  |
| BS receiver | Advanced receiver, with baseline scheme is MU-MIMO (e.g., has the capability of spatial differentiation) Companies to provide analysis of complexity between baseline vs. advanced receivers |  |  |