3GPP TSG RAN WG1 Meeting #94	R1-1808302

Gothenburg, Sweden, August 20th - 24th, 2018

Agenda Item:	7.2.6.2

Source: 	Fujitsu

Title: 	Discussion on uplink pre-emption indication

Document for:	Discussion/Decision

Introduction

Uplink (UL) multiplexing of transmissions with different reliability requirement was discussed in Rel-15 but no consensus was reached, due to time limitation. The following agreements have been achieved in previous meetings:

Agreements in RAN1 #92: - Study the options to support dynamic resource sharing between eMBB UL and URLLC UL from different UEs (comparing with existing techniques) - Option 1: eMBB UE cancels UL transmission when an indication is detected. Details to be discussed/clarified - UE processing timeline for cancelation - UE monitoring periodicity - Group common or UE specific signalling (including the possibility to use eMBB scheduling DCI) - reliability of indication - Any impact due to timing advance - Option 2: UL power control. URLLC UE transmits over the same resource with eMBB UE transmission. The transmission power for URLLC UL is boosted and/or transmission power for eMBB UL is reduced. Details need to be discussed/clarified - Performance impact to eMBB/URLLC transmission - How to signal the URLLC transmission power boosting - How to signal the eMBB transmission power reduction after UL grant - UE monitoring periodicity - Processing timeline - Feasibility of changing eMBB Tx power during the transmission - reliability of indication - Any impact due to timing advance - Other options including gNB receiver interference cancelation schemes are not precluded - Aspects to be included in the study - Processing timeline for grant-based procedure for URLLC in UL - Applicability of the options to TDD and/or FDD can be studied - Cases for GB-based & GF-based Conclusion in RAN1 #92b: • There is no consensus in Rel-15 to support handling inter-UE UL multiplexing of transmission with different reliability requirements (including the potential need for UL UE preemption)

In the RAN plenary #80 meeting, it was agreed that Rel-16 will continue to study enhanced multiplexing considering different latency and reliability requirements, especially the UL inter UE Tx prioritization/ multiplexing [1]. In this contribution, we will discuss the design of inter-UE preemption indication for UL based on that for DL, to have minimal spec impact.

Discussion

Considering the 10-5 reliability requirement for URLLC, we support the option 1 introduced in section 1 to cancel the corresponding transmission of eMBB UE when a UL preemption indication (PI) is detected. Fig. 1 illustrates the main concept of the UL preemption indication, and the main procedures for the UL preemption indication are stated as follows:

- When a UE has UL URLLC data, it sends a URLLC scheduling request (SR) to the gNB by a dedicated channel.

- The gNB generates the UL preemption indicator signal which is sent to the eMBB UE using a DL control channel.

- The UL preemption indicator signal includes the information of the preempted UL resource. This signal enables eMBB UEs to timely stop any ongoing transmission scheduled in the preempted UL resource.

- The related eMBB UEs will not transmit their UL data in the preempted resource if the UL pre-emption indicator is successfully decoded.

Fig.1 illustration of UL resource preemption.

UE monitoring periodicity

To avoid interference to URLLC data, the eMBB UE needs to cancel the corresponding UL transmission before the URLLC UE starts to transmit its UL data. In view of the 1ms latency requirement of the sporadic URLLC traffic, the slot-level monitoring for the UL preemption indicator may not be feasible to enable eMBB UE stop its transmission in a timely way, especially when the subcarrier spacing is small. Thus, to avoid collision of URLLC traffic and eMBB traffic, the non-slot level monitoring of potential UL PI would be required for eMBB UE. This is feasible [2]. Non-slot level monitoring of PDCCH has already been supported by high-layer parameter monitoringSymbolsWithinSlot.

Proposal 1. The non-slot level monitoring of the UL preemption indication would be required for eMBB UEs.

UE processing timeline for cancellation

Fig.2 illustrates the processing timeline of eMBB UE and URLLC UE when the uplink preemption occurs. After receiving the SR from the URLLC UE, gNB decodes the SR and prepares the UL grant for the URLLC UE. Then, gNB generates the UL PI for the eMBB UE. The time of the UL PI transmission is earlier or the same as that of the UL grant transmission. To satisfy the low latency requirement of the URLLC, the slot offset  between the UL grant and the scheduled PUSCH of the URLLC UE should be smaller than that of the eMBB UE , i.e., <. Due to the impact of the propagation time Tp of UL PI and timing advance (TA) of the canceled PUSCH, the time of processing UL grant and preparing for  PUSCH  is smaller than, i.e., < . To avoid the collision between the URLLC PUSCH and the eMBB PUSCH, the time of eMBB UE processing UL PI and preparing for PUSCH cancellation should be smaller than or equal to the time  of the URLLC UE processing UL grant and preparing for the PUSCH transmission, i.e.,≤.

In summary, the relationship among the above parameters are≤< < .

Fig.2 The processing timeline of eMBB UE and URLLC UE when uplink preemption occurs.

Observation 1. The time  of the eMBB UE processing the UL PI and preparing for the PUSCH cancellation should be smaller than or equal to the time  of the URLLC UE processing UL grant and preparing for the PUSCH transmission, i.e., ≤.

Group-common signaling

To achieve low latency, the URLLC traffic usually occupies a very large frequency band, which may affect multiple eMBB UEs. Thus, to reduce the signaling overhead of the UL PI, a group-common signaling is recommended for transmitting the UL PI.

Proposal 2. A group-common signaling is recommended for UL PI to reduce the signaling overhead.

Reliability of UL PI

In comparison with the DL PI, the UL PI requires much higher reliability. Because the DL PI is used to indicate the eMBB UE the already canceled PDSCH from gNB, while the UL PI is designed to indicate the eMBB UE to cancel the ongoing or impending PUSCH before the URLLC UE starts to transmit its PUSCH. Therefore, to avoid collision with the eMBB UE and achieve the 10-5 overall reliability of the URLLC, the reliability of the UL PI should also be in the order of 10-5 or even higher. The following two kinds of methods can be considered to improve the reliability of the UL PI.

- More time-frequency resource

Allocating more time-frequency resource for the UL PI is a direct method to improve its reliability. For example, it can adopt a higher aggregation level for the PDCCH carried the UL PI. Similarly, the frequency repetition or time repetition can be utilized to improve the reliability. Nevertheless, allocating more rime-frequency resource may cause a high blocking probability of PDCCH that sharing the same search space with the PDCCH carried the UL PI.

- Smaller payload size

Another method is to reduce the payload size of UL PI, for example, to utilize a coarser granularity to indicate the preempted time-frequency resource. As an extreme instance, only 1 bit is used to indicate the whole BWP in one slot, even if URLLC UE only occupies a part of BWP in a few symbols. Specifically, after decoding the UL PI, eMBB UE will stop the PUSCH transmission in the whole BWP in the rest symbols of that slot if it has started the PUSCH transmission in that slot. On the other hand, if eMBB UE has not started to transmit its PUSCH, the eMBB UE will cancel the scheduled PUSCH transmission.

Proposal 3. Higher aggregation level, PDCCH repetition, and coarser granularity can be considered to enhance the reliability of the UL PI.

Conclusions

In this contribution, the following observations and proposals are summarized:

Observation 1. The time  of the eMBB UE processing the UL PI and preparing for the PUSCH cancellation should be smaller than or equal to the time  of the URLLC UE processing UL grant and preparing for the PUSCH transmission, i.e., ≤.

Proposal 1. The non-slot level monitoring of the UL preemption indication would be required for eMBB UEs.

Proposal 2. A group-common signaling is recommended for UL PI to reduce the signaling overhead.

Proposal 3. Higher aggregation level, PDCCH repetition, and coarser granularity can be considered to enhance the reliability of the UL PI.

- References

[1] RP-181477, SID on Physical Layer Enhancements for NR URLLC.

[2] 3GPP TS 38.213, NR; Physical layer procedures for control.