3GPP TSG RAN WG1 Meeting #94                                                               R1-1808317

Gothenburg, Sweden, August 20th – 24th, 2018

Source:            ZTE

Title:                 Simulation Methodology and Preliminary Results for NR-U

Agenda Item:   7.2.2.1

Document for: Discussion and Decision

# Introduction

Some conclusions were reached at RAN1#92bis and #93, which includes the following agreements [1][2]:

Agreement:

- For sub7 GHz outdoor scenario, adopting the following

  - Macro deployment with ISD=200×A meters

  - Each operator randomly drops 1 micro-layer TRP within each macro cell sector with minimum distance between micro-layer TRPs equals 57.9×A meters

  - Independent dropping between two operators

    - Use 10 meters as the inter-operator micro-layer TRP minimum distance

    - For the inter-operator micro-layer TRP maximum distance

      - Outdoor scenario 1: 30

      - Outdoor scenario 2: No limit as long as the TRP is within the macro cell

  - UE randomly dropped within macro cell sector with a minimum serving cell RSSI of -82dBm

  - All UEs dropped outdoor

  - Try A>=1 and find the A that satisfies serving cell received power distribution satisfies (10+X)% to (15+X)%] UEs below -72dBm

  - Other parameters follow the table below

In RAN#80, the revised NR-U SID [3] was approved and the parts of high frequency (i.e. above 7GHz) were removed. The following revisions are related to system simulation.

  - Coexistence methods within NR-based and between NR-based operation in unlicensed and LTE-based LAA and with other incumbent RATs in accordance with regulatory requirements in e.g., 5GHz , 6G bands

    - Coexistence methods already defined for 5GHz band in LTE-based LAA context should be assumed as the baseline for 5GHz operation. Enhancements in 5GHz over these methods should not be precluded. NR-based operation in unlicensed spectrum should not impact deployed Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier;

In this contribution, we give further considerations on simulation scenarios for NR-U operation. Some preliminary simulation results are also provided.

# Further Considerations on Scenarios and Methodology

- Scenarios

In order to better align the simulation results from different companies, more details should be further discussed and decided.

From the co-existence interference perspective, in addition to the co-channel interference, the adjacent channel interference also needs to be considered, especially for the cross-link interference of the inter-operator scenarios. How to model the adjacent channel scenarios needs to be studied carefully.

For component carrier (CC)/bandwidth part (BWP) number, according to the agreements reached in last RAN1 meeting, the configuration of the single CC/BWP can be as a baseline at least for sub-7GHz. However, we expect that most if not all eMBB NR-UEs which are used for NR-U will have capability supporting carrier aggregation. Therefore, multiple CCs/BWPs should also be a candidate configuration in addition to single CC/BWP. To simplify the simulation, two CCs/BWPs can be taken as a basic configuration and more CCs/BWPs are not precluded.

Regarding co-existence scenarios, we think the following aspects should be considered.

- For 5 GHz, extensive work has been done for the co-existence between LTE-LAA and WiFi. Most conclusions are expected to be similar for the co-existence between NR-U and WiFi.

- Regarding 6 GHz, the regulation is still in discussion and no WiFi network are deployed. So, there is no strong motivation to evaluate the scenarios of coexistence between NR-U cells and WiFi APs.

In addition, considering that LTE-LAA network is not yet deployed on a large scale so far, those co-existence scenarios involving LTE-LAA can be of low priority as well.

Therefore, we think a prioritization on the co-existence scenarios of NR-U to NR-U is necessary for this NR-U SI.

Proposal 1: When performing a coexistence simulation, besides the co-channel interference, the adjacent channel interference also needs to be considered.

Proposal 2: Multiple CCs/BWPs should also be a candidate configuration in addition to single CC/BWP. Two CCs/BWPs can be taken as a basic configuration.

Proposal 3: A prioritization on the co-existence scenarios of NR-U to NR-U is necessary.

- Bandwidth

On bandwidth size per CC, the following conclusions have been agreed [1].

- Baseline for study: If absence of Wi-Fi cannot be guaranteed (e.g. by regulation) in the band (sub-7 GHz) where NR-U is operating, the NR-U operating bandwidth is an integer multiple of 20MHz

- At least for band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), LBT can be performed in units of 20 MHz.

  - FFS: details on how to perform LBT for as single carrier with bandwidth greater than 20 MHz, i.e., integer multiples of 20 MHz.

So for sub-7GHz band, the value of 20 MHz still can be baseline even wider bandwidth is support in NR.

Moreover, as we know, the BWP aspect was introduced in NR. A reasonable assumption is that LBT should be executed with a BWP granularity where the bandwidth of BWP is integer multiples of 20 MHz, which means that NR-U can occupy one BWP independently rather than have to occupy a whole wider bandwidth carrier with multiple BWPs.

Propose 4:  LBT should be performed with a BWP granularity for as single carrier with bandwidth greater than 20 MHz, i.e., integer multiples of 20 MHz.

- Metrics

The evaluation assume that NR-U is mainly used for eMBB and the evaluation need to identify at least the following.

    - The benefit that NR-U provided;

    - The performance impact to other coexistence system.

The base performance metrics include distributional statistics of the user perceived throughput (UPT) in low, medium and high traffic loads. The base metrics also include resource utilization (RU) ratio and the ratio of traffic packet offered/served.

Regarding User Perceived Throughput (UPT), the definition introduced in LTE-LAA [4] should be reused, which means that the unfinished files should be incorporated in the UPT calculation.

To measure the performance of LBT, the performance metrics of latency and average buffer occupancy (BO) which have be used for LTE-LAA should also be included. The details of BO definition can refer to LAA SID TR [4]. In addition, statistics of the success ratio of LBT can also be considered.

Proposal 5: The metrics for NR-U evaluation should include:

    - Base metrics: UPT in low, medium and high traffic loads, RU ratio and the ratio of traffic packet offered/served.

    - Specific metrics for NR-U: BO, latency and the success ratio of LBT.

    - Unfinished files should be incorporated in the UPT calculation.

# Preliminary Simulation Results for sub7 GHz NR-U Indoor Scenario

In this section, we will provide some preliminary evaluation results. The detailed simulation assumptions are given in Appendix A.

The following are the simulation results for the co-existence scenarios of NR-U to NR-U, including the cases of DL-only and DL+UL.

Table 1   DL-only performance for sub-7GHz indoor scenario

|  |  | (OpA) | (OpB) | (OpA) | (OpB) | (OpA) | (OpB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DL UPT[Mbps] | mean | 55.7509 | 78.7147 | 44.2571 | 32.6266 | 24.9352 | 33.8568 |
|  | 5% | 12.2241 | 13.6174 | 2.1142 | 4.8954 | 2.1958 | 2.0964 |
|  | 50% | 31.1695 | 20.8904 | 9.5958 | 16.9743 | 7.2753 | 14.7276 |
|  | 95% | 85.0891 | 147.2310 | 102.3169 | 86.4760 | 42.4925 | 51.0083 |
| Latency[s] | mean | 0.140 | 0.131 | 0.790 | 0.516 | 1.068 | 0.863 |
|  | 5% | 0.019 | 0.019 | 0.019 | 0.025 | 0.023 | 0.020 |
|  | 50% | 0.081 | 0.074 | 0.215 | 0.246 | 0.398 | 0.381 |
|  | 95% | 0.356 | 0.324 | 4.759 | 1.876 | 4.735 | 3.802 |
| RU |  | 6.020 | 5.572 | 45.051 | 41.703 | 54.921 | 66.003 |
| BO |  | 6.240 | 5.884 | 50.776 | 49.234 | 74.623 | 84.316 |
| Lambda |  | 0.1files/s |  | 0.4 files/s |  | 0.5 files/s |  |

Table 2   DL+UL performance for indoor sub-7GHz scenario

|  |  | (OpA) | (OpB) | (OpA) | (OpB) | (OpA) | (OpB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DL UPT[Mbps] | mean | 46.4266 | 31.4709 | 41.5710 | 37.8691 | 31.4682 | 26.5346 |
|  | 5% | 1.9464 | 3.5684 | 1.5680 | 1.7528 | 0.6745 | 0.8541 |
|  | 50% | 20.1597 | 23.9521 | 6.7811 | 5.8394 | 6.7145 | 7.9889 |
|  | 95% | 169.5652 | 49.0922 | 82.4830 | 89.0629 | 99.4950 | 76.2040 |
| DL Latency[s] | mean | 0.583 | 0.509 | 0.539 | 1.142 | 1.333 | 1.747 |
|  | 5% | 0.023 | 0.038 | 0.023 | 0.035 | 0.024 | 0.048 |
|  | 50% | 0.207 | 0.201 | 0.141 | 0.285 | 0.523 | 0.586 |
|  | 95% | 2.396 | 1.577 | 2.078 | 3.976 | 6.407 | 6.911 |
| DL RU |  | 12.054 | 11.766 | 19.675 | 26.771 | 45.778 | 48.846 |
| DL BO |  | 13.323 | 13.070 | 23.601 | 31.416 | 53.375 | 57.412 |
| UL UPT[Mbps] | mean | 10.6049 | 9.4471 | 9.7204 | 9.7826 | 9.1334 | 8.8290 |
|  | 5% | 0.7285 | 1.4514 | 0.7757 | 1.8207 | 0.6378 | 1.4864 |
|  | 50% | 4.3441 | 4.1623 | 4.1776 | 6.8439 | 4.0758 | 3.4718 |
|  | 95% | 24.5820 | 20.3904 | 21.2747 | 21.0135 | 21.1493 | 20.6186 |
| UL Latency[s] | mean | 1.759 | 0.913 | 1.356 | 1.057 | 1.623 | 1.098 |
|  | 5% | 0.136 | 0.148 | 0.144 | 0.149 | 0.145 | 0.156 |
|  | 50% | 0.899 | 0.511 | 0.849 | 0.629 | 0.723 | 0.576 |
|  | 95% | 5.847 | 2.067 | 5.006 | 2.601 | 5.752 | 2.541 |
| UL RU |  | 5.547 | 4.709 | 8.398 | 7.605 | 12.476 | 11.358 |
| UL BO |  | 7.151 | 3.690 | 8.213 | 6.360 | 14.483 | 9.457 |
| Lambda |  | 0.05files/s |  | 0.07files/s |  | 0.1 files/s |  |

Observation 1: The two operators deploying NR-U can coexist well if LBT mechanism is adopted.

The scheme of using higher CCA threshold within LAA system for increasing frequency reuse factor was introduced in LTE-LAA. This scheme can also be reused in NR-U. The following is the simulation results using different thresholds.

Table 3   DL-only performance for various CCA thresholds

| CCA threshold |  | -52dbm |  | -62dbm |  | -72dbm |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (OpA) | (OpB) | (OpA) | (OpB) | (OpA) | (OpB) |
| DL UPT[Mbps] | mean | 45.0588 | 40.0317 | 44.2571 | 32.6266 | 36.3534 | 31.7035 |
|  | 5% | 1.7926 | 4.9162 | 2.1142 | 4.8954 | 7.9267 | 4.7826 |
|  | 50% | 20.3526 | 14.9856 | 9.5958 | 16.9743 | 22.6806 | 14.1580 |
|  | 95% | 102.1643 | 98.9896 | 102.3169 | 86.4760 | 78.5832 | 51.5402 |
| Latency[s] | mean | 0.926 | 0.348 | 0.790 | 0.516 | 1.140 | 0.471 |
|  | 5% | 0.019 | 0.019 | 0.019 | 0.025 | 0.033 | 0.019 |
|  | 50% | 0.205 | 0.198 | 0.215 | 0.246 | 0.238 | 0.247 |
|  | 95% | 4.937 | 1.129 | 4.759 | 1.876 | 5.083 | 2.034 |
| RU |  | 37.457 | 35.655 | 45.051 | 41.703 | 33.381 | 33.682 |
| BO |  | 38.597 | 39.437 | 50.776 | 49.234 | 53.663 | 56.147 |
| Lambda |  | 0.4 files/s |  |  |  |  |  |

The above simulation results show that the performances increase along with the increase of CCA threshold and the two operators can still coexist very well.

Observation2: The performances increase along with the increase of CCA threshold and the two operators can still coexist very well.

Proposal 6: The scheme of using higher CCA thresholds within NR-U system for increasing frequency reuse factor should be reused.

# Conclusion

In this contribution, further considerations on simulation scenarios for NR-U operation are given. Some preliminary simulation results are also provided. Based on these, we have the following proposals and observations:

Proposal 1: When performing a coexistence simulation, besides the co-channel interference, the adjacent channel interference also needs to be considered.

Proposal 2: Multiple CCs/BWPs should also be a candidate configuration in addition to single CC/BWP. Two CCs/BWPs can be taken as a basic configuration.

Proposal 3: A prioritization on the co-existence scenarios of NR-U to NR-U is necessary.

Propose 4:  LBT should be performed with a BWP granularity for as single carrier with bandwidth greater than 20 MHz, i.e., integer multiples of 20 MHz.

Proposal 5: The metrics for NR-U evaluation should include:

    - Base metrics: UPT in low, medium and high traffic loads, RU ratio and the ratio of traffic packet offered/served.

    - Specific metrics for NR-U: BO, latency and the success ratio of LBT.

    - Unfinished files should be incorporated in the UPT calculation.

Observation 1: The two operators deploying NR-U can coexist well if LBT mechanism is adopted.

Observation2: The performances increase along with the increase of CCA threshold and the two operators can still coexist very well.

Proposal 6: The scheme of using higher CCA thresholds within NR-U system for increasing frequency reuse factor should be reused.

# Reference

- Draft report of 3GPP TSG RAN WG1#92bis v0.1.0, Sanya, China, April 16th - 20th

- Draft_Minutes_report_RAN1#93_v020, Busan, Korea, May 21st – 25th, 2018

- RP-181339, Revised SID on NR-based Access to Unlicensed Spectrum, 3GPP TSG RAN Meeting #80, San Diego, USA, June	11-15, 2018

- 3GPP TR 36.889 V13.0.0(2015-06)

# Appendix A: NR-U Indoor sub7 GHz Simulation Assumptions

| Parameters | Indoor Sub-7GHz |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB per 20MHz |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| DL slotNum: UL slotNum | 4:1 |
| Traffic model | Use 36.889 Table A.1.1 |
| DL traffic load: UL traffic load | 1:1 |

# Appendix B: NR-U Outdoor sub7 GHz Simulation Calibration

For scenario 1 and scenario 2, due to different ways to drop gNBs between the operators, the max AP2AP received power CDF will be different. We provide more detailed calibration results here for reference.

## B.1 Results for NR-U Outdoor sub7 GHz Simulation Calibration: AP-to-AP

Percentile of -72dBm point for Max AP2AP received power for scenario 1

|  | A=1.0 | A=1.2 | A=1.4 | A=1.6 | A=1.8 | A=2.0 |
| --- | --- | --- | --- | --- | --- | --- |
| ZTE | 0% | 0% | 0.2% | 0.1% | 0.6% | 1% |

Percentile of -72dBm point for Max AP2AP received power for scenario 2

|  | A=1.0 | A=1.2 | A=1.4 | A=1.6 | A=1.8 | A=2.0 |
| --- | --- | --- | --- | --- | --- | --- |
| ZTE | 2.1% | 6.6% | 12.8% | 21.5% | 29.5% | 36.8% |

The above results show that

- for scenario 1, almost 0% max AP2AP links are below -72dBm as the APs are dropped within the hotspot;

- for scenario 2, more MAX AP2AP links below -72dBm are observed with higher value of A.

## B.2 Simulation parameters for the simulations in B.1

| Parameters | Outdoor Sub-7GHz |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB per 20MHz |
| SCS | To be reported together simulation results |
| Channel Model | NR UMi street canyon |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |
| gNB to gNB link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |