3GPP TSG RAN WG1 Meeting #94        　　	                                 R1-1808447

Gothenburg, Sweden 20th - 24th Aug. 2018

Source:	Mitsubishi Electric

Title:	Discussions on UL inter-UE Tx multiplexing

Agenda Item:	7.2.6.2	UL inter UE Tx prioritization/multiplexing

Document for: 	Discussion/Decision

Introduction

In RAN Plenary #80, a study item was created to investigate performance of pre-emption for UL multiplexing with different reliability requirements [1].

Enhanced multiplexing considering different latency and reliability requirements (RAN1): UL inter UE Tx prioritization/multiplexing

In this contribution, we discuss how to handle multliplexing between UEs with different reliability, i.e., URLLC and eMBB UEs for UL.

Need for UL preemption

URLLC (Ultra reliable low latency communication) in 5G NR is envisioned to be utilized in applications where communication with high reliability is required. Such use cases can be found in applications such as communication between a high speed train conductor and train operation centre [2]. URLLC can include emergency communications which can occur unpredictably. Communication with sensors installed in infrastructures can also be considered as one of the potential use cases for URLLC.

When gNB supports both URLLC and eMBB UEs, there is a need to consider inter-UE prioritization between URLLC and eMBB transmission to reduce interference between two applications. As explained earlier, URLLC may be more spontaneous and unpredictable than eMBB, which will require a mechanism in the specification to dynamically allow URLLC transmission over eMBB transmission. Due to the spontaneous nature of envisioned URLLC communication, dynamic multiplexing through pre-empting has an advantage in spectrum efficiency over fixed scheduling approaches in which dedicated resources are reserved for URLLC transmission.

Thus, we recommend NR to consider UL pre-emption, in which part of eMBB transmission is suspended for URLLC transmission from another UE, when handling UL multiplexing with different reliability requirements.

Proposal 1: NR supports UL pre-emption when handling UL multiplexing with different reliability requirements

Details related to pre-emption

Grant-free based pre-emption

In this section, we list several approaches for pre-emption. There are grant or grant-free based pre-emption. As illustrated in Figure 1, in the grant-free mode, pre-emption indicator (PI) in PDCCH can be used to notify eMBB UEs possible locations for URLLC transmission. The eMBB UE can blank resources indicated by the PI where URLLC transmission make take place. The resources to be blanked can be indicated dynamically using PI or semi-statically via RRC. The bitmap indicator, embedded in PDCCH, similar to the one used in DL, can be used to indicate the blanked resources.

Figure 1 Grant free pre-emption between eMBB and URLLC UE （Examples based on grant-based, pre-emption request to be added）

Figure 2　Grant based pre-emption between eMBB and URLLC UE

Figure 3　Pre-emption request and grant based pre-emption between eMBB and URLLC UE

Grant based pre-emption

For the grant-based pre-emption, as illustrated in Figure 2, URLLC UE sends an SR to gNB to receive a grant. The design and transmission timing of SR need careful consideration. The SR must be reliable and timing must be flexible enough to cater to urgent transmission of URLLC. For the grant-based transmission, the SR can be transmitted in PUCCH. After receiving the PUCCH, gNB can send PI to eMBB and pre-empt the eMBB resources overlapping with the URLLC PUCCH.

Alternatively, instead of a grant, the URLLC UE can send a dedicated request for PI, namely pre-emption request (PR), to the gNB, as illustrated in Figure 3. The gNB can schedule UL transmission for eMBB UE according to the received pre-emption request. While resources need to be reserved periodically for SR transmission in PUCCH, pre-emption request can be transmitted on-demand basis, allowing flexible scheduling and better spectrum efficiency compared to the current grant-based transmission. Furthermore, the on-demand based approach is more suitable for URLLC transmission since occurrence of the transmission is rather sporadic compared to eMBB.

The content of the request can be simple so that it is easy to decode by gNB. Sequence design for the pre-emption request can be considered. For example, considering sensor-based URLLC communication, Zadoff-Chu based low PAPR sequence designs can be considered for pre-emption notification.

Given the nature of unpredictable and time-sensitive URLLC transmission, the pre-emption schedule request can be sent in or outside of PUCCH depending on urgency of the need for transmission.

Proposal 2: Pre-emption request can be used to allocate blank resources during UL eMBB transmissions

Proposal 3: Pre-emption request can be used instead of scheduling request

Whether repetitions of PUSCH over multiple slots in URLLC transmission can be supported or not should be discussed. In addition, whether pre-emption can be applied along with the repetitions of should be discussed. Repetitions of URLLC transmission bring benefits such as increase in SNR or robustness in the transmission in case some of PUSCH are dropped.

Observation 1: Repetitions of PUSCH in URLLC over multiple slots should be discussed.

UE monitoring periodicity

When eMBB monitors for PI, monitoring periodicity should depend on the frame format of the URLLC frame format. For example, if the URLLC frame format is non-slot, the monitoring periodicity should be at non slot level. On the other hand, if URLLC transmission is slot-based, the monitoring periodicity can be either mini-slot or slot level.

Table 1 Possible combinations of pre-empted slot types between eMBB and URLLC

| Case # | eMBB | URLLC |
| --- | --- | --- |
| 1 | Slot | Non-slot |
| 2 | Non-slot | Non-slot |
| 3 | Slot | Slot |
| 4 | Non-slot | Slot |

Possible combinations of pre-empted slot types between eMBB and URLLC are shown in Table 1. For the above four cases, pre-emption handling should be considered. For case #1, we should consider symbol-level based pre-emption. Dropping the entire eMBB slot will lead to inefficient operation and loss of valuable RS, e.g., SRS, DMRS and PTRS. Even some of PUSCH are pre-empted, channel estimation or phase noise estiation obtained from the RS should be kept for the future slots.  Based on the above analysis, we make the following proposal:

Proposal 4: Symbol-level pre-emption should be studied

For the symbol-level pre-emption, pre-empting procedure for RS should be discussed.

Symbol-level pre-emption and RS for UL: UE behavior

As mentioned in the previous section, when considering pre-emption of slot/non-slot with different size, symbol-level pre-emption can be considered instead of dropping the entire victim slot/non-slot to improve the efficiency of the operation.

Observation 2: Different pre-emption procedures should be considered for different length of slot/non-slot

When considering pre-empting eMBB transmission, pre-empting DMRS and PTRS in eMBB transmission should be avoided to prevent serious performance degradation. Missing DMRS or PTRS will lead to loss of performance in channel estimation. Thus, location to be pre-empted in eMBB transmission should be located between frontload DMRS/PTRS and additional DMRS/PTRS, if additional DMRS is present.

If there are not enough resources to be pre-empted, RS or PUCCH must be pre-empted. If RS or PUCCH for eMBB UE are dropped due to the pre-emption, the dropped PUCCH or RS should be scheduled following the URLLC transmission within the same slot or in the subsequent slot. For example, if SRS or PUCCH are pre-empted, since they tend to be located close to the end of the slot, they may need to be rescheduled in the next slot. Furthermore, RS or PUCCH types that can be pre-empted should be discussed. For example, it may be possible to pre-empt SRS, additional DMRS or PUCCH while forbidding preemption of the frontloaded DMRS.

Observation 3: Pre-empted resources in UL eMBB transmission should be located between frontloaded DMRS and additional DMRS, if additional DMRS is present to prevent dropping DMRS

Observation 4: Whether RS or PUCCH can be pre-empted in UL transmission should be discussed.

Conclusion

In this contribution we make the following proposals and observations:

Observation 1: Repetitions of PUSCH in URLLC over multiple slots should be discussed.

Observation 2: Pre-emption rules should be considered for different length of slot/non-slot

Observation 3: Pre-empted resources in UL eMBB transmission should be located between frontloaded DMRS and additional DMRS, if additional DMRS is present to prevent dropping DMRS

Observation 4: Whether RS or PUCCH can be pre-empted in UL transmission should be discussed.

Proposal 1: NR supports UL pre-emption when handling UL multiplexing with different reliability requirements

Proposal 2: Pre-emption request can be used to allocate blank resources during UL eMBB transmissions

Proposal 3: Pre-emption request can be used instead of scheduling request

Proposal 4: Symbol-level pre-emption should be studied

- References

- [1]	RP-181477, “SID on Physical Layer Enhancements for NR URLLC,” La Jolla, USA, June, 2018.

- [2] 	TR 38.802, “Study on New Radio Access Technology Physical Layer Aspects,” V14.0.0, March 2017.