3GPP TSG RAN WG1 Meeting #94	R1-1808562

Gothenburg, Sweden, August 20-24, 2018

Agenda Item:	6.1.5.4

Source:	Huawei, HiSilicon

Title:	Corrections on additional SIB1-NB transmission

Document for:	Discussion and decision

# Introduction

In NB-IoT Rel-15, the agreements related to additional SIB1-NB transmission are summarized in [1]. However, some agreements are not well captured in TS 36.213 [3]. In this paper, we provide our views on those issues.

# Indication of the presence of additional SIB1-NB

An always-present Boolean type parameter additionalTransmissionSIB1 is introduced in TS 36.331 [2] to indicate the presence of additional SIB1-NB, which is shown as follows:

Extracted from Section 6.7.2 of TS 36.331 [2] MasterInformationBlock-NB -- ASN1START MasterInformationBlock-NB ::= SEQUENCE { systemFrameNumber-MSB-r13 BIT STRING (SIZE (4)), hyperSFN-LSB-r13 BIT STRING (SIZE (2)), schedulingInfoSIB1-r13 INTEGER (0..15), systemInfoValueTag-r13 INTEGER (0..31), ab-Enabled-r13 BOOLEAN, operationModeInfo-r13 CHOICE { inband-SamePCI-r13 Inband-SamePCI-NB-r13, inband-DifferentPCI-r13 Inband-DifferentPCI-NB-r13, guardband-r13 Guardband-NB-r13, standalone-r13 Standalone-NB-r13 }, additionalTransmissionSIB1-r15 BOOLEAN, spare BIT STRING (SIZE (10)) } … … MasterInformationBlock-NB field descriptions. …. additionalTransmissionSIB1 Indicates whether additional SIB1-NB transmissions are present. Value TRUE indicates that additional SIB1-NB transmissions are present. See TS 36.211 [21] and TS 36.213 [23]. E-UTRAN only configures additionalTransmissionSIB1 to TRUE if schedulingInfoSIB1 indicates that the number of NPDSCH repetitions is 16, see TS 36.213 [23, Table 16.4.1.3-3]..

In TS 36.213 [3], the indication of the presence of additional SIB1-NB is described as follows:

Extracted from Section 16.4 of TS 36.213 [3] - except when the UE is configured with higher layer parameter additionalTransmissionSIB1, subframe #3 not containing additional SystemInformationBlockType1-NB transmission is assumed as a NB-IoT DL subframe if the UE monitors a NPDCCH UE-specific search space or decodes NPDSCH transmission scheduled by NPDCCH in the UE-specific search space.

additionalTransmissionSIB1 is a Boolean type parameter and will always be configured, so the description on additionalTransmissionSIB1 in TS 36.213 [3] is inaccurate. As a result, we propose to revise the associated description to make the indication of the presence of additional SIB1-NB accurate.

Proposal 1: Adopt the TP in Annex A for TS 36.213 to make the indication of the presence of additional SIB1-NB accurate.

# Conclusion

Proposal 1: Adopt the TP below in TS 36.213 to make the indication of the presence of additional SIB1-NB accurate.

# References

R1-1807590, “RAN1 agreements for Rel-15 NB-IoT after RAN193”, WI rapporteur

R2-1809276, “Introduction_of_NB-IoT_Enhancements_other_than_EDT_in 36.331”, Huawei, HiSilicon

3GPP TS 36.213 V15.2.0 (2018-06)

# Annex A. TP for additional SIB1-NB in TS 36.213

---------------------------------------------- Start of Text Proposal ------------------------------------------

-------------------------------------------- Unchanged parts omitted -----------------------------------------

## 16.4	Narrowband physical downlink shared channel related procedures

A NB-IoT UE shall assume a subframe as a NB-IoT DL subframe if

-	except when the UE is configured with higher layer parameter additionalTransmissionSIB1 set to TRUE, subframe #3 not containing additional SystemInformationBlockType1-NB transmission is assumed as a NB-IoT DL subframe if the UE monitors a NPDCCH UE-specific search space or decodes NPDSCH transmission scheduled by NPDCCH in the UE-specific search space.

-------------------------------------------- Unchanged parts omitted -----------------------------------------

----------------------------------------------- End of Text Proposal ------------------------------------------