3GPP TSG RAN WG1 Meeting #94	                                                	            R1-1808710

Gothenburg, Sweden, August 20th – 24th, 2018

| Source | : | Intel Corporation |
| --- | --- | --- |
| Title | : | Spectral Efficiency Evaluation for NR eMBB |
| Agenda Item | : | 7.2.7.2 Others |
| Document for | : | Discussion and Decision |

# Introduction

This paper is related to the “New Study Item on Self-Evaluation towards IMT-2020 submission” [1]. In this paper, we provide self-evaluation results for the simulation based metrics of Average Spectral Efficiency and 5-th Percentile Spectral Efficiency for NR eMBB deployment configurations for sub-6 GHz and mmWave cases. We present the detailed assumptions considered for this evaluation and show that NR can meet IMT-2020 minimum requirements [2].

# Evaluation Methodology

In this paper, we evaluate the NR eMBB average cell spectral efficiency and the 5th percentile spectral efficiency. System level simulation with full-buffer traffic model is used for these evaluations. According to [3], the two metrics can be evaluated as follows.

Average Spectral Efficiency: For the case of average spectral efficiency, consider a system withusers and TRxPs. If  denotes the successfully received bits from user i uplink) or by user i (downlink) over a bandwidth W and time T, the average spectral efficiency of the system over  number of drops is given by

,

where  denotes the successfully received bits by (from) user from drop.

5th Percentile Spectral Efficiency: Considering a similar system over  number of drops, the spectral efficiency of the i-th user is denoted by:

.

Running the system level simulation over the multiple drops yields  number of data points and the 5th percentile spectral efficiency is evaluated as the lowest 5th percentile point of the CDF of these data points.

In this paper, based on the above definitions and using the simulation methodology for the different eMBB test environments outlined in [3], we show that NR can meet the IMT-2020 requirements for all environments in sub-6GHz. The case of mmWave, an SLS implementation with single panel operation, where all panels (or TXRUs) use the same analog beam, it can be shown that Indoor Hotspot eMBB (Configuration B in [3]) meets requirements for both downlink and uplink.

The evaluations for spectral efficiency should account for L1 and L2 overhead. The system level overhead assumptions for FR1 and FR2 are provided in Appendix 7.1, while the detailed system level simulation assumptions are provided in Appendices 7.2 and 7.3 for FR1 and FR2 respectively.

# Evaluation Results

In this section, we present the system level evaluation results for Frequency Range 1 and Frequency Range 2 operation of NR eMBB based on the test environments outlined in [3]. For the case of FR1 [5] i.e., carrier frequency below 6 GHZ (4GHz and 700MHz), we present evaluation results for IMT channel model A for the respective test environments. In this case, we consider a sub-carrier spacing of 15 kHz. For the case of FR2 [6] i.e., carrier frequency in the mmWave range (30GHz), we present results for IMT channel model B for the respective test environments. In this case, we consider a subcarrier spacing of 120 kHz. For the case of uplink simulation results, we also report the open power control parameters used for each test environment. For TDD operation, we consider the frame-structure ‘DSUUD’ with S having 6 downlink symbols, 2 guard symbols and 6 uplink symbols.

## Frequency Range 1

In this section, the results for frequency range 1 corresponding to evaluation configurations in [3] with carrier frequencies below 6GHz, are presented for each test environment. The detailed overhead assumption for FDD and TDD are presented in Appendix 6.1 and the system level simulation assumptions are provided in Appendix 6.2. Finally, for the case of frequency range 1, we provide results based on IMT Channel Model A [3] for each test environment.

### Indoor Hotspot eMBB

| Downlink Spectral Efficiency (bits/s/Hz) | Indoor Hotspot eMBB Configuration A CF = 4GHz, Channel Model A |  |  |  |
| --- | --- | --- | --- | --- |
|  | 12 TRxP |  | 36 TRxP |  |
|  | FDD | TDD | FDD | TDD |
| 5th Percentile | 0.443 | 0.418 | 0.452 | 0.426 |
| Average | 11.832 | 11.163 | 14.011 | 13.219 |

It can be seen from the above table that the IMT-2020 requirement of 0.3 bits/s/Hz for 5th percentile downlink spectral efficiency and 9 bits/s/Hz for downlink average spectral efficiency is easily fulfilled by NR Indoor Hotspot deployment scenario for downlink operation in FR1 for all configurations defined in [3] with IMT channel model A.

| Uplink Spectral Efficiency (bits/s/Hz) | Indoor Hotspot eMBB Configuration A CF = 4GHz, Channel Model A |  |  |  |
| --- | --- | --- | --- | --- |
|  | 12 TRxP |  | 36 TRxP |  |
|  | FDD | TDD | FDD | TDD |
| 5th Percentile | 0.590 | 0.564 | 0.465 | 0.445 |
| Average | 13.903 | 13.293 | 15.122 | 14.458 |
| Power Control Parameters |  |  |  |  |

It can be seen from the above table that the IMT-2020 requirement of 0.21 bits/s/Hz for 5th percentile uplink spectral efficiency and 6.75 bits/s/Hz for average uplink spectral efficiency is easily fulfilled by NR Indoor Hotspot deployment scenario for uplink operation in FR1 for all configurations defined in [3] with IMT channel model A.

### Dense Urban eMBB

| Downlink Spectral Efficiency (bits/s/Hz) | Dense Urban eMBB Configuration A CF = 4GHz, Channel Model A |  |
| --- | --- | --- |
|  | FDD | TDD |
| 5th Percentile SE | 0.598 | 0.564 |
| Average SE | 16.491 | 15.587 |

It can be seen from the above table that the IMT-2020 requirement of 0.225 bits/s/Hz for 5th percentile downlink spectral efficiency and 7.8 bits/s/Hz for average downlink spectral efficiency is easily fulfilled by NR Dense Urban Macro deployment scenario for downlink operation in FR1 for all configurations defined in [3] with IMT channel model A.

| Uplink Spectral Efficiency (bits/s/Hz) | Dense Urban eMBB Configuration A CF = 4GHz, Channel Model A |  |
| --- | --- | --- |
|  | FDD | TDD |
| 5th Percentile SE | 0.526 | 0.502 |
| Average SE | 22.406 | 21.422 |
| Power Control Parameters |  |  |

It can be seen from the above table that the IMT-2020 requirement of 0.15 bits/s/Hz for 5th percentile uplink spectral efficiency and 5.4 bits/s/Hz for average uplink spectral efficiency is easily fulfilled by NR Dense Urban Macro deployment scenario for uplink operation in FR1 for all configurations defined in [3] with IMT channel model A.

### Rural Macro eMBB

| Downlink Spectral Efficiency (bits/s/Hz) | Rural Macro eMBB - Channel Model A |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Configuration A CF = 700 MHz, ISD = 1732 m |  | Configuration B CF = 4GHz, ISD = 1732 m |  | Configuration C (LMLC) CF = 700 MHz, ISD = 6000 m |  |
|  | FDD | TDD | FDD | TDD | FDD | TDD |
| 5th Percentile SE | 0.525 | 0.496 | 0.524 | 0.495 | 0.469 | 0.442 |
| Average SE | 17.014 | 16.052 | 20.045 | 18.912 | 14.082 | 13.285 |

It can be seen from the above table that the IMT-2020 requirement of 0.12 bits/s/Hz for 5th percentile downlink spectral efficiency and 3.3 bits/s/Hz for average downlink spectral efficiency is easily fulfilled by NR Rural Macro deployment scenario for downlink operation in FR1 for all configurations defined in [3] with IMT channel model A.

| Uplink Spectral Efficiency (bits/s/Hz) | Rural Macro eMBB - Channel Model A |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Configuration A CF = 700 MHz, ISD = 1732 m |  | Configuration B CF = 4GHz, ISD = 1732 m |  | Configuration C (LMLC) CF = 700 MHz, ISD = 6000 m |  |
|  | FDD | TDD | FDD | TDD | FDD | TDD |
| 5th Percentile SE | 0.630 | 0.602 | 0.282 | 0.270 | 0.087 | 0.083 |
| Average SE | 15.499 | 14.819 | 22.934 | 21.927 | 11.406 | 10.905 |
| Power Control Parameters |  |  |  |  |  |  |

It can be seen from the above table that the IMT-2020 requirement of 0.045 bits/s/Hz for 5th percentile downlink spectral efficiency and 1.6 bits/s/Hz for average downlink spectral efficiency is easily fulfilled by NR Rural Macro deployment scenario for downlink operation in FR1 for all configurations defined in [3] with IMT channel model A.

## Frequency Range 2

In this section, the results for frequency range 2 with carrier frequency of 30 GHz is presented for Indoor Hotspot eMBB (downlink and uplink). The detailed overhead assumptions are presented in Appendix 6.1 and the system level simulation assumptions are provided in Appendix 6.3.

### Indoor Hotspot eMBB

| Downlink Spectral Efficiency (bits/s/Hz) | Indoor Hotspot eMBB Configuration B CF = 30 GHz, Channel Model B |  |
| --- | --- | --- |
|  | 12 TRxP | 36 TRxP |
| 5th Percentile | 0.417 | 0.356 |
| Average | 10.308 | 10.567 |

It can be seen from the above table that the IMT-2020 requirement of 0.3 bits/s/Hz for 5th percentile downlink spectral efficiency and 9 bits/s/Hz for downlink average spectral efficiency is fulfilled by NR Indoor Hotspot deployment scenario for downlink operation in FR2.

| Uplink Spectral Efficiency (bits/s/Hz) | Indoor Hotspot eMBB Configuration B CF = 30 GHz, Channel Model B |  |
| --- | --- | --- |
|  | 12 TRxP | 36 TRxP |
| 5th Percentile | 0.438 | 0.319 |
| Average | 11.755 | 10.688 |
| Power Control Parameters |  |  |

It can be seen from the above table that the IMT-2020 requirement of 0.21 bits/s/Hz for 5th percentile uplink spectral efficiency and 6.75 bits/s/Hz for average uplink spectral efficiency is easily fulfilled by NR Indoor Hotspot deployment scenario for uplink operation in FR2.

# Conclusions

In this paper, we presented system level evaluations of average cell spectral efficiency and the 5th percentile spectral efficiency for the frequency range 1 and 2. It was shown that NR can easily meet the requirements for all test environments and evaluation configuration in frequency range 1 for both downlink and uplink. For the case of frequency range 2, NR can meet the IMT-2020 requirements for Indoor Hotspot eMBB Configuration B for both downlink and uplink.

# References

- RP-170832, New Study Item on Self-Evaluation towards IMT-2020, Huawei, Ericsson, 3GPP TSG RAN Meeting #75, Dubrovnik, Croatia, March 6-9, 2017.

- ITU-R M.2410-0, Minimum requirements related to technical performance for IMT-2020 radio interface(s)

- ITU-R M.2412-0, Guidelines for evaluation of radio interface technologies for IMT-2020

- ITU-R M.2134, Requirements related to technical performance for IMT-Advanced radio interface(s)

- 3GPP TS 38.101-1 v15.1.0, User Equipment radio transmission and reception: Part 1: Range 1 Standalone.

- 3GPP TS 38.101-2 v15.1.0, User Equipment radio transmission and reception: Part 2: Range 2 Standalone.

- 3GPP TS 38.306 v15.1.0, User Equipment (UE) Radio Access Capabilities.

- 3GPP TS 38.214 v15.1.0, NR Physical Layer Procedures for Data (Release 15)

- 3GPP TS 38.211 v15.1.0, NR Physical Channels and Modulation (Release 15)

- RT-170019, Summary of email discussion “[ITU-R AH 01] Calibration for self-evaluation, Huawei, 3GPP TSG RAN ITU-R Ad Hoc.

- R1-1808714, Summary of Email Discussion on NR SLS Overhead Calculation for IMT-2020 Self-Evaluation, Intel Corporation, 3GPP WG RAN1 #94, Gothenburg, Sweden, August 20-24, 2018.

# Appendix

## System Level Overhead Assumptions

Table 5: Overhead Assumptions for FR1 (TDD/FDD)

| Parameter |  | Downlink | Uplink |
| --- | --- | --- | --- |
| Max # Transmit Layers |  | 12 (up to 4 per UE in MU-MIMO) | 12 (up to 4 per UE in MU-MIMO) |
| DM-RS |  | 2 symbol Type 2 FL DMRS in each slot with RS on 3 combs: 24 REs/PRB/slot | 2 symbol Type 2 FL DMRS in each slot with RS on 3 combs: 24 REs/PRB/slot |
| SRS |  | - | 12 RE/PRB/period Periodicity 10 slots |
| CSI-RS (for CSI) | TDD | 10 4-port CSI-RS resource per slot 40 RE/PRB/period Periodicity 10ms | - |
|  | FDD | 32-port CSI-RS resource per slot 32 RE/PRB/period Periodicity 10ms |  |
| CSI-IM |  | 4 RE/PRB/period Periodicity 10ms | - |
| TRS |  | 2(burst length)*4 RE/PRB/periodicity Periodicity = 20ms | - |
| SSB |  | 1 SSB 240x4 RE/period Periodicity = 10ms |  |
| PDCCH |  | 8 CCE (Aggregation Level 8) + 2 symbols (24 RE) in in every slot 12x48 RE/slot | - |
| PUCCH |  | - | Short – 2 symbols, 8 PRBs 24 x 8 x 5 = 960 REs/period Periodicity 2 slots |

In order to provide a tangible calculation with the above assumptions, we consider an SCS of 15 kHz and 10 MHz system BW with 52 PRBs as shown in Table 3. Considering a block of 10 slots with a total duration of 10ms i.e., a total of 87360 REs, we provide an initial calculation in the attached worksheet in [11]. It is seen that a downlink overhead of 24.359% (FDD), 28.635% (TDD), and an uplink overhead of 16.09% (FDD) and 19.784% (TDD) can be achieved.

Table 6: Overhead Assumptions for FR2 (TDD)

| Parameter |  | Downlink | Uplink |
| --- | --- | --- | --- |
| Max # Transmit Layers |  | 6/12 (up to 4 per UE in MU-MIMO) | 6/12 (up to 4 per UE in MU-MIMO) |
| DM-RS | PTRS OFF | 2 symbol Type 2 FL DMRS in each slot with RS on 3 combs 24 REs/PRB/slot | 2 symbol Type 2 DMRS in each slot with RS on 3 combs 24 REs/PRB/slot |
|  | PTRS ON | 1 symbol Type 2 FL DMRS in each slot with RS on 3 combs 12 REs/PRB/slot | 1 symbol Type 2 DMRS in each slot with RS on 3 combs 12 REs/PRB/slot |
| SRS |  | - | 12 RE/PRB/period Periodicity 2 slots |
| CSI-RS (for CSI) |  | 10 4-port CSI-RS resource per slot 40 RE/PRB/period Periodicity 10 ms | - |
| CSI-IM |  | 4 RE/PRB/period Periodicity 1 ms | - |
| TRS |  | 1(burst length)*4 RE/PRB/period Periodicity = 2ms | - |
| PTRS (if ON) |  | 4x3 = 12 RE/PRB/slot | 4x3 = 12 RE/PRB/slot |
| CSI-RS BM |  | TBD | - |
| SSB |  | 12 SSB 12x240x4 RE/period Periodicity 20ms |  |
| PDCCH |  | 8 CCE (Aggregation Level 8) + 2 symbols in every slot 12x48 RE/slot | - |
| PUCCH |  | - | Short – 2 symbols, 8 PRBs 24 x 8 x 5 = 960 REs/period Periodicity 2 slots |

For FR2, we assume a SCS of 120 kHz and system BW of 100 MHz with 66 PRBs as shown in Table 4. Considering a block of 80 slots with a total duration of 10ms i.e., a total of 887040 REs, we provide an initial calculation in the attached worksheet in [11]. It is seen that a DL overhead of 26.933% and a UL overhead of 23.128% is achievable under the assumptions in Table 6. It can be seen that the overhead with or without PTRS remains the same. However, in the presence of PTRS only up to 6 MIMO layers can be supported at the gNB. Furthermore, the overhead assumptions for CSI-RS for BM can be further discussed.

## Simulation Assumptions for FR1

| Simulation Parameters | Scenario |  |  |  |
| --- | --- | --- | --- | --- |
|  | Dense Urban Macro eMBB | Indoor Hotspot eMBB | Rural Macro eMBB Configuration A, Configuration C (LMLC) | Rural Macro eMBB Configuration B |
| Carrier Frequency | 4GHz | 4GHz | 700 MHz | 4GHz |
| Simulation BW | 10MHz (FDD); 20 MHz (TDD) |  |  |  |
| Sub-carrier Spacing | 15 kHz |  |  |  |
| Channel Model | IMT UMa A | IMT InH A | IMT RMa A | IMT RMa A |
| Inter-Site Distance | 200m | 20m | Configuration A: 1732 m Configuration C: 6000 m | 1732 m |
| BS Antenna Configuration | (1,1,8,16,2; 1,16) 32 TXRU | For 12 TRxP: (1,1,4,4,2; 4,4) 32 TXRU | (1,1,4,8,2; 1,8) 16 TXRU | (1,1,8,16,2; 1,16) 32 TXRU |
|  |  | For 36 TRxP: (1,1,8,16,2; 2,8) 32 TXRU |  |  |
| BS Antenna Spacing | (0.8, 0.5) | (0.5, 0.5) | (0.8, 0.5) | (0.8, 0.5) |
| UE Antenna Configuration | (1,1,1,2,2; 1,2) 4 TXRU | (1,1,1,2,2; 1,2) 4 TXRU | (1,1,1,2,2; 1,1): 4 TXRU | (1,1,1,2,2; 1,2) 4 TXRU |
| UE Antenna Spacing | (0.5, 0.5) | (0.5, 0.5) | (0.5, 0.5) | (0.5, 0.5) |
| Beam set at BS | Single beam at 102° electronic downtilt | Single beam Downtilt 12 TRxP - 90° 36 TRxP - 110° | Single beam Electronic downtilt: Configuration A - 100° Configuration C - 96° | Single beam at 100° electronic downtilt |
| Beam Set at UE | Single Beam Omni-Directional |  |  |  |
| UE Deployment | 80% Indoor; 20% Outdoor | 100% Indoor | 50% Indoor; 50% Outdoor |  |
| Traffic Model | Full Buffer |  |  |  |
| Highest Modulation | 256 QAM for DL and UL |  |  |  |
| Channel Coding | LPDC |  |  |  |
| Transmission/Reception Scheme | Multi-user MIMO with 12 layers at BS for DL/UL |  |  |  |
| Scheduler | DL: MU-PF with sub-band scheduling, rank adaptation and beam sweeping at BS UL: MU-PF with non-contiguous scheduling, rank adaptation and beam sweeping at BS |  |  |  |
| Feedback | Type II Codebook based CSI for DL NR UL Codebook for UL |  |  |  |
| Impairment Modeling | Channel Estimation Error; UL MPR modeled as per TS 38.101 (Additional 2dB back-off for non-contiguous UL scheduling) |  |  |  |

## Simulation Assumptions for FR2

| Simulation Parameters | Indoor Hotspot eMBB |
| --- | --- |
| Carrier Frequency | 30GHz |
| Simulation BW | 50MHz (TDD) |
| Sub-carrier Spacing | 120 kHz |
| Channel Model | IMT InH B |
| Inter-Site Distance | 20m |
| BS Antenna Configuration | For 12 TRxP: (1,1,8,8,2; 4,4) 32 TXRU |
|  | For 36 TRxP: (1,1,8,16,2; 2,8) 32 TXRU |
| BS Antenna Spacing | (0.5, 0.5) |
| UE Antenna Configuration | (1,2,2,4,2; 1,1) 4 TXRU Panels:[0° 180°] |
| UE Antenna Spacing | (0.5, 0.5) |
| Beam set at BS | DFT Beam Selection with 2x Horizontal/Vertical Oversampling |
| Beam Set at UE | DFT Beam Selection |
| UE Deployment | 100% Indoor |
| Traffic Model | Full Buffer |
| Highest Modulation | 256 QAM for DL and UL |
| Channel Coding | LPDC |
| Transmission/Reception Scheme | Multi-user MIMO with up to 8 layers at BS for DL/ and up to 6 layers at the BS for UL |
| Scheduler | DL: Multi-panel MU-PF with sub-band scheduling, rank adaptation and beam sweeping at BS |
|  | UL: MU-PF with non-contiguous scheduling, rank adaptation and beam sweeping at BS |
| Feedback | DL: Type II Codebook based CSI for single beam transmissions Type I SP Codebook based CSI for multi-beam transmissions |
|  | UL: NR UL Codebook |
| Impairment Modeling | Channel Estimation Error; UL MPR modeled as per TS 38.102 |