3GPP TSG RAN WG1 Meeting #94		R1-1808780

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda item:	7.2.4.2

Source: 	Samsung

Title: 				    Discussion on Uu enhancements for advanced V2X use cases

Document for:	Discussion and Decision

Introduction

In the approved SID for NR V2X [1], the following objective for NR/LTE Uu enhancements is included.

2: Uu enhancements for advanced V2X use cases [RAN1, RAN2, RAN3]: - Evaluate whether Rel-15 NR Uu and LTE Uu interfaces will support advanced V2X use cases - Identify enhancements, if any, that are needed to meet advanced V2X use cases NOTE: Also consider other Rel-16 NR and LTE SI/WI enhancements to avoid overlap.

This contribution discusses advanced V2X use cases and candidate issues for NR/LTE Uu enhancements.

Discussion

Requirements for advanced V2X use cases

In [2], the use cases for advanced V2X services are addressed and categorized into platooning, advanced driving, remote driving, and extended sensors as shown in Table 1.

Table 1: Mapping of use cases to use case group [2, Table A-1]

| Use case group | Section number | Use case name |
| --- | --- | --- |
| Platooning | 5.1 | eV2X support for Vehicle Platooning |
|  | 5.2 | Information exchange within platoon |
|  | 5.5 | Automated Cooperative Driving for Short distance Grouping |
|  | 5.12 | Information sharing for limited automated platooning |
|  | 5.13 | Information sharing for full automated platooning |
|  | 5.17 | Changing Driving-Mode |
| Advanced Driving | 5.9 | Cooperative Collision Avoidance (CoCA) |
|  | 5.10 | Information sharing for limited automated driving |
|  | 5.11 | Information sharing for full automated driving |
|  | 5.20 | Emergency Trajectory Alignment |
|  | 5.22 | Intersection Safety Information Provisioning for Urban Driving |
|  | 5.23 | Cooperative lane change (CLC) of automated vehicles |
|  | 5.25 | 3D video composition for V2X scenario |
| Remote driving | 5.4 | eV2X support for Remote Driving |
|  | 5.21 | Teleoperated Support (TeSo) |
| Extended Sensor | 5.3 | Automotive: Sensor and State Map Sharing |
|  | 5.6 | Collective Perception of Environment |
|  | 5.16 | Video data sharing for automated Driving |
| General | 5.7 | Communication between vehicles of different 3GPP RATs |
|  | 5.8 | Multi-PLMN environment |
|  | 5.15 | Use case on Multi-RAT |
|  | 5.19 | Use case out of 5G coverage |
|  | 5.14 | Dynamic Ride Sharing |
|  | 5.18 | Tethering via Vehicle |
|  | 5.24 | Proposal for secure software update for electronic control unit |

First of all, among the above use cases, RAN1 needs to discuss which use cases should be targeted for Uu enhancement rather than sidelink.

Proposal 1: Discuss the use cases for advance V2X applications using Uu.

For each use case, performance requirements in the perspective of payload size, transmission rate, E2E latency, reliability, data rate, and communication range are provided below.

| Communication scenario |  |  |  | Payload (Bytes) | Max end-to-end latency (ms) | Reliability (%) | Data rate (Mbps) | Communication range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.16 Video data sharing for automated Driving | Between UEs supporting V2X application | Driver control/ Limited automated driving | [CPR.E-007] |  | [50] | 90 | [10] | [100] |
|  |  | Fully automated driving | [CPR.E-008] |  | [10] | 99.99 | [700] | [500] |

RAN1 needs to discuss how those requirements defined in [2] could be interpreted in physical layer. For example, when max end-to-end latency is given as 50 ms for video data sharing, it is not clear yet what the maximum allowable latency in physical layer is. Moreover, it is observed that many use cases for advanced V2X applications require high reliability and low latency. These aspects for LTE/NR Uu will be studied in other SI/WI such as eURLLC as described in SID to avoid overlap.

Proposal 2: High reliability and low latency aspects Uu interface are studied in eURLLC SI.

Uu enhancement: Multicast operations via NR Uu

Figure 1: Use case of Uu multicast from gNB to many vehicles in the same group/platoon

One possible enhancement of NR Uu is groupcast operations. Compared to LTE supporting eMBMS and SC-PTM, NR does not support multicast and broadcast for data transmission yet. Group communications over Uu can be used for platooning applications under the gNB coverage for better reliability than sidelink. When introducing multicast/broadcast over Uu, rather than eMBMS, the approach like SC-PTM will have less specification impact and help to reduce time budget to study/work on it. Also, similar to NR sidelink groupcast, HARQ and CSI feedback mechanisms could be also studied to increase throughput and reliability.

Proposal 3: Study SC-PTM-like group communications over NR Uu.

Proposal 4: Study the feasibility and candidate techniques of HARQ and CSI feedback mechanisms for group communications over NR Uu.

Conclusions

This contribution considered aspects for Uu enhancements, and the followings are proposed.

Proposal 1: Discuss the use cases for advance V2X applications using Uu.

Proposal 2: High reliability and low latency aspects of Uu interface are studied in eURLLC SI.

Proposal 3: Study SC-PTM-like group communications over NR Uu.

Proposal 4: Study the feasibility and candidate techniques of HARQ and CSI feedback mechanisms for group communications over NR Uu.

- References:

- RP-181480, “New SID: Study on NR V2X”

- 3GPP TR22.886, V16.0.0, “Study on enhancement of 3GPP Support for 5G V2X Services”