3GPP TSG RAN WG1 Meeting #94                   R1-1808862

Gothenburg, Sweden, 20th – 24th, August 2018

Source: MediaTek Inc.

Title: Evaluation results and observations of UL spectral efficiency for IMT2020 self

evaluation

Agenda Item: 7.2.7

Document for: Discussion

- Introduction

IMT2020 systems, being as new mobile systems for future IMT, are expected to have enhanced capabilities beyond those of IMT2000 and IMT-Advanced. The service capabilities of average spectral efficiency and 5th percentile spectral efficiency are two of the critical features, and need to be evaluated with the new radio interfaces to achieve the technique performance requirements addressed in ITU-R M.2410 [1]. To exploit and develop IMT2020 systems, a common understanding of the evaluation assumptions for different scenarios is described in ITU-R M2412 [2]. In this contribution, the evaluation results and observations, considering the summary of offline discussion [3], of eMBB UL spectral efficiency in dense urban, indoor hotspot and rural scenarios with FDD duplexing are provided.

- Preliminary

2.1 Adoption of different MIMO modes

Two MIMO modes, MU-MIMO and SU-MIMO, are considered in the evaluation of NR UL spectral efficiency. The rank that a UE can use on a single resource element is eventually bounded by the number of receiver antenna ports. If multiple UEs can be paired on the same resource element in terms of better spectral efficiency, the overall ranks gathered from these paired UEs will reveal better resource utilization and typically result in higher spectral efficiency. However, the paired multiple UEs do not decrease their UL transmitting power, and the increased inter-cell interference may cause more performance losses that dominate the benefits from better resource utilization. The observations of using SU-MIMO and MU-MIMO will be described in the evaluation section.

2.2 Overhead modeling

The resource utilization of reference signals, synchronization signal and the control channel should be considered in the evaluation of spectral efficiency. In this contribution, the UL overheads originated from the resource allocations for DMRS, SRS as well as PUCCH are considered as shown in Appendix.

- Evaluation result

The evaluation results of UL spectral efficiency in different eMBB environments are provided in Table 3.1. It is noted that the results in Table 3.1 are evaluated by using SU-MIMO mode, and they can fulfill the ITU requirements in all sub 6 GHz UL eMBB scenarios. On the other hand, in the UL evaluations with MU-MIMO scheme and high UE transmission power, it is observed that the inter-cell interferences are very large such that it causes failure UL transmissions in most cases and can’t meet ITU requirements eventually. The phenomenon will become more severe in the cell edge and it is a challenge to use MU-MIMO in the sub 6 GHz UL eMBB scenarios. The detailed evaluation assumptions can be found in the Appendix below.

Table 3.1 FDD UL spectral efficiency evaluation in different eMBB scenarios

| Test environment | Evaluation configuration | Average spectral efficiency (bps/Hz/TRxP) |  | 5th percentile spectral efficiency (bps/Hz) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SU-MIMO | ITU Req. | SU-MIMO | ITU Req. |
| Dense Urban | Configuration A 4GHz | 8.53 | 5.4 | 0.49 | 0.15 |
| Indoor Hotspot | Configuration A 4GHz | 8.56 | 6.75 | 0.57 | 0.21 |
| Rural | Configuration A 700MHz | 4.50 | 1.6 | 0.224 | 0.045 |
| Rural | Configuration C 700MHz | 4.06 | 1.6 | 0.096 | 0.045 |

Observation 1: NR with sub 6 GHz can fulfill UL spectral efficiency requirements in all test environments.

Observation 2: SU-MIMO scheme is feasible for sub 6 GHz NR UL cases to meet the spectral efficiency requirements.

Observation 3: Due to serious inter-cell interferences, it is a high challenge to use MU-MIMO in the sub 6 GHz UL eMBB scenarios to meet ITU spectral efficiency requirements.

- Conclusion

In this document, the evaluation results of UL spectral efficiency for NR sub 6 GHz in all test environments are provided. Based on the evaluation results, the following observations are summarized.

Observation 1: NR with sub 6 GHz can fulfill UL spectral efficiency requirements in all test environments

Observation 2: SU-MIMO scheme is feasible for sub 6 GHz NR UL cases to meet the spectral efficiency requirements.

Observation 3: Due to serious inter-cell interferences, it is a high challenge to use MU-MIMO in the sub 6 GHz UL eMBB scenarios to meet ITU spectral efficiency requirements.

- Appendix: Evaluation assumption

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
| Test environment |  | Indoor Hotspot – eMBB | Dense Urban – eMBB | Rural - eMBB |
| Evaluation configuration |  | Configuration A | Configuration A | Configuration A Configuration C |
| Channel model |  | InH_A | UMa_A | RMa_A |
| Subcarrier spacing |  | FDD: 15 kHz | FDD: 15 kHz | FDD: 15 kHz |
| Symbols number per slot |  | 14 | 14 | 14 |
| Number of antenna elements per TRxP |  | 32Rx cross-polarized antenna (M,N,P,Mg,Ng) = (4,4,2,1,1) | 128Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) | 64Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,4,2,1,1) |
| Number of TXRU per TRxP |  | 16TXRU Vertical 1-to-2 | 16TXRU Vertical 1-to-8 | 8TXRU Vertical 1-to-8 |
| Number of antenna elements per UE |  | 2Tx with 0°,90° polarization | 2Tx with 0°,90° polarization | 1Tx |
| UE power class |  | 23 dBm | 23 dBm | 23 dBm |
| Mechanic tilt |  | 180deg in GCS (pointing to the ground) | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | 90deg in LCS | 105deg in LCS | 100deg in LCS |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 |
| Scheduling |  | SU-PF | SU-PF | SU-PF |
| MIMO mode |  | SU-MIMO with rank 2/4 adaptation | SU-MIMO with rank 2/4 adaptation | SU-MIMO |
| BS receiver type |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| UE precoder scheme |  | Codebook based | Codebook based | Codebook based |
| UL CSI derivation |  | Non-precoded SRS based, with delay | Non-precoded SRS based, with delay | Non-precoded SRS based, with delay |
| Power control |  | α= 0.9, P0 =-86 dBm | α= 0.6, P0 =-60 dBm | α= 0.8, P0 =-76 dBm |
| RB allocation for Power backoff model |  | Continuous: follow TS 38.101; Non-continuous: additional 2 RB reduction | Continuous: follow TS 38.101; Non-continuous: additional 2 RB reduction | Continuous: follow TS 38.101; Non-continuous: additional 2 RB reduction |
| Overhead | PUCCH | 2 RBs, 14 OFDM symbols | 2 RBs, 14 OFDM symbols | 2 RBs, 14 OFDM symbols |
|  | DMRS | Type II, 1 additional DMRS symbol, and no FDM with PUSCH | Type II, 1 additional DMRS symbol, and no FDM with PUSCH | Type II, 1 additional DMRS symbol, and no FDM with PUSCH |
|  | SRS | 2 symbols per 5 slots, 8 RBs per symbol | 2 symbols per 5 slots, 8 RBs per symbol | 2 symbols per 5 slots, 8 RBs per symbol |
| Channel estimation |  | Non-ideal | Non-ideal | Non-ideal |
| Waveform |  | OFDM | OFDM | OFDM |

- Reference

- [1] Report ITU-R M.2410 - Minimum requirements related to technical performance for IMT-2020 radio interface(s).

- [2] Report ITU-R M.2412 - Guidelines for evaluation of radio interface technologies for IMT-2020.

- [3] R1-1803386, “Summary of offline discussion for IMT-2020 self evaluation”, RAN1 #92, Huawei, Athens

- [4] REPORT ITU-R M.2134 Requirements related to technical performance for IMT-Advanced radio interface(s)