3GPP TSG RAN WG1 Meeting #94	 R1-1808950

Gothenburg, Sweden, August 20th – 24th , 2018

Agenda Item:	7.2.8

Source:	Huawei, HiSilicon

Title:	Enhancements on Type II CSI for more than 2 layers

Document for:	Discussion and decision

Introduction

Based on the WID on NR MIMO enhancements for Rel-16 in RAN meeting #80 [1], it has been agreed that

- Perform study and, if needed, specify extension of Type II CSI feedback to rank >2

It is known that only up to rank 2 is supported in type II codebook in Rel-15. However, up to rank 4 is supported per user in MU MIMO transmission. That means the MU-MIMO performance will be limited by the max rank of type-II codebook, especially in the case of UE with 4 Rx antennas.

In this contribution, we discuss whether rank-3 and 4 is needed for the type-II codebook design and also provide performance evaluation.

Probability of rank-3/4 for Type II CSI feedback

In NR Rel.15, up to 32 CSI-RS ports at gNB side has been supported and UEs with 4Rx are becoming more and more popular. Fig. 1 shows the distribution of ranks of CSI reports, with the assumption of 32Tx at gNB and 4Rx at UE without any rank restriction. Simulation results show that the possibility of UE reporting rank 3/4 is 25%-30% for both UMa and UMi scenarios. On the other hand, for SU-MIMO case, since type II CSI feedback has higher resolution than type I, supporting rank 3/4 for type II can improve UE experience compared with type I CSI feedback. Therefore, the extension of type II codebook for rank 3/4 is a good candidate of CSI feedback enhancement.

Figure 1. Distribution of feedback ranks in UMa and UMi scenarios.

Observation 1: The probability of UE reporting rank 3/4 is 25%-30% in both UMa and UMi scenarios.

One straightforward method to design high rank codebook is to separately quantize and feedback the combining coefficients in W2 for each layer. In other words, a type-II-like codebook for rank 3/4 can be obtained by simply extending the type II codebook design for rank 1/2 with each layer quantized independently and same linear combination codebook for each layer.

However, the CSI reporting overhead with simply extension of type II codebook to high rank will be enormous. Therefore, compressing the feedback overhead of high rank codebook shall be considered, for example [2], so that CSI feedback overhead using high resolution codebook for low and high rank CSI reporting can be similar by balancing system performance gain and codebook design requirements.

Performance evaluations of codebooks with rank-3/4

Preliminary system-level simulation results of rank adaptation are illustrated in Fig. 2. The performance gain of type II in Rel-15 and type-II-like codebook over type I codebook in Rel-15 is compared. In terms of codebook design of type-II-like codebook, Rel-15 type II codebook is reused for rank 1/2 and the design for type II codebook is simply extended to rank 3/4 for each layer of rank 3/4. In the simulation, the maximum rank of type-II-like codebook is 4, while the maximum rank of Rel-15 type II is 2, as supported in Rel-15. For the baseline using type I codebook, the maximum rank of Type I codebook is 4.

It can be seen that type II codebook has a significant performance gain over type I codebook. The performance for type-II-like codebook has about 8% gain over the Rel-15 type II codebook with maximum rank 2, which means it is necessary to extend type II codebook for rank 3 and 4.

Figure 2. Performance comparison of type I, type II codebook in Rel-15 and type-II-like codebook.

Observation 2: Extending type II codebook with rank 3 and 4 can provide around additional 8% cell average gain compared with Rel-15 type II with rank 1/2 only, at least for the scenario with high RU.

Proposal 1: Type II codebook should be extended to rank 3 and 4 in Rel-16.

Potential Type-II codebook enhancement for rank-3/4

In the above sections, we have shown the benefits for type-II with extension to rank-3 and rank-4. For further optimizing the codebook design with considering performance and CSI feedback overhead, in the following, we discuss potential Type-II codebook enhancement for rank 3 and rank 4 for Type-II CSI feedback.

For beam combination codebook, such as type II codebook, the performance of using that codebook can be considerably better than that of using beam selection codebook such as type-I codebook, at the expense of obviously increased overhead. Considering both of performance and overhead, a hybrid codebook design with both beam selection and beam combination may be beneficial. Beam combination codebook is utilized within the first one or two layers and beam selection is utilized within layers except the first or two layers.

Fig. 3 is an illustration of hybrid codebook design. For the layers with beam combination, the beams for combination are selected from an orthogonal 2D-DFT beam group. As type II codebook in NR, wideband amplitude, subband amplitude and subband phase are feedback to represent the coefficients of linear combination. For each layer with beam selection, a beam can be selected from the same orthogonal 2D-DFT beam group with a co-phasing term between polarizations, but selected beam may be same or different from beams used for other layers with beam combination.

Figure 3. Illustration of hybrid codebook design.

Proposal 2: The hybrid design of Type-II codebook with beam combination and beam selection can be considered in Rel-16, taking into account both performance gain and CSI feedback overhead of high rank Type II codebook design.

Conclusions

The contribution discusses the type II codebook enhancement for rank>2 in Rel-16, based on which the following observations and proposal are made.

Observation 1: The probability of UE reporting rank 3/4 is 25%-30% in both UMa and UMi scenarios.

Observation 2: Extending type II codebook with rank 3 and 4 can provide around additional 8% cell average gain compared with Rel-15 type II with rank 1/2 only, at least for the scenario with high RU.

Proposal 1: Type II codebook should be extended to rank 3 and 4 in Rel-16.

Proposal 2: The hybrid design of Type-II codebook with beam combination and beam selection can be considered in Rel-16, taking into account both performance gain and CSI feedback overhead of high rank Type II codebook design.

- References

RP-181453, “WI Proposal on NR MIMO Enhancements”, 3GPP RAN#80, Samsung, La Jolla, USA, 11-14 June, 2018.

R1-1808949, “Enhancements on CSI reporting and codebook design”, 3GPP RAN1#94, Huawei, HiSilicon, Gothenburg, Sweden, 20-24 August, 2018.

- Appendix

| Parameters | UMa | UMi |
| --- | --- | --- |
| Carrier frequency | 4GHz |  |
| Subcarrier spacing | 15kHz |  |
| System Bandwidth | 10MHz |  |
| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites, 570 UEs |  |
| ISD | 500m | 200m |
| Minimum distance | 35m | 10m |
| TP antenna height | 25m | 10m |
| TP Tx power | 46dBm/10MHz | 41dBm/10MHz |
| TP antenna configuration | X-pol (+/-45), (dH, dV) = (0.5, 0.8) λ, (M, N, P, Mg, Ng) = (4,4,2,1,1) |  |
| UE antenna configuration | X-Pol (M, N, P, Mg, Ng) = (1,2,2,1,1) |  |
| UE antenna pattern | Omnidirectional |  |
| UE antenna height | 1.5m |  |
| UE dropping | indoor UE 80%, outdoor UE 20% |  |
| UE speed | indoor UE 3km/h, outdoor UE 30km/h |  |
| UE receiver | MMSE-IRC |  |
| Scheduler | PF |  |
| Traffic model | Non full buffer |  |