3GPP TSG-RAN WG1 Meeting #94	Tdoc R1- 1809042

Göteborg, Sweden, August 20th – 24th 2018

Agenda Item:	7.1.6

Source:	Ericsson

Title:	Maintenance of Rel-15 URLLC

Document for:	Discussion, Decision

# 1	Introduction

In RAN1#93, NR Rel-15 standardization has been completed for physical layer, including the URLLC feature. In this contribution we point out corrections to the specifications for Rel-15 URLLC. It is proposed that the text proposals be adopted for the maintenance of URLLC.

# 2	Discussion

## 2.1	Impact on TS 38.300

TS 38.300 contains the overall description of NR. Currently, the overview of URLLC in Section 16.1 of TS 38.300 [1] has only a list of RAN2 features. It is necessary that RAN1 features are added to provide a sufficient understanding of the Rel-15 URLLC. Specifically, the following list of RAN1 features can be captured:

- Reliability enhancements, such as

  - CQI and MCS tables for URLLC;

- Latency enhancements, such as:

  - DL pre-emption;

  - UL configured grants (i.e., grant-free access);

In section 3.1 of this contribution, the text proposal for TS 38.300 describes the features above. It is recommended that this text proposal be adopted to TS 38.300 Section 16.1.

Adopt text proposal in section 3.1 of the contribution to TS 38.300 Section 16.1.

## 2.2	Impact on TS 38.214

It has been agreed that the ‘new-RNTI’ is introduced to indicate the usage of the new 64-QAM MCS table, i.e., Table 5.1.3.1-3 of TS 38.214. For TS 38.214 [2], the new-RNTI has to be added everywhere together with C-RNTI since new-RNTI and C-RNTI in general are used for user data. The following parts of TS 38.214 are affected:

- Section 5.1;

- Table 5.1.2.1.1-1;

- Section 5.1.2.3;

- Section 5.1.6.2;

- Table 6.1.2.1.1-1.

Considering brevity of the contribution, text proposals for this editorial change is not included.

Add “new-RNTI” along with “C-RNTI” in TS38.214 Sections 5.1, 5.1.2.3, 5.1.6.2 texts as well as in tables 5.1.2.1.1-1 and 6.1.2.1.1-1.

Introduction of new MCS tables for URLLC has a potential  impact on PT-RS sections in [2]. The presence of PT-RS is determined by the entry in the MCS table when performing scheduling, unless the higher layer parameters timeDensity and frequencyDensity have been configured. Effectively, PT-RS is default not present for BPSK or QPSK modulation. However, since the aim is not spectral efficiency for URLLC, rather robustness, it is preferable if PT-RS is present in the downlink, by default. Similar reasoning applies to uplink. Still, it can be useful to not transmit PT-RS for the very small scheduling bandwidths of 1 or 2 RB. Hence, the specification change is minimal and the text proposal for this change can be found in subsection 3.2 of this contribution.

Adopt text proposal from section 3.2 of the contribution to TS 38.214 sections 5.1.6.3 and 6.2.3.1.

## 2.3	Impact on TS 38.211

In TS38.211 [3], “new-RNTI” should be used instead of C-RNTI for scrambling and sequence generation in the relevant procedures. It impacts the following sections of TS 38.211:

- Section 6.3.1.1 ”Scrambling” of PUSCH,

- Section 6.4.1.1.1.1 ”Sequence generation when transform precoding is disabled”

- Section 6.4.1.2.2.1 ” Precoding and mapping to physical resources if transform precoding is not enabled”

- Section 7.3.1.1 ”Scrambling” of PUSCH,

- Section 7.3.2.3 ”Scrambling” of PDCCH,

- Section 7.4.1.1.1 ”Sequence generation” of PDSCH DMRS

Considering brevity of the contribution, text proposals for this editorial change is not included.

Add “new-RNTI” in TS38.211 Sections 6.3.1.1, 6.4.1.1.1.1, 6.4.1.2.2.1, 7.3.1.1, 7.3.2.3 and 7.4.1.1.1 text along with “C-RNTI”.

## 2.3	Impact on TS38.213

In TS38.213 [4], the new-RNTI has to be added together with C-RNTI since new-RNTI and C-RNTI in general are used for user data.

- Section 9.1 “HARQ-ACK codebook determination”:

- Section 10.1 ”UE procedure for determining physical downlink control channel assignment”

Considering brevity of the contribution, text proposals for this editorial change is not included.

Add “new-RNTI” in TS38.213 Sections 9.1 and 10.1 texts along with “C-RNTI”.

# 3	Text Proposals

## 3.1	TS 38.300

====================== Start of Text Proposal to TS 38.300 ======================

### 16.1.4	CQI and MCS

For channel state reporting, A CQI table for target block error rate 10-5 is introduced.

For scheduling data packets with higher reliability, 64QAM MCS tables containing entries with lower spectral efficiency are introduced for both downlink and uplink. The tables are different for CP-OFDM and DFT-s-OFDM. The MCS tables can be configured semi-statically or dynamically. The dynamic signalling of MCS table is supported by configuring UE with “new-RNTI”, where “new-RNTI” indicates the 64QAM MCS tables with entries of lower spectral efficiency.

### 16.1.5	Downlink pre-emption

Downlink pre-emption is supported, where ongoing data transmission(s) with already allocated resources are overridden by high-priority transmission. To remedy the impact to ongoing data transmission, pre-emption indication DCI or bit-flag in retransmission DL grant can be used.

### 16.1.6	Configured uplink grants

In uplink, two types of configured uplink grants are defined: Type1 Configured Uplink Grants and Type 2 Configured Uplink Grants. When either Type 1 or Type 2 is configured, the UE can autonomously start the uplink data transmission according to the configured periodicity and radio resources.

====================== End of Text Proposal to TS 38.300 ======================

## 3.2	TS38.214

====================== Start of Text Proposal to TS 38.214 ======================

#### 5.1.6.3	PT-RS reception procedure

<…>

If a UE is configured with the higher layer parameter phaseTrackingRS in DMRS-DownlinkConfig,

-	the higher layer parameters timeDensity and frequencyDensity in PTRS-DownlinkConfig indicate the threshold values ptrs-MCSi, i=1,2,3 and NRB,i , i=0,1, as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2, respectively.

-	if either or both of the additional higher layer parameters timeDensity and frequencyDensity are configured, and the RNTI equals new-RNTI, C-RNTI or CS-RNTI, the UE shall assume the PT-RS antenna ports' presence and pattern are a function of the corresponding scheduled MCS of the corresponding codeword and scheduled bandwidth in corresponding bandwidth part as shown in Table 5.1.6.3-1, and Table 5.1.6.3-2, and Table 5.1.6.3-3,

<…>

-	otherwise, if neither of the additional higher layer parameters timeDensity and frequencyDensity are configured and the RNTI equals new-RNTI, C-RNTI or CS-RNTI, the UE shall assume the PT-RS is present with LPT-RS = 1, KPT-RS = 2, and

-	The UE shall assume PT-RS is not present when,

-	the scheduled MCS from Table 5.1.3.1-1 is smaller than 10, or

-	the scheduled MCS from Table 5.1.3.1-2 is smaller than 5, or

-	the number of scheduled RBs is smaller than 3, or

<…>

### 6.2.3	UE PT-RS transmission procedure

If a UE is not configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig, the UE shall not transmit PT-RS. The PTRS may only be present if RNTI equals new-RNTI, C-RNTI, CS-RNTI, SP-CSI-RNTI.

#### 6.2.3.1	UE PT-RS transmission procedure when transform precoding is not enabled

<…>

The higher layer parameter PTRS-UplinkConfig provides the parameters ptrs-MCSi, i=1,2,3 and with values in 0-29 when MCS Table 5.1.3.1-1 or Table 5.1.3.1-3 is configured, and 0-28 when MCS Table 5.1.3.1-2 is configured, respectively. ptrs-MCS4 is not explicitly configured by higher layers but assumed 29 when MCS Table 5.1.3.1-1 or Table 5.1.3.1-3 is configured, and 28 when MCS Table 5.1.3.1-2 is configured. The higher layer parameter PTRS-UplinkConfig provides the parameters NRBi i=0,1 with values in range 0-276.

<…>

====================== End of Text Proposal to TS 38.214 ======================

## 3.3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt text proposal in section 3.1 of the contribution to TS 38.300 Section 16.1.

Proposal 2	Add “new-RNTI” along with “C-RNTI” in TS38.214 Sections 5.1, 5.1.2.3, 5.1.6.2 texts as well as in tables 5.1.2.1.1-1 and 6.1.2.1.1-1.

Proposal 3	Adopt text proposal from section 3.2 of the contribution to TS 38.214 sections 5.1.6.3 and 6.2.3.1.

Proposal 4	Add “new-RNTI” in TS38.211 Sections 6.3.1.1, 6.4.1.1.1.1, 6.4.1.2.2.1, 7.3.1.1, 7.3.2.3 and 7.4.1.1.1 text along with “C-RNTI”.

Proposal 5	Add “new-RNTI” in TS38.213 Sections 9.1 and 10.1 texts along with “C-RNTI”.

# References

3GPP TS 38.300, NR and NG-RAN Overall Description, Stage 2, V15.2.0, 2018-06

3GPP TS 38.214, V15.2.0 (2018-06)

3GPP TS 38.211, V15.2.0 (2018-06)

3GPP TS 38.213, V15.2.0 (2018-06)