3GPP TSG RAN WG1 #94																				     R1-1809079

Gothenburg, Sweden, August 20th – 24th, 2018

Source: 	InterDigital Inc.

Title:	Complexity Analysis of IDMA NOMA Reception

Agenda item:	7.2.1.2

Document for:	Discussion and Decision

# Introduction

In RAN1 WG1 Meeting #93 [1], RAN1 continued the discussion for evaluation of NOMA schemes. Moreover, some aspects related to receiver processing and complexity for NOMA was discussed, and the following related agreement was reached,

- In performing performance evaluation, companies should provide analsysis of receiver complexity. Particularlly (with details FFS): - Detector complexity - Decoding complexity - Interference cancellation complexity, if any - Number of iteration(s), if any - Other receiver optimization, if any - Complexity for the preamble/DMRS detection - Memory requirements - Latency - FFS which simulation cases to be selected for evaluation - Discuss further next meeting potential template capturing the complexity analysis, especially regarding the level of details in the analysis

In this contribution, we describe the complexity of IDMA MUD receiver. More specifically, we discuss the required processing in terms of core processing usage and latency.

# Discussion

# Complexity of IDMA Receiver Processing

IDMA belongs to a family of NOMA schemes which operate at the chip level. It allows to multiplex multiple users on the same time-frequency resources. At its core, IDMA relies on user specific interleaver to distinguish the overlapping signals from multiple users and low-rate channel coding to recover the signal affected by multiple user interference. Simple low-complexity implementation, flexible architecture for adapting to required use cases and spectral efficiency, high overloading potential, and the possibility to operate in some asynchronous transmission scenarios are among advantages of IDMA.

As demonstrated in Figure 1, in an IDMA receiver, the combined signal is jointly processed in a recursive manner through a two-step process of elementary signal estimator (ESE) and decoding [2]. A complex baseband representation of the received signal at l th, antenna  l∈1, 2, …, L at the input of the ESE can be expressed as,

Figure 1 Simplified structure of an IDMA receiver

rlj=k=1Khk, ljxkj+nj,

= k=1Khk,lRe(i)xkRe(j)-hk,lIm(j)xkIm(j)+hk,lRe(j)xkIm(j)-hk,lIm(j)xkRe(j)+nj,  j=1, 2, …, J

rlj is the received signal at chip j. K is the number of multiplexed users/streams. hk,lRe(j) and xk,lRe(j) represent the real parts of the channel gains hk, lj and transmitted signal xkjon chip j. hk,lIm(i) and xk,lIm(i) are the corresponding imaginary parts. n(i) is the received noise sample of variance σ2 per dimension. The above presentation can be simplified to

rlj=hk, l(j) xkj+ ζk, l(j)

where ζk, lj  is the interference-plus-noise.

In the first step of detection process, the ESE function provides updates on a-priori log likelihood ratio (LLR) estimates required for the decoding based on the measurements performed on the received signal and the a-posteriori LLR estimate of previous decoding attempts, if available. Then, the receiver enters the second step by de-interleaving, and attempting to decode each user, yielding updated a-posteriori LLR estimates, 𝑒𝐷𝐸𝐶 (𝑥𝑘 (𝑗)), for the next round of ESE updates. The turbo-like cycle of updates is simultaneously iterated several times across the users to complete the detection process. Details of the receiver processing are summarized and captured in the Appendix. Based on the discussion and provided details in the Appendix, the following observations can be made;

Observation 1: In IDMA MUD algorithm, most of the operations can be performed on a chip-by-chip basis.

Observation 2: In IDMA MUD, the complexity cost per information bit per user is independent from the number of users.

### Systemic versus Implementation Comparison

Based on the provided details in the Appendix, Table 1 summarizes the overall complexity of an IDMA receiver. In estimating the complexity of a receiver, it is important to limit the scope of the complexity and latency analysis to the systemic operation of the receiver. In other words, implementation and processing latency of certain functions in a receiver depends on the available processing power of a given hardware such as: processing clock, memory size, availability of dedicated basic functions, etc. However, the systemic aspect of receiver processing of a given scheme remains unchanged regardless of the implementation choice. Therefore, the scope of the complexity analysis should be constrained to estimating complexity of core detector block (ESE, EPA, etc.) and number of outer loops. As such, the complexity of the FEC decoding is not relevant as it is a basic NR function.

Proposal 1: The scope of the complexity analysis should be constrained to estimating complexity of core detector block (ESE, EPA, etc.) and number of outer loops. As such, the complexity of the FEC decoding is not relevant as it is a basic NR function.

| Table 1 – Summary of complexity estimation for IDMA |  |  |  |
| --- | --- | --- | --- |
| Computational complexity |  | Type | Processing Complexity |
| Detector algorithm |  | ESE | NA=(28 + 2L)K Additions NM=43K Multiplications |
| Decoder algorithm |  | LDPC | NLDPC= BP with flooding (50 iterations per block) |
| Number of iterations |  | Outer loop iterations | Nouter |
| Total complexity |  |  | Nouter (NA+ NM + NLDPC) |
| Memory requirements |  |  | Implementation dependent |
| Processing Latency |  |  | Depends on processing clock |

# Conclusions

In this contribution we have provided details on IDMA MUD. It has been observed that the complexity cost per information bit per user is independent of the number of users. Also it has been observed that MUD detection can be performed on chip-by-chip basis, reducing the complexity of the MUD significantly.  Our main observations and proposal are listed below.

Observation 1: In IDMA MUD algorithm, most of the operations can be performed on  chip-by-chip basis.

Observation 2: In IDMA MUD, the complexity cost per information bit per user is independent from the number of users.

Proposal 1: The scope of the complexity analysis should be constrained to estimating complexity of core detector block (ESE, EPA, etc.) and number of outer loops. As such, the complexity of the FEC decoding is not relevant as it is a basic NR function.

# References

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #93, Busan, South Korea, May 2018

- L. Ping, L. Liu, K. Wu and W.K. Leung, “Interleave-Division Multiple-Access,” IEEE Transaction on Wireless Communications, vol. 5, no. 4, April 2006

# Appendix

## IDMA MUD Detection Steps

## Algorithm: IDMA multi-user detection

Step (1): Initialization

Evaluate ExkRe(j),ExkIm(j), VarxkRe(j), and VarxkIm(j) assuming no prior knowledge about transmitted information bits, i.e.,   eDECRexkj=0,  eDECImxkj=0, j=1, 2, …, J

ExkRe(j)=tanheDECRexkj2 ………… (1)

VarxkRe(j)=1-ExkRej………… (2)

ExkIm(j)=tanheDECImxkj2 ………… (3)

VarxkIm(j)=1-ExkImj………… (4)

Step (2): Iteration: for t=1:Nitr, do

Step (2.1) Estimate mean and the variance of inference

ErlRej=k=1Khk,  lRe(j) ExkRe(j)-hk, lIm(j)ExkIm(j) ………… (5)

ErlImj=k=1Khk,  lRe(j) ExkIm(j)-hk, lIm(j)ExkRe(j) ………… (6)

ERehk, l(j) ζk, l(j)=hk, lRe(j)ErlRe(j)+hk, lIm(j)ErlIm(j)-hk,l(j)2ExkRe(j)  ………… (7)

VarrlRe(j)= k=1Khk, lRe(j)2 VarxkRe(j)+ hk, lIm(j)2 VarxkIm(j)+σ2………… (8)

VarrlIm(j)= k=1Khk, lIm(j)2 VarxkRe(j)+ hk, lRe(j)2 VarxkIm(j)+σ2………… (9)

ψRej= k=1Khk, lRe(j)hk, lIm(j)VarxkRe(j)- VarxkIm(j)………… (10)

VarRehk, l(j)ζk, lj= hk, lRe(j)2VarrlRej+hk, lIm(j)2VarrlImj+2 hk, lRe(j)hk, lIm(j)ψRej                                                                   -hk, l(j)4VarxkRe(j)………… (11)

EImhk,ljζk,lj=hk,lReErImj-hk,lImErRej-hk,lj2ExkRej………… (12)

ψImj=k=1Khk, lRe(j)hk, lIm(j)VarxkIm(j)- VarxkRe(j)………… (13)

VarImhk,l(j)ζk,l(j)=hk,lRe(j)2VarrIm(j) + hk,lIm(j)2VarrRe(j)+2hk,lRejhk,lImjψImj-                                                  hk,lj4VarxkImj………… (14)

Step (2.2): Calculate ESE LLRs

eESERexkImjl=2hk, lRe(j)2Rehk, l (j) rl(j)-ERehk, l jζk, l(j)VarRehk, l(j)ζk, l(j)  ………… (15)

eESExkImjl=2hk,lj2Imhk, l (j) rl j-EImhk,ljζk,ljVarImhk,l(j)ζk,l(j)  ………… (16)

Step (2.3): LLR combining

eESExkRej=l=1LeESExkRejl………… (17)

eESExkImj=l=1LeESExkImjl………… (18)

Step (2.4): Calculate eDECxkj, j=1, 2, …, J, and k=1, 2, …, K

Step (2.5): Update ExkRe(j),ExkIm(j), VarxkRe(j), and VarxkIm(j) using (1) – (3)

end

## IDMA MUD Complexity Analysis

Table 1 summarize the number of operations to be performed for each operation in the algorithm.

Table 1: IDMA MUD complexity

| Eq. # | Step shared by all UEs | Additions per chip | Multiplications per chip | Divisions per chip | Exponentials per chip |
| --- | --- | --- | --- | --- | --- |
| 4 | No | 2K |  | K | 2K |
| 5 | No | K |  |  |  |
| 6 | No | 2K |  | K | 2K |
| 7 | No | K |  |  |  |
| 8 | Yes | 2K | 2K |  |  |
| 9 | yes | 2K | 2K |  |  |
| 10 | No | 2K | 4K |  |  |
| 11 | yes | 2K | 4K |  |  |
| 12 | Yes | 2K | 4K |  |  |
| 13 | yes | K | 2K |  |  |
| 14 | No | 3K | 11K |  |  |
| 15 | No | 2K | 4K |  |  |
| 16 | Yes | K | 2K |  |  |
| 17 | No | 3K | 2K |  |  |
| 18 | No | K | 3K | K |  |
| 19 | No | K | 3K | K |  |
| 20 | No | LK |  |  |  |
| 21 | No | LK |  |  |  |
| Total |  | (28 + 2L)K | 43K | 4K | 4K |