3GPP TSG RAN WG1 Meeting #94								    R1-1809080

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:	7.2.1.3

Source:	InterDigital Inc.

Title:	On NOMA Related Procedures in NR

Document for:	Discussion and Decision

# Introduction

In the SID [2], the study on non-orthogonal multiple access (NOMA) will further progress on the evaluation of NOMA schemes focusing on uplink and provide recommendation on the NOMA scheme(s) to be specified later. Agreements, observations and evaluation assumption in Rel-14 study shall be the starting point. The detailed objectives are to study the non-orthogonal multiple transmission scheme for the following:

- Transmitter side signal processing schemes for non-orthogonal multiple access

- Receivers for non-orthogonal multiple access

- Procedures related to the non-orthogonal multiple access

- Link and system level performance evaluation or analysis for non-orthogonal multiple access

A common solution for mMTC, URLLC and eMBB small packet is targeted. NOMA studies in 3GPP started with the NOMA SI in Rel 14 where different NOMA schemes were investigated. Up to RAN1#86b meeting, preliminary simulations were performed and the NOMA schemes were shown to provide significant benefits and gains in throughput, overloading and handling large packet arrival rates. The results were described and summarized in [1] and captured in TR38.802 which includes the simulation assumptions. At RANP #75, a new Study Item on NOMA was approved and further revised at RANP #76 [2].

In this contribution, we discussed the aspects of procedures related to NOMA.

# Procedures Related to Non-Orthogonal Multiple Access

According to SID [2], the objective of procedures related to NOMA is to study the following:

- UL transmission detection

- HARQ, including transmission scheme, feedback scheme and combining scheme

- Link adaptation for MA signature allocation and selection

- Synchronous and asynchronous operation

- Adaptation between orthogonal and non-orthogonal multiple access

When NOMA is designed for each of these use cases and scenarios, e.g., URLLC, mMTC and eMBB, specific solutions regarding scheduling, initial transmission and retransmission are required. One particular problem is the multiple access (MA) collision issue for NOMA. This may occur when multiple UEs select the same MA signature and/or demodulation reference signal (DMRS) in the same frequency/time resource. In case of collision, gNB cannot detect or distinguish one UE from the other UE.

Collision, either MA signature or DMRS collision could significantly degrade MA performance for NOMA. Especially for high overloading condition where many UEs may be active and accessing the same resource at the same time. The likelihood that UEs may select the same MA signature or DMRS could be high. The higher the overloading factor of system is, the higher the likelihood that users may have selected the same MA signature or DMRS, and as a result, the higher likelihood of collision for UEs could occur.

For grant-based approach, MA signature or DMRS can be assigned individually to each user, therefore collision can be mostly avoided by proper scheduling of gNB. However, for grant-free based approach, MA signature or DMRS cannot be assigned individually to each user since no grant is used for such purpose, therefore collision cannot be avoided by scheduling of gNB. Collision may be mitigated by proper configuration or pre-configuration of gNB for MA signature or DMRS. However, there may still be possibility that collision could occur and users may still select the same MA signature or DMRS.

Configuration or pre-configuration for MA signature or DMRS from gNB to avoid collision may be possible for use cases or scenarios such as URLLC or eMBB in which less number of users may be allocated in the same resource. For mMTC scenario or use case, due to large number of connectivity, configuration or pre-configuration of gNB for MA signature or DMRS to avoid collision may become less feasible or less possible. Especially, overloading factor for mMTC could be very high. Therefore, collision mitigation should be considered for NOMA for different scenarios or use cases. In general, in order to optimize the NOMA performance, collision mitigation methods should be studied for evaluation use cases or scenarios at least for URLLC and mMTC and possibly for eMBB.

Proposal 1: Impact of collision for DMRS and MA signature on NOMA performance should be studied.

# Collision Handling for NOMA

In order to handle collision in NOMA, one possible way may be to utilize MA signature and DMRS grouping. For example, multiple access (MA) signatures may be divided or partitioned into two MA groups. MA signature Group A may be “dedicated” MA signatures and MA signature Group B may be “shared” MA signatures.

A UE may be assigned a MA signature for transmission and/or retransmission using group A. For example, to reduce the latency due to retransmission failure, UE may be assigned a dedicated MA signature from group A to avoid collision. UE may use a shared MA signature from group B for initial transmission to increase channel access efficiency. In order to trade off between channel access efficiency, latency and performance for transmission and retransmission, MA signature groups may be used in different combinations of transmission and retransmission. The association of MA signature groups and transmission/retransmission may be predetermined, configured or indicated by signaling either implicitly or explicitly, in a semi-static or dynamic fashion. MA groups may be configured and the size of group may also be configured.

Proposal 2: How to handle collision should be studied for NOMA.

# NOMA (Re)-Transmission

Several aspects related to (re)-transmission for NOMA may be the following:

- Grant-based and grant-free utilization

- OMA and NOMA resource utilization and adaptation

- Resource for initial transmission and retransmission

- Resource size for initial transmission and retransmission

One of the issues is that how to ensure the performance of NOMA (re)-transmission for different use cases and scenarios. Solutions regarding enhanced NOMA (re)-transmission for different use cases and scenarios are required. Grant-based and grant-free approaches can be used for NOMA. However, how to efficiently utilize grant-based and grant-free approaches to enhance NOMA operation should be studied. Transmission and retransmission with either grant-free or grant-based approaches can be considered for NOMA. In addition, resource utilization for OMA or NOMA resources should be studied for performance enhancement. For example, in order to have both low latency and high reliability, both initial transmission and retransmission should be as low latency and as high reliable and high performance as possible. To achieve low latency and high performance, one possibility could be that grant-free may be employed for initial transmission and grant-based may be employed for retransmission.

Another aspect is that both orthogonal multiple access (OMA) and NOMA can be possibly used by UEs in a system. Depending on scenarios or use cases, adaptation between OMA and NOMA may be needed. By proper adaptation between OMA and NOMA resources, performance may be improved for a targeted evaluation scenario e.g., URLLC or eMBB.

Transmission and retransmission may employ different resources for NOMA. For example, one set of resources can be dedicated for transmission while another set of resources can be dedicated for retransmission. By doing so, transmission and retransmission can be distinguished.

Transmission and retransmission may employ different size of resources for NOMA. For example, transmission may employ resource of one resource size while retransmission may employ resource of another resource size. For example, retransmission may utilize larger size for resource to reduce the collision for MA signature or DMRS during retransmission.

Proposal 3: Efficient (re)-transmission for NOMA should be investigated.

# Assistance Information for NOMA

When multiple UEs select the same MA signature or DMRS, collision may occur during HARQ retransmission. How to avoid the collision for NOMA retransmission is essential for the success of HARQ operation for NOMA. In order to avoid the collision, one way is to extend the resource size or utilize grant-based approach. Another way may be to provide additional information to UE regarding collision information such that UE could avoid using the same MA signature(s) or DMRS(s) that have been already in use.

For example, UE may receive assistance information to management collision. One possibility is that gNB may communicate with UEs regarding the already used MA signatures or DMRSs, such that UEs could have the prior knowledge about what resources are available and what are not. UE may not be allowed to select the MA signature or DMRS from the already used MA signature and DMRS which are considered not available MA signature and DMRS. Instead, UE may select the MA signature or DMRS from the still available MA signature and DMRS in the MA signature and DMRS pool.

# Conclusions

In this contribution, we considered and discussed the procedures related to NOMA. We have the following proposals:

Proposal 1: Impact of collision for DMRS and MA signature on NOMA performance should be studied.

Proposal 2: How to handle collision should be studied for NOMA.

Proposal 3: Efficient (re)-transmission for NOMA should be investigated.

# References

- R1-1611251, “Summary of simulation results for NR MA”, 3GPP TSG RAN WG1 Meeting #87, November 2016, Reno, USA

- RP-171043, “Revision of Study on 5G Non-Orthogonal Multiple Access”, 3GPP RAN#76, Dubrovnik, Croatia, March 6 - 9, 2017.