3GPP TSG RAN WG1 Meeting #94			R1-1809171

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	NTT DOCOMO, INC.

Title:	Preliminary evaluation results of reliability for IMT-2020 self-evaluation

Agenda Item:	7.2.7.2

Document for: 	Discussion

- Introduction

Reliability is the successful probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality. The minimum requirement for reliabilty is 1-10-5 success probability of transmitting a layer 2 PDU of 32 bytes within 1 ms in channel quality of coverage edge [1].

Methodolgy of reliability evaluation is showed as follow [2].

Methodology of reliability evaluation:

- Step 1:  Run downlink or uplink full buffer system-level simulations of candidate RITs/SRITs using the evaluation parameters of Urban Macro-URLLC test environment see § 8.4.1 below, and collect overall statistics for downlink or uplink SINR values, and construct CDF over these values.

- Step 2:	. Use the CDF for the Urban Macro-URLLC test environment to save the respective 5th percentile downlink or uplink SINR value

- Step 3:	Run corresponding link-level simulations for either NLOS or LOS channel conditions using the associated parameters in the Table 8-3 of this Report, to obtain success probability, which equals to (1-Pe), where Pe is the residual packet error ratio within maximum delay time as a function of. SINR taking into account retransmission

- Step 4:	The proposal fulfils the reliability requirement if at the 5th percentile downlink or uplink SINR value of Step 2 and within the required delay, the success probability derived in Step 3 is larger than or equal to the required success probability. It is sufficient to fulfil the requirement in either downlink or uplink, using either NLOS or LOS channel conditions.

Methodology of reliability evaluation:

- Step 1:  Run downlink or uplink full buffer system-level simulations of candidate RITs/SRITs using the evaluation parameters of Urban Macro-URLLC test environment see § 8.4.1 below, and collect overall statistics for downlink or uplink SINR values, and construct CDF over these values.

- Step 2:	. Use the CDF for the Urban Macro-URLLC test environment to save the respective 5th percentile downlink or uplink SINR value

- Step 3:	Run corresponding link-level simulations for either NLOS or LOS channel conditions using the associated parameters in the Table 8-3 of this Report, to obtain success probability, which equals to (1-Pe), where Pe is the residual packet error ratio within maximum delay time as a function of. SINR taking into account retransmission

- Step 4:	The proposal fulfils the reliability requirement if at the 5th percentile downlink or uplink SINR value of Step 2 and within the required delay, the success probability derived in Step 3 is larger than or equal to the required success probability. It is sufficient to fulfil the requirement in either downlink or uplink, using either NLOS or LOS channel conditions.

At the RAN1#93 meeting, the details of reliability evaluation methodology and parameters were agreed [3]. Based on it, we present the initial evaluation results of reliability of UL.

- System level evaluation results of pre-processing SINR

In this section, we provide the pre-processing SINR on Urban Macro URLLC ConfigA and ConfigB.

In Figure 1 and 2, the CDF curves for UL pre-processing SINR on Urban Macro URLLC Config A and B scenario are provided. To align the bandwidth between system level and link level simulation for PUSCH transmission, in which different modulation and coding schemes lead different bandwidth occupancy, we provide the UL pre-processing SINRs considering bandwidth of 10/20/35 RBs. The evaluation assumptions could be found in Annex 1.

Figure 1． CDF curve for UL pre-processing SINR on Urban Macro URLLC ConfigA

Figure 2． CDF curves for UL pre-processing SINR on Urban Macro URLLC ConfigB

In Figure 3, the CDF curves for DL pre-processing SINR on Urban Macro URLLC ConfigA and ConfigB are provided. The evaluation assumption could be found in Appendix 1.

Figure 3． CDF curves for DL pre-processing SINR on Urban Macro URLLC

Based on the above figures, the SINR values of 5% percentile of CDF of different scenarios are shown in the following table.

TABLE 1

| Test Environment | UL/DL | Configuration |  | 5% CDF SINR(dB) |
| --- | --- | --- | --- | --- |
| Urban Macro URLLC | UL | ConfigA | RB=10 | -2.4 |
|  |  | ConfigA | RB=20 | -2.9 |
|  |  | ConfigA | RB=35 | -4.2 |
|  |  | ConfigB | RB=10 | -1.5 |
|  |  | ConfigB | RB=20 | -1.5 |
|  |  | ConfigB | RB=35 | -1.6 |
|  | DL | ConfigA |  | -2.8 |
|  |  | ConfigB |  | -2.1 |

Observation 1: The 5% percentile SINR is -4.2dB to -2.4dB for Urban Macro-URLLC Config A, and -2.1 dB to -1.5dB for Urban Macro-URLLC Config B.

- Link level evaluation results

In this section, we provide the initial link level results of PDCCH and PUSCH. The assumptions for LLS of PDCCH and PUSCH could be found in Annex 2. Payload size of 40bit, excluding 24bit CRC, is assumed for PDCCH.  Different aggregation levels for PDCCH are assumed. Packet size of 32 bytes for PUSCH was simulated considering mini-slot configurations with 4 symbol and 7 symbol per slot with different code rates.

3.1 URLLC config.A

For URLLC config.A, the performance of BLER of PDCCH and PUSCH to received SINR is shown in Figure 4 and 5, respectively.

Figure 4. BLER of PDCCH to received SINR

Figure 5. BLER of PUSCH to received SINR

3.2 URLLC config.B

For URLLC config.B, the performance of BLER of PDCCH and PUSCH to received SINR is shown in Figure 6 and 7, respectively.

Figure 6. PDCCH average BLER as a function of received SINR

Figure 7. PUSCH average BLER as a function of received SINR

- Performance of UL reliability

4.1 Methodology of UL reliability calculation

For UL transmission, reliability and latency performance based on grant free mechanism is calculated. We utilize Alternative 1 in [3] to derive reliability under a given SINR, i.e. each transmission is independent and soft combining is not considered. Table 2 and 3 shows the descriptions of terminologies for calculation of successful probability and latency, respectively.

Table 2 Successful probability of PDCCH/PUSCH transmission

| Successful Probability | Description |
| --- | --- |
| pPDCCH | Successful probability of PDCCH transmission |
| pPUSCH | Successful probability PUSCH transmission |

Table 3 Transmission and processing time for PDCCH/PUSCH

| Time | Description |
| --- | --- |
| tPDCCH | PDCCH transmission time |
| tPUSCH | PUSCH transmission time |
| TgNB | gNB processing time for PUSCH |
| TUE | UE prepare time for PUSCH |

Then, we can describe the total reliability as:

We can describe the total latency as:

Where M is repetition times of initial PUSCH transmission, N is repetition times of PUSCH retransmission and K-1 is the number of re-transmission times triggered by DCI.

4.2  Performacne of UL reliability

When we calculate the reliability performance, two assumptions of different gNB/UE L1 processing time are considered.

- Assumption 1

- gNB processing time for PUSCH:   5.5 OFDM symbol  for SCS=30KHz

- UE prepare time for PUSCH:   5.5 OFDM symbol  for SCS=30KHz

- UE prepare time only for re-transmission

- Assumption 2

- gNB processing time for PUSCH:   1 OFDM symbol for SCS=30KHz

- UE prepare time for PUSCH:    1 OFDM symbol for SCS=30KHz

- UE prepare time only for re-transmission

4.2.1 Performacne of URLLC config.A

For 1 OFDM symbol PDCCH with aggregation level 8, pPDCCH = 1. The reliability evaluation results considering Assumption 1 and 2 are shown in Table 4 and Table 5, respectively.

Table 4 Relibility evalution results for Config.A with Assumption.1

| OS | RB | Code rate | pPUSCH | M | N | K | PUL | TUL (ms) | Meet ITU requirement |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OS=4 | 18 | 1/5 | 0.7513 | 5 | \ | 1 | 0.9990 | 0.912 | No |
|  | 35 | 1/10 | 0.8928 | 5 | \ | 1 |  | 0.912 | No |
| OS=7 | 9 | 1/5 | 0.7531 | 3 | \ | 1 | 0.9849 | 0.95 | No |
|  | 18 | 1/10 | 0.9614 | 3 | \ | 1 |  | 0.95 | No |
|  | 35 | 1/20 | 0.9881 | 3 | \ | 1 |  | 0.95 | Yes |

Table 5 Relibility evalution results for Config.A with Assumption.2

| OS | RB | Code rate | pPUSCH | M | N | K | PUL | TUL (ms) | Meet ITU requirement |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OS=4 | 18 | 1/5 | 0.7513 | 3 | 3 | 2 |  | 1 | No |
|  | 35 | 1/10 | 0.8928 | 3 | 3 | 2 |  | 1 | Yes |
| OS=7 | 9 | 1/5 | 0.7531 | 2 | 1 | 2 | 0.9849 | 0.89 | No |
|  | 18 | 1/10 | 0.9614 | 2 | 1 | 2 |  | 0.89 | No |
|  | 35 | 1/20 | 0.9881 | 2 | 1 | 2 |  | 0.89 | Yes |

4.2.2 Performance of URLLC config.B

For 1 OFDM symbol PDCCH with aggregation level 8, pPDCCH =0.9995. The reliability evaluation results considering Assumption 1 and 2 are shown in Table 6 and Table 7, respectively.

Table 6. Relibility evalution results for Config.B with Assumption.1

| OS | RB | Code rate | pPUSCH | M | N | K | PUL | TUL (ms) | Meet ITU requirement |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OS=4 | 18 | 1/5 | 0.9106 | 5 | \ | 1 |  | 0.912 | Yes |
|  | 35 | 1/10 | 0.9955 | 4 | \ | 1 |  | 0.768 | Yes |
| OS=7 | 9 | 1/5 | 0.8937 | 3 | \ | 1 | 0.9988 | 0.95 | No |
|  | 18 | 1/10 | 0.9918 | 3 | \ | 1 |  | 0.95 | Yes |
|  | 35 | 1/20 | 0.99994 | 2 | \ | 1 |  | 0.697 | Yes |

Table 7. Relibility evalution results for Config.B with Assumption.2

| OS | RB | Code rate | pPUSCH | M | N | K | PUL | TUL (ms) | Meet ITU requirement |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OS=4 | 18 | 1/5 | 0.9106 | 2 | 3 | 2 |  | 1 | Yes |
|  | 35 | 1/10 | 0.9955 | 3 | 3 | 2 |  | 1 | Yes |
| OS=7 | 9 | 1/5 | 0.8937 | 2 | 1 | 2 | 0.9988 | 0.89 | No |
|  | 18 | 1/10 | 0.9918 | 2 | 1 | 2 |  | 0.89 | Yes |
|  | 35 | 1/20 | 0.99994 | 2 | 1 | 2 |  | 0.89 | Yes |

Observation 2：With Assumption 1, IMT-2020 reliability requirements for Configuration A (4 GHz) UL can be met with 7-OS mini-slot configuration.

Observation 3: With Assumption 2, IMT-2020 reliability requirements for Configuration A (4 GHz) UL ca nbe met with both 7-OS mini-slot and 4-OS mini-slot configuration.

Observation 4：With Assumption 1, IMT-2020 reliability requirements for Configuration B (700 MHz) UL can be met with both 4-OS mini-slot and 7-OS mini-slot configuration.

Observation 5：With Assumption 2, IMT-2020 reliability requirements for Configuration B (700 MHz) UL can be met with 4-OS mini-slot and 7-OS mini-slot configuration.

- Summary

In this contribution, the initial evaluation results of reliability are provided. The following observation are made.

Observation 1: The 5% percentile SINR is -4.2dB to -2.4dB for Urban Macro-URLLC Config A, and -2.1 dB to -1.5dB for Urban Macro-URLLC Config B.

Observation 2：With Assumption 1, IMT-2020 reliability requirements for Configuration A (4 GHz) UL can be met with 7-OS mini-slot.

Observation 3: With Assumption 2, IMT-2020 reliability requirements for Configuration A (4 GHz) UL ca nbe met with both 7-OS mini-slot and 4-OS mini-slot.

Observation 4：With Assumption 1, IMT-2020 reliability requirements for Configuration B (700 MHz) UL can be met with both 4-OS mini-slot and 7-OS mini-slot.

Observation 5：With Assumption 2, IMT-2020 reliability requirements for Configuration B (700 MHz) UL can be met with 4-OS mini-slot and 7-OS mini-slot.

References

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, June 2017.

- R1-1807760, “Summary on offline discussion on URLLC evaluation method and parameters for IMT-2020 self-evaluation”, Huawei, May 2018.

Annex 1: Assumptions of system level simulations

| Urban Macro - URLLC | Config. A | Config. B |
| --- | --- | --- |
| Carrier frequency for evaluation | 4 GHz | 700 MHz |
| BS antenna height | 25 m | 25 m |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 100% low loss (applies to Channel model B) | 100% low loss (applies to Channel model B) |
| Inter-site distance | 500 m | 500 m |
| Number of antenna elements per TRxP | 64 Tx/Rx = (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 16 Tx/Rx = (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 8TXRU=(Mp,Np,P,Mg,Ng) = (1,4,2,1,1) | 2TXRU=(Mp,Np,P,Mg,Ng) = (1,1,2,1,1) |
| Number of UE antenna elements | 2Tx/Rx = (M,N,P,Mg,Ng) = (1,1,2,1,1), (dH,dV) = (0.5, 0.5)λ 0°, 90° polarization | 2Tx/Rx = (M,N,P,Mg,Ng) = (1,1,2,1,1), (dH,dV) = (0.5, 0.5)λ 0°, 90° polarization |
| Number of TXRU per UE | 2TXRU (1-to-1 mapping) | 2TXRU (1-to-1 mapping) |
| Device deployment | 80% outdoor, 20% indoor Randomly and uniformly distributed over the area | 80% outdoor, 20% indoor Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction |
| UE speeds of interest | 3 km/h for indoor and 30 km/h for outdoor | 3 km/h for indoor and 30 km/h for outdoor |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi |
| UE antenna element gain | 0 dBi | 0 dBi |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | Full buffer | Full buffer |
| Simulation bandwidth | 2MHz/4MHz/7Mhz | 2MHz/4Mhz/7Mhz |
| UE density | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height | 1.5 m | 1.5 m |
| Channel model variant | Channel model B | Channel model B |
| TRxP number per site | 3 | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | [95°] in LCS | [99°] in LCS |
| Handover margin (dB) | 0 | 0 |
| TRxP boresight | 30 / 150 / 270 degrees | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula) from port 0 | Based on RSRP (formula) from port 0 |
| Wrapping around method | Geographical distance based wrapping | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=10m | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 |

Annex 2: Assumptions of link level simulations

Aussumptions of PDCCH

| Parameters | Value |
| --- | --- |
| DCI payload (excluding 24bits CRC) | 40bits |
| System bandwidth | 20MHz |
| Carrier Frequency | 4GHz, 700MHz |
| Number of symbols for CORESET | 1,2 |
| CORESET BW (contiguous PRB allocation) | 20MHz |
| Subcarrier spacing | 30KHz |
| Aggregation level | 4, 8, 16. |
| Transmission type | Interleaved (interleaver row =6) |
| REG bundling size | 6 |
| Resource mapping | Distributed transmission |
| Channel coding | Polar code (DCI) |
| Transmission scheme | 1-port precoder cycling |
| Channel estimation | Realistic(MMSE) |
| Channel model | TDL-C (delay spread: 300ns,UE speed: 3 km/h) |
| Number of BS antennas | 2Tx |
| Number of UE antennas | 4Rx for 4GHz, 2Rx for 700MHz |

Aussumptions of PUSCH

| Parameters |  | Value |
| --- | --- | --- |
| DCI payload (excluding 24bits CRC) |  | 256bits |
| System bandwidth |  | 20MHz |
| Carrier Frequency |  | 4GHz, 700MHz |
| Modulation |  | QPSK |
| Code rate |  | 1/20,1/10,1/5 |
| Subcarrier spacing |  | 30KHz |
| Waveform |  | CP-OFDM |
| Number of symbol for PUSCH |  | 4(1xDMRS+3xData), 7(1xDMRS+6xData), |
| DMRS configuration |  | Front-loaded DMRS (1/4 for OS4, 1/7 for OS7) |
| Channel coding |  | LDPC ( BP for decoding) |
| Channel estimation |  | Realistic(LS-CE) |
| Channel model |  | TDL-C (delay spread: 300ns,UE speed: 3km/h) |
| Number of BS antennas |  | 2Rx |
| Number of UE antennas |  | 1Tx |
| Number of PRB allocated to UE | OS=4,CR=1/5 | 18 |
|  | OS=4,CR=1/10 | 35 |
|  | OS=7,CR=1/5 | 9 |
|  | OS=7.CR=1/10 | 18 |
|  | OS=7,CR=1/20 | 35 |