3GPP TSG RAN WG1 Meeting #94 	R1-1809253

# Gothenburg, Sweden, August 20th – 24th 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.885 | CR | 0001 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of optional antenna element patterns for vehicular UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer HHI, Fraunhofer IIS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_LTE_NR_V2X_eval |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Incomplete information on 3D extrapolation of radiation patterns, - usage of wrong symbols, - parameter tables are inserted as figures instead of tables, - vertical cut of radiation power pattern is discontinous at the poles, - the antenna patterns for Option 2 already incorporate the orientation of the antennas wrt the vehicle, while the vehicle is headed towards the negative x-axis |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added paragraph on 3D extrapolation of radiation patterns, Removed incomplete information from the tables, Added reference about extrapolation method - corrected symbols - inserted parameter tables as tables instead of figures - replaced the two vertical cuts (front and back) by one continous function - added note clarifying the orientation of the vehicle wrt to the antennas |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - not possible to use 3D extrapolation method to generate 3D radiation pattern - possible misunderstandings - no easy extraction of parameters possible - the discontinuities will results in unwanted spikes in the resulting 3D pattern, further advantage is that both cuts are now based on the same Fourier series - antennas point in the opposite direction as anticipated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2 References, 6.1.4 Antenna model |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Unaffected text was omitted

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-170837: "New SI proposal: Study on evaluation methodology of new V2X use cases for LTE and NR".

[3]	3GPP TR 22.886: "Study on enhancement of 3GPP Support for 5G V2X Services".

[4]	3GPP TR 38.913: "Study on Scenarios and Requirements for Next Generation Access Technologies".

[5]	3GPP TR 38.802: "Study on New Radio Access Technology; Physical Layer Aspects".

[6]	ITU-R M.1452-2 (05/2012): "Millimetre wave vehicular collision avoidance radars and radiocommunication systems for intelligent transport system applications"

[7]	ECC/DEC/(09)01: "The harmonised use of the 63-64 GHz frequency band for Intelligent Transport Systems (ITS)"

[8]	ETSI TR 102 400 v1.2.1(2006-07): "Electromagnetic compatibility and Radio spectrum Matters (ERM); Short Range Devices (SRD); Intelligent Transport Systems (ITS); Road Traffic and Transport Telematics (RTTT); Technical characteristics for communications equipment in the frequency band from 63 GHz to 64 GHz; System Reference Document".

[9]	ETSI EN 302 686 V1.1.1(2011-02): "Intelligent Transport Systems (ITS); Radiocommunications equipment operating in the 63 GHz to 64 GHz frequency band; Harmonized EN covering the essential requirements of article 3.2 of the R&TTE Directive"

[10]	ERMTG37(17)026012: "ITS in 60GHz proposal, FBConsulting, ETSI ERM-TG37#26"

[11]	IEEE 802.18-17/0097: "ITS in 60GHz - An updated proposal discussed in ETSI TC BRAN and TC ITS, Friedbert Berens, Markus Mueck"

[12]	Republic of Korea table of frequency allocations, https://spectrum.or.kr:5017/download.php?dnfile=%EC%A3%BC%ED%8C%8C%EC%88%98%EB%B6%84%EB%B0%B0%EB%8F%84%ED%91%9C%282015%EB%85%84+1%EC%9B%94%29.pdf&file=/www/spectrum_or_kr/webapp/../upload_dir/banner/175666235554bdd5483c499.pdf

[13]	3GPP TR 36.885: "Study on LTE-based V2X services".

[14]	3GPP TR 36.873: "Study on 3D channel model for LTE".

[15]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

[16]	3GPP TR 36.872: "Small cell enhancements for E-UTRA and E-UTRAN - Physical layer aspects".

[17]	3GPP TR 36.814: "Further advancements for E-UTRA physical layer aspects".

[18]	3GPP TR 22.872: "Study on positioning use cases".

[19]	F. Gil et al., “A 3-D extrapolation model for base station antenna’s radiation patterns,” in Proc. IEEE VTC-Fall, Amsterdam, The Netherlands, Sep. 1999, pp. 1341–1345.

Omitted unchanged text

-	Option 2: Two panels are placed in the vehicle as follows and the antenna pattern for each location is given by Tables 6.1.4-10 and, 6.1.4-11. The antenna array configuration is given by Table 6.1.4-12.

-	For vehicle type 1, one panel at the front bumper and one panel at the rear bumper

-	For vehicle type 2, one panel at the front rooftop and one panel at the rear rooftop

-	For vehicle type 3, one panel at the front rooftop and one panel at the rear rooftop

The 3D radiation power patterns AdB''θ,ϕ for the antenna elements proposed for Option 2 shall be generated using the extrapolation method presented in [19], i.e.

AdB''θ,ϕ=4θ1θ2GV2π-θϕ1+GVθϕ2+θ1GH(ϕ)+θ2G1θϕ1ϕ2π4θ1θ2+0.5ϕ1ϕ2

for 0≤θ≤π and 0≤ϕ≤2π with

θ1=θfor 0≤θ≤π2π-θfor π2≤θ≤π

θ2=π2-θfor 0≤θ≤π2θ-π2for π2≤θ≤π

ϕ1=ϕfor 0≤ϕ≤π2π-ϕfor π≤ϕ≤2π

ϕ2=π-ϕfor 0≤ϕ≤πϕ-πfor π≤ϕ≤2π

G1θ=GVθ=0for 0≤θ≤π2GVθ=πfor π2≤θ≤π

and GVθ and GHϕ being the vertical and horizontal cut of the radiation power patterns defined for 0≤θ≤2π and 0≤ϕ≤2π respectively.

The cuts are approximated by a finite sum of weighted sinusoids in the following format.

GVθ=aV02+n=1NVaVncosnθ+⁡bVnsinnθ,  θ∈0,2π

GHϕ=aH02+n=1NHaHncosnϕ+⁡bHnsinnϕ,  ϕ∈0,2π

The Fourier coefficients aV0, aVn, bVn and aH0, aHn, bHn are given in the following tables.

Note that θ=π2, ϕ=π points to the front of the car, while θ=π2, ϕ=0 points to the back of the car.

Table 6.1.4-10: Antenna element pattern coefficients for vehicle UE in Option 2 for 6 GHz

| front rooftop n; aVn; bVn; aHn; bHn. 0; -21.3409; -; -23.438; -. 1; 9.3741; -2.0156; -2.4504; 0.0053. 2; 0.5169; 0.2506; -0.9825; -0.0508. 3; 1.1775; 0.0714; -0.0419; -0.0225. 4; -1.4612; 0.1592; 0.2384; 0.0065. 5; 0.598; -0.2366; 0.2406; -0.0023. 6; -0.284; -0.0715; 0.1532; 0.006. 7; 0.3897; 0.1117; 0.1285; 0.0062. 8; -0.2194; 0.2427; 0.1407; 0.0038. 9; -0.1151; -0.1035; -; -. 10; 0.0395; -0.2105; -; -. 11; 0.3162; 0.0647; -; -. 12; -0.0845; 0.285; -; -. 13; -0.228; -0.0648; -; -. 14; 0.0439; -0.253; -; -. Max direct. gain of the antenna element 3 dBi | rear rooftop n; aVn; bVn; aHn; bHn. 0; -22.3857; -; -24.798; -. 1; 9.3108; -1.1454; -0.6788; -0.0034. 2; 0.6144; 0.0641; -1.025; -0.0364. 3; 1.2791; -0.6854; -0.0755; -0.0404. 4; -1.7109; 0.2784; 0.4216; 0.014. 5; 0.4281; 0.2538; -0.6139; -0.0053. 6; -0.0082; -0.0814; 0.2297; 0.0063. 7; 0.4084; -0.5153; -0.4418; -0.0038. 8; -0.4479; -0.0801; 0.0055; -0.0141. 9; 0.0993; 0.2204; -; -. 10; 0.0694; 0.0423; -; -. 11; 0.0855; 0.0312; -; -. 12; 0.0543; 0.0349; -; -. 13; 0.0616; 0.006; -; -. Max direct. gain of the antenna element 3 dBi |
| --- | --- |
| front bumper n; aVn; bVn; aHn; bHn. 0; -37.4957; -; -41.586; -. 1; 8.7507; -1.9074; -3.4026; 0.0721. 2; -1.7605; -2.2022; 1.3366; 0.1213. 3; -0.3562; -2.051; 2.1764; -0.0595. 4; -1.0966; 2.6102; 0.5881; 0.0102. 5; 0.5013; 0.6833; -2.4068; 0.0675. 6; 1.043; 0.2344; 1.7561; -0.1513. 7; -0.4879; 0.6558; 0.7899; -0.0133. 8; 0.5513; -0.7286; -1.0144; 0.0177. 9; 0.287; -1.0163; -; -. 10; -0.3768; 0.0164; -; -. 11; 0.1484; 0.0793; -; -. Max direct. gain of the antenna element 13 dBi | rear bumper n; aVn; bVn; aHn; bHn. 0; -34.0987; -; -35.928; -. 1; 8.137; 2.2526; 4.1952; 0.0073. 2; -0.1282; 2.9871; -0.3746; -0.0202. 3; 2.3735; 1.3294; 1.2325; 0.005. 4; -0.5764; 0.7505; 0.2862; -0.0764. 5; 1.0287; 0.5362; -0.2796; 0.0252. 6; -0.8903; 0.6445; -0.063; -0.0114. 7; -0.0733; -0.2058; -1.0205; 0.0298. 8; 0.0468; -0.0472; -0.2964; 0.0095. 9; -0.0445; -0.2659; -; -. 10; -0.0067; -0.0511; -; -. 11; -0.0165; -0.1558; -; -. 12; -0.0052; -0.0531; -; -. 13; -0.0089; -0.1178; -; -. 14; 0.0001; -0.0514; -; -. 15; -0.0044; -0.096; -; -. 16; 0.0035; -0.0474; -; -. 17; -0.0016; -0.0825; -; -. 18; 0.0059; -0.0429; -; -. 19; 0.0004; -0.0738; -; -. 20; 0.0079; -0.0383; -; -. Max direct. gain of the antenna element 11 dBi |

Table 6.1.4-11: Antenna element pattern coefficients for vehicle UE in Option 2 for 30 and 63 GHz

| front rooftop n; aVn; bVn; aHn; bHn. 0; -45.0201; -; -32.17; -. 1; 18.8098; -6.2227; -7.903; 0.1101. 2; -5.2966; -0.0117; -0.2972; -1.4639. 3; -3.6007; 1.9867; 0.4083; -0.0866. 4; 1.9433; -0.7799; 0.6706; 1.0598. 5; 1.7541; -1.6535; -0.9804; -2.5714. 6; -1.147; 0.2912; -0.9085; 0.1326. 7; 0.0295; 0.0457; 0.1433; -0.1372. 8; 0.6647; -1.0202; -0.1282; -0.2697. 9; -0.1398; -0.2275; -; -. 10; -0.5274; 0.6719; -; -. 11; -0.1036; 0.1338; -; -. 12; 0.2962; 0.0476; -; -. 13; -0.1365; -0.2312; -; -. 14; 0.2767; 0.9589; -; -. 15; -0.0865; 0.2435; -; -. 16; 0.1157; -0.1965; -; -. 17; -0.0617; 0.014; -; -. 18; 0.1013; -0.0948; -; -. 19; -0.0498; -0.0177; -; -. 20; 0.088; -0.063; -; -. Max direct. gain of the antenna element 11.5 dBi | rear rooftop n; aVn; bVn; aHn; bHn. 0; -50.2015; -; -36.942; -. 1; 20.4382; -2.1651; 3.6106; -0.1824. 2; -4.6106; -1.1891; -1.4888; 1.3614. 3; -5.211; -1.5541; 0.5772; 0.5331. 4; 1.5492; 2.1171; 1.7125; 1.3205. 5; 1.73; 0.7044; -0.0573; -2.5904. 6; -0.7497; -1.4473; 0.0628; -1.0024. 7; -0.8466; -1.1446; -1.0526; -1.2366. 8; 0.0089; 0.2879; 0.0433; -0.1443. 9; 0.3271; 0.1418; -; -. 10; -0.3845; -1.2483; -; -. 11; -0.3356; -0.442; -; -. 12; -0.2952; 0.6602; -; -. 13; -0.2619; 0.8404; -; -. 14; -0.0033; 0.2918; -; -. 15; -0.2742; 0.0759; -; -. 16; 0.1196; 0.1237; -; -. 17; -0.1562; 0.0354; -; -. 18; 0.1094; 0.0931; -; -. 19; -0.1053; 0.0186; -; -. 20; 0.0932; 0.0765; -; -. Max direct. gain of the antenna element 12.5 dBi |
| --- | --- |
| front bumper n; aVn; bVn; aHn; bHn. 0; -29.0394; -; -29.424; -. 1; 2.0111; -0.85; -0.814; -0.0126. 2; -4.238; -1.808; 4.5025; -0.4615. 3; 0.1995; 0.1304; 0.6572; -0.0404. 4; 1.2891; -0.291; 0.3252; 0.5985. 5; 0.0422; 0.1732; -2.0187; 0.0372. 6; 1.2135; -0.2515; -1.6555; 0.2589. 7; -0.1313; -0.7555; -0.843; 0.6858. 8; -0.0194; -0.065; -0.2954; -0.2211. Max direct. gain of the antenna element 16.6 dBi | rear bumper n; aVn; bVn; aHn; bHn. 0; -25.5522; -; -25.484; -. 1; 1.0994; -0.4289; -1.2007; 0.2451. 2; -4.8356; -3.4364; 4.1916; -0.5651. 3; -0.6327; 1.1004; 0.1901; 0.453. 4; 1.0957; -0.7866; 0.4235; 0.6884. 5; -0.7695; 0.002; -1.3848; -0.4145. 6; 0.9043; -0.0634; -0.2381; 0.2875. 7; -0.4003; -0.7493; 0.4488; 0.5441. 8; -0.0282; 0.0601; 0.1606; -0.2555. 9; -0.205; -0.279; -; -. 10; -0.0953; 0.0287; -; -. 11; -0.2132; 0.0383; -; -. 12; 0.2122; 0.1832; -; -. 13; 0.0414; -0.1473; -; -. 14; 0.1703; -0.0179; -; -. Max direct. gain of the antenna element 13.6 dBi |

Unaffected text was omitted