3GPP TSG-RAN WG1 Meeting #94	R1-1809265

Gothenburg, Stockholm, August 20th – 24th, 2018	(resubmission of R1-1806438)

Source:	Ericsson

Title:	IMT-2020 self-evaluation: Mobility evaluations for NR

Agenda Item:	7.2.7.2

Document for:	Discussion

# Introduction

In this contribution, we show preliminary evaluation results for NR regarding the IMT-2020 mobility requirements. After a brief review of the requirements and evaluation methodology, system-level SINR distributions and link-spectral efficiency results are provided. The results indicate that that the IMT-2020 requirements can be met.

# The Requirement and the evaluation methodology

Table 1 list IMT-2020 requirements [1] on spectral efficiency in mobility scenarios. Each test environment further contains configurations with different carrier frequencies, 4, 30 and 70GHz for Indoor Hotspot, 4GHz and 30GHz for Dense Urban, and 700MHz and 4GHz for Rural. For each test environment, the requirement needs to be reached for at least one configuration.

The evaluation methodology in [2] involves first generating median (50th percentile) uplink SINR from system-level simulations of the test environments above. Link-level simulations for the corresponding test environments are then used to verify that the required normalized traffic channel link data rates can be reached at the median SINR values.

The requirements are for the uplink, but proponents are encouraged to evaluate also the downlink.

Table 1	Traffic channel link data rates normalized by bandwidth (from [1])

| Test environment | Normalized traffic channel link data rate (Bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 1.5 | 10 |
| Dense Urban – eMBB | 1.12 | 30 |
| Rural – eMBB | 0.8 | 120 |
|  | 0.45 | 500 |

# System evaluations – median SINR values

This section presents SINR distributions and median SINR values for the evaluated test environments.  Models and assumptions are according to the ITU guidelines [2]. It should be noted that a rather basic system configuration is assumed, not utilizing the full antenna configurations allowed, and with power control and scheduling parameters not specifically targeting high median SINR values, A selection of parameters is presented in

Table 2	Models and assumptions

| Parameter | Indoor hotspot | Dense urban | Rural |
| --- | --- | --- | --- |
| Frequency | 4GHz | 4GHz | 700MHz |
| BS antenna | 8x8x2 | 8x8x2 | 8x4x2 |
| UE antenna | Isotropic | Isotropic | Isotropic |
| Beamforming and transmission scheme | Wideband eigen beamforming, SIMO | Wideband eigen beamforming, SIMO | Wideband eigen beamforming, SIMO |
| Power control | SNR target 15dB alpha = 1.0 | SNR target 10dB alpha = 1.0 | SNR target 10dB alpha = 1.0 |
| BW allocation | Full system BW | Full system BW | 10 resource blocks |

SINR distributions for downlink and uplink for the different test scenarios, and for propagation model A and B, are presented in Figure 1 - Figure 4. Downlink and uplink median SINR values are summarized in Table 3.

Figure 1. Downlink and uplink SINR distributions for the indoor hotspot scenario with one sector per site

Figure 2. Downlink and uplink SINR distributions for the indoor hotspot scenario with three sectors per site

Figure 3. Downlink and uplink SINR distributions for the dense urban scenario

Figure 4. Downlink and uplink SINR distributions for the rural scenario

Table 3	Downlink and uplink median SINR values

| Test environment | Median downlink SINR [model A / B] | Median uplink SINR [model A/B] |
| --- | --- | --- |
| Indoor Hotspot, 1 sector/site | 14.5 / 14.1dB | 14.2 / 13.8dB |
| Indoor Hotspot, 3 sectors/site | 11.0 / 10.7 dB | 14.8 / 14.5dB |
| Dense Urban | 13.0 / 12.8dB | 8.6 / 8.8dB |
| Rural | 14.9 / 15.3dB | 8.6 / 8.6dB |

# Link results – required SNR values

In the presented link level evaluations, due to the high doppler and low SNR working point, a conservative approach was taken to link adaptation; potential gains of following fast channel variations and SU-MIMO using dense CSI feedback are not reflected. A fixed bandwidth corresponding to roughly 10 MHz was assumed but as requirements are set in the spectral efficiency domain, this may be of secondary importance. The required SINR for reaching the spectral efficiency target should consequently be from the point in the curves where the throughput matches the target spectral efficiency. Reference signals, channel codes, etc. were implemented according to the approved RAN1 specification (with overhead accounted for). Higher layer protocols, including MAC, PDCP, RLC were not included in the evaluations.

The NR standard supports a large set of DMRS densities, and the optimal DMRS pattern is a function of speed (among many other things). DMRS patterns were selected according to the analysis in [3] to match the receiver speed in the evaluation scenario. Depending on the carrier frequency, different sub-carrier spacing was chosen – this improves resilience to high Doppler and will, for of the evaluated deployments, also provide sufficient robustness to inter symbol interference. More detailed simulation assumptions are provided in Section 4.1.

## Models and Assumptions

TDL models [2] are used for the different scenarios according to Table 4 where delay spreads and K-factors for line-of-sight (LOS) channels are chosen according to [4].

Table 4 Test environment, carrier frequency, and corresponding channel model parameters as used in the evaluations

| Test environment | Carrier frequency [GHz] | TDL channel | Delay spread [ns] | K-factor [dB] |
| --- | --- | --- | --- | --- |
| Indoor Hotspot – eMBB, NLOS | 4 | TDL-i (TDL-A) | 39 | - |
| Indoor Hotspot – eMBB, LOS | 4 | TDL-iv (TDL-D) | 20 | 13.3 |
| Indoor Hotspot – eMBB, NLOS | 30 | TDL-i (TDL-A) | 26 | - |
| Indoor Hotspot – eMBB, LOS | 30 | TDL-iv (TDL-D) | 19.5 | 13.3 |
| Indoor Hotspot – eMBB, NLOS | 70 | TDL-i (TDL-A) | 20 | - |
| Indoor Hotspot – eMBB, LOS | 70 | TDL-iv (TDL-D) | 19.5 | 13.3 |
| Dense Urban – eMBB, NLOS | 4 | TDL-iii (TDL-C) | 363 | - |
| Dense Urban – eMBB , LOS | 4 | TDL-v (TDL-E) | 93 | 22 |
| Dense Urban – eMBB, NLOS | 30 | TDL-iii (TDL-C) | 263 | - |
| Dense Urban – eMBB , LOS | 30 | TDL-v (TDL-E) | 79 | 22 |
| Rural – eMBB, NLOS | 0.7 | TDL-iii (TDL-C) | 37 | - |
| Rural – eMBB , LOS | 0.7 | TDL-v (TDL-E) | 32 | 22 |
| Rural – eMBB , NLOS | 4 | TDL-iii (TDL-C) | 37 | - |
| Rural – eMBB, LOS | 4 | TDL-v (TDL-E) | 32 | 22 |

For the link layer simulations, the layer-one is configured differently for 700 MHz, 4.0 GHz, 30 GHz and 70 GHz, according to Table 5-8, respectively.  Note that a different, and denser DMRS pattern, is used for the scenario with 500 km/h at 4.0 GHz as compared the other scenarios.

As we chose to not use the option for frequency division multiplexing (FDM) of data and DMRS, and overhead for control is assumed to occupy one full symbol, the payload per slot can only occupy resources corresponding to 11 full symbols per slot when using a 2-symbol DMRS allocation, and 9 full symbols per slot when using a 4-symbol DMRS (500 km/h @ 4.0GHz) configurations.

The layer-one configurations reflect a FDD deployment, where  control channel overhead for UL/DL and DMRS overhead is accounted for. For TDD, guard-symbols should be added to enable DL/UL switching.  It is here assumed that not more than an additional 2 guard-symbols per DL/UL switching is needed for this purpose.

The added over-head for TDD, compared to FDD, depend on the UL/DL switching configuration.  As an example, consider a DL/UL switching guard period overhead of 2 symbols in every slot (note that UL/DL switching does not require any guard symbols). For instance, in a slot containing one DL/UL switch, the available resource elements for data would then be reduced from 11 to 9 full symbols for the 2-symbol DMRS configuration and 9 to 7 full symbols for the 4-symbol DMRS configuration (remember that the over-head for a 2-symbol DMRS is 2 full symbols and the over-head for a 4-symbol DMRS is four full symbols when not allowing for FMD of DMRS and data).

To be on the safe side we consider an aggressive approach where DL/UL switching occur every slot.  Consider then that the cost of DL/UL switching over-head is a pure DL over-head cost per definition. Consequently, for the TDD DL, the throughput evaluation results provided in Section 4.2, should be reduced by

- 18% for the 2-symbol DMRS configuration

- 22% for the 4-symbol DMRS configuration

to model the overhead impact in the case of very frequent DL/UL switching. Note however that, especially for sub-carrier spacing larger than 15kHz, it is reasonable to assume a much lower DL/UL switching rate in many deployments, which would result in much less TDD switching OH.

For TDD operation we suggest reducing the throughput in the evaluation results provided in Section 4.2 with 18%, or 22%, depending on the DMRS allocation, to assess the TDD switching OH impact on spectral efficiency in the DL.

Table 5	Simulation parameters for 700 MHz evaluations

| Numerology, waveform | Subcarrier spacing 15kHz, CP-OFDM |
| --- | --- |
| Codec | NR LDPC codec including HARQ. |
| Transmission rank | 1, fixed |
| DMRS | 2 symbol DMRS (front loaded and one additional) with configuration type 1, no FDM with data and full power utilization in DMRS symbols. |
| Antenna | Tx1, Rx2 |
| Other overhead | 1 symbol control overhead per 14 symbols, one directional data allocation |
| Channel estimator | Practical, slot based |
| Allocation | 14 symbol slots, with 52 RB allocated |

Table 6 Simulation parameters for 4.0GHz evaluations

| Numerology, waveform | Subcarrier spacing 30kHz, CP-OFDM (9,36 MHz) |
| --- | --- |
| Codec | NR LDPC codec including HARQ. |
| Transmission rank | 1, fixed |
| DMRS | Speed = 10,30,120 km/h 2 symbol DMRS (front loaded and one additional) with configuration type 1, no FDM with data and full power utilization in DMRS symbols. Speed = 500 km/h 4 symbol DMRS (front loaded + 3 additional) with configuration type 1, no FDM with data and full power utilization in DMRS symbols |
| Antenna | Tx1, Rx2 |
| Other overhead | 1 symbol control overhead per 14 symbols, one directional data allocation |
| Channel estimator | Practical, slot based |
| Allocation | 14 symbol slots, with 26 RB allocated (9,36 MHz) |

Table 7 Simulation parameters for 30.0GHz evaluations

| Numerology, waveform | Subcarrier spacing 60kHz, CP-OFDM |
| --- | --- |
| Codec | NR LDPC codec including HARQ. |
| Transmission rank | 1, fixed |
| DMRS | 2 symbol DMRS (front loaded and one additional) with configuration type 1, no FDM with data and full power utilization in DMRS symbols. |
| Antenna | Tx1, Rx2 |
| Other overhead | 1 symbol control overhead per 14 symbols, one directional data allocation |
| Channel estimator | Practical, slot based |
| Allocation | 14 symbol slots, with 13 RB allocated (9.36 MHz) |

Table 8 Simulation parameters for 70.0GHz evaluations

| Numerology, waveform | Subcarrier spacing 120kHz, CP-OFDM |
| --- | --- |
| Codec | NR LDPC codec including HARQ. |
| Transmission rank | 1, fixed |
| DMRS | 2 symbol DMRS (front loaded and one additional) with configuration type 1, no FDM with data and full power utilization in DMRS symbols. |
| Antenna | Tx1, Rx2 |
| Other overhead | 1 symbol control overhead per 14 symbols, one directional data allocation |
| Channel estimator | Practical, slot based |
| Allocation | 14 symbol slots, with 7RB allocated (10.08 MHz) |

## Numerical evaluations

In this section evaluation results of layer-one throughput are presented for the different Test environments (Indoor Hotspot/Dense Urban/Rural scenario) LOS and NLOS channels and for carrier frequencies selected from {700 MHz, 4GHz, 30 GHz, 70 GHz}, depending on the scenario. Also, in these plots, the scaled throughput curves showing performance with frequent DL/UL switching is included, together with the corresponding IMT-2020 requirement, assuming a 10 MHz bandwidth. The required SNR values to meet the normalized data rate targets are summarized in Table 3.

As noted before, it is sufficient that one configurations per Test environment fulfils the IMT2020 requirements. Thus, here we focus on the best-performing configuration per Test environment for without and with DL/UL switching over-head; in Table 9, the Required SNR is noted for cases without and with frequent DL/UL switching over-head and for LOS as well as NLOS.

Table 9 Required SNR values

| Test environment | Requirement | Required SNR (NLOS/LOS) | Required SNR with DL/UL switching OH (NLOS/LOS) |
| --- | --- | --- | --- |
| Indoor Hotspot – eMBB | 1.5bps/Hz at 10km/h | 8.2dB / 6.5dB | 10.5dB / 9dB |
| Dense Urban – eMBB | 1.12bps/Hz at 30km/h | 7dB / 3.5dB | 9dB / 5dB |
| Rural – eMBB | 0.8bps/Hz at 120km/h | 7dB / 1dB | 8dB / 2.5dB |
|  | 0.45bps/Hz at 500km/h | 4.5dB / -1dB | 6dB / 0dB |

Figure 5	Layer 1 throughput as a function of SNR for the Indoor Hotspot scenario without TDD switching OH (green cross) and with TDD frequency switching OH (black circle). Evaluations at 4.0MHz (top), 30 GHz (middle) and 70GHz (bottom) carrier frequency at 10 km/h for NLOS (left) and LOS (right). For 4GHz, the requirement of 1.5bps/Hz (15Mbps in 10MHz) is reached at 8.2dB SNR for NLOS and 6.5dB for LOS (with OH scaling for frequent TDD switching we get 10.5dB SNR for NLOS and 9dB for LOS for the same configuration).

Figure 6 Layer 1 throughput as a function of SNR for the Dense Urban scenario without TDD switching OH (green cross) and with frequent TDD switching OH (black circle). Evaluations at 4.0GHz(top) and 30GHz (bottom) carrier frequency at 30 km/h for NLOS (left) and LOS (right). For 4GHz, the requirement of 1.12bps/Hz (11.2Mbps in 10MHz) is reached at 7dB SNR for NLOS and 3.5dB for LOS (with OH scaling for frequent TDD switching we get 9dB SNR for NLOS and 5dB for LOS for the same configuration).

Figure 7	Layer 1 throughput as a function of SNR for the Rural scenario without TDD switching OH (green cross) and with frequent TDD switching OH (black circle). Evaluations at 700 MHz (top) and 4GHz (bottom) carrier frequency at 120 km/h for NLOS (left) and LOS (right). For 700MHz, the requirement of 0.8bps/Hz (8Mbps in 10MHz) is reached at 7dB SNR for NLOS and 1dB for LOS (with OH scaling for frequent TDD switching we get 8dB SNR for NLOS and 2.5dB for LOS for the same configuration).

Figure 8	Layer 1 throughput as a function of SNR for the Rural scenario without TDD switching OH (green cross) and with frequent TDD switching OH (black circle). Evaluations at 700 MHz (top) and 4GHz (bottom) carrier frequency at 500 km/h for NLOS (left) and LOS (right). For 4.0 GHz, the requirement of 0.45bps/Hz (4.5Mbps in 10MHz) is reached at 4.5dB SNR for NLOS and -1dB for LOS (with OH scaling for frequent TDD switching we get 6dB SNR for NLOS and 0dB for LOS for the same configuration).

# Summary

The required SINR levels for reaching the spectral efficiency requirements and the and achieved median SINR values are given by Table 10 below for the case when TDD UL/DL switching is ignored (valid for FDD UL/DL and TDD UL). In Table 11 below the case when the impact of TDD UL/DL switching is included (valid for TDD DL) Note that it is sufficient that one configurations per Test environment fulfils the requirements in the UL to claim that the IMT-2020 targets are fulfilled.

Table 10	Required and achieved SNR values for the case without OH for DL/UL switching

| Test environment | Requirement | Required SNR (NLOS/LOS) | Achieved median downlink SINR [model A/B] | Achieved median uplink SINR [model A/B] |
| --- | --- | --- | --- | --- |
| Indoor Hotspot – eMBB, 1sector/site | 1.5bps/Hz at 10km/h | 8.2dB / 6.5dB | 14.5dB / 14.1dB | 14.2dB / 13.8dB |
| Indoor Hotspot – eMBB, 3 sectors/site | 1.5bps/Hz at 10km/h | 8.2dB / 6.5dB | 11.0dB / 10.7 dB | 14.8dB / 14.5dB |
| Dense Urban – eMBB | 1.12bps/Hz at 30km/h | 7dB / 3.5dB | 13.0dB / 12.8dB | 8.6dB / 8.8dB |
| Rural – eMBB | 0.8bps/Hz at 120km/h | 7dB / 1dB | 14.9dB / 15.3dB | 8.6dB / 8.6dB |
|  | 0.45bps/Hz at 500km/h | 4.5dB / -1dB | 14.9dB / 15.3dB | 8.6dB / 8.6dB |

Observation: The achieved median SINR values are higher than the required SNR values. Hence, the IMT-20202 mobility requirements are fulfilled.

Table 11	Required and achieved SNR values for the case accounting for over-head for frequent DL/UL switching (only DL comparisons needed, UL results can be found in Table 10)

| Test environment | Requirement | Required SNR (NLOS/LOS) UL/DL switching OH accounted for | Achieved median downlink SINR [model A/B] |
| --- | --- | --- | --- |
| Indoor Hotspot – eMBB, 1sector/site | 1.5bps/Hz at 10km/h | 10.5 dB/ 9dB | 14.5dB / 14.1dB |
| Indoor Hotspot – eMBB, 3 sectors/site | 1.5bps/Hz at 10km/h | 10.5 dB / 9dB | 11.0dB / 10.7 dB |
| Dense Urban – eMBB | 1.12bps/Hz at 30km/h | 9dB / 5dB | 13.0dB / 12.8dB |
| Rural – eMBB | 0.8bps/Hz at 120km/h | 8dB / 2.5dB | 14.9dB / 15.3dB |
|  | 0.45bps/Hz at 500km/h | 6dB / 0dB | 14.9dB / 15.3dB |

Observation: The achieved median down-ling SINR values are higher than the required SNR values even when over-head for frequent DL/UL switching is accounted for in the link evaluation results. Hence, the IMT-2020 mobility requirements are fulfilled even when accounting for over-head for aggressive TDD switching.

# References

[1] ITU-R Report M.2410, “IMT-2020 Minimum Requirements related to technical performance for IMT-2020 radio interfaces”, November 2017

[2] ITU-R Report M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, December 2017

[3] R1-1720952, “IMT-2020 self-evaluation: Mobility evaluations for NR”, 3GPP TSG-RAN WG1 #91, Ericsson

[4] R1-1805643 “Summary of discussion on mobility evaluation method and parameters”, Huawei, 3GPP TSG RAN WG1 Meeting #92Bis, Sanya, China, April 16 – 20, 2018