3GPP TSG-RAN WG1 Meeting #94	R1-1809272

Gothenburg, Sweden, August 20th – 24th, 2018					   (Revision of R1-1806439)

Source:	Ericsson

Title:	IMT-2020 self-evaluation: Peak data rate and peak spectrum efficiency evaluations for NR

Agenda Item:	7.2.7.2

Document for:	Discussion

# Introduction

In this paper we show evaluation results of NR w.r.t. peak data rate, peak spectrum efficiency, and bandwidth. The ITU and 3GPP targets for NR w.r.t. above metrics are summarized in the table below.

Table 1: ITU and 3GPP targets on peak data rate, peak spectral efficiency, and bandwidth

| Performance Measure | ITU | 3GPP |
| --- | --- | --- |
| Peak data rate | DL: 20 Gb/s UL: 10 Gb/s | DL: 20 Gb/s UL: 10 Gb/s |
| Peak spectral efficiency | DL: 30 bps/Hz UL: 15 bps/Hz | DL: 30 bps/Hz UL: 15 bps/Hz |
| Bandwidth | At least 100MHz, up to 1GHz for higher frequency bands | Reference to ITU |

This contribution is a revision of R1-1806439.

# Bandwidth

RAN4 agreed on spectrum utilization numbers listed in Table 1 and Table 2 for below and above 6 GHz, respectively [1].

Table 2: Maximum spectrum utilization (in PRB) for various bandwidth/subcarrier spacing combinations for below 6 GHz (BW in MHz, SCS in kHz)

| SCS\BW | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 25 | 52 | 79 | 106 | 133 | 216 | 270 | NA | NA | NA |
| 30 | 11 | 24 | 38 | 51 | 65 | 106 | 133 | 162 | 217 | 273 |
| 60 | NA | 11 | 18 | 24 | 31 | 51 | 65 | 79 | 107 | 135 |

Table 3: Maximum spectrum utilization (in PRB) for various bandwidth/subcarrier spacing combinations for above 6 GHz (BW in MHz, SCS in kHz)

| SCS\BW | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- |
| 60 | 66 | 132 | 264 | NA |
| 120 | 32 | 66 | 132 | 264 |

One can see from Table 2 and Table 3 that several configurations support bandwidths of 100 MHz and above. The largest component carrier bandwidth is 400 MHz. NR supports carrier aggregation of up to 16 component carriers in which case the supported NR carrier bandwidth exceeds 1 GHz.

# Peak data rate and spectral efficiency

## Downlink

For the assessment of DL peak data rate overhead due to SSB, TRS, PDCCH, DM-RS, PT-RS, and CSI-RS is considered, see Table 4. To achieve peak data rates of 20 Gb/s, bandwidth in the order of 400 MHz is needed which only exists above 6 GHz, therefore we focus the evaluation on above 6 GHz.

Table 4: Evaluation assumptions for peak data rate and peak spectral efficiency in DL

| Parameter | Setting |
| --- | --- |
| SSB | 8 SSB per 20 ms |
| TRS | min(52, BW in PRB) PRB wide, occurs every 20 ms |
| PDCCH | 4 CCE in every slot |
| DM-RS | 2 complete symbols per slot |
| CSI-RS | 8 RE per PRB, occurs every 10 ms |
| PT-RS | 1 subcarrier every 4th PRB, every symbol |
| Number of layers | 8 |
| Modulation format | 256QAM |
| Code rate | 0.93 |

The achievable DL peak data rates assuming the assumptions from Table 4 are shown in Table 5. For a 400 MHz wide component carrier the peak data rate is 17.49 Gb/s. Aggregating two such component carriers requires a bandwidth of 800 MHz and gives a peak data rate well beyond 20 Gb/s.

Table 5: DL peak data rates in Gb/s (BW in MHz, SCS in kHz)

| SCS\BW | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- |
| 60 | 2.11 | 4.32 | 8.73 | NaN |
| 120 | 1.98 | 4.25 | 8.66 | 17.49 |

The peak spectral efficiency is obtained by normalizing the peak data rates shown in Table 5 by the bandwidth. The resulting peak spectral efficiency values are tabulated in Table 6. All values are well above the target of 30 bps/Hz.

Table 6: DL peak spectral efficiency in bps/Hz (BW in MHz, SCS in kHz)

| SCS\BW | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- |
| 60 | 42.23 | 43.16 | 43.63 | NaN |
| 120 | 39.56 | 42.50 | 43.31 | 43.72 |

## Uplink

The evaluation parameters for UL are listed in Table 7. In UL the overhead due to DM-RS, PT-RS, SRS, and PUCCH is considered. The 3GPP and ITU peak data rate targets are e.g. fulfilled for carrier aggregation of two 400 MHz wide component carrier, see Table 8. All shown combinations exceed the peak spectral efficiency target of 15 bps/Hz, see Table 9.

Table 7: Evaluation assumptions for peak data rate and peak spectral efficiency in UL

| Parameter | Setting |
| --- | --- |
| DM-RS | 1 complete symbol per slot |
| PT-RS | 1 subcarrier every 4th PRB, every symbol |
| SRS | 1 complete symbol every 10 ms |
| PUCCH | Long PUCCH with 2 PRB over slot in every slot |
| Number of layers | 4 |
| Modulation format | 256QAM |
| Code rate | 0.93 |

Table 8: UL peak data rates in Gb/s (BW in MHz, SCS in kHz)

| SCS\BW | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- |
| 60 | 1.16 | 2.35 | 4.74 | NaN |
| 120 | 1.08 | 2.31 | 4.71 | 9.50 |

Table 9: UL peak spectral efficiency in bps/Hz (BW in MHz, SCS in kHz)

| SCS\BW | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- |
| 60 | 23.12 | 23.52 | 23.72 | NaN |
| 120 | 21.61 | 23.14 | 23.54 | 23.74 |

# Conclusion

In this contribution, we evaluate NR w.r.t. supported bandwidth, peak data rate, and peak spectral efficiency. It is shown that NR fulfills ITU and 3GPP targets w.r.t. to these three metrics.

# References

R4-1806932, “TS 38.104 Combined updates (NSA) from RAN4 #86bis and RAN4 #87,” Ericsson