3GPP TSG RAN WG1 Meeting #94	R1-1809358

Gothenburg, Sweden, August 20th- 24th, 2018

Agenda Item:	6.1.6.3

Source:	Huawei, HiSilicon

Title:	Clarification on UE procedure for receiving PDSCH when configured with repetition

Document for:	Discussion and decision

# Introduction

This contribution provides a text proposal to supplement UE procedure for receiving PDSCH when configured with repetition but the number of transmission is set to 1 which was not captured in section 7.1 of TS 36.213 based on the latest version [1].

# Discussion

It was agreed that the “repetition number” field (indicating the number of PDSCH transmission k) is added into DCI formats 1A and 7-1A/1B/1C/1D/1E/1F/1G which however can be set to 1. During the discussion of standardization for DL HARQ-less in the LTE HRLLC WI, it was assumed the common understanding that the UE procedure for PDSCH receiving when the number of PDSCH transmission is set to 1 shall follow the case of not configured with repetition as preceding Rel-15.

Accordingly, the following proposal is given as below for clarification in section 7.1 of TS 36.213 [1].

Proposal 1: Endorse the following text proposal:

------------------------------------------------Start of Text Proposal to 36.213-----------------------------------------

## 7.1	UE procedure for receiving the physical downlink shared channel

Except the subframes indicated by the higher layer parameter mbsfn-SubframeConfigList or by mbsfn-SubframeConfigList-v1250 or by mbsfn-SubframeConfigList-v14xy or by laa-SCellSubframeConfig of serving cell , a UE shall

-	upon detection of a PDCCH of the serving cell with DCI format 1, 1A, 1B, 1C, 1D, 2, 2A, 2B, 2C, or 2D intended for the UE in a subframe, or

-	upon detection of an EPDCCH of the serving cell with DCI format 1, 1A, 1B, 1D, 2, 2A, 2B, 2C, or 2D intended for the UE in a subframe, or

-	upon detection of a PDCCH of the serving cell with DCI format 7-1A, 7-1B, 7-1C, 7-1D, 7-1E, 7-1F, 7-1G intended for the UE in the first slot/subslot of a subframe

-	upon detection of a SPDCCH of the serving cell with DCI format 7-1A, 7-1B, 7-1C, 7-1D, 7-1E, 7-1F, 7-1G intended for the UE in a slot/subslot

decode the corresponding PDSCH in the same subframe/slot/subslot with the restriction of the number of transport blocks defined in the higher layers, unless specified otherwise.

For a given serving cell, if the UE is configured with higher layer parameter blindSubframePDSCH-Repetitions, the UE shall upon detection of a PDCCH/EPDCCH with DCI format 1A with CRC scrambled by C-RNTI in UE-specific search space of subframe n, intended for the UE, decode, starting from subframe n, the corresponding PDSCH transmission in k consecutive DL subframe(s) according to the PDCCH information, where the value of k is determined by the repetition number field in the corresponding DCI.

- For k=1,

- the UE shall assume the corresponding PDSCH is in the same subframe as the PDCCH/EPDCCH with DCI format 1A with CRC scrambled by C-RNTI in UE-specific search space.

- For k>1,

- the UE shall assume the truncated modulation and coding scheme field is interpreted such that the modulation and coding scheme corresponding to DCI format 1A is determined from MCS indices 0 through 15 if the higher layer parameter MCS-restrictionSubframePDSCH-Repetitions is set to ‘1’.

- The UE shall assume the“PDSCH RE Mapping and Quasi-Co-Location Indicator”field is applied to all the k PDSCH data transmissions signaled via PDCCH/EPDCCH with DCI format 2D.

- The UE shall assume all the k PDSCH data transmissions are received in the same resource blocks.

- For TDD cell, if the UE is configured with higher layer parameter subframeAssignment, the k consecutive DL subframes include the k DL subframes or special subframes according to the DL-reference UL/DL configuration.

For a given serving cell, if the UE is configured with higher layer parameter blindSlotSubslotPDSCH-Repetitions, the UE shall upon detection of a PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G with CRC scrambled by C-RNTI in slot/sublot n, intended for the UE, decode, starting from slot/subslot n, the corresponding PDSCH transmission in k consecutive DL slot(s)/subslot(s) according to the PDCCH/SPDCCH information, where the value of k is determined by the repetition number field in the corresponding DCI.

- For k=1,

- the UE shall assume the corresponding PDSCH is in the same slot/subslot as the PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G with CRC scrambled by C-RNTI.

- For k>1,

- the UE is not expected to receive the PDSCH data transmissions with more than two transmission layers.

- if the k consecutive DL slots/subslots cross two consecutive subframes with different downlink transmission modes, the UE is not expected to receive the PDSCH data transmissions after the former subframe.

- for DCI format 7-1F/7-1G, the UE shall assume the value of the DMRS position indicator field (defined in 3GPP TS 36.212 [4]) is set to ‘0’.

- the UE shall assume the modulation and coding scheme corresponding to DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G is determined from MCS indices 0 through 15 if the higher layer parameter MCS-restrictionSlotSubslotPDSCH-Repetitions is set to ‘1’.

- the UE shall assume the “PDSCH RE Mapping and Quasi-Co-Location Indicator” field is applied to all the k PDSCH data transmissions signaled via PDCCH/SPDCCH with DCI format 7-1G.

- the UE shall assume all the k PDSCH data transmissions are received in the same resource blocks.

- For FDD cell,

  - the UE may assume that the same precoder applies to all scheduled k PDSCH transmissions.

  - subslot 0 of a subframe is not counted as a DL subslot for k PDSCH transmissions if the CFI value is 2 or 3.

- For TDD cell, the UE may assume that the same precoder applies to those of the k scheduled PDSCH transmissions occur between two adjacent special subframes.

<Unchanged parts are omitted>

------------------------------------------------End of Text Proposal to 36.213-----------------------------------------

# Reference

- 3GPP TS 36.213, (E-UTRA); Physical layer procedure (Release 15); V15.2.0 (2018-06).