3GPP TSG-RAN WG1 Meeting #94	Tdoc R1- 1809378

Gothenburg, Sweden, 20th – 24th August 2018

Agenda Item:	6.1.1

Source:	Ericsson

Title:	Clarification of number of PDSCH repetitions when R_CSI is 1 in LTE-MTC

Document for:	Discussion and Decision

# Background

RAN1#84 made the following for Rel-13 BL/CE UEs:

- If the parameter R_CSIMax is: - >1: repetition for PDSCH is allowed - =1, no repetition for PDSCH

The above agreement is captured for the TDD case in 36.213 section 7.3.2.1 (“TDD HARQ-ACK reporting procedure for same UL/DL configuration”) in the following way (where parameter csi-NumRepetitionCE-r13 below corresponds to R_CSIMax above):

If the BL/CE UE is configured in CEModeA, and if the PDSCH is assigned by or semi-statically scheduled by a MPDCCH with DCI format 6-1A, the UE shall assume no PDSCH repetition if the higher layer parameter csi-NumRepetitionCE-r13 indicates one subframe.

However, there is no similar statement in 36.213 for the FDD case. Therefore, it is in principle unclear whether the stated restriction actually applies in both FDD and TDD.

# Proposal

We propose to discuss in RAN1 whether to e.g. introduce the statement quoted above in 36.213 section 7.3.1 (“FDD HARQ-ACK reporting procedure”) and in what release (Rel-13/14/15). If needed, we will then provide a corresponding CR.