3GPP TR 38.889 V0.0.2 (2018-05)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Study on NR-based Access to Unlicensed Spectrum;

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

NR, radio, unlicensed spectrum, physical layer

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Regulatory requirements	5

4.1	Regulatory requirements for 5GHz band	5

4.1.1	ITU Region 1	7

4.1.2	ITU Region 2	12

4.1.3	ITU Region 3	16

4.2	Regulatory requirements for [X]GHz band	30

4.4	Applicability of DFS requirements	30

4.4.1	DFS according to ECC	31

4.4.2	DFS in the Part 15 rules	31

4.4	Applicability of DFS requirements	32

4.4.1	DFS according to ECC	33

4.4.2	DFS in the Part 15 rules	33

5	Spectrum considerations	35

6	Deployment scenarios	35

7	Design targets, functionalities and solutions	35

7.1	Design targets and functionalities	35

7.2	Solutions for operation in unlicensed spectrum	35

7.2.1	Physical layer aspects	35

7.2.2	Higher layer aspects	35

8	Performance evaluations	35

8.1	Scenarios and methodology	35

8.2	Channel access schemes	35

8.3	Evaluation results	35

9	Conclusions	35

Annex A: Evaluation methodology	35

A.1	General evaluation assumptions	35

Annex B:  Evaluation results f	35

Annex C: Change history	36

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

A study item, “Study on NR-based Access to Unlicensed Spectrum”, was approved at 3GPP TSG RAN #77 [2] and revised in 3GPP TSG RAN #80 [30]. This study is to determine a single global solution for NR-based access to unlicensed spectrum, to be compatible with the NR concepts.

The objectives of the study include:

- Study NR-based operation in unlicensed spectrum (RAN1, RAN2, RAN4) including

  - Physical channels inheriting the choices of duplex mode, waveform, carrier bandwidth, subcarrier spacing, frame structure, and physical layer design made as part of the NR study and avoiding unnecessary divergence with decisions made in the NR WI

    - Consider unlicensed bands below 7GHz

    - Consider similar forward compatibility principles made in the NR WI

  - Initial access, channel access. Scheduling/HARQ, and mobility including connected/inactive/idle mode operation and radio-link monitoring/failure

  - Coexistence methods within NR-based and between NR-based operation in unlicensed and LTE-based LAA and with other incumbent RATs in accordance with regulatory requirements in e.g., 5GHz , 6GHz bands

    - Coexistence methods already defined for 5GHz band in LTE-based LAA context should be assumed as the baseline for 5GHz operation. Enhancements in 5GHz over these methods should not be precluded. NR-based operation in unlicensed spectrum should not impact deployed Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier;

The above study will address the following architectural scenarios (RAN2):

  - An NR-based LAA cell(s) connects with an LTE or NR anchor cell operating in licensed spectrum

    - The study assumes the techniques for linking between Pcell (LTE or NR licensed CC) and Scell (NR unlicensed CCs) according to the NR WI

  - An NR-based cell operating standalone in unlicensed spectrum, connected to a 5G-CN network, e.g., for private network deployments;

  - Study how to ensure from a RAN level that connection and security management can be integrated with the E-UTRAN, NG RAN and 5G CN architecture, including service continuity requirements for users moving between cells of licensed and unlicensed frequency bands, liaising with SA2 as required

The results and findings of the study are documented in this technical report.

# 1	Scope

The present document contains the results and findings from the study item, “Study on NR-based Access to Unlicensed Spectrum” [2]. The purpose of this TR is to document the identified NR enhancements and corresponding evaluations for a single global solution framework for NR based access to unlicensed spectrum.

This document addresses evaluation methodology and possible scenarios for NR based unlicensed deployments.

This technical report documents the existing regulatory requirements for unlicensed spectrum deployment in the 5GHz bands, [and other bands]

This document identifies and captures coexistence evaluations of physical layer options and enhancements to NR and, if necessary, NR RAN protocols to meet the requirements and targets for unlicensed spectrum deployments.

This document contains an assessment of the feasibility of base station and terminal operation of 5GHz band (based on regulatory limits) in conjunction with relevant licensed frequency bands.

This document is a ‘living’ document, i.e. it is permanently updated and presented to TSG-RAN meetings.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-172021: “Revised SID on NR-based Access to Unlicensed Spectrum”.

[3]	FCC Part 15 ruling, http://www.ecfr.gov/cgi-bin/text-idx?SID=3c5e2d1533490603e0131fcdc041030d&node=pt47.1.15&rgn=div5

[4] 	FCC 13-22, “Notice of proposed rulemaking” , Feb 20, 2013.

[5] 	ETSI EN 301 893, Harmonized European Standard, “Broadband Radio Access Networks (BRAN); 5 GHz high performance RLAN”

[6] 	ETSI EN 302 502, Harmonized European Standard, “Broadband Radio Access Networks (BRAN); 5,8 GHz fixed broadband data transmitting systems”

[7] 	ETSI EN 302 571, Harmonized European Standard, “Intelligent Transport Systems (ITS); Radio communications equipment operating in the 5 855 MHz to 5 925 MHz frequency band”

[8]	“Commission decision of 11 July 2005 on the harmonised use of radio spectrum in the 5 GHz frequency band for the implementation of wireless access systems including radio local area networks (WAS/RLANs)” (2005/513/EC).

[9]	“Commission decisions of 12 February 2007 amending Decision 2005/513/EC on the harmonised use of radio spectrum in the 5 GHz frequency band for the implementation of Wireless Access Systems including Radio Local Area Networks (WAS/RLANs)” (2007/90/EC).

[10]	ECC/DEC (04)08, “ECC Decision of 09 July 2004 on the harmonised use of the 5 GHz frequency bands for the implementation of Wireless Access Systems including Radio Local Area Networks (WAS/RLANs)”

[11] 	KDB 443 999, FCC Office of Engineering and Technology – Laboratory Division: “Interim Plans to Approve UNII Devices Operating in the 5470-5725 MHz Band with Radar Detection and DFS Capabilities” (14th October 2010).

[12] 	FCC 12-148, “Notice of Proposed Rulemaking and Order: amendment of the Commission’s Rules with regard to Commercial Operations in the 3550-3650 MHz Band” (GN Docket No. 12-354), adopted and released December 12, 2012.

[13] 	FCC 13-154, “Public Notice. Commission Seeks Comment on Licensing Models and Technical Requirements in the 3550-3650 MHz Band”, released November 1, 2013

[14] 	ECC Recommendation ECC/REC (06)04: “Use of the band 5725- 5875 MHz for Broadband Fixed Wireless Access (BFWA)”

[15] 	Commission Decision 2008/671/EC of 5th August2008 on the harmonised use of radio spectrum in the 5875-5905 MHz frequency band for safety related application of Intelligent Transport Systems (IOTS)

[16] 	ECC Decision (08)01: "ECC Decision of 14 March 2008 on the harmonized use of the 5875-5925 frequency band for Intelligent Transport Systems (ITS)”

[17] 	ECC Recommendation (08)01:"Use of band 5855-5875 MHz for Intelligent Transport Systems (ITS)".

[18] 	ETSI EN 300 440-1 v1.6.1: “Electromagnetic compatibility and Radio spectrum Matters (ERM); Short range devices; Radio equipment to be used in the 1 GHz to 40 GHz frequency range; Part 1: Technical characteristics and test methods”

[19] 	Document RSCOM 13-32rev3: “Mandate to CEPT to study and identify harmonised compatibility and sharing conditions for Wireless Access Systems including Radio Local Area Networks in the bands 5350-5470 MHz and 5725-5925 MHz ('WAS/RLAN extension bands') for the provision of wireless broadband services”

[20] 	Group of Administrative Co-operation Under the R&TTE Directive (ADCO R&TTE): Report on the 5th joint cross-border R&TTE Market Surveillance Campaign on WLAN 5 GHz (2013)

[21] 	RSS-210 Issue 8 (December 2010): Licence-exempt Radio Apparatus (All Frequency Bands): Category I Equipment

[22] 	National Frequency Allocation Plan 2011 (In-Force): http://www.wpc.gov.in/WriteReadData/userfiles/file/National_Frequency_Allocation_Plan-2011.pdf

[23] 	http://legislacao.anatel.gov.br/resolucoes/2008/104-resolucao-506 , Resolução nº 506, de 1º de julho de 2008, Regulamento sobre Equipamentos de Radiocomunicação de Radiação Restrita.

[24]	3GPP TR 36.872: “Small cell enhancements for E-UTRA and E-UTRAN - Physical layer aspects”.

[25]	Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications, IEEE Std 802.11-2012.

[26]	https://mentor.ieee.org/802.11/dcn/14/11-14-0571-08-00ax-evaluation-methodology.docx

[27]	3GPP TR 36.942 V11.0.0, “Radio Frequency (RF) system scenarios,” Sept. 2012.

[28]	3GPP TR 38.802, "Study on new radio access technology Physical layer aspects"

[29]	3GPP TS 38.901, "Study on channel model for frequencies from 0.5 to 100 GHz"

[30]	3GPP RP-181339: “Revision of Study on NR-based Access to Unlicensed Spectrum”.

[31]

[32]

[33]

[34]

[35]	GSR No. 46(E) Dated: 28th Jan. 2005, Rule - Indoor Use of low power wireless equipment in the frequency band 5 GHz (Exemption from Licensing Requirement) Rules, 2005.

[36] 	GSR No. 37(E) Dated: 10th Jan. 2007, Rule - Indoor use of low power wireless equipment in the frequency band 5 GHz (Exemption from Licensing Requirement) Amendment Rules, 2006. (GSR No. 46(E) Ammendment).

[37] 	GSR No. 38(E) Dated: 19th Jan. 2007, Rule - the Outdoor Use of wireless Equipment  (Exemption from Licensing Requirement) Rules, 2007.[38]	RESOLUTION 229 (Rev. WRC-12), “Use of the bands 5 150-5 250, 5 250-5 350 MHz and 5 470-5 725 MHz by the mobile service for the implementation of wireless access systems including radio local area networks”.

[39]	Recommendation ITU-R M.1652-1 (05/2011) “Dynamic frequency selection in wireless access systems including radio local area networks for the purpose of protecting the radiodetermination service in the 5 GHz band”.

[40]	http://www.ncc.gov.tw/chinese/law_detail.aspx?site_content_sn=260&is_history=0&law_sn=1807&sn_f=1807, the regulations for low-power transmitters in Taiwan, June 28, 2011 (in Chinese).

[41]	http://www.motc.gov.tw/post/home.jsp?id=369&parentpath=0,364, the frequency allocation in Taiwan (in Chinese)

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Regulatory requirements

## 4.1	Regulatory requirements for 5GHz band

The range 5150-5925 MHz, or parts thereof, is potentially available for license-assisted access to unlicensed operation. This represents a significant amount of spectrum that can be used by operators to augment their service offerings in licensed bands. The range above can be operated under a license-exempt regime or ISM but must be shared with existing mobile services and other incumbent services. The quality of service offered by a licensed regime can therefore not be matched. Hence, unlicensed access is viewed as complementary, and does not reduce the need for additional allocations for licensed operation in view of the increased demand for wireless broadband access.

It is relevant to consider the global (International) ITU-R allocations and technical provisions first. These could be basis for defining globally harmonised bands for LAA and starting points for requirements and limits before the local variations are considered.

5150-5350 and 5470-5725 MHz

WRC 2003 allocated the bands 5 150-5 350 MHz and 5 470-5 725 MHz on a co-primary basis to the mobile service for the implementation of “wireless access systems (WAS), including radio local area networks (RLANs)”. This was subject to technical and regulatory provisions included in the radio regulations given in Resolution 229 (WRC-03), which was subsequently revised at WRC-12 to Resolution 229 (Rev. WRC-12) [38]. These provisions are followed by many Administrations and resolves:

1)	that the use of these bands by the mobile service will be for the implementation of WAS, including RLANs, as described in the most recent version of Recommendation ITU-R M.1450;

2)	that in the band 5 150-5 250 MHz, stations in the mobile service shall be restricted to indoor use with a maximum mean e.i.r.p.  of 200 mW and a maximum mean e.i.r.p. density of 10 mW/MHz in any 1 MHz band or equivalently 0.25 mW/25 kHz in any 25 kHz band;

3)	that administrations may monitor whether the aggregate pfd levels given in Recommendation ITU-R S.1426 have been, or will be exceeded in the future, in order to enable a future competent conference to take appropriate action;

4)	that in the band 5 250-5 350 MHz, stations in the mobile service shall be limited to a maximum mean e.i.r.p. of 200 mW and a maximum mean e.i.r.p. density of 10 mW/MHz in any 1 MHz band. Administrations are requested to take appropriate measures that will result in the predominant number of stations in the mobile service being operated in an indoor environment. Furthermore, stations in the mobile service that are permitted to be used either indoors or outdoors may operate up to a maximum mean e.i.r.p. of 1 W and a maximum mean e.i.r.p. density of 50 mW/MHz in any 1 MHz band, and, when operating above a mean e.i.r.p. of 200 mW, these stations shall comply with the following e.i.r.p. elevation angle mask where  is the angle above the local horizontal plane (of the Earth):

–13					dB(W/MHz)		for		0	<  < 8

–13 – 0.716(-8)		dB(W/MHz)		for		8	<  < 40

–35.9 – 1.22(-40)	dB(W/MHz)		for		40	<  < 45

–42					dB(W/MHz)		for		45	< ;

5)	that administrations may exercise some flexibility in adopting other mitigation techniques, provided that they develop national regulations to meet their obligations to achieve an equivalent level of protection to the EESS (active) and the SRS (active) based on their system characteristics and interference criteria as stated in Recommendation ITU-R RS.1632;

6)	that in the band 5 470-5 725 MHz, stations in the mobile service shall be restricted to a maximum transmitter power of 250 mW3 (administrations with existing regulations prior to WRC 03 may exercise some flexibility in determining transmitter power limits) with a maximum mean e.i.r.p. of 1 W and a maximum mean e.i.r.p. density of 50 mW/MHz in any 1 MHz band;

7)	that in the bands 5 250-5 350 MHz and 5 470-5 725 MHz, systems in the mobile service shall either employ transmitter power control to provide, on average, a mitigation factor of at least 3 dB on the maximum average output power of the systems, or, if transmitter power control is not in use, then the maximum mean e.i.r.p. shall be reduced by 3 dB;

8)	that, in the bands 5 250-5 350 MHz and 5 470-5 725 MHz, the mitigation measures found in Annex 1 to Recommendation ITU-R M.1652-1 shall be implemented by systems in the mobile service to ensure compatible operation with radiodetermination systems,

This resolution makes DFS as described in the Annex 1 of ITU-R Recommendation M.1652-1 [39] mandatory, the basis for the DFS requirements developed e.g. in Europe and the US.

WAS is defined as end-user radio connections to public or private core networks, while primary allocation means that the services can claim protection from services of the secondary service. However, the WAS/RLAN services must protect the incumbent primary services.

Even if primary in the International table of allocations, this may not be the case in all countries. The bands are not allocated on a primary basis in the US allocation table, but to the Part 15 rules that provide for operation of low power radio transmitters without a license (secondary service operated on a non-interference basis).

5725-5850 MHz

The 5725-5875 MHz is allocated for ISM applications by means a footnote in the allocation table. Radiolocation is allocated on primary basis up to 5850 MHz so DFS is required up to this limit. Operation in 5725-5850 MHz is allowed in the US under the Part 15 rules (15.247 and 15.407).

5850-5925 MHz

The band is allocated to the mobile service on a primary basis in all regions. In Europe it has been decided (2008) to harmonise the use of the 5875-5925 MHz frequency band for Intelligent Transport Systems (ITS). Similarly, according to the US allocation table, the use of the non-Federal mobile service in the band 5850-5925 MHz is limited to Dedicated Short Range Communications operating in the Intelligent Transportation System radio service.

### 4.1.1	ITU Region 1

#### 4.1.1.1	Europe

The European regulation is determined by the European Commission and the ECC. The relevant regulations for the 5 GHz bands are found in two Commission Decisions [8, 9] and one ECC Decision [10]. These are interpreted by ETSI and used as a basis for harmonized standards, which are used for conformance declaration when products are placed on the European market. Harmonized European standards have a higher regulatory relevance than other product standards, since they are produced based on a mandate from the Commission with reference to an EU directive. They also go through a public enquiry and voting process, and are cited by the Commission. The European requirements on 5 GHz unlicensed deployment are specified in three ETSI harmonized standards [5, 6, and 7]. Figure 4.1.1.1-1 summarizes the relevant parts of the 5 GHz band set aside for unlicensed spectrum usage. The 5150-5350 MHz and the 5470-5725 MHz bands are referred here as the broadband radio access networks (BRAN) bands where the wireless access systems (WAS) including RLAN equipment are operating in. Moreover, the 5725-5875 MHz band (in the BRAN domain) is used by the fixed wireless access (FWA) networks and finally the intelligent transport systems (ITS) utilize the 5855-5925 MHz band.

The BFWA and the ITS are designated by the ECC for use as parts of the 5 GHz band and the relevant regulations are found in:

-	An ECC Recommendation for FBWA [14], and

-	A Commission Decision [15], an ECC Decision [16] and an ECC Recommendation for ITS [17].

General purpose SRD devices can also operate in the band 5725-5875 MHz under the provisions of the ETSI harmonised standard EN 300 440 [18], but with reduced max EIRP of 25 mW.

Figure 4.1.1.1-1: 5 GHz spectrum allocations in Europe.

The European Commission has recently submitted to CEPT a mandate to study the conditions for the extension of the 5 GHz range designated for WAS/RLANs [19] in order to allow the use by WAS/RLANs of the whole 5150-5925 MHz band.

ECC approved CEPT Report 57 in March 2015 in response to the mandate based on the results of the Public Consultation. CEPT has carried out a significant amount of work but studies on mitigation techniques have not been completed in the timeframe and there are still a number of open issues ongoing.

Figure 4.1.1.1-2: Summary of existing and proposed EU regulations for WAS/RLANs in the 5GHz band

In the rest of this section, the specified ETSI requirements for the WAS/RLAN and FWA bands are summarized. Table 4.1.1.1-1 provides the limits on the transmit power control (TPC), the RF output power and power density given by the mean EIRP and the mean EIRP density at the highest power level. Additionally the requirements on the transmitter out of band emissions are listed in Table 4.1.1.1-2, Figures 4.1.1.1-3a, and Figure 4.1.1.1-3b. Table 4.1.1.1-4 illustrates the DFS requirements for some of these bands in Europe. Moreover, the 5150-5350 MHz is restricted to indoor deployments.

Transmit Power Control (TPC) is a mechanism to be used by the RLAN device to ensure a mitigation factor of at least 3 dB on the aggregate power from a large number of devices. This requires the RLAN device with TPC to have a TPC range for which the lowest value is at least 6 dB below the values for mean EIRP given in Table 4.1.1.1-1.

In ETSI EN 301 893 [5], the requirements on the Nominal Channel Bandwidth and the Occupied Channel Bandwidth are defined for unlicensed spectrum in the 5 GHz region. The Nominal Channel Bandwidth, i.e., the widest band of frequencies inclusive of guard bands assigned to a single channel, shall be at least 5MHz at all times. The Occupied Channel Bandwidth, i.e., the bandwidth containing 99 % of the power of the signal, shall be between 80 % and 100 % of the declared Nominal Channel Bandwidth. During an established communication, a device is allowed to operate temporarily in a mode where its Occupied Channel Bandwidth may be reduced to as low as 40 % of its Nominal Channel Bandwidth with a minimum of 4 MHz. The Occupied Channel Bandwidth is determined by the test procedure defined in Section 5.3.3.2 in [5].

ETSI mandates the usage of DFS in some bands as shown in Table 4.1.1.1-4. Furthermore, a Listen-Before-Talk (LBT) mechanism is requested independently of whether the channel is occupied or not, the LBT parameters being given in Table 4.1.1.1-5a and 4.1.1-5b. Both the requirements on frame-based equipment and the requirements on load-based equipment are summarized in Table 4.1.1.1-5a and 4.1.1.1-5b, respectively. Note that no LBT requirement is requested in [6] for the FWA band.

Table 4.1.1.1-1: TPC, Transmit power and power spectral density requirements in Europe

|  | Freq. range (MHz) | Max Mean EIRP (dBm) | Max Mean EIRP density (dBm/MHz) | Comment |
| --- | --- | --- | --- | --- |
| WAS/RLAN | 5150-5350 | 23 | 10 | 20 MHz and 40 MHz channels |
|  | 5470-5725 | 30 | 17 |  |
| FWA | 5725-5875 | 33 | 23 | 10 MHz channels |
|  | 5725-5875 | 36 | 23 | 20 MHz channels |
| Transmit Power Control (TPC): TPC ensures an average reduction in the aggregated transmission power by at least 3 dB (5 dB for FWA) compared with the maximum permitted transmission power. TCP is not required for channels within the band 5150-5250 MHz. Without TPC, the highest permissible average EIRP (density) are reduced by 3 dB. |  |  |  |  |

Table 4.1.1.1- 2: Requirements on out of band emissions in Europe

|  | Frequency range | Max. power | Bandwidth |
| --- | --- | --- | --- |
| WAS/RLAN, FWA | 30-47 MHz | -36 dBm | 100 kHz |
|  | 47-74 MHz | -54 dBm | 100 kHz |
|  | 75-87.5 MHz | -36 dBm | 100 kHz |
|  | 87.5-118 MHz | -54 dBm | 100 kHz |
|  | 118-174 MHz | -36 dBm | 100 kHz |
|  | 174-230 MHz | -54 dBm | 100 kHz |
|  | 230-470 MHz | -36 dBm | 100 kHz |
|  | 470-862 MHz | -54 dBm | 100 kHz |
|  | 0.862-1 GHz | -36 dBm | 100 kHz |
|  | 1-5.15 GHz | -30 dBm | 1 MHz |
|  | 5.35-5.5.47 GHz | -30 dBm | 1 MHz |
|  | 5.725-26 GHz | -30 dBm | 1 MHz |

NOTE:	dBc is the spectral density relative to the maximum spectral power density of the transmitted signal.

Figure 4.1.1.1-3a: Transmit spectral power mask for RLAN equipment operating within the frequency bands 5150-5250 MHz; 5250-5350 MHz or 5470-5725 MHz

NOTE1:	0 dB Reference Level is the spectral density relative to the maximum spectral power density of the transmitted signal.

NOTE2:	On the Frequency Offset axis, the figures apply to ChS = 20 MHz whereas the figures in parentheses apply to ChS = 10 MHz.

NOTE3:	Emissions that fall outside the lower and upper band frequency limits of 5 725 MHz and 5 875 MHz, respectively shall instead meet the unwanted emission limits of clause 4.3.1 [6].

Figure 4.1.1.1-3b: Transmit spectral power mask for FBWA equipment operating within the frequency band 5725-5875 MHz

FWA devices in the 5.8 GHz range are also subject to an additional requirement of EIRP spectral density limit in the elevation plane, see Table 5.

Table 4.1.1.1-3: EIRP spectral density limits in the elevation plane (5.8 GHz frequency range)

| EIRP spectral density | Elevation angle |
| --- | --- |
| For sectorised (e.g. P-MP Central or Base Station) and Omni-directional deployments: |  |
| −7 dB(W/MHz) | 0° ≤ θ <4° |
| −2.2 - (1.2*θ) dB(W/MHz) | 4° ≤ θ ≤ 15° |
| −18.4 - (0.15*θ) dB(W/MHz) | θ > 15° |
| For P-MP Customer Terminal Station and P-P deployments: |  |
| −7 dB(W/MHz) | for 0° ≤ θ <8° |
| −2.68 -(0.54*θ) dB(W/MHz) | 8° ≤ θ < 32° |
| −20 dB(W/MHz) | 32° ≤ θ ≤50° |
| −10 - (0.2*θ) dB(W/MHz) | θ > 50° |

Table 4.1.1.1-4: DFS requirements in Europe

| Parameter | Requirement | Comments |
| --- | --- | --- |
| DFS Threshold (dBm) for WAS/RLAN | -62(dBm) + 10(dBm/MHz) - EIRP Spectral density (dBm/MHz) + G(dBi) | *No DFS requirements on 5150 MHz – 5250 MHz *G denotes antenna gain |
| DFS Threshold (dBm) for FWA | -69 (dBm) + 23(dBm/MHz) - EIRP Spectral density (dBm/MHz) + G(dBi) | *No DFS requirements on 5850 MHz – 5875 MHz *G denotes antenna gain |
| Channel Availability check | 60 seconds outside 5600-5650 MHz | Master mode |
|  | 10 minutes inside 5600-5650 MHz |  |
| Channel move time | 10 seconds | Master and slave modes |
| Non-occupancy time | 30 minutes | After radar detection in either channel availability check or in-service monitoring |
| Uniform Spreading is required across the frequency ranges 5150 -5350 MHz and 5470-5725 MHz. Uniform Spreading is not applicable for equipment that only operates in 5150-5250 MHz band. |  |  |

Table 4.1.1.1-5a: LBT requirements for Frame-Based-Equipment in Europe

| Parameter | Requirement | Comment |
| --- | --- | --- |
| Clear Channel Assessment (CCA) time | Minimum 20μs |  |
| Channel Occupancy time | Minimum 1 ms, maximum10 ms |  |
| Idle period | Minimum 5% of channel occupancy time |  |
| Fixed frame period | Equals to Channel Occupancy time + Idle Period |  |
| Short control signaling transmission time | Maximum duty cycle of 5% within an observation period of 50ms | Part of Channel occupancy time |
| CCA Energy detection threshold | Assuming receive antenna gain G=0dBi: If EIRP=23dBm at transmitter Threshold ≤ -73 dBm/MHz Otherwise (different transmit power levels, PH) Threshold = -73(dBm/MHz) + 23(dBm) – PH(dBm) | For WAS/RLAN |

Table 4.1.1.1-5b: LBT requirements for Load-Based-Equipment in Europe

| Parameter | Requirement | Comment |
| --- | --- | --- |
| Clear Channel Assessment (CCA) time | Minimum 20μs | Also referred to as CCA time slot |
| N (number of clear idle slots) in extended CCA | N shall be randomly selected in the range 1..q every time, q=4…32 |  |
| Channel Occupancy time | <= (13/32) × q ms |  |
| Idle period | At least the duration of a random factor N multiplied by the CCA time slot. |  |
| Short control signaling transmission time | Maximum duty cycle of 5% within an observation period of 50ms | Part of Channel occupancy time |
| CCA Energy detection threshold | Assuming receive antenna gain G=0dBi: If EIRP=23dBm at transmitter Threshold ≤ -73 dBm/MHz Otherwise (different transmit power levels, PH) Threshold = -73(dBm/MHz) + 23(dBm) – PH(dBm) | For WAS/RLAN |

Interference to Weather Radars

Interference to Weather Radars is also a hot topic in the EU. The two last versions of EN 301 893 (1.6.1 and 1.7.1) have included amendments to better protect these radars, like the prohibition to give the user access to the configuration control settings that would allow him to disconnect the DFS functionality.

The use of the band 5.60-5.65 GHz by WLANs is allowed in Europe, and a Market Surveillance campaign on WLANs 5 GHz has been led at EU level by the ADCO R&TTE Group. The report [20] of the campaign has proposed specific recommendations to improve the situation. These recommendations do not require amendments to the last version of EN 301 893.

#### 4.1.1.2	Israel

In Israel the bands 5150-5250 MHz and 5250-5350 MHz are open to RLANs.

#### 4.1.1.3	Russia

In the Russian Federation the bands 5150-5350 MHz, the band 5470-5725 MHz above 5650 MHz and the band 5725-5825 MHz [20] are allowed to RLANs. Use of DFS is not mandated.

#### 4.1.1.4	South Africa

In South Africa the bands 5150-5250 MHz and 5250-5350 MHz are available to RLANs but restricted to indoor use. The band 5470-5725 MHz is also open to RLANs.

#### 4.1.1.5	Turkey

In Turkey the bands 5150-5250 MHz and 5250-5350 MHz are restricted to indoor use. DFS and TPC are mandated in the band 5470-5725 MHz.

### 4.1.2	ITU Region 2

#### 4.1.2.1	USA

The use of unlicensed 5 GHz spectrum in USA is governed by FCC part 15 regulations [3]. In Feb 2013, potential new rules were proposed in FCC 13-22 [4]. Figure 4.1.2.1-1 summarizes the relevant part 15 rules for 5GHz unlicensed spectrum usage:

Figure 4.1.2.1-1 : Summary of existing and proposed new FCC part 15 rules for 5GHz unlicensed spectrum usage

In Figure 4.1.2.1-1, U-NII-x bands denote frequency bands for Unlicensed National Information Infrastructure devices usage that are governed by §15.407 [3]. As shown in the figure, there is also an overlapping ruling of §15.247 from 5.725 to 5.85 GHz. A device could choose to follow either U-NII rulings or §15.247 rulings when operating within the frequency range.

In the rest of this section, we summarize FCC paragraphs 15.407 and 15.247 rules. In general either frequency hopping or digital modulation techniques are permitted under part 15 rules. Since LTE is not designed as a frequency hopping system, the rest of the document will focus on regulations related to digital modulation.

Paragraph 15.247 rules relevant for LTE point to multi-point communications are summarized in 4 aspects:

-	Transmission Bandwidth:

-	The minimum 6 dB bandwidth shall be at least 500 kHz.

-	Maximum Transmit Power:

-	Peak conducted output power shall not exceed 1 W. An alternative to peak power measurements is maximum conducted output power, which is the total transmit power over all antennas and antenna elements when the transmitter is operating at its maximum power control level.

-	Note that, the conducted output power limit is based on the use of antennas with directional gains that do not exceed 6 dBi. If transmitting antennas of directional gain greater than 6 dBi are used, the conducted output power from the intentional radiator shall be reduced by the amount in dB that the directional gain of the antenna exceeds 6 dBi.

-	Out of Band Emission:

-	In any 100 kHz bandwidth outside the frequency band, the radio frequency power that is produced by the intentional radiator shall be at least 20 dB below that in the 100 kHz bandwidth within the band that contains the highest level of the desired power, based on either an RF conducted or a radiated measurement. If the transmitter complies with the conducted power limits based on the use of RMS averaging over a time interval, the attenuation required under this paragraph shall be 30 dB instead of 20 dB.

-	Power Spectrum Density:

-	The power spectral density conducted from the intentional radiator to the antenna shall not be greater than 8 dBm in any 3 kHz band during any time interval of continuous transmission. The same method of determining the conducted output power shall be used to determine the power spectral density.

Paragraph15.407 rules for UNII devices are summarized in following tables. In Table 4.1.2.1-1, the maximum transmit power, PSD and out of band emission requirements are listed for UNII-1/2/3 bands. In Table 4.1.2.1-2, the dynamic frequency selection requirements for radar detection are summarized for UNII-2 devices.

Table 4.1.2.1-1: Transmit power requirements for UNII devices

|  |  | UNII-1 | UNII-2A | UNII-2C | UNII-3 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency Range (GHz) |  | 5.15 – 5.25 | 5.25-5.35 | 5.47-5.725 | 5.725-5.85 |  |
| Max conducted output power < min(a, b) (dBm) | a | eNB: 30 UE: 24 | 24 | 24 | 30 |  |
|  | b |  | 11+10logB | 11+10logB |  | B is the 26-dB emission bandwidth in MHz |
| Peak PSD (dBm/MHz) |  | eNB: 17 UE: 11 | 11 | 11 | 30dBm in 500kHz |  |
| Assumed Antenna Gain (dBi) |  | 6 | 6 | 6 | 6* | Peak power is reduced by G-6 dB for directional antennas with gain > 6 dBi; * UNII-3 fixed point to point operation power scaling threshold is 23 dBi |
| Out of band emission | Frequency Support (GHz) | Outside 5.15 – 5.35 | Outside 5.15 – 5.35 | Outside 5.47-5.725 | Outside 5.715-5.865 |  |
|  | EIRP (dBm/MHz) | -27 | -27 | -27 | -27 | Resolution bandwidth 1 MHz |
|  | Frequency Support (GHz) |  |  |  | 5.715-5.725 5.85-5.86 |  |
|  | EIRP (dBm/MHz) |  |  |  | -17 | Resolution bandwidth 1 MHz |
| Transmit Power Control |  | N/A | TPC to 6 dB below a mean EIRP of 30 dBm. No TPC for mean EIRP < 27 dBm |  | N/A |  |

Table 4.1.2.1-2: DFS requirements for UNII-2 devices

|  | Levels | Comments |
| --- | --- | --- |
| Max EIRP (dBm) | 23 to 30 | * DFS power is averaged in 1 micro-second for 0 dBi antenna. * Uniform spread over available channels. |
| DFS Threshold (dBm) | -64 |  |
| Max EIRP (dBm) | <23 |  |
| DFS Threshold (dBm) | -62 |  |
| Channel Availability check | 60 seconds | Master mode |
| Channel move time | 10 seconds | Master and slave modes |
|  | 200 ms normal operation |  |
| Non-occupancy time | 30 minutes | After radar detection in either channel availability check or in-service monitoring |

Interference to Weather Radars

In order to resolve interference to Terminal Doppler Weather Radar (TDWR) the FCC has defined interim plans to approve UNII devices operating in the 5470-5725 MHz band [11]. These interim plans provide specific actions for equipment authorization and installation, as detailed below. The main elements in the interim plan are:

-	Master devices shall not transmit on channels overlapping with the range 5600-5650 MHz band used by TDWRs;

-	Professional installation of equipment operating within the band 5470-5720 MHz;

-	Prohibition to include configuration controls (like country code settings) that would allow to change the frequency of operations to any frequency other than those specified on the grant of certification for US operation.

In parallel FCC is continuing its work to develop long-term equipment authorization test procedures that will ensure that the devices comply with the rules that protect the TDWR operations.

#### 4.1.2.2	Canada

In Canada, the use of RLANs is forbidden in the band 5600-5650 MHz in order to protect the meteorological radars from interference caused by RLANs. The regulations for RLANs in the 5 GHz range in Canada are defined in RSS-210 Annex 9 [21]. Table 4.1.2.2-1 presents the transmit power requirements for RLAN devices while Table 4.1.2.2-1 provides the EIRP spectral density limits in the elevation plane required from RLAN devices operating with an EIRP level higher than 200 mW in the frequency range 5250-5350 MHz.

Table 4.1.2.2-1: Transmit power requirements for RLAN devices in Canada

| Frequency Range (GHz) |  | 5.15 – 5.25 | 5.25-5.35 | 5.47-5.60 and 5.65-5.725 | 5.725-5.825 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Max conducted output power < min(a, b) (dBm) | a |  | 24 | 24 | 30 |  |
|  | b |  | 11+10logB | 11+10logB | 17+10logB | B is the 26-dB emission bandwidth in MHz |
| Peak PSD (dBm/MHz) |  | 4 | 11 | 11 | 17 | Resolution bandwidth 1 MHz |
| Max e.i.r.p. < min(a, b) (dBm) | a | 23 | 30 | 30 | 36 |  |
|  | b | 10+10logB | 17+10logB | 17+10logB | 23+10logB |  |
| Max e.i.r.p density (dBm/MHz) |  | 10 |  |  |  | Resolution bandwidth 1 MHz |
| Out of band emission | Frequency Support (GHz) | Outside 5.15 – 5.25 | Outside 5.25 – 5.35 | Outside 5.47-5.725 | Outside 5.725-5.825 |  |
|  | EIRP (dBm/MHz) | -27 | -27 | -27 | -17 within 5.715-5.725 and 5.825-5.835; -27 outside | Resolution bandwidth 1 MHz |
| Transmit Power Control |  | N/A | TPC to 6 dB below a mean EIRP of 30 dBm. No TPC for mean EIRP < 27 dBm |  |  |  |
| DFS |  | N/A | Required |  |  |  |
| e.i.r.p. elevation mask |  | N/A | If e.i.r.p. > 23 dBm compliance with e.i.r.p. elevation mask required | N/A | N/A |  |

Table 4.1.2.2-1:  EIRP spectral density limits in the elevation plane for devices with EIRP > 200 mW (5.25-5.35 GHz frequency range)

| EIRP spectral density | Elevation angle |
| --- | --- |
| −13 dB(W/MHz) | 0° ≤ θ <8° |
| –13 – 0.716(-8) dB(W/MHz) | 8° ≤ θ < 40° |
| –35.9 – 1.22(-40) dB(W/MHz) | 40° ≤ θ < 45° |
| –42 dB(W/MHz) | 44° ≤ θ |

#### 4.1.2.3	Brazil

In Brazil, nearly all the 5 GHz spectrum is allocated for Restricted Radiation, which means low-power unlicensed bands (i.e. any low-power-device can use it on a secondary basis). The bands 5250-5350 MHz, 5470-5650 MHz, 5650-5725 MHz and 5725-5850 MHz are allowed to RLANs [22]. Bands 5150-5250 MHz and 5250-5350 MHz are restricted to indoor use, and DFS is mandated in the bands 5250-5350 MHz and 5470-5725 MHz.

The relevant restrictions by band are as follows [23]:

| From (MHz) | To (MHz) | Service | Restriction [insert reference] |
| --- | --- | --- | --- |
| 5150 | 5350 | Restricted radiation | Indoor use only, EIRP limited to 200mW, EIRP spectral power density limited to 10mW/MHz. DFS mandated between 5250-5350MHz. |
| 5350 | 5470 | Unregulated |  |
| 5470 | 5650 | Restricted radiation | DFS mandated. Max transmitter output power limited to 250mW, EIRP limited to 1W, EIRP spectral power density limited to 50mW/MHz |
| 5650 | 5725 | Restricted radiation or amateur radio | DFS mandated. Max transmitter output power limited to 250mW, EIRP limited to 1W, EIRP spectral power density limited to 50mW/MHz |
| 5725 | 5875 | Restricted radiation (ISM Band) | Max transmitter output power limited to 1W, max EIRP EMF density of 50,000 microvolt per meter (measured at 3 meter distance) |

NOTE:	Note that the 5350-5470MHz band is not regulated.

#### 4.1.2.4	Mexico

In Mexico, the bands 5150-5250 MHz, 5250-5350 MHz, 5470-5600 MHz, 5650-5725 MHz and 5725-5875 MHz are open to RLANs [22].

### 4.1.3	ITU Region 3

#### 4.1.3.1	China

The 5150-5350 MHz frequency band is open to unlicensed WAS/RLANs indoor deployment in China. Furthermore, mandatory DFS / TPC (no less than 6 dB) or DFS only with a 3 dB backoff of the max mean EIRP, Power spectrum density and max emission is required for 5250-5350 MHz.

The key regulatory restrictions include:

-	EIRP: ≤200mW

-	Power Spectrum Density: ≤10dBm/MHz (EIPR)

-	Max Emission at edges of the used frequency:  ≤-80dBm/Hz (EIRP)

-	Spurious Emission (corresponding to frequency range outside 2.5*carrier bandwidth )

-	30-1000MHz: -36dBm/100kHz

-	48.5-72.5MHz, 76-118MHz, 167-223MHz, 470-798MHz: -54dBm/100kHz

-	2400-2483.5MHz: -40dBm/1MHz

-	5150-5350MHz: -33dBm/100kHz

-	5470-5850MHz: -40dBm/1MHz

-	Other frequency in 1-40GHz: -30dBm/1MHz

The 5725-5850 MHz frequency band was assigned as light licensed in 2009, shared among operators and traffic control bureau, open for both WAS (wireless access system) and RLAN, for both indoor and outdoor deployment in China. The key regulatory restrictions as below:

-	Transmit Power: ≤500mW and ≤27dBm;

-	EIRP: ≤2W and  ≤33dBm

-	Power Spectrum Density: ≤13dBm/MHz and ≤19dBm/MHz(EIRP)

-	Out of Band Emission:  ≤-80dBm/Hz(≤5725MHz and ≥5850MHz)

-	Spurious emission

-	30-1000MHz: ≤-36dBm/100kHz

-	2400-2483.5MHz: ≤-40dBm/1MHz

-	3400-3530MHz: ≤-40dBm/1MHz

-	5725-5850MHz: ≤-33dBm/100kHz

-	corresponding to frequency range outside 2.5*carrier bandwidth

-	Other frequency in 1-40 GHz: -30dBm/1MHz

In the end of 2014, the regulation requirements of this band were adjusted from light license to fully unlicensed. Meanwhile, some restrictions for equipment were proposed to be updated such as spurious emission etc., which was publicly inquired on the website of ministry of industry and information technology of China without formal issued so far. The proposed key regulation restrictions update in 5725-5850MHz can be found in the table below.

Table 4.1.3.1-1 Proposed key regulatory restrictions update in 5725-5850MHz

| Parameter | Requirement |
| --- | --- |
| EIRP | ≤25mW for Micro power(short range) station ≤2W for others |
| Power Spectrum Density(EIRP) | ≤19dBm/MHz (other than ITS system) |
| Max Emission at edges of the used frequency | ≤-80dBm/Hz（EIRP） |
| Spurious Emission (corresponding to frequency range outside 2.5*carrier bandwidth) | -36dBm/100kHz (30-1000MHz) -54dBm/100kHz (48.5-72.5MHz, 76-118MHz, 167-223MHz, 470-798MHz) -40dBm/1MHz (2400-2483.5MHz, 5150-5350MHz) -33dBm/100kHz (5470-5850MHz) -30dBm/1MHz(Other frequency in 1-40GHz) |

The band 5470-5725 MHz has not yet been officially open for WAS/RLAN (is put on hold). However, this band as a potential WAS/RLAN frequency band has been widely discussed, while the related regulation restrictions are also publicly inquired on the website of ministry of industry and information technology of China in the year of 2014. In order to protect the incumbent services (such as radio-determination service), DFS and TPC (no less than 6dB) are strictly required and DFS function can not be closed. The details of the proposed key regulation restrictions in 5470-5725MHz are listed as table below.

Table 4.1.3.1-2 Proposed key regulatory restrictions in 5470-5725MHz

| Parameter | Requirement |
| --- | --- |
| EIRP | ≤1W |
| Power Spectrum Density(EIRP) | ≤50mW/MHz |
| Max Emission at edges of the used frequency | ≤-80dBm/Hz（EIRP） |
| Spurious Emission (corresponding to frequency range outside 2.5*carrier bandwidth) | -36dBm/100kHz (30-1000MHz) -54dBm/100kHz (48.5-72.5MHz, 76-118MHz, 167-223MHz, 470-798MHz) -40dBm/1MHz (2400-2483.5MHz) -33dBm/100kHz (5470-5850MHz) -30dBm/1MHz (Other frequency in 1-40GHz) |

#### 4.1.3.2	Japan

With regard to the use of 5 GHz spectrum for RLAN in Japan, the following frequency bands are available:

-	5150-5250 MHz;

-	5250-5350 MHz;

-	5470-5725 MHz.

Table 4.1.3.2-1 summarizes overview of technical regulatory requirements in Japan based on those for IEEE 802.11a/n/ac. As shown in the table, it should be noted that “Max Burst Length” is specified as less than 4 msec for RLAN systems in Japan.

Table 4.1.3.2-1: Summary of basic regulatory requirements

| Frequency |  | 5.15-5.25 GHz | 5.25-5.35 GHz | 5.47~5.725 GHz |
| --- | --- | --- | --- | --- |
| Location |  | Limited to indoor |  | Indoor and outdoor |
| Channel bandwidth |  | 20/40/80/160 MHz |  |  |
| Modulation | 20/40/80/160 MHz | OFDM |  |  |
| Maximum output power | 20/40/80/160 MHz | 10/5/2.5/1.25 mW/MHz |  |  |
| Maximum antenna gain |  | Any |  |  |
| Maximum e.i.r.p | 20/40/80/160 MHz | 10/5/2.5/1.25 mW/MHz |  | 50/25/12.5/6.25 mW/MHz |
| Carrier sense | 20/40/80/160 MHz | Required |  |  |
| Max Burst Length |  | < 4 ms |  |  |
| DFS, TPC (Note 1) |  | Not required | Required (only for master control station) |  |
| Connection topology |  | Any | Any (connection between the stations not controlled by master control station is not allowed) |  |
| NOTE 1: DFS（Dynamic Frequency Selection）, TPC（Transmitter Power Control) |  |  |  |  |

Figure 4.1.3.2-1 indicates the spectrum and channelling arrangements available for different channel bandwidth. This figure also provides information on incumbent systems to be coexisting with RLAN systems in Japan.

Figure 4.1.3.2-1: Spectrum and channel arrangement

Further detailed regulatory requirements, such as ACLR, SEM and peak data rate for the respective frequency bands are provided in the following sub-sections. Note that for “78 MHz < Occupied bandwidth ≤ 158 MHz” (covers the 160 MHz system), there are no ACLR1 and ACLR2 requirements specified.

##### 4.1.3.2.1	5150-5250 and 5250-5350 MHz

Table 4.1.3.2.1-1: Adjacent Channel Leakage Ratio (ACLR) 1

| Occupied bandwidth | ≤ 18MHz | > 18MHz and ≤ 19MHz | > 19MHz and ≤ 38MHz | > 38MHz and ≤ 78MHz |
| --- | --- | --- | --- | --- |
| ACLR 1 | ≥ 25 dB | ≥ 25 dB | ≥ 25 dB | ≥ 25 dB |
| Measurement bandwidth | 18 MHz | 19 MHz | 38 MHz | 78 MHz |
| Adjacent channel centre frequency offset [MHz] | +20 / -20 | +20 / -20 | +40 / -40 | +80 / -80 |

Table 4.1.3.2.1-2: Adjacent Channel Leakage Ratio (ACLR) 2

| Occupied bandwidth | ≤ 18MHz | > 18MHz and ≤ 19MHz | > 19MHz and ≤ 38MHz | > 38MHz and ≤ 78MHz |
| --- | --- | --- | --- | --- |
| ACLR 2 | ≥ 40 dB | ≥ 40 dB | ≥ 40 dB | - |
| Measurement bandwidth | 18 MHz | 19 MHz | 38 MHz | - |
| Adjacent channel centre frequency offset [MHz] | +40 / -40 | +40 / -40 | +80 / -80 | - |

Occupied bandwidth ≤ 18 MHz:

Table 4.1.3.2.1-3: SEM: Occupied bandwidth ≤ 18 MHz

| Centre Frequency (MHz) | Frequency range(f) (MHz) | Frequency difference between centre frequency – the edges of frequency range(MHz) | e.i.r.p/MHz |
| --- | --- | --- | --- |
| 5240 | 5140 ≤ f ≤ 5142 | ≥ 98 and ≤ 100 | ≤ 2.5 W |
|  | 5142 < f ≤ 5150 | ≥ 90 and < 98 | ≤ 15 W |
|  | 5250 ≤ f < 5251 | ≥ 10 and < 11 | ≤ 101-(f-9) mW |
|  | 5251 ≤ f < 5260 | ≥ 11 and < 20 | ≤ 10-1-(890)(f-11) mW |
|  | 5260 ≤ f < 5266.7 | ≥ 20 and < 26.7 | ≤ 10-1.8-(650)(f-20) mW |
|  | 5266.7 ≤ f ≤ 5360 | ≥ 26.7 and ≤ 120 | ≤ 2.5 W |
| 5260 | 5140 ≤ f ≤ 5233.3 | ≥ 26.7 and ≤ 120 | ≤ 2.5 W |
|  | 5233.3 < f ≤ 5240 | ≥ 20 and < 26.7 | ≤ 10-1.8-(650)(f-20) mW |
|  | 5240 < f ≤ 5249 | ≥ 11 and < 20 | ≤ 10-1-(890)(f-11) mW |
|  | 5249 < f ≤ 5250 | ≥ 10 and < 11 | ≤ 101-(f-9) mW |
|  | 5350 ≤ f ≤ 5360 | ≥ 90 and ≤ 100 | ≤ 2.5 W |

18 MHz < Occupied bandwidth ≤ 19 MHz:

Table 4.1.3.2.1-4: SEM: 18 MHz < Occupied bandwidth ≤ 19 MHz

| Centre Frequency (MHz) | Frequency range(f) (MHz) | Frequency difference between centre frequency – the edges of frequency range(MHz) | e.i.r.p/MHz |
| --- | --- | --- | --- |
| 5180 | 5135 ≤ f ≤ 5142 | ≥ 38 and ≤ 45 | ≤ 2.5 W |
|  | 5142 < f ≤ 5150 | ≥ 30 and < 38 | ≤ 15 W |
| 5240 | 5250 ≤ f < 5251 | ≥ 10 and < 11 | ≤ 101-(f-9) mW |
|  | 5251 ≤ f < 5260 | ≥ 11 and < 20 | ≤ 10-1-(890)(f-11) mW |
|  | 5260 ≤ f < 5266.7 | ≥ 20 and < 26.7 | ≤ 10-1.8-(650)(f-20) mW |
|  | 5266.7 ≤ f ≤ 5365 | ≥ 26.7 and ≤ 125 | ≤ 2.5 μW |
| 5260 | 5135 ≤ f ≤ 5233.3 | ≥ 26.7 and ≤ 125 | ≤ 2.5 μW |
|  | 5233.3 < f ≤ 5240 | ≥ 20 and < 26.7 | ≤ 10-1.8-(650)(f-20) mW |
|  | 5240 < f ≤ 5249 | ≥ 11 and < 20 | ≤ 10-1-(890)(f-11) mW |
|  | 5249 < f ≤ 5250 | ≥ 10 and < 11 | ≤ 101-(f-9) mW |
| 5320 | 5350 ≤ f ≤ 5365 | ≥ 30 and ≤ 45 | ≤ 2.5 W |

19 MHz < Occupied bandwidth ≤ 38 MHz:

Table 4.1.3.2.1-5: SEM: 19 MHz < Occupied bandwidth ≤ 38 MHz

| Centre Frequency (MHz) | Frequency range(f) (MHz) | Frequency difference between centre frequency – the edges of frequency range(MHz) | e.i.r.p/MHz |
| --- | --- | --- | --- |
| 5190 | 5100 ≤ f ≤ 5141.6 | ≥ 48.4 and ≤ 90 | ≤ 2.5 W |
|  | 5141.6 < f ≤ 5150 | ≥ 40 and < 48.4 | ≤ 15 W |
| 5230 | 5250 ≤ f < 5251 | ≥ 20 and < 21 | ≤ 10-f-20+log(1/2) mW |
|  | 5251 ≤ f < 5270 | ≥ 21 and < 40 | ≤ 10-8190f-21-1+log(1/2) mW |
|  | 5270 ≤ f < 5278.4 | ≥ 40 and < 48.4 | ≤ 10-350f-40-1.8+log(1/2) mW |
|  | 5278.4 ≤ f ≤ 5400 | ≥ 48.4 and ≤ 170 | ≤ 2.5 W |
| 5270 | 5100 ≤ f ≤ 5210 | ≥ 60 and ≤ 170 | ≤ 2.5 W |
|  | 5210 < f ≤ 5221.6 | ≥ 48.4 and < 60 | ≤ 2.5 W |
|  | 5221.6 < f ≤ 5230 | ≥ 40 and < 48.4 | ≤ 10-350f-40-1.8+log(1/2) mW |
|  | 5230 < f ≤ 5249 | ≥ 21 and < 40 | ≤ 10-8190f-21-1+log(1/2)mW |
|  | 5249 < f ≤ 5250 | ≥ 20 and < 21 | ≤ 10-f-20+log(1/2) mW |
| 5310 | 5350 ≤ f < 5358.4 | ≥ 40 and < 48.4 | ≤ 15 W |
|  | 5358.4 ≤ f ≤ 5400 | ≥ 48.4 and ≤ 90 | ≤ 2.5 W |

38 MHz < Occupied bandwidth ≤ 78 MHz:

Table 4.1.3.2.1-6: SEM: 38 MHz < Occupied bandwidth ≤ 78 MHz

| Centre Frequency (MHz) | Frequency range(f) (MHz) | Frequency difference between centre frequency – the edges of frequency range(MHz) | e.i.r.p/MHz |
| --- | --- | --- | --- |
| 5210 | 5020 ≤ f ≤ 5123.2 | ≥ 86.8 and ≤ 190 | ≤ 2.5 W |
|  | 5123.2 < f ≤ 5150 | ≥ 60 and < 86.8 | ≤ 15 W |
|  | 5250 ≤ f < 5251 | ≥ 40 and < 41 | ≤ 10-f-40+log(1/4) mW |
|  | 5251 ≤ f < 5290 | ≥ 41 and < 80 | ≤ 10-8390f-41-1+log(1/4) mW |
|  | 5290 ≤ f < 5296.7 | ≥ 80 and < 86.7 | ≤ 10-3100f-80-1.8+log(1/4) mW |
|  | 5296.7 ≤ f ≤ 5480 | ≥ 86.7 and ≤ 270 | ≤ 2.5 W |
| 5290 | 5020 ≤ f ≤ 5203.3 | ≥ 86.7 and ≤ 270 | ≤ 2.5 W |
|  | 5203.3 < f ≤ 5210 | ≥ 80 and < 86.7 | ≤ 10-3100f-80-1.8+log(1/4) mW |
|  | 5210 < f ≤ 5249 | ≥ 41 and < 80 | ≤ 10-8390f-41-1+log(1/4) mW |
|  | 5249 < f ≤ 5250 | ≥ 40 and < 41 | ≤ 10-f-40+log(1/4) mW |
|  | 5350 ≤ f < 5376.8 | ≥ 60 and < 86.8 | ≤ 15 W |
|  | 5376.8 ≤ f ≤ 5480 | ≥ 86.8 and ≤ 190 | ≤ 2.5 W |

78 MHz < Occupied bandwidth ≤ 158 MHz:

Table 4.1.3.2.1-7: SEM: 78 MHz < Occupied bandwidth ≤ 158 MHz

| Centre Frequency (MHz) | Frequency range(f) (MHz) | Frequency difference between centre frequency – the edges of frequency range(MHz) | e.i.r.p/MHz |
| --- | --- | --- | --- |
| 5250 | 4916 ≤ f ≤ 5099.6 | ≥ 150.4 and ≤ 334 | ≤ 2.5 W |
|  | 5099.6 < f ≤ 5150 | ≥ 100 and < 150.4 | ≤ 15 W |
|  | 5350 ≤ f < 5400.4 | ≥ 100 and < 150.4 | ≤ 15 W |
|  | 5400.4 ≤ f ≤ 5584 | ≥ 150.4 and ≤ 334 | ≤ 2.5 W |

Table 4.1.3.2.1-8: Peak data rate

| Occupied bandwidth(MHz) | Peak data rate (Mbps) |
| --- | --- |
| ≤ 19 | ≥ 20 |
| > 19 and ≤ 38 | ≥ 40 |
| > 38 and ≤ 78 | ≥ 80 |
| > 78 and ≤ 158 | ≥ 160 |

##### 4.1.3.2.2	5470-5725 MHz

Table 4.1.3.2.2-1: Adjacent Channel Leakage Ratio (ACLR) 1

| Occupied bandwidth | ≤ 19.7MHz | > 19.7MHz and ≤ 38MHz | > 38MHz and ≤ 78MHz |
| --- | --- | --- | --- |
| ACLR 1 | ≥ 25 dB | ≥ 25 dB | ≥ 25 dB |
| Measurement bandwidth | 19 MHz | 38 MHz | 78 MHz |
| Adjacent channel centre frequency offset [MHz] | +20 / -20 | +40 / -40 | +80 / -80 |

Table 4.1.3.2.2-2: Adjacent Channel Leakage Ratio (ACLR) 2

| Occupied bandwidth | ≤19.7MHz | > 19.7MHz and ≤ 38MHz | > 38MHz and ≤ 78MHz |
| --- | --- | --- | --- |
| ACLR 2 | ≥ 40 dB | ≥ 40 dB | - |
| Measurement bandwidth | 19 MHz | 38 MHz | - |
| Adjacent channel centre frequency offset [MHz] | +40 / -40 | +80 / -80 | - |

Occupied bandwidth ≤ 19.7 MHz:

Table 4.1.3.2.2-3: SEM: Occupied bandwidth ≤ 19.7 MHz

| Frequency range(f) (MHz) | e.i.r.p/MHz |
| --- | --- |
| 5455 ≤ f ≤ 5460 | ≤ 2.5 W |
| 5460 < f ≤ 5470 | ≤ 12.5 W |
| 5725 ≤ f < 5740 | ≤ 12.5 W |
| 5740 ≤ f ≤ 5745 | ≤ 2.5 W |

19.7 MHz < Occupied bandwidth ≤ 38 MHz:

Table 4.1.3.2.2-4: SEM: 19.7 MHz < Occupied bandwidth ≤ 38 MHz

| Frequency range(f) (MHz) | e.i.r.p/MHz |
| --- | --- |
| 5420 ≤ f ≤ 5460 | ≤ 12.5 W |
| 5460 < f ≤ 5470 | ≤ 50 W |
| 5725 ≤ f ≤ 5760 | ≤ 12.5 W |

38 MHz < Occupied bandwidth ≤ 78 MHz:

Table 4.1.3.2.2-5: SEM: 38 MHz < Occupied bandwidth ≤ 78 MHz

| Frequency range(f) (MHz) | e.i.r.p/MHz |
| --- | --- |
| 5340 ≤ f ≤ 5460 | ≤ 12.5 W |
| 5460 < f ≤ 5469.5 | ≤ 50 W |
| 5469.5 < f ≤ 5470 | ≤ 51.2 W |
| 5725 ≤ f ≤ 5800 | ≤ 12.5 W |

78 MHz < Occupied bandwidth ≤ 158 MHz:

Table 4.1.3.2.2-6: SEM: 78 MHz < Occupied bandwidth ≤ 158 MHz

| Frequency range(f) (MHz) | e.i.r.p/MHz |
| --- | --- |
| 5236 ≤ f ≤ 5419.6 | ≤ 12.5 W |
| 5419.6 < f ≤ 5470 | ≤ 50 W |
| 5725 ≤ f ≤ 5904 | ≤ 12.5 W |

Table 4.1.3.2.2-7: Peak data rate

| Occupied bandwidth(MHz) | Peak data rate (Mbps) |
| --- | --- |
| ≤ 19.7 | ≥ 20 |
| > 19.7 and ≤ 38 | ≥ 40 |
| > 38 and ≤ 78 | ≥ 80 |
| > 78 and ≤ 158 | ≥ 160 |

##### 4.1.3.2.3	Simultaneous use of 5150-5250 and 5470-5725 MHz / 5250-5350 and 5470-5725 MHz

When simultaneous use of non-contiguous two channels in 5150-5250 and 5470-5725 MHz or that in 5250-5350 and 5470-5725MHz is employed, the following regulatory requirements are also applied. It should be note that total channel bandwidth is less and equal to 160 MHz in these usages.

Table 4.1.3.2.3-1: SEM: 78 MHz < Occupied bandwidth ≤ 158 MHz

| Case | Combination of simultaneous transmission Centre Frequencies |  |
| --- | --- | --- |
|  | Centre Frequency 1 (MHz) | Centre Frequency 2 (MHz) |
| 1 | 5210 | 5530 |
| 2 |  | 5610 |
| 3 | 5290 | 5530 |
| 4 |  | 5610 |

Side conditions

-	Occupied bandwidth for respective Carrier for Centre frequency is > 38 MHz and ≤ 78 MHz.

-	Maximum Output power ≤ 1.25 mW

-	e.i.r.p.≤ 1.25 mW

Table 4.1.3.2.3-2: Adjacent Channel Leakage Ratio (ACLR)

| Occupied bandwidth | > 38MHz and ≤ 78MHz |
| --- | --- |
| ACLR | ≥ 25 dB |
| Measurement bandwidth | 78 MHz |
| Adjacent channel centre frequency offset [MHz] | +80 / -80 |
| Note: Applicable to respective carrier |  |

Table 4.1.3.2.3-3: SEM: Case 1 and 2

| Centre Frequency (MHz) | Frequency range(f) (MHz) | Frequency difference between centre frequency – the edges of frequency range(MHz) | e.i.r.p/MHz |
| --- | --- | --- | --- |
| 5,210 | 5,020 ≤ f ≤ 5,134.8 | ≥ 75.2 and ≤ 190 | ≤ 2.5 W |
|  | 5,134.8 < f ≤ 5,150 | ≥ 60 and < 75.2 | ≤ 12.5 W |
|  | 5,250 ≤ f < 5,251 | ≥ 40 and < 41 | ≤ 10-f-40+log⁡(1/8) mW |
|  | 5,251 ≤ f < 5,285.2 | ≥ 41 and < 75.2 | ≤ 10-8390f-41-1+log⁡(1/8) mW |
|  | 5,285.2 ≤ f < 5,370 | ≥ 75.2 and < 160 | ≤ 2.5 W |
| 5,530 | 5,370 ≤ f ≤ 5,454.8 | ≥ 75.2 and ≤ 160 | ≤ 2.5 W |
|  | 5,454.8 < f ≤ 5,470 | ≥ 60 and < 75.2 | ≤ 15 W |
| 5,610 | 5,725 ≤ f ≤ 5,800 | ≥ 115 and ≤ 190 | ≤ 15 W |

Table 4.1.3.2.3-4: SEM: Case 3 and 4

| Centre Frequency (MHz) | Frequency range(f) (MHz) | Frequency difference between centre frequency – the edges of frequency range(MHz) | e.i.r.p/MHz |
| --- | --- | --- | --- |
| 5,290 | 5,020 ≤ f ≤ 5,214.8 | ≥ 75.2 and ≤ 270 | ≤ 2.5 W |
|  | 5,214.8 < f ≤ 5,249 | ≥ 41 and < 75.2 | ≤ 10-8390f-41-1+log⁡(1/8) mW |
|  | 5,249 < f ≤ 5,250 | ≥ 40 and < 41 | ≤ 10-f-40+log⁡(1/8) mW |
|  | 5,350 ≤ f < 5,365.2 | ≥ 60 and < 75.2 | ≤ 15 W |
|  | 5,365.2 ≤ f < 5,410 | ≥ 75.2 and < 120 | ≤ 2.5 W |
| 5,530 | 5,410 ≤ f ≤ 5,454.8 | ≥ 75.2 and ≤ 120 | ≤ 2.5 W |
|  | 5,454.8 < f ≤ 5,470 | ≥ 60 and < 75.2 | ≤ 15 W |
| 5,610 | 5,725 ≤ f ≤ 5,800 | ≥ 115 and ≤ 190 | ≤ 15 W |

Table 4.1.3.2.3-5: Peak data rate

Peak data rate (Mbps)

≥ 160

#### 4.1.3.3	Korea

Table 4.1.3.3-1: Transmit power requirements for 5GHz Devices

| Frequency Range(MHz) | 5100-5250 |  | 5250-5350 |  | 5470-5650 |  | 5725-5825, 2400~2483.5 (Note 7) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average PSD requirement (mW/MHz) | BW(MHz) | PSD | BW(MHz) | PSD | BW(MHz) | PSD | BW(MHz) | PSD |
|  | 0.5-20 | ≤2.5 | 0.5-20 | ≤10 | 0.5-20 | ≤10 | 0.5-26 | ≤10 |
|  | 20-40 | ≤1.25 | 20-40 | ≤5 | 20-40 | ≤5 | 26-40 | ≤5 |
|  | 40-80 | ≤0.625 | 40-80 | ≤2.5 | 40-80 | ≤2.5 | 40-80 | ≤2.5 |
|  |  |  |  |  |  |  | 40-60 (Note 1) | ≤0.1 (Note 1) |
| Assumed antenna gain G (dBi) (Note 4) | 6 |  | 7 |  | 7 |  | 6, 20 (PTP) (Notes 2, 3) |  |
| Out of band emission, EIRP (dBm/MHz) | -27 |  | -27 |  | -27 |  | For any 100kHz in outside band, less than -30dBm(2400-2483.5MHz) -27dBm/MHz (5725-5825MHz) |  |
| DFS (Note 5) | No |  | Yes |  | Yes |  | No |  |
| TPC (Note 6) | No |  | Yes |  | Yes |  | No |  |
| NOTE 1: 2400-2483.5MHz devices NOTE 2: Fixed point to point operation power scaling threshold is 20 dBi NOTE 3: PTP: Point to Point communication NOTE 4: Peak power is reduced by G-THR dB for directional antennas with gain > THR dBi (THR = 6 dBi for 51005250MHz and 5725-5825MHz or 7dBi for 5250-5350MHz and 5470-5650MHz) NOTE 5: DFS: Dynamic Frequency Selection NOTE 6: TPC: Transmit Power Control NOTE 7: 5725-5825MHz, 2400-2483.5MHz are not allowed to be used for point-to-multipoint service for the same information and omni-directional electro-magnetic wave transmission |  |  |  |  |  |  |  |  |

Table 4.1.3.3-2: DFS requirements for 5GHz Devices

|  | Levels | Comments |
| --- | --- | --- |
| DFS Threshold (dBm) for interference detection | -62dBm | Average power considering antenna gain: <10mW/MHz |
|  | -64dBm | Average power considering antenna gain: 10mW/MHz ~50mW/MHz |
| Channel availability check time | > 60 seconds |  |
| Channel move time | < 10 seconds |  |
| Non-occupancy time | > 30 minutes | After radar detection in either channel availability check or in-service monitoring |

Korean regulatory requirements are summarized as follows:

-	Average PSD requirement (mW/MHz)

-	It covers both power spectral density and Max transmission power

-	The maximum PSD is defined for a given spectrum range

-	Maximum transmission power :

-	50mW, 200mW, 200mW and 200/260mW for 5150-5250, 5250-5350, 5470-5650, and 5725-5825 MHz band  respectively

-	Antenna gain assumed:

-	6 dBi for 5150-5250 and 5725-5825MHz band

-	7 dBi for 52505350 and 5470-5650MHz band

-	Out of Band Emission (EIRP (dBm/MHz) )

-	<-27 dBm/MHz

-	DFS & TPC

-	DFS is defined for 5250-5350MHz and 5470-5650MHz

-	TPC is defined for 5250-5350MHz and 5470-5650MHz

-	TPC ensures wireless devices with average Tx power including antenna gain larger than 25mW/MHz can reduce its average Tx power by at least 3dB (below 12.5mW/MHz)

-	Maximum bandwidth occupancy

-	< 80MHz for 5GHz

-	Maximum power spectral density for bandwidth aggregation (contiguous or non-contiguous)

-	Among 5150-5250, 5250-5350, 5470-5650, and 5725-5825 MHz bands, multiple of 80MHz bandwidth can be aggregated in contiguous or non-contiguous manner to form 160MHz bandwidth

-	In this case, maximum power spectral density for 5150-5250MHz should be lower than 0.625mW/MHz while it should be lower than 1.25mW/Hz in the other bands

-	Modulation scheme

-	Digital modulation for 5GHz

-	Difference from FCC regulation

-	Maximum transmission power in case of 5725 to 5825MHz is still 200mW for 20MHz bandwidth (a bit lower compared to 1W power in UNII-3)

-	5825-5850MHz is not for WAS (Wireless Access System)

#### 4.1.3.4	India

In India the bands 5150-5250 MHz, 5250-5350 MHz, 5570-5725 MHz and 5725-5875 MHz are open to RLANs [22], [35], [36], [37]. Table 4.1.3.4-1 and Table 4.1.3.4-2 summarize regulatory requirements in India for indoor and outdoor deployments respectively. Some parts of the 5 GHz band shall follow usage “on non-interference, non-protection and shared (non-exclusive) basis”. These are the 5725-5825 MHz band open to licensed WAS including RLANs, and the 5150-5350 MHz and 5725-5875 MHz bands open for unlicensed WAS including RLANs for indoor deployment.

Table 4.1.3.4-1: Summary of regulatory requirements for indoor deployment in India

| Regulation code | NFAP2011-IND 67, GSR No 46E, 37E | NFAP2011-IND 69 |
| --- | --- | --- |
| Band (MHz) | 5150-5350, 5725-5875 | 5570-5725 |
| License Type | Unlicensed | Licensed |
| Maximum mean EIRP | 200mW (23dBm) | 1W (30dBm) |
| Maximum mean EIRP density | 10 mW/MHz | 50 mW/MHz |
| Band usage | Low power WAS including RLAN | Low power WAS including RLAN |

Table 4.1.3.4-2: Summary of regulatory requirements for outdoor deployment in India

| Regulation code | NFAP2011-IND 68 | NFAP2011-IND 69 | NFAP2011-IND 71 | NFAP2011-IND 72, GSR No 38E |
| --- | --- | --- | --- | --- |
| Band (MHz) | 5150-5250 | 5570-5725 | 5725-5825 | 5825-5875 |
| License Type | Licensed | Licensed | Licensed | Unlicensed |
| Maximum transmitter output power |  |  | 1 W (30dBm) in spread of 10 MHz or higher | 1 W (30dBm) in spread of 10 MHz or higher |
| Maximum mean EIRP | 200mW (23dBm) | 1W (30dBm) | 4W (36dBm) | 4W (36dBm) |
| Maximum mean EIRP density | 10 mW/MHz | 50 mW/MHz |  |  |
| Band usage | Low power WAS including RLAN | Low power WAS including RLAN | Low power WAS including RLAN and Dedicated Short Range Communications (DSRC) for Intelligent Transport Networks | Low power WAS including RLAN |

.

#### 4.1.3.5	Taiwan

In Taiwan the bands 5250-5350 MHz, 5470-5600 MHz, 5650-5725 MHz and 5725-5850 MHz are allocated to RLANs [22][40]. Table 4.1.3.5-1 and Table 4.1.3.5-2 summarize the current regulatory requirements for transmit power and DFS in Taiwan [40]. DFS is mandate for 5470-5725 MHz [41]. Recently, work for specifying requirements for allowing RLANs in 5150-5250 MHz and 5600-5650 MHz has started but the detailed regulatory requirements for this has not yet been specified. Additionally specification work for allowing 5250-5350 MHz outdoor has started, this assumes that DFS will be performed, but detailed requirements are not yet defined.

Table 4.1.3.5-1 Transmit power requirements for 5GHz band in Taiwan

| Frequency Range (GHz) |  | 5.25-5.35 * | 5.47-5.60 and 5.65-5.725 | 5.725-5.825 | *For indoor use only |
| --- | --- | --- | --- | --- | --- |
| Peak transmit power < min(a, b) (dBm) | A | 17 | 24 | 30 |  |
|  | B | 4+10logB | 11+10logB | 17+10logB | B is the 26-dB emission bandwidth in MHz |
| Peak PSD (dBm/MHz) |  | 4 | 11 | 17 | Resolution bandwidth 1 MHz |
| Assumed Antenna Gain (dBi) |  | 6 | 6 | 6** | Peak power is reduced by G-6 dB for directional antennas with gain > 6 dBi; ** Fixed point to point operation power scaling threshold is 23 dBi |
| Out of band emission | Frequency Support (GHz) | Outside 5.25 – 5.35 | Outside 5.47-5.725 | Outside 5.715-5.835 |  |
|  | EIRP (dBm/MHz) | -27 | -27 | -27 | Resolution bandwidth 1 MHz |
|  | Frequency Support (GHz) |  |  | 5.715-5.725 5.825-5.835 |  |
|  | EIRP (dBm/MHz) |  |  | -17 | Resolution bandwidth 1 MHz |
| Transmit Power Control |  | N/A | TPC to 6 dB below a mean EIRP of 30 dBm. No TPC for mean EIRP < 27 dBm | N/A |  |
| DFS |  | Required | Required | N/A |  |

Table 4.1.3.5-2 DFS requirements for 5.470-5.725GHz band in Taiwan

|  | Levels | Comments |
| --- | --- | --- |
| DFS Detection Threshold (dBm) | -64 for EIRP between 200mW and 1W | DFS power is averaged in 1 micro-second for 0 dBi antenna. Uniform spread over available channels. |
|  | -62 for EIRP < 200mW |  |
| Channel availability check time | 60 seconds | Master mode |
| Channel move time | <10 seconds | Master and slave mode After detection of radar signal: at most 200ms for normal communication manage and control signals can be transmitted discontinuously in the remaining time within 10 seconds |
| Non-occupancy time | >30 minutes | After radar detection in either channel availability check or in-service monitoring |

#### 4.1.3.6	Singapore

In Singapore the bands 5150-5250 MHz, 5250-5350 MHz, 5470-5725MHz and 5725-5850 MHz are open to RLANs.

#### 4.1.3.7	Australia

In Australia the band bands 5150-5250 MHz, 5250-5350 MHz and 5470-5725 MHz, except the sub-band 5600-5650 MHz, are open to RLANs. DFS and TPC are mandatory in the bands 5250-5350 MHz and 5470-5725 MHz.

### 4.1.4	Applicability of DFS requirements

The concept surfaced around 2001 in the ECC for handling uniform spread of WLAN interference into satellite and radar services, but was soon extended to include methods for discovery and avoidance of frequencies used by the radar service in the preparation work for WRC 2003.

Use of DFS in accordance with Annex 1 of ITU-R Recommendation M.1652 [39] is mandated as per Resolution 229 [38]. The 5 GHz Harmonized European Standard developed by ETSI TC BRAN was the first to include these DFS rules (EN 301 893 V1.2.3 in 2003), the DFS test specification included in this standard therefore became the basis for the development of the FCC DFS test specification and other test specifications in other countries.

For NR-U, it appears that conformance testing would only have to be performed for the BS. Already at the outset in ITU-R M.1652 it was made clear that full DFS functionality may not have to be implemented in all devices, only those controlling the transmission [39]:

2.1 Detection requirements

The DFS mechanism should be able to detect interference signals above a minimum DFS detection threshold of –62 dBm for devices with a maximum e.i.r.p. of < 200 mW and –64 dBm for devices with a maximum e.i.r.p. of 200 mW to 1 W3 averaged over 1 s.

This is defined as the received signal strength (RSS) (dBm), normalized to the output of a 0 dBi receive antenna, that is required to be detected within the WAS channel bandwidth.

2.2 Operational

with the footnote 3 stating:

In practice, it may not be necessary for each device to implement full DFS functionality, provided that such devices are only able to transmit under the control of a device that ensures that all DFS requirements are fulfilled.

This is reflected in both regulation for Europe and the Part 15 rules by the FCC. The harmonized ETSI BRAN standard cited in the EC commission rules is also followed by many other countries outside Europe.

#### 4.1.4.1	DFS according to ECC

In accordance with the ECC Decision [10], “Every Master Device will use the Radar Interference Detection function in order to check for any co-channel radar signal prior to use a channel but also during normal operation. In addition to this Radar Interference Detection function, every Master Device shall also implement a channel selection mechanism to ensure a near uniform spread of the loading of available spectrum.” In the latest version of the EN 301 893 [5], the applicability of the DFS requirements is listed as follows:

Table 5 lists the DFS related technical requirements and their applicability for every operational mode. If the RLAN device is capable of operating in more than one operational mode then every operating mode shall be assessed separately.

Table 5 (from EN 301 893): Applicability of DFS requirements

| Requirement | DFS Operational mode |  |  |  |
| --- | --- | --- | --- | --- |
|  | Master | Slave without radar detection (see table D.2, note 2) | Slave with radar detection (see table D.2, note 2) |
| Channel Availability Check |  | Not required | (see note 2) |
| Off-Channel CAC (see note 1) |  | Not required | (see note 2) |
| In-Service Monitoring |  | Not required |  |
| Channel Shutdown |  |  |  |
| Non-Occupancy Period |  | Not required |  |
| Uniform Spreading |  | Not required | Not required |
| NOTE 1: Where implemented by the manufacturer. NOTE 2: A slave with radar detection is not required to perform a CAC or Off-Channel CAC at initial use of the channel but only after the slave has detected a radar signal on the Operating Channel by In-Service Monitoring. |  |  |  |

The radar detection requirements specified in clauses 4.7.2.2 to 4.7.2.4 assume that the centre frequencies of the radar signals fall within the central 80 % of the Occupied Channel Bandwidth of the RLAN channel (see clause 4.3).

Notice also the assumption that the radar signal falls within 80% of the occupied bandwidth, which is linked to the requirements on occupied emission bandwidth in EN 801 893 (clause 4.7).

#### 4.1.4.2	DFS in the Part 15 rules

The rules with regard to implementation of DFS functionality are similar in the Part 15 rules [3]; the required functionality is limited for the slave,

(2) Radar Detection Function of Dynamic Frequency Selection (DFS). U-NII devices operating with any part of its 26 dB emission bandwidth in the 5.25-5.35 GHz and 5.47-5.725 GHz bands shall employ a DFS radar detection mechanism to detect the presence of radar systems and to avoid co-channel operation with radar systems. Operators shall only use equipment with a DFS mechanism that is turned on when operating in these bands. The device must sense for radar signals at 100 percent of its emission bandwidth. The minimum DFS detection threshold for devices with a maximum e.i.r.p. […]

(i) Operational Modes. The DFS requirement applies to the following operational modes:

(A) The requirement for channel availability check time applies in the master operational mode.

(B) The requirement for channel move time applies in both the master and slave operational modes.

(ii) Channel Availability Check Time. A U-NII device shall check if there is a radar system already operating on the channel before it can initiate a transmission on a channel and when it has to move to a new channel. The U-NII device may start using the channel if no radar signal with a power level greater than the interference threshold values listed in paragraph (h)(2) of this section, is detected within 60 seconds.

(iii) Channel Move Time. After a radar's presence is detected, all transmissions shall cease on the operating channel within 10 seconds. Transmissions during this period shall consist of normal traffic for a maximum of 200 ms after detection of the radar signal. In addition, intermittent management and control signals can be sent during the remaining time to facilitate vacating the operating channel.

Similar to the ETSI BRAN harmonized standard, there is also a requirement that “the device must sense for radar signals at 100 percent of its emission bandwidth”. In deriving the minimum DFS detection threshold for the WAS receiver, it was assumed that above this threshold the emissions within the RLAN bandwidth would desensitize the radar receiver. Reciprocity was thus assumed, the WAS should be able to detect a radar within its emission bandwidth. The bandwidth of the WAS affects the probability of detecting a radar signal and its side lobes and the probability of causing interference (the WAS emissions).

## 4.2	Regulatory requirements for [X]GHz band

# 5	Spectrum considerations

For the NR-U study item, different unlicensed bands or shared bands have been discussed, such as 2.4 GHz band, 3.5 GHz band, 5 GHz band, and 6 GHz band. Some bands are available globally and some bands are only available in some specific regions.

Follow the NR design principles, from RAN1 design perspective, this NR-U study is not limited to a particular unlicensed band. Instead, the target NR-U design is applicable to a set of frequency ranges to be further defined. There is no prioritization between unlicensed bands. On the other hand, the NR-U study does not target sub-1 GHz unlicensed bands, due to the lack of spectrum in the band to support efficient NR-U operation.

# 6	Deployment scenarios

This section describes possible deployment scenarios for NR-U.

Five deployment scenarios have been identified:

- Scenario A: Carrier aggregation between licensed band NR (PCell) and NR-U (SCell)

  - NR-U SCell may have both DL and UL, or DL-only.

- Scenario B: Dual connectivity between licensed band LTE (PCell) and NR-U (PSCell)

- Scenario C: Stand-alone NR-U

- Scenario D: An NR cell with DL in unlicensed band and UL in licensed band

- Scenario E: Dual connectivity between licensed band NR (PCell) and NR-U (PSCell)

# 7	Design targets, functionalities and solutions

In the discussions in the NR-U study item, references to sub-7 GHz are intended to include unlicensed bands in the 6 GHz region that are being discussed in regulatory discussions which may have some region exceeding 7 GHz (e.g., 7.125 GHz).

## 7.1	Design targets and functionalities

The NR-U study targets identification of additional functionality needed for a PHY layer design (except channel access procedures) for operation in unlicensed spectrum that may be applicable over a particular frequency range (e.g., sub-7 GHz, 7-52.6 GHz, > 52.6 GHz). The definition of the frequency ranges is to be further defined. On the other hand, the optimizations for a particular frequency band may be necessary due to different requirements, such as PSD limitation or OCB requirement, for each band. Channel bandwidths below 5 MHz are not targeted in this study.

The study targets the design of channel access procedures for frequency bands based on coexistence and regulatory considerations applicable to the band. The study includes identification of procedures for technology neutral channel access for frequency bands that may become available subject to regulations. The study assumes regulation will provide the framework concerning the protection for the technologies not using unlicensed access in those bands.

## 7.2	Solutions for operation in unlicensed spectrum

### 7.2.1	Physical layer aspects

According to the study item description [30], physical channels inheriting the choices of duplex mode, waveform, carrier bandwidth, subcarrier spacing, frame structure, and physical layer design made as part of the NR study and avoiding unnecessary divergence with decisions made in the NR WI.

NR-U supports both Type-A and Type-B mapping already supported in NR.

Initial active DL/UL BWP is approximately 20MHz for 5GHz band, though the final value will be quantized to number of PRBs. Initial active DL/UL BWP is approximately 20MHz for 6GHz band if similar channelization as 5GHz band is used for 6GHz band.

#### 7.2.1.1	Frame structure

Single and multiple DL to UL and UL to DL switching points within a shared gNB COT is identified to be beneficial and can be supported. LBT requirements to support single or multiple switching points need to be identified

- For gap of less than 16us: no-LBT can be used, though restrictions/conditions on when no-LBT option can be used will be further identified, in consideration of fair coexistence.

- For gap of above 16us but does not exceed 25us: one-shot LBT can be used, though restrictions/conditions on when one-shot LBT option can be used will be further identified, in consideration of fair coexistence.

- For single switching point, for the gap from DL transmission to UL transmission exceeds 25us: one-shot LBT is used, while further study is needed on how many one-shot LBT attempts is allowed for granted UL transmission.

#### 7.2.1.2	Physical layer channel designs

For physical layer channel design, NR design will be used as baseline, and the following potential design changes are to be studied to support the following channels/signals in NR-U.

- PDCCH/PDSCH

- PUCCH/PUSCH

- PSS/SSS/PBCH

- PRACH

- DL and UL reference signals applicable to the operational frequency range

For PSS/SSS/PBCH transmission, NR-U should have a signal that contains at least SS/PBCH block burst set transmission. The design of this signal should consider the following characteristics specific to unlicensed band operation:

- There are no gaps within the time span the signal is transmitted at least within a beam

- The occupied channel bandwidth is satisfied (although this may not be a requirement)

- Strive to minimize the channel occupancy time of the signal

- Characteristics that may facilitate fast channel access

For UL waveform for PUSCH, PUCCH, and PRACH, it has been identified that an interlaced waveform can have benefits in some scenarios including link budget limited cases with given PSD constraint, and as one option to efficiently meet the occupied channel bandwidth requirement.

On the other hand, it is RAN1’s understanding that the temporal allowance of not meeting occupied channel bandwidth by regulation can be exploited if the minimum bandwidth requirement, e.g., 2 MHz, is satisfied. Therefore, a waveform contiguous in frequency may be adequate in some scenarios, which implies that Release 15 NR contiguous allocation designs can be used for NR-U as well.

Support for Rel-15 NR PUSCH can be considered. However, it has been identified that block-interlaced based PUSCH can be beneficial.

Support for Rel-15 NR PUCCH formats can be considered, however, not necessarily all Release 15 NR PUCCH formats are applicable to NR-U. It is RAN1’s understanding that certain formats do not meet the minimum bandwidth requirement by regulation. Exclusion of the support of certain formats is to be identified.

When new block interlace waveform for PUCCH is to be defined, it is beneficial to use the same block interlace structure for PUCCH and PUSCH. For the block interlace waveform design for PUCCH, the following aspects can be considered:

- Flexible number of OFDM symbols

- Flexible payload size

- User multiplexing

- Number of formats

Support for Rel-15 NR PRACH formats can be considered, however, not necessarily all Release 15 NR PRACH formats are applicable to NR-U. It is RAN1’s understanding that certain formats do not meet the minimum bandwidth requirement by regulation. Exclusion of the support of certain formats is to be identified.

It is identified that interlaced based PRACH can be beneficial. The following aspects can be considered for Interlace waveform based PRACH design for 4-step random access:

- Interlacing based on PRB or REs

- Targeted cell sizes

- Targeted PRACH capacity

- Targeted false alarm and detection rates

- Targeted timing estimation accuracy

- Number of formats

- Multiplexing with other channels such as block interlaced PUCCH and PUSCH

#### 7.2.1.3	Physical layer procedure designs

##### 7.2.1.3.1	Channel access procedures

If absence of Wi-Fi cannot be guaranteed (e.g. by regulation) in the band (sub-7 GHz) where NR-U is operating, the baseline assumption is, the NR-U operating bandwidth is an integer multiple of 20MHz.

Channel access mechanisms need to comply with regulations and may therefore need to be adapted for particular frequency ranges.

For channel access mechanism, LTE-LAA LBT mechanism is adopted as baseline for 5GHz band and adopted as the starting point of the design for 6GHz band. At least for band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), LBT can be performed in units of 20 MHz.

For 5GHz band, a no-LBT option is beneficial for NR-U, such as for supporting fast A/N feedback, and is permitted per regulation. Restrictions/conditions on when no-LBT option can be used will be further identified, e.g., in consideration of fair coexistence. No-LBT option can be applied to 6GHz band if allowed by regulation. Restrictions/conditions on when no-LBT option can be used will be further identified, if fair coexistence criterion is defined for 6GHz band.

##### 7.2.1.3.2	Initial access and mobility

For initial access and mobility procedures, the main issue identified for NR operation in unlicensed band is the reduced transmission opportunities for different signalings due to LBT failure.

The following modifications to initial access procedures have been identified as beneficial:

- Modifications to initial access procedures considering limitations on access to the channel based on LBT. NR-U needs to develop techniques to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure.

- Enhancement to 4-step RACH, including developing mechanisms to handle reduced msg 1/2/3/4 transmission opportunities due to LBT failure.

- 2-step RACH procedure.

Potential modifications to RLM/RRM procedures due to reduced transmission opportunities for DL signals and channels due to LBT failure should be identified and studied.

Modifications to paging procedures due to reduced transmission opportunities for paging due to LBT failure are beneficial and should be identified and studied.

##### 7.2.1.3.3	Potential HARQ enhancements

NR-U uses NR HARQ feedback mechanisms as baseline, and enhancements can be identified.

Transmission of HARQ A/N for the corresponding data in the same shared COT is identified as beneficial. For NR-U, the design strives to support transmitting all HARQ A/N for the corresponding data in the same shared COT, if possible, considering the current NR UE processing time required. However, it is understood in some cases, the HARQ A/N has to be transmitted in a separate COT from the one the corresponding data was transmitted.

Techniques to handle reduced HARQ A/N transmission opportunities for a given HARQ process due to LBT failure are identified as beneficial. Potential techniques include mechanisms to provide multiple and/or supplemental time and/or frequency domain transmission opportunities.

When UL HARQ feedback is transmitted on unlicensed band, NR-U considers mechanisms to support flexible triggering and multiplexing of HARQ feedback for one or more DL HARQ processes

Scheduling multiple TTIs for PUSCH each using a separate UL grant in the same PDCCH monitoring occasion is identified as beneficial. Scheduling multiple TTIs for PUSCH using a single UL grant is identified as beneficial and should be supported in NR-U.

##### 7.2.1.3.4	Potential enhancements to configured grant

The following modifications to the configured grant procedures are beneficial.

- Removing dependencies of HARQ process information to the timing. This can be achieved by introducing UCI on PUSCH to carry HARQ process ID, NDI, RVID

- Introducing Downlink Feedback Information (DFI) including HARQ feedback for configured grant transmission

- Increased flexibility on time domain resource allocation for the configured grant transmissions

- Supporting retransmissions without explicit UL grant

### 7.2.2	Higher layer aspects

# 8	Performance evaluations

For performance evaluation, coexistence with other networks will be evaluation, such as WiFi, LTE-LAA, or other NR-U network.

When coexistence with WiFi is evaluated, following the study item description [2], NR-based operation in unlicensed spectrum should not impact deployed Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier, where the deployed Wi-Fi includes 11ac in 5GHz band. The fairness criterion for coexistence with 11ax can be further discussed.

For sub-7 GHz bands, coexistence simulations will be performed using technology neutral assumptions (eg. channel access mechanism) at an arbitrary carrier frequency in 5GHz band for application to bands other than 5GHz which may become available subject to regulations. The study assumes regulation will provide the framework concerning the protection for the technologies not using unlicensed access in those bands.

## 8.1	Scenarios and methodology

For the NR-U study evaluation, to reuse the simulation platform already developed for NR study, the 5GCM in [29] is used for NR-U simulation evaluation. The evaluation deployment scenarios are derived from NR evaluation deployment scenarios as defined in [28] with proper modifications to introduce the second operator.

NR-U simulation evaluation considers the following layout scenarios:

- Indoor sub-7GHz, 2 operators

- Outdoor Sub-7 GHz, 2 operators

- Stadium scenario for sub-7GHz, 2 operators, can be optionally considered by interested companies.

The following deployment scenarios for simulation are identified:

- CA between NR licensed cell and NR unlicensed cell

- DC (with LTE and with NR)

- SA

- An NR cell with DL in unlicensed band and UL in licensed band

In the simulations, only unlicensed cell(s) is to be simulated. The licensed cell may not be explicitly modelled in the simulation. Necessary assumptions regarding the presence of the licensed carriers can be made and provided.

It was also noted that a single set of evaluations may be applicable to multiple deployment scenarios. For example, DC and SA deployment scenarios can share the same set of simulations, possibly with some minor differences on how the overhead (say for system information delivery) is captured in the result.

### 8.1.1	Sub7 GHz indoor scenario

For sub7 GHz indoor simulation evaluation, two operators each with 3 gNBs are deployed in a room of size 120 meters by 80 meters as shown in Figure 1. In the figure, the gNB of the same color belongs to the same operator. The parameters are of value a=20 meters, b=40 meters, c=20 meters, and d=40 meters. The deployment scenario is selected to achieve a target serving link RSSI distribution with 10%-15% serving link below -72dBm.

Figure 1. Indoor sub7 simulation office layout

Other parameters are as given in Table 1. Other parameters not explicitly included in the table will use values defined in [28] and [29].

Table 1. Evaluation parameters for sub7 GHz indoor scenario

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline , 80MHz optional |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB per 20MHz |
| SCS | To be reported together simulation results |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0dBi |
| UE/STA Antenna gain | 0dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ Optional Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

### 8.1.2	Sub7GHz outdoor scenario

For sub 7 GHz outdoor simulation evaluation, two scenarios are considered. The two scenarios differ in the inter-gNB distance within the same macro cell.

- Scenario 1: Within each macro cell coverage, two operators each drop one micro gNB with a maximum distance of 30 meters between the gNBs

- Scenario 2: Within each macro cell coverage, two operators each drop one micro gNB with no limitation on the inter-gNB distance.

The layout of the gNBs are defined as follows:

- Macro deployment with ISD=200×A meters

- Each operator randomly drops 1 micro-layer TRP within each macro cell sector with minimum distance between micro-layer TRPs equals 57.9×A meters

- Independent dropping between two operators

  - Use 10 meters as the inter-operator micro-layer TRP minimum distance

  - For the inter-operator micro-layer TRP maximum distance

    - Outdoor scenario 1: 30

    - Outdoor scenario 2: No limit as long as the TRP is within the macro cell

The layout of UEs are defined as follows:

- UE randomly dropped within macro cell sector with a minimum serving cell RSSI of -82dBm

- All UEs dropped outdoor

Other parameters of the simulation follow the table below

Table 2.  Evaluation parameters for sub7 GHz outdoor scenario

| Parameters | Outdoor Sub-7GHz |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline , 80MHz optional |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB per 20MHz |
| SCS | To be reported together simulation results |
| Channel Model | NR UMi street canyon |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ Optional Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |
| gNB to gNB link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |

## 8.2	Channel access schemes

## 8.3	Evaluation results

# 9	Conclusions

# Annex A:
Evaluation methodology

## A.1	General evaluation assumptions

# Annex B: 
Evaluation results

# Annex C:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2018-02-26 | RAN1#92 | R1-1803272 |  |  | First version of TR |  | 0.0.1 |
| 2018-05-21 | RAN1#93 | R1-1807383 |  |  | Update to capture the agreements reached in RAN1 #92 and #92bis | 0.0.1 | 0.0.2 |