3GPP TSG-RAN WG1 Meeting RAN1 94	R1-1809443

Gothenburg, Sweden

20th August – 24th August 2018.

Agenda item:	7.2.3.2

Source: 	              Qualcomm Incorporated

Title: 	Evaluation Methodology for NR IAB

Document for:	Discussion/Decision

# Introduction

This contribution focuses on topology creation algorithms of IAB network.

# Overall Approach

During the last meeting, RAN1 agreed to the evaluation parameters for NR IAB [1]. For baseline comparison, RAN1 agreed to the following.

| Topology formation | In addition to RSRP, other factors to avoid the backhaul link congestion can also be included for parent node selection. The detailed algorithm is up to companies’ choice and should be reported by companies. |
| --- | --- |

In this contribution, we focus on following two topology creation algorithms and compare their performance, in terms of number of hops.

- RSRP based spanning tree.

- RSRP & load based spanning tree.

The next few sections proceed in the following way. We first review RSRP based topology creation. We then review how load can be considered while generating the topology. We then discuss simulation scenario and show simulation results. After discussing how IAB node capability can influence the topology management discussion, we conclude the contribution by listing the proposals.

# Review: RSRP based Topology Creation

Figure 1: RSRP based Topology Creation

Figure 1 shows the RSRP based topology generation method. During each step of the process, the network looks at all links between the set of connected nodes and the unconnected nodes and then picks the link with the strongest RSRP. This process iterates until all nodes are connected.

RSRP based topology management only considers signal strength while adding an unconnected node to the set of connected nodes. It does not consider load, hop count and other metrics.

# RSRP & Load based Topology Creation

As shown in the previous section, RSRP based topology creation does not consider load of the base stations. This may lead to large hop counts and poorer UE rates. We expect the scheme to perform better if load is considered as well while generating topology.

‘Load’ can be considered in many ways. We use ‘geometric mean rate’ as the optimization criteria to consider load during topology management. That means, instead of selecting the link with the strongest RSRP in each step of topology creation procedure, we select the link that maximizes the geometric mean of UEs that are associated with the set of connected nodes and the candidate unconnected node.

The motivation to use ‘geometric mean’ to illustrate the influence of load is because: geometric mean based resource allocation is the proportional fair resource allocation [2] [3]. A ‘geometric mean rate maximization’ method considers the link gain of both ‘good’ UEs and ‘poor’ UEs; and tries to ‘balance’ the resource allocation among the UEs.

Table 1 and 2 show our algorithms for RSRP & Load based topology creation. A detailed mathematical description of the geometric mean maximization problem in IAB networks can be found in our previous work [4].

Table 1: Geometric mean rate maximization optimization algorithms

| Input: - Base station (BS) and user equipment (UE) drops - Fiber drops (anchor base stations) | Output: - RSRP and load based spanning tree topology in the network |
| --- | --- |
| Step 0 (Initialization): - Consider the anchor nodes (the base stations with fiber drops) to be the set of connected nodes - Consider the remaining nodes to be the set of unconnected nodes ……………….. Step N: - Find all edges between the set of currently connected nodes and the set of currently unconnected nodes - For each edge, run the optimization problem of table 2 - Pick the edge that maximizes the geometric mean rate of UEs that are associated with already connected node set and the associated unconnected node - Bring the associated unconnected node to the set of connected nodes ---- Iterate until all nodes are connected. |  |

Table 2: Geometric mean rate maximization optimization algorithms

| Input: - Base station (BS) and user equipment (UE) drops - Fiber drops (anchor base stations) - Set of already connected nodes - Link capacity - Candidate link between a connected node and an unconnected node | Output: - Achievable geometric mean rate of UEs that are associated with already connected node set and the unconnected node associated with the candidate link |
| --- | --- |
| Optimization objective: Maximize geometric mean rate of UEs that are associated with connected node set and the unconnected node associated with the candidate link Constraints: - Capacity constraint per link - Flow conservation per base station - Half duplex constraint - Time division multiplexing constraint |  |

# Simulation Scenario

We focus on nineteen cells with three and seven fiber drops. Figure 2 and 3 show these scenarios. The details of the simulation assumptions can be found in the appendix.

In these scenarios, we investigate how RSRP based spanning tree and RSRP & load based spanning tree impact performance.

Figure 2: Nineteen cells with three fiber drops

Figure 3: Nineteen cells with seven fiber drops

# Simulation Results

Figure 4: Backhaul hop count CDF of RSRP based spanning tree and RSRP & load based spanning tree

Figure 4 compares the backhaul hop count CDF of RSRP based spanning tree and RSRP plus load based spanning tree. As expected, the number of backhaul hops with three fiber drops is greater than that with seven fiber drops.

Besides, the backhaul hop count CDF of RSRP plus load based spanning tree is to the left of RSRP based spanning tree. The below table shows that with 3 fiber drops, in 75 percentile scenario, RSRP and load based spanning tree lead to two backhaul hops; whereas RSRP based spanning tree leads to four backhauls.

Table 3: Number of backhaul hops in 75 percentile scenario

| 7 fiber drops |  | 3 fiber drops |  |
| --- | --- | --- | --- |
| RSRP spanning tree | RSRP & load based spanning tree | RSRP spanning tree | RSRP & load based spanning tree |
| 2 | 1 | 4 | 2 |

This illustrates that RSRP and load based spanning tree reduce the number of backhaul hops, with respect to RSRP based spanning tree.

Observation 1: Consideration of load, while generating topology, reduces the number of backhaul hops in IAB networks.

Observation 2: Additional parameters, e.g. hop count, load, node capability, etc., need to be considered for parent node selection to reduce backhaul link congestion in IAB networks.

# Conclusion

Observation 1: Consideration of load, while generating topology, reduces the number of backhaul hops in IAB networks.

Observation 2: Additional parameters, e.g. hop count, load, node capability, etc., need to be considered for parent node selection to reduce backhaul link congestion in IAB networks.

# Appendix: Simulation Assumptions

Table 4: Channel Model and Simulation Assumptions

| Parameter | Assumptions |
| --- | --- |
| Layout | Homogenous urban micro Single layer: 19 micro sites with wrap-around |
| Number of IAB donors (Ndonor) | 3 or 7 |
| Number of IAB nodes | 19 – Ndonor |
| Inter-BS distance | 200 m |
| Sectorization | 3 sectors, only one active at a time |
| Topology formation | RSRP based, RSRP+load based |
| Large-scale channel parameters | - Micro-to-UE: UMi-Street canyon - Micro-to-Micro: UMi-Street canyon (hUE =10m) |
| BH link bonus | The path loss for links between the IAB node and candidate serving IAB nodes/donors is determined based on N =3 independent large-scale channel realizations (taking into account LOS/NLOS probability and shadow fading) |
| Fast fading parameters | - Micro-to-UE: UMi-Street canyon - Micro to Micro: UMi-Street canyon O-to-O (hUE =10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 |
| UE distribution | 100% outdoor |
| Number of RSRP realizations | - 1 for access network - 100 for BH network |

Table 5: Simulation Configuration

| Parameter | Assumptions |  |
| --- | --- | --- |
|  | Access | Backhaul |
| Carrier Frequency | 30 GHz | 30 GHz |
| Simulated BW | 400 MHz | 400 MHz |
| Overhead | 35% | 35% |
| Antenna element gain | BS: 8 dBi UE: 5dBi | 8 dBi |
| BS antenna array | {M, N, P} = {16, 8, 1}, horizontal polarization (slant angle = 90) |  |
| UE antenna array | {M, N, P} = {2, 2, 1}, horizontal polarization, single panel | N/A |
| BS TX power | 33 dBm |  |
| UE TX power | 23 dBm | N/A |
| BS analog BF codebook | L3: 33 beams, 3 rows on azimuth with 11 beams each (equally spaced DFT beams) | L3: 33 beams, 3 rows on azimuth with 11 beams each (equally spaced DFT beams) |
| UE analog BF codebook | L3: 4 beams, 2 rows of 2 beams each (equally spaced DFT beams) | N/A |
| Noise figure | BS Rx: 7 dB UE Rx: 13 dB | gNB Rx: 7 dB |
| Traffic Model | Full buffer, DL only | Aggregated access traffic |
| UE speed | 3 km/hr, all UEs outdoor | N/A |
| Max effective SNR | 30 dB | 30 dB |
| MU-MIMO enabled | No | No |
| Number of layers | 1 | 1 |
| Implementation margin | 0 dB | 0 dB |
| SNR to rate conversion | Shannon’s capacity equation | Shannon’s capacity equation |

Table 5: BS and UE codebooks used for access and backhaul

# 9. Reference

- 3GPP TSG RAN1 93 Chairman’s Notes.

- Congestion Control 2: Utility, Fairness and Optimization in Resource Allocation. https://www.cs.helsinki.fi/u/ldaniel/mm_cn/lec1.2_cc_resource_allocation.pdf

- An axiomatic theory of fairness. http://www.princeton.edu/~chiangm/fairnesstalk.pdf.

- Investigation of Performance in Integrated Access and Backhaul Networks. https://arxiv.org/pdf/1804.00312.pdf