3GPP TSG RAN WG1 Meeting #94	R1-1809445

Gothenburg, SWEDEN, 20th August – 24th August 2018

Agenda Item:	7.2.3.3

Source: 	Qualcomm Incorporated

Title: 	Network synchronization for multi-hop IAB

Document for:	Discussion and decision

# Introduction

One important aspect of NR-IAB is network synchronization, as captured in TR 38.874:

5.2.6  Network Synchronization

Time synchronization between IAB nodes is also very essential e.g. to support TDD system and some potential features which need network synchronization. IAB may have additional requirement on network synchronization, which includes in-band wireless backhaul and multi-hops backhauling.

RAN1 #93 also achieved the following agreement

Agreements: - IAB supports TA-based synchronization between IAB nodes, including across multiple backhaul hops - Enhancements to existing mechanisms can be further studied - The following cases should be further studied: - Case 1: DL transmission timing alignment across IAB nodes and donor nodes - Case 2: DL and UL transmission timing is aligned within an IAB node - Case 3: DL and UL reception timing is aligned within an IAB node - Case 4: within an IAB node, when transmitting using case 2 while when receiving using case 3 - Case 5: Case 1 for access link timing and Case 4 for backhaul link timing within an IAB node in different time slots - Further study the following levels of alignment between IAB nodes/donor nodes or within an IAB node: - Slot alignment - Symbol-level alignment - No alignment - Further consider the impact of different cases on TDM/FDM/SDM multiplexing of access and backhaul links, cross-link interference, and impact on access UEs

In this contribution, we discuss

- Accuracy of the over-the-air (OTA) synchronization

- Timing alignment across multi-hop NR-IAB networks.

# OTA synchronization

In [1], we discussed a TA-based OTA synchronization technique can be adopted to provide NR-IAB network synchronization. OTA can help increase the synchronization robustness in indoor or urban settings where the GNSS coverage can be spotty or can replace the GNSS altogether. As shown in Figure 1, in a multi-hop NR-IAB network with hierarchical topology, an IAB-node can synchronize to its parent node(s) using the available synchronization mechanism of the Uu interface (i.e. tracking downlink receive timing, and adjusting uplink transmit timing using the provided TA command). This would naturally synchronize the whole network to the IAB-donors.

Figure 1: multi-hop over-the-air synchronization

One potential issue with this technique could be the fact that the accumulated errors over multiple hops may lead to a network that is not tightly synchronized – please see Figure 2. Here,  represents the OTA synchronization error range relative to the reference source.

Figure 2: accumulated OTA sync error across multiple hops

From Figure 2, we see the resulting cross cell timing error in an Nhop-hop IAB network could be as large as

maxcross cell timing error= ±2Nhopϵ.

In [1], we calculated the maximum timing error over a single link (ϵ) by taking into account various error components (like detection error, transmission timing error accuracy, TA granularity, TA adjustment error) and their maximum allowable values based on NR spec. Table 1 provides the result for different SCS values. We also used the above formula to find the maximum supportable number of hops, assuming the 3-usec cell phase accuracy requirement (38.133).

Table 1: maximum time deviation per hop, and maximum number of allowable hops

Observation 1: TA-based OTA synchronization can support multi-hop IAB network (4~5 hops) for mmw bands. TA-based OTA synchronization is not sufficient to support multiple hops in lower bands.

Proposal 1: To tighten the OTA timing error, especially in lower bands, RAN1 should consider the following solutions:

- Using wider band signals (UL and DL) to achieve more accurate timing estimation

- Enhancing TA, e.g. by reducing its granularity and increasing the number of bits

Note that the proposed multi-hop OTA synchronization technique ends up synchronizing the IAB-nodes to the IAB-donor(s) who are at the roots of the IAB network topology. To achieve synchronization across the network, the IAB-donors should also be synchronized – e.g. using other techniques like GNSS or PTP (IEEE 1582 precision time protocol).

Also given the accumulated timing error, large scale IAB networks cannot achieve tight network synchronization by relying solely on the OTA techniques. Instead, the network should be partitioned into clusters of nodes where the timing of the center of a cluster is disciplined by GNSS or PTP and the rest of the nodes may choose OTA techniques.

Proposal 2: IAB TR should mention TA-based OTA synchronization can support up to [5] hops in MMW bands.

Proposal 3: IAB network should use other synchronization techniques, such as GNSS and PTP, along with OTA techniques to achieve tight network synchronization.

Proposal 4: it should be further studied how timing adjustment of an IAB-node (following OTA synchronization) impacts the operation of its child IAB-nodes and UEs.

# Multi-hop timing alignment

Over the multi-hop backhaul network, an IAB-node is served by its parent IAB-node, and serves one or multiple UEs or child IAB-nodes. For communication with the parent IAB-node, the IAB-node acts as a UE and follows the DL/UL timing reference acquired through its parent. However, the IAB-node should also choose a timing reference for communicating with its children.

In general, an IAB-node may choose any reference for its DL/UL TX/RX timing towards its children. RAN1 #93 identified the following solutions:

- Case 1: DL transmission timing alignment across IAB nodes and donor nodes

- Case 2: DL and UL transmission timing is aligned within an IAB node

- Case 3: DL and UL reception timing is aligned within an IAB node

- Case 4: within an IAB node, when transmitting using case 2 while when receiving using case 3

- Case 5: Case 1 for access link timing and Case 4 for backhaul link timing within an IAB node in different time slots

We note that case 1 results in slot alignment across the multi-hop IAB network and supports synchronous operation. Furthermore, case 1 is more resilient to the change – e.g. if an IAB-node changes its parent node or if the propagation time changes (due to mobility of the nodes or environment), the timing reference used by the IAB-node and all the nodes below it will not change.

Figure 3: case 1 timing alignment

However, a potential issue of case 1 (that motivated the other cases) is the difficulty of supporting concurrent communications (MU-MIMO) of an IAB-node with its parent and children, due to the timing misalignment.

Figure 4: case 2 timing alignment

We note that although cases 2-5 allow simultaneous transmissions and/or receptions across two adjacent hops, and hence can improve performance (in terms of throughput and latency), they result in a relative timing error that may grow every hop (as seen in Figure 4). This in turn leads to either more inter-cell interference or a need for a larger guard period (and hence loss of resource efficiency). Moreover, any change to the timing of an IAB-node towards its parent impacts all other IAB-nodes/UEs beneath the IAB-node.

Table 2 summarizes the comparison between various cases.

Table 2: comparison of different timing alignment cases

| Timing Alignment | Pros | Cons |
| --- | --- | --- |
| Case 1 | - Allows synchronous operation without cross link interference (due to time misalignment) - When the there is a change in timing of the BH link, timing relative to the access UEs and IAB child nodes remain the same. | - Simultaneous Tx to parent & children and Rx from the parent & children creates interference due to misalignment |
| Cases 2-5 | - Allows simultaneous transmissions and/or receptions to the parent node and the IAB child nodes/UEs. | - With every hop, the relative timing error may grow - Asynchronous operation - Requires longer guard periods to minimize cross link interference - When there is a change in timing of the BH link, the new timing reference needs to be communicated to the children. |

Based on these observations, we propose case 1 to be adopted as the baseline solution, while other cases may also be adopted (left to implementation) in cases where they can provide meaningful benefits.

Proposal 5: the timing alignment case 1 (where slot boundaries across multi-hop IAB network are aligned) is adopted as the baseline.

- The other cases that allow simultaneous transmissions and/or receptions over two adjacent hops should also be supported.

- It is left to network, which case to select.

# Conclusion

In this contribution, we discussed synchronization across multi-hop IAB network, and made the following observation and proposals:

Observation 1: TA-based OTA synchronization can support multi-hop IAB network (4~5 hops) for mmw bands. TA-based OTA synchronization is not sufficient to support multiple hops in lower bands.

Proposal 1: To tighten the OTA timing error, especially in lower bands, RAN1 should consider the following solutions:

- Using wider band signals (UL and DL) to achieve more accurate timing estimation

- Enhancing TA, e.g. by reducing its granularity and number of bits

Proposal 2: IAB TR should mention TA-based OTA synchronization can support up to [5] hops in MMW bands.

Proposal 3: IAB network should use other synchronization techniques, such as GNSS and PTP, along with OTA techniques to achieve tight network synchronization.

Proposal 4: it should be further studied how timing adjustment of an IAB-node (following OTA synchronization) impacts the operation of its child IAB-nodes and UEs.

Proposal 5: the timing alignment case 1 (where slot boundaries across multi-hop IAB network are aligned) is adopted as the baseline.

- The other cases that allow simultaneous transmissions and/or receptions over two adjacent hops should also be supported.

- It is left to network, which case to select.

# References

- R1-1807396, “Network synchronization for multi-hop IAB”, Qualcomm.