3GPP TSG RAN WG1 Meeting 94 			R1-1809528

Gothenburg, Sweden, Aug 20th – Aug 24th, 2018

Source:	Sierra Wireless

Title:	LTE-M Preconfigured UL Resources Summary

Agenda Item:	6.2.1.2

Document for:	Discussion and Decision

# Introduction

The following tdoc provides a summary of the open issues and proposals based submitted tdoc’s to the “Support for transmission in preconfigured UL resources” agenda item for RAN1#94. The following is the WID objective:

Improved UL transmission efficiency and/or UE power consumption:

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance[RAN1, RAN2, RAN4]

  - Both shared resources and dedicated resources can be discussed

  - Note: This is limited to orthogonal (multi) access schemes

# Timing Advance

The WID states “…for UEs with a valid timing advance”.   This section contains issues related to TA.

## TA Validity Definition

Issue:  What is the definition of what is “Valid TA” is. e.g. what % of CP is considered valid.

Proposal #1: Send LS to RAN4 to decide

Supported by:

Proposal #2: Study and decide within RAN1

Supported by:

## TA Validation Mechanism

A mechanism to determine if/when the UE has valid TA is needed. At least, the following TA validation criterion could be studied:

RRM measure delta

Time (i.e. how long a configured TA is valid)

SNR

Previous mobility events (i.e. NW controlled)

- Others not precluded.

Consider a LS to RAN4

Proposal #1: Study the above TA validation criteria’s

Supported by:

Proposal #2: Send LS to RAN4

Supported by: Sierra Wireless

# Dedicated and/or Shared Resources

The WID indicates “Both shared resources and dedicated resources can be discussed” so RAN1 must decide if shared and/or dedicated resources are supported.

## Dedicated Pre-configured UL Resources (PUR)

Issue: Whether to support dedicated PUR or not. Dedicated resources would have no collisions thus no need for contention resolution needed.

Considerations:

Best suited for applications that have periodic transmissions of the same size

Best for UE power consumption – exact TBS (no padding), no collisions, minimal signaling

No need for eNB blind decoding

Best for capacity – no collisions

No need for contention resolution

UE may be mandated to transmit on each assigned PUR whether they have data or not (Or at least be charged)

Can consider increasing multiplexing of dedicated resources by:

Sub-PRB

MU-MIMO

CDMA

- Others not precluded.

- Proposal #1: Dedicated pre-configured UL resources is supported

Supported by: Sierra Wireless(priority), ZTE (priority),  Qualcomm, Huawei, Samsung(priority), LGE, Sony

## Shared Pre-configured UL Resources (PUR)

Issue: Whether to support shared PUR or not. Shared PUR will require the UE to autonomously decided on which shared resources to use from a pool of shared resources. A collision/contention resolution is needed.

Conidiations:

Best for applications that have a-periodic transmissions with different data packet sizes

Collisions will cause UE power consumption and capacity degradation

eNB requires blind decoding of TBS sizes and repeats – this may limit TBS options

Need a contention resolution mechanism

Combining HARQ re-transmission is more complex

- Proposal #1: Shared pre-configured UL resources is supported

- Supported by: Nokia, Sony, Huawei, Qualcomm

# Connected and Idle mode support

The WID indicates “idle and/or connected mode” so RAN1 must decide if idle and/or connected mode is supported.

## Idle mode Pre-configured Shared UL Resources (PUR) Support

Issue: Whether to support Idle mode PUR or not.

- Proposal #1: Idle mode based pre-configured UL resources is supported

- Supported by: Sierra,  Nokia (priority), Qualcomm(priority), Huawei, Samsung, LGE(priority), Sony

Care consider – NTT Docomo

## Connected Pre-configured Shared UL Resources (PUR) mode support

Issue: Whether to support Connect mode PUR or not. LTE-M already supports SPS in connected mode  A so some possible enhancements could include:

Mode B support

NR type SPS

Proposal #1: Connected mode based pre-configured UL resources is supported

Supported by: Intel, NTT Docomo

Proposal #2: Connected mode based pre-configured UL resources is NOT supported

Supported by: Samsung

# General

## Maximum Idle mode PUR Period

Issue: Legacy PUR mechanisms like SPS have a very short period (i.e. 640ms). Some companies have indicated that idle mode PUR for LTE-M should support a much longer periodicity of at least hours. Since this may change the design, agreeing on a maximum periodicity range may be useful.

Proposal #1 – The periodicity of dedicated preconfigured UL resources in RRC-idle mode should support at least hours.

Supported by: Sierra Wireless, Samsung

## Oversubscription of PUR

Issue: A UE may request and be granted more PUR than it uses (e.g. UE has PUR every 30 seconds but only sends data 1 per hour). For dedicated resource, this would be very inefficient use of resources. Some mechanisms to consider may include:

Mandate UE to transmit on all assigned PUR (this may not prevent UE which are AC power from oversubscribing)

Charge $$ UE for data in each PUR regardless if UE sends data, blank data, or not

## Mandatory or Optional Transmission for dedicated PUR

Issue:  For legacy SPS, a UE is mandated to transmit in the PUR even if it has no data. RAN1 needs to decide if LTE-M UEs will mandated to transmit in its dedicated PUR or not. Some considerations:

If not mandated to transmit

eNB needs a reliable DTX detection

If mandated to transmit

Increases battery life

## Update Mechanisms

Issue:  Some companies have indicated that new mechanisms should be considered to update some of the PUR parameters especially for changing coverage and TA. Mechanisms to dynamically and efficiently update the following PUR parameters could be studied:

TA

PA Power

Repetition

MCS

Time and frequency resource

- Others not precluded.

## Increasing Multiple Access

Issue:  For dedicated and shared resources, increase multiple access will improve capacity. At least, the following mechanisms to improve multiple access could be studied:

Sub-PRB

CDMA

MU-MIMO (also study increased DMRS patterns)

- Others not precluded.

## Configuration of pre-configured resources (PUR)

Issue:  The aspects that can be pre-configured for PUR need to be agreed. At least, the following list of configuration aspects could be studied:

Frequency domain resources

Time Domain (period and offset)

TBS options

Coverage factor: MCS and repeats

Frequency hopping

HARQ retransmission MPDCCH search space

Pre-grant window (see [5] for details)

- Others not precluded.

## Retransmission

Issue:  Several issues have been identified and discussed WRT to re-transmission and HARQ. These issues can be studied but can be deprioritized for this meeting.

# Tdocs

- R1-1808035 Support for transmission in preconfigured UL resources in LTE-MTC Ericsson

- R1-1808118 UL transmission in preconfigured resource Huawei, HiSilicon

- R1-1808348 Transmission in preconfigured UL resources Sony

- R1-1808357 Pre-configured UL Resources Design Considerations Sierra Wireless, S.A.

- R1-1808431 Transmission in preconfigured UL resources Nokia, Nokia Shanghai Bell

- R1-1808466 Discussion on preconfigured UL resources in MTC LG Electronics

- R1-1808632 Support for transmission in preconfigured UL resources for MTC ZTE

- R1-1808656 UL transmission in preconfigured resources for eMTC Intel Corporation

- R1-1808731 Discussion on transmission in preconfigured UL resources for MTC Samsung

- R1-1809023 Support for transmission in preconfigured UL resources Qualcomm Incorporated

- R1-1809129 UL transmission scheme in preconfigured resources NTT DOCOMO, INC.