3GPP TSG-RAN WG1 Meeting #94	R1-1809531

Gothenburg, Sweden, August 20th – 24th 2018

Agenda Item:	6.1.4.1

Source:	Ericsson

Title:	Feature summary of 6.1.4.1 Maintenance for Resynchronization signal

Document for:	Discussion, Decision

# 1	Introduction

Three companies have submitted contributions regarding maintenance for the resynchronization signal, [1], [2], [3].

# 2	Discussion

The content of the different contributions is summarized below. Based on the contributions, feature lead proposals are presented.

### RSS overlapping PSS, SSS or PBCH

All companies propose to puncture RSS REs overlapping PSS or SSS.

RSS REs overlapping with PSS or SSS are punctured.

Furthermore, one company proposed to drop subframes overlapping with PBCH.

RSS PRB pairs overlapping with PBCH PRB pairs are dropped.

### RSS in SIB1-BR or other SI subframes

All companies propose to drop RSS transmissions in PRB pairs where SIB1-BR and other SI is transmitted.

RSS PRB pairs overlapping with SIB1-BR PRB pairs or PRB pairs carrying other SI are dropped.

RSS overlapping with MPDCCH or non-SI PDSCH

One company proposes to puncture MPDCCH and non-SI PDSCH transmissions for resource elements occupied by RSS.

A UE may assume that MPDCCH REs and PDSCH REs not carrying SI overlapping with RSS are punctured.

RSS overlapping with WUS

One company recognizes that RSS and WUS can be configured to be separated in the frequency domain.

Further discussions may be needed regarding RSS PRB pairs overlapping with WUS PRB pairs.

Relation between RSS and CRS power regarding #CRS ports

One company proposes that the ce-RSS-PowerBoost-Config parameter should be scaled according to the number of CRS ports:

1 CRS port:		ce-RSS-PowerBoost-Config

2 CRS ports:	ce-RSS-PowerBoost-Config+3 dB

4 CRS ports:	ce-RSS-PowerBoost-Config+6 dB

Further discussions may be needed regarding ce-RSS-PowerBoost-Config scaling with respect to #CRS ports.

# 3	Conclusion

Based on the discussion in the previous sections about submitted contributions, the feature lead proposes the following:

Proposal 1	RSS REs overlapping with PSS or SSS are punctured.

Proposal 2	RSS PRB pairs overlapping with PBCH PRB pairs are dropped.

Proposal 3	RSS PRB pairs overlapping with SIB1-BR PRB pairs or PRB pairs carrying other SI are dropped.

Proposal 4	A UE may assume that MPDCCH REs and PDSCH REs not carrying SI overlapping with RSS are punctured.

Proposal 5	Further discussions may be needed regarding RSS PRB pairs overlapping with WUS PRB pairs.

Proposal 6	Further discussions may be needed regarding ce-RSS-PowerBoost-Config scaling with respect to #CRS ports.

# References

R1-1808984, Maintenance issues for Resynchronization signal for Rel-15 LTE-MTC, Ericsson, RAN1 #94, Gothenburg, Sweden, August 2018.

R1-1809008, Maintenance for RSS, Qualcomm, RAN1 #94, Gothenburg, Sweden, August 2018.

R1-1808623, Remaining collision issues for RSS transmission, ZTE, RAN1 #94, Gothenburg, Sweden, August 2018.