3GPP TSG RAN WG1 Meeting #94	R1-1809578

Gothenburg, Sweden, 20th - 24th August, 2018	(Revision of R1-1809384)

Agenda item:	6.1.2.2.4

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Corrections to CSI reporting for shorter TTI in 36.213 – TP#1

Document for:	Decision

# 1	Reason for needed changes

RAN1 agreed to make the 256QAM and 1024QAM operation independently configurable for shorter TTI and RAN2 defined the following signaling accordingly in 36.331:

SlotOrSubslotPDSCH-Config information element

-- ASN1START

SlotOrSubslotPDSCH-Config-r15 ::= CHOICE {

release							NULL,

setup							SEQUENCE {

altCQI-TableSTTI-r15			ENUMERATED {

allSubframes, csi-SubframeSet1,

csi-SubframeSet2, spare1}				OPTIONAL, -- Need ON

altCQI-Table1024QAM-STTI-r15	ENUMERATED {

allSubframes, csi-SubframeSet1,

csi-SubframeSet2, spare1}				OPTIONAL, -- Need ON

resourceAllocation-r15			ENUMERATED {

resourceAllocationType0,resourceAllocationType2}	OPTIONAL, -- Need ON

tbsIndexAlt-STTI-r15			ENUMERATED {a33}			OPTIONAL, -- Need ON

tbsIndexAlt2-STTI-r15			ENUMERATED {b33}			OPTIONAL, -- Need ON

tbsIndexAlt3-STTI-r15			ENUMERATED {a37}			OPTIONAL, -- Need ON

...

}

}

-- ASN1STOP

This needs to be also taken account in the related CSI reporting in Sec. 7.2 of 36.213.

# 2	TP to TS 36.213 V15.2.0, Sec. 7.2

co

## 7.2	UE procedure for reporting Channel State Information (CSI)

< Unchanged parts are omitted >

In case both periodic and aperiodic CSI reporting would occur in the same subframe, the UE shall only transmit the aperiodic CSI report in that subframe. If the aperiodic CSI reporting occurs on an LAA SCell, the UE shall assume that the UL channel access procedure, as described in clause 15.2, is successful to determine whether periodic and aperiodic CSI reporting would occur in the same subframe.

If the higher layer parameter altCQI-TableSTTI-r15 is configured and is set to allSubframes and aperiodic CSI is triggered through DCI format 7-0A or 7-0B,

-	the UE shall report CQI according to Table 7.2.3-2.

Else if the higher layer parameter altCQI-Table1024QAM-STTI-r15 is configured and is set to allSubframes and aperiodic CSI is triggered through DCI format 7-0A or 7-0B,

-	the UE shall report CQI according to Table 7.2.3-4.

Else if the higher layer parameter altCQI-TableSTTI-r15 is configured and is set to csi-SubframeSet1 or csi-SubframeSet2 and aperiodic CSI is triggered through DCI format 7-0A or 7-0B,

-	the UE shall report CQI according to Table 7.2.3-2 for the corresponding CSI subframe set configured by altCQI-TableSTTI-r15

-	the UE shall report CQI for the other CSI subframe set according to Table 7.2.3-1.

Else if the higher layer parameter altCQI-Table1024QAM-STTI-r15 is configured and is set to csi-SubframeSet1 or csi-SubframeSet2 and aperiodic CSI is triggered through DCI format 7-0A or 7-0B,

-	the UE shall report CQI according to Table 7.2.3-4 for the corresponding CSI subframe set configured by altCQI-Table1024QAM-STTI-r15

-	the UE shall report CQI for the other CSI subframe set according to Table 7.2.3-1.

Else if aperiodic CSI is triggered through DCI format 7-0A or 7-0B,

-	the UE shall report CQI according to Table 7.2.3-1.

Else if the higher layer parameter altCQI-Table-r12 is configured and is set to allSubframes,

-	the UE shall report CQI according to Table 7.2.3-2.

Else if the higher layer parameter altCQI-Table-1024QAM-r15 is configured and is set to allSubframes

-	the UE shall report CQI according to Table 7.2.3-4.

Else if the higher layer parameter altCQI-Table-r12 is configured and is set to csi-SubframeSet1 or csi-SubframeSet2,

-	the UE shall report CQI according to Table 7.2.3-2 for the corresponding CSI subframe set configured by altCQI-Table-r12

-	the UE shall report CQI for the other CSI subframe set according to Table 7.2.3-1.

Else if the higher layer parameter altCQI-Table-1024QAM-r15 is configured and is set to csi-SubframeSet1 or csi-SubframeSet2,

-	the UE shall report CQI according to Table 7.2.3-4 for the corresponding CSI subframe set configured by altCQI-Table-1024QAM-r15

-	the UE shall report CQI for the other CSI subframe set according to Table 7.2.3-1.

Else

-	the UE shall report CQI according to Table 7.2.3-1.

< Unchanged parts are omitted >