3GPP TSG RAN WG1 Meeting #94 	R1-1809647

# Gothenburg, Sweden, 20 – 24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0002 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on starting position of Partial PUSCH Mode 1 for FeLAA in 37.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_unlic-Core |  |  |  |  |  |  | Date: |  | 2018-08-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current version of 37.213 specifies that if the LBT fails at the first slot of a subframe, the UE shall attempt to transmit at the second slot with an offset of symbols. This means if the starting position at the second slot is an offset to the starting position indicated for the first slot, which does not match the agreement where the starting position at the first slot is indicated by DCI, but the starting position at the second slot is symbol 7. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct that the candidate staring position for partial PUSCH Mode 1 at the second slot is symbol 7. In addition, some editorial corrections are made. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The candidate starting point for partial PUSCH mode 1 on the second slot is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

4.2.1	Channel access procedure for uplink transmission(s)

The UE can access a carrier on which LAA Scell(s) UL transmission(s) are performed according to one of Type 1 or Type 2 UL channel access procedures. Type 1 channel access procedure is described in sub clause 4.2.1.1. Type 2 channel access procedure is described in sub clause 4.2.1.2.

If an UL grant scheduling a PUSCH transmission indicates Type 1 channel access procedure, the UE shall use Type 1 channel access procedure for transmitting transmissions including the PUSCH transmission unless stated otherwise in this sub clause.

A UE shall use Type 1 channel access procedure for transmitting transmissions including the PUSCH transmission on autonomous UL resources unless stated otherwise in this sub clause.

If an UL grant scheduling a PUSCH transmission indicates Type 2 channel access procedure, the UE shall use Type 2 channel access procedure for transmitting transmissions including the PUSCH transmission unless stated otherwise in this sub clause.

The UE shall use Type 1 channel access procedure for transmitting SRS transmissions not including a PUSCH transmission. UL channel access priority class =1 is used for SRS transmissions not including a PUSCH.

If the UE is scheduled to transmit PUSCH and SRS in subframe , and if the UE cannot access the channel for PUSCH transmission in subframe , the UE shall attempt to make SRS transmission in subframe  according to uplink channel access procedures specified for SRS transmission.

Table 4.2.1-1: Channel Access Priority Class for UL

| Channel Access Priority Class () |  |  |  |  | allowed sizes |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 7 | 2 ms | {3,7} |
| 2 | 2 | 7 | 15 | 4 ms | {7,15} |
| 3 | 3 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| 4 | 7 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| NOTE1: For , =10ms if the higher layer parameter 'absenceOfAnyOtherTechnology-r14' indicates TRUE, otherwise, =6ms. NOTE 2: When =6ms it may be increased to 8 ms by inserting one or more gaps. The minimum duration of a gap shall be 100 µs. The maximum duration before including any such gap shall be 6 ms. |  |  |  |  |  |

If the UL duration and offset' field configures an 'UL offset'  and an 'UL duration'  for subframe , then

the scheduled UE may use channel access Type 2 for transmissions in subframes where , irrespective of the channel access Type signalled in the UL grant for those subframes, if the end of UE transmission occurs in or before subframe .

If the 'UL duration and offset' field configures an 'UL offset'  and an 'UL duration'  for subframe  and the 'COT sharing indication for AUL' field is set to true, then a UE configured with autonomous UL may use channel access Type 2 for autonomous UL transmissions corresponding to any priority class in subframes where , if the end of UE autonomous UL transmission occurs in or before subframe  and the autonomous UL transmission between  and  shall be contiguous.

If the 'UL duration and offset' field configures an 'UL offset'  and an 'UL duration' for subframe  and the 'COT sharing indication for AUL' field is set to false, then a UE configured with autonomous UL shall not transmit autonomous UL in subframes  where .

If the UE scheduled to transmit transmissions including PUSCH in a set subframes  using PDCCH DCI Format 0B/4B, and if the UE cannot access the channel for a transmission in subframe , the UE shall attempt to make a transmission in subframe  according to the channel access type indicated in the DCI, where , and is the number of scheduled subframes indicated in the DCI.

If the UE is scheduled to transmit transmissions without gaps including PUSCH in a set of subframes  using one or more PDCCH DCI Format 0A/0B/4A/4B and the UE performs a transmission in subframe  after accessing the carrier according to one of Type 1 or Type 2 UL channel access procedures, the UE may continue transmission in subframes after  where .

If the beginning of UE transmission in subframe immediately follows the end of UE transmission in subframe , the UE is not expected to be indicated with different channel access types for the transmissions in those subframes.

If the UE is scheduled to transmit transmissions including PUSCH Mode 1 using the Type 1 channel access procedure indicated in DCI, and if the UE cannot access the channel for a transmission according to the PUSCH starting position indicated in the DCI, the UE shall attempt to make a transmission at symbol 7 in the same subframe according to Type 1 channel access procedure. There is no limit on the number of attempts the UE can make using Type 1 channel access procedure.

If the UE is scheduled to transmit transmissions including PUSCH Mode 1 using the Type 2 channel access procedure indicated in DCI, and if the UE cannot access the channel for a transmission according to the PUSCH starting position indicated in the DCI, the UE may attempt to make a transmission at symbol 7 in the same subframe and according to Type 2 channel access procedure. The number of attempts the UE should make within the consecutively scheduled subframes including the transmission is limited to , where  is the number of consecutively scheduled subframes using Type 2 channel access procedure.

< Unchanged parts are omitted >