3GPP TSG RAN WG1 Meeting #94	R1-1809684

Gothenburg, Sweden, August 20 - 24, 2018

Agenda Item:	6.1.5.1.2

Source:	Huawei, HiSilicon

Title:	Updated Feature lead summary of EDT during RACH in feNB-IoT

Document for:	Discussion and decision

WID objective

A-1. Further latency and power consumption reduction - Evaluate power consumption/latency gain and specify necessary support for DL/UL data transmission on a dedicated resource during the Random Access procedure after NPRACH transmission and before the RRC connection setup is completed. [RAN2, RAN1, RAN3]

Background

Early data transmission during RACH for Rel-15 NB-IoT has been discussed in Rel-15 and was concluded completed in RAN1#93. The related agreements are attached in the Appendix for reference. In RAN1#94, two contributions are proposed in the maintenance session of Release 15 EDT for NB-IoT.

Summary of proposals on maintenance of EDT for NB-IoT

Timing relationship of EDT

In R1-1808129, it is proposed to capture the following agreements in RAN1#93 into the specification.

Agreements:

- The timing for monitoring DCI is based on the number of Msg3 NPUSCH repetitions indicated in RAR/DCI

The corresponding TP is also provided as following:

---------------------------------------------- Start of Text Proposal --------------------------------------- -------------------------------------------- Unchanged parts omitted -------------------------------------- ## 16.6 Narrowband physical downlink control channel related procedures -------------------------------------------- Unchanged parts omitted -------------------------------------- If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig - and if the UE has a NPUSCH transmission ending in subframe n, - the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and - the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1 to subframe n+3; else if the UE is not configured with higher layer parameter edt-parameters or if the UE is configured with higher layer parameter edt-parameters and - if the NB-IoT UE has a NPUSCH transmission ending in subframe n , the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3. otherwise - If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n’+1 to subframe n+3. -------------------------------------------- Unchanged parts omitted -------------------------------------- ----------------------------------------------- End of Text Proposal ---------------------------------------

- Capture the agreements on timing in RAN1#93 and adopt the above TP.

Clarification on wording in section 16.3.3 of TS 36.213

A draft CR (R1-1809126) is proposed for clarification on the wording in section 16.3.3 of TS 36.213. The proposal on the change of the specification is attached in the following table:

< Unchanged parts are omitted > - 16.3.3 Narrowband random access response grant The higher layers indicate the Nr-bit UL Grant to the physical layer, as defined in 3GPP TS 36.321 [8]. This is referred to as the Narrowband Random Access Response Grant in the physical layer. Nr-bit =15, and the content of these 15 bits starting with the MSB and ending with the LSB are as follows: - Uplink subcarrier spacing is '0'=3.75 kHz or '1'=15 kHz – 1 bit - Subcarrier indication field as determined in Subclause 16.5.1.1 – 6 bits - Scheduling delay field () as determined in Subclause 16.5.1 with k0 = 12 for IDelay = 0 , where NB-IoT DL subframe n is the last subframe in which the NPDSCH associated with the Narrowband Random Access Response Grant is transmitted – 2 bits - Msg3 repetition number as determined in Subclause 16.5.1.1 – 3 bits - MCS index indicating TBS, modulation, and number of RUs for Msg3 – 3 bits The redundancy version for the first transmission of Msg3 is 0. If the UE is not configured with higher layer parameter edt-parameters, or the UE is configured with higher layer parameter edt-parameters and , - the TBS, modulation, and number of RUs for Msg3 are determined according to Table 16.3.3-1 otherwise, - if the UE is configured with higher layer parameter edt-smallTBS-Enabled set to ‘false’, - the TBS is given by higher layer parameter edt-TBS - otherwise, - the UE selects a TBS from the allowed TBS values according to Table 16.3.3-2 - the repetition number for Msg3 is the smallest integer multiple of L value that is equal to or larger than where is the selected TBS for Msg3, and is given by higher layer parameter edt-TBS - if , otherwise, is used in subclause 16.3.3 and 16.5.1.2 - the number of RUs for Msg3 are determined according to Table 16.3.3-3 - π/4 QPSK modulation for or and ; QPSK modulation for and < Unchanged parts are omitted > - 16.5.1.2 Modulation order, redundancy version and transport block size determination To determine the modulation order, redundancy version and transport block size for the NPUSCH, the UE shall first - read the "modulation and coding scheme" field () in the DCI, and - read the "redundancy version" field () in the DCI, and - read the "resource assignment" field () in the DCI, and - compute the total number of allocated subcarriers (), number of resource units (), and repetition number () according to Subclause 16.5.1.1. If the UE is configured with higher layer parameter edt-parameters, and for a NPUSCH retransmission of the same transport block including EDT as part of the contention based random access procedure with in the DCI, - the modulation order is set to . - if the UE is configured with higher layer parameter edt-smallTBS-Enabled set to ‘true’, the repetition number for the NPUSCH retransmission is the smallest integer multiple of value that is equal to or larger than where is the TBS corresponding to the NPUSCH retransmission, and is given by the higher layer parameter edt-TBS. elseif the UE is configured with higher layer parameter edt-parameters, and if the DCI indicates a retransmission as part of the contention based random access procedure with and the most recent PUSCH transmission including a transport block with EDT, - the TBS and modulation are determined according to Table 16.3.3-1 in Subclause 16.3.3, for and the transport block does not include EDT otherwise, the UE shall use modulation order, = 2 if . The UE shall useand Table 16.5.1.2-1 to determine the modulation order to use for NPUSCH if . < Unchanged parts are omitted >

- The usage of parameter L is described in section 16.5.1.2 and in section 16.3.3, the specification just specify how to determine value of L. Therefore it seems not needed to add the reference to section 16.3.3.

- Regarding the wording change from ‘integer number’ to ‘integer multiple’, it is also included in the TP proposed in section 3.4.

Discussion on fallback to legacy Msg3

In RAN1#93, it was agreed that the DCI format that schedules the retransmission of EDT Msg3 can indicate the UE to transmit legacy Msg3 as fallback.

Agreement

- DCI format for scheduling retransmission of Msg3 can be used to indicate UE to transmit a legacy Msg3 to fallback to non-EDT Msg3 transmission when the indicated TBS is 88 bits;

  - Note: it is up to spec. editor which state to choose for EDT retransmission

  - Note: Whether to introduce higher layer support for this mechanism is up to RAN2.

It should be noticed that whether to adopt this feature in higher layers depends on RAN2’s decision. In R1-1808129, it refers to RAN2 discussion that it may impact the UE to introduce the fallback to legacy Msg3 transmission unless the timing relationships are defined sufficiently conservatively. To resolve RAN2’s concern, it is proposed in R1-1808129 as following that for EDT fallback to legacy Msg3, the minimum scheduling delay is reinterpreted as 12 ms instead of 8 ms, when fallback is indicated in DCI format N0. This is based on how long the UE is given to prepare a new NPUSCH transmission under normal circumstances of receiving RAR.

Proposal 6: For EDT fallback to legacy Msg3, the minimum scheduling delay is reinterpreted as 12 ms instead of 8 ms. Proposal 7: Adopt the TP in Annex C for TS 36.213. ---------------------------------------------- Start of Text Proposal --------------------------------------- -------------------------------------------- Unchanged parts omitted -------------------------------------- #### 16.5.1.2 Modulation order, redundancy version and transport block size determination -------------------------------------------- Unchanged parts omitted -------------------------------------- elseif the UE is configured with higher layer parameter edt-parameters, and if the DCI indicates a retransmission as part of the contention based random access procedure with and the most recent PUSCH transmission including a transport block with EDT, - the TBS and modulation are determined according to Table 16.3.3-1 in Subclause 16.3.3, for and the transport block does not include EDT - Scheduling delay field () as determined in Subclause 16.5.1 with k0 = 12 for IDelay = 0 , -------------------------------------------- Unchanged parts omitted -------------------------------------- ----------------------------------------------- End of Text Proposal ---------------------------------------

- It is suggested to further discuss the issue.

Discussion on collision issue due to EDT retransmission

In R1-1808129, another issue is raised. As shown in Figure 1, for EDT Msg3 retransmission eNB can assign resource based on the detected TBS rather than the broadcasted TBS. This enables eNB to reduce the resource overhead of EDT Msg3 retransmission, rather than always allocating the large maximum resource that can convey the maximum TBS.

However, as shown in the figure, the actual used TBS (TBSMsg3) may be different from the detected TBS (TBSMsg3,detected). When TBSMsg3,detected < TBSMsg3, the duration of NPUSCH transmission by UE may be larger than the duration allocated by eNB. In this case, there is collision between EDT Msg3 retransmission and the transmission from other UEs.

Figure 1 the Figure in R1-1808129 to show the issue

To resolve the issue, R1-1808129 proposed to indicate the detected TBS in the DCI format N0 that schedules the EDT Msg3 retransmission. The corresponding proposals and TP are attached in the following table.

Proposal 3: Re-interpret the field of ‘Modulation and coding scheme’(IMCS) in DCI format N0 and use 3≤IMCS≤6 to indicate one of the up to 4 actual TBS candidates for scheduling EDT Msg3 retransmission. The interpretation of other fields in DCI format N0 is unchanged. IMCS; Broadcast maximum TBS (edt-TBS). 328; 408; 504; 584; 680; 808; 936; 1000. 3; 328; 328; 328; 328; 328; 328; 328; 328. 4; 408; 408; 408; 456; 504; 504; 536. 5; 504; 504; 584; 680; 712; 776. 6; 584; 680; 808; 936; 1000. Proposal 4: In EDT Msg3 retransmission, if the indicated TBS in DCI( TBSindicated) is smaller than the TBS the UE selected (TBSMsg3), the UE uses TBSMsg3 to retransmit the Msg3 and scales repetition number by TBSMsg3, indicated/TBSMsg3, max instead of TBSMsg3,detected/TBSMsg3. Proposal 5: Adopt the TP in Annex B for TS 36.213. - Annex B. TP on EDT retransmission ---------------------------------------------- Start of Text Proposal --------------------------------------- -------------------------------------------- Unchanged parts omitted -------------------------------------- - 16.5.1.2 Modulation order, redundancy version and transport block size determination To determine the modulation order, redundancy version and transport block size for the NPUSCH, the UE shall first - read the "modulation and coding scheme" field () in the DCI, and - read the "redundancy version" field () in the DCI, and - read the "resource assignment" field () in the DCI, and - compute the total number of allocated subcarriers (), number of resource units (), and repetition number () according to Subclause 16.5.1.1. If the UE is configured with higher layer parameter edt-parameters, and for a NPUSCH retransmission of the same transport block including EDT as part of the contention based random access procedure with 3≤IMCS≤6 in the DCI, - the modulation order is set to . - if the UE is configured with higher layer parameter edt-smallTBS-Enabled set to ‘true’, - the TBS is determined according to Table 16.5.1.2-3. - if TBSindicated≥ TBSMsg3 , the repetition number for the NPUSCH retransmission is the smallest integer multiple of that is equal to or larger than where is the TBS corresponding to the NPUSCH retransmission, and is given by the higher layer parameter edt-TBS; - if TBSindicated< TBSMsg3, the repetition number for the NPUSCH retransmission is the smallest integer multiple of that is equal to or larger than where is the TBS corresponding to the NPUSCH retransmission, and is given by the higher layer parameter edt-TBS. elseif the UE is configured with higher layer parameter edt-parameters, and if the DCI indicates a retransmission as part of the contention based random access procedure with and the most recent PUSCH transmission including a transport block with EDT, - the TBS and modulation are determined according to Table 16.3.3-1 in Subclause 16.3.3, for and the transport block does not include EDT otherwise, the UE shall use modulation order, = 2 if . The UE shall useand Table 16.5.1.2-1 to determine the modulation order to use for NPUSCH if . -------------------------------------------- Unchanged parts omitted -------------------------------------- Table 16.5.1.2-2: Transport block size (TBS) table for NPUSCH. 0; 1; 2; 3; 4; 5; 6; 7. 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1000. 7; 104; 224; 328; 472; 584; 712; 1000; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1384. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1000; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 1000; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. Table 16.5.1.2-3: Transport block size (TBS) table of for NPUSCH retransmission of Msg3 including EDT IMCS; edt-TBS. 328; 408; 504; 584; 680; 808; 936; 1000. 3; 328; 328; 328; 328; 328; 328; 328; 328. 4; 408; 408; 408; 456; 504; 504; 536. 5; 504; 504; 584; 680; 712; 776. 6; 584; 680; 808; 936; 1000. ----------------------------------------------- End of Text Proposal --------------------------------------

- It is suggested to discuss the issue.

Others

Corrections on Title of Table 16.3.3-2

In eMTC, the title of Table 8.6.2-1 and Table 8.6.2-2. A similar change should be also applied to Table 16.3.3-2. The related TP is proposed as following:

---------------------------------------------- Start of Text Proposal ---------------------------------------

- 16.3.3	Narrowband random access response grant

-------------------------------------------- Unchanged parts omitted --------------------------------------

Table 16.3.3-2: EDT TBS for Msg3 NPUSCH with edt-smallTBS-Enabled set to ‘true’

| edt-TBS | edt-SmallTBS-Subset | Allowable TBS values |
| --- | --- | --- |
| 408 | not configured | 328, 408 |
| 504 | not configured | 328, 408, 504 |
| 504 | enabled | 408, 504 |
| 584 | not configured | 328, 408, 504, 584 |
| 584 | enabled | 408, 584 |
| 680 | not configured | 328, 456, 584, 680 |
| 680 | enabled | 456, 680 |
| 808 | not configured | 328, 504, 680, 808 |
| 808 | enabled | 504, 808 |
| 936 | not configured | 328, 504, 712, 936 |
| 936 | enabled | 504, 936 |
| 1000 | not configured | 328, 536, 776, 1000 |
| 1000 | enabled | 536, 1000 |

----------------------------------------------- End of Text Proposal --------------------------------------

Clarification on TBS for retransmission of EDT Msg3

Another TP is also provided in R1-1809663.

R1-1809663	Clarification of TBS for retransmission of EDT Msg3	Sharp

- < Unchanged parts are omitted >

#### 16.5.1.2	Modulation order, redundancy version and transport block size determination

To determine the modulation order, redundancy version and transport block size for the NPUSCH, the UE shall first

-	read the "modulation and coding scheme" field () in the DCI, and

-	read the "redundancy version" field () in the DCI, and

-	read the "resource assignment" field () in the DCI, and

-	compute the total number of allocated subcarriers (), number of resource units (), and repetition number () according to Subclause 16.5.1.1.

If the UE is configured with higher layer parameter edt-parameters, and for a NPUSCH retransmission of the same transport block including EDT as part of the contention based random access procedure with  in the DCI,

-	the modulation order is set to .

-	if the UE is configured with higher layer parameter edt-smallTBS-Enabled set to ‘true’, the repetition number for the NPUSCH retransmission is the smallest integer number of  value that is equal to or larger than where  is the TBS corresponding to the NPUSCH transmission scheduled by the Narrowband Random Access Response Grant, and  is given by the higher layer parameter edt-TBS.

elseif the UE is configured with higher layer parameter edt-parameters, and if the DCI indicates a retransmission as part of the contention based random access procedure with  and the most recent PUSCH transmission including a transport block with EDT,

-	the TBS and modulation are determined according to Table 16.3.3-1 in Subclause 16.3.3, for  and the transport block does not include EDT

otherwise, the UE shall use modulation order, = 2 if . The UE shall useand Table 16.5.1.2-1 to determine the modulation order to use for NPUSCH if .

- < Unchanged parts are omitted >

- References

| No. | Tdoc | Source | Title |
| --- | --- | --- | --- |
| 1 | R1-1808129 | Corrections on EDT for NB-IoT | Huawei, HiSilicon |
| 2 | R1-1809126 | Clarification on the cyclic repetition L of Early Data Transmission | NTT DOCOMO, INC |
| 3 | R1-1809663 | Clarification of TBS for retransmission of EDT Msg3 | Sharp |

- Appendix: Agreements regarding EDT in Rel-15

Agreements in RAN1#90bis - From RAN1 point of view, it is feasible to support early UL data transmission in Msg3 from an NB-IoT UE using some TBS value(s) from the TBS range specified for NB-IoT in Rel-13 with a maximum total TBS of 1000 bits. - FFS if and how there will also be a larger supported maximum total TBS - The detailed value(s) should consider the payload size of early data packets from RAN2. - From RAN1 perspective, the physical layer design will assume eNB is not required to always provide a grant of a larger TBS for Msg3 and can decide to just provide a grant for 88 bits instead - Send LS to RAN2 informing the above (Xiaolei, HiSilicon, R1-1719100) (including eMTC agreements) Agreements in RAN1#91 - The number of MCS/TBS/RU states that can be used for EDT will be chosen from - Limited MCS/TBS/RU states - Alt. 0: 5 unused MCS/TBS/RU states and 0 bit in SIB - Alt. 1: As many as supported by using 1 spare bit from RAR and 0 bit in SIB - Alt. 2: As many as supported by using 2 spare bits from RAR and 0 bit in SIB - Alt. 3: As many as supported by using 2 bits in SIB and 0 spare bit in RAR - Alt. 4: As many as supported by using maximum TBS value in SIB and 0 spare bit in RAR - Alt. 5: 1 spare bit in RAR used for new/modified UL grant and 0 bit in SIB - From RAN1’s point of view, - Uplink subcarrier spacing field, subcarrier indication field, scheduling delay field and Msg3 repetition number field in RAR UL Grant for uplink EDT in Msg3 do not need to be changed according to current RAN2 agreements. - The above applies to above Alts. 1-4 Send reply LS (R1-1721227) – Matthew (Huawei) indicating RAN1 will select from Rel-13 NB PUSCH TBS values. For NB-IoT, it is feasible to support at least 5 MCS/TBS/RU size combinations. - Ask RAN2 how many TBS values are needed. Note: R1-1721227 is in principle agreed. Update for eMTC will be taken into account in R1-1721242 - Final LS is agreed in R1-1721255 Agreements in RAN1#92 Agreement - The maximum TBS broadcasted in system information are selected from 8 values which are taken from the Rel-13 NPUSCH tables. - The up to 4 possible TBS which is smaller than or equal to the maximum broadcast TBS values for the UE to choose among are FFS. FFS: How the UE obtains the up to 4 possible values. Agreement Support NW enabling the use of TBS smaller than the maximum configured. FFS details. Agreements in RAN1#92bis Agreement - The 8 maximum TBS is {1000, 936, 808, 680, 584, 504, 408, 328}. - Only values of NRU and values of TBS in legacy Rel-13 NPUSCH table can be used for EDT - For each of the 8 maximum TBS values - For each of the Rel-13 reserved ‘MCS index’ states there is one number of RUs - The number of repetitions changes depending on the actual TBS, potentially including numbers of repetitions which are not included in legacy NPUSCH repetition numbers (FFS how) - Potential new numbers of repetitions may include multiple of 4 or 8 - The 8 maximum TBS values are: 1000, 936, 808, 680, 584, 504, 408, 328 bits - For 1000 bits max TBS: I_RU = 3, 4, 5, 6, 7 - For 936 bits max TBS: I_RU = 3, 4, 5, 6, 7 - For 808 bits max TBS: I_RU = 3, 4, 5, 6, 7 - For 680 bits max TBS: I_RU = 2, 3, 4, 6, 7 - For 584 bits max TBS: I_RU = 2, 3, 4, 5, 6 - For 504 bits max TBS: I_RU = 2, 3 4, 5, 6 - For 408 bits max TBS: I_RU = 2, 3, 4, 5, 6 - For 328 bits max TBS: I_RU = 2, 3, 4, 5, 6 - The 3 legacy MCS indices are used for fallback to non-EDT transmission; Agreement - The use of TBS smaller than the maximum configured is configured per CE level in SIB. - Per cell, in the below, Ti < Ti+1, eNB can configure that the UE chooses from: - When there are 4 permitted actual transmitted TBS {T1, T2, T3, T4} - T2 or T4 - T1, or T2, or T3, or T4 - When there are 3 permitted actual transmitted TBS {T1, T2, T3} - T2 or T3 - T1 or T2 or T3 - When there are 2 permitted actual transmitted TBS {T1, T2} - T1 or T2 Agreement - Ti with the maximum value of i is the maximum TBS in SIB. Agreement Up to 4 TBS values are defined based on only the maximum broadcast TBS R1-1805409 Updated Feature lead summary of EDT during RACH in feNB-IoT Huawei, HiSilicon Agreement The following table is taken as baseline for up to 4 TBS values for each maximum broadcast TBS when the use of smaller TBS is enabled, which can be approved by an email discussion. 328; 408; 504; 584; 680; 808; 936; 1000. T1; 328; 328; 328; 328; 328; 328; 328; 328. T2; 408; 408; 408; 456; 504; 504; 536. T3; 504; 504; 584; 680; 712; 776. T4; 584; 680; 808; 936; 1000. Conclusion Further study aspects related to Msg. 3 retransmission until RAN1#93 meeting Agreements in RAN1#93 Agreement The repetition number of actual TBS（Ti）for initial transmission of Msg3 is determined by scaling the repetition number indicated in the corresponding RAR as following: - The repetition number indicated in the corresponding RAR is set based on the broadcast maximum TBS; - The scaling factor is the ratio of the actual TBS(Ti) and the broadcast maximum TBS for Msg3; - The repetition number of the actual TBS is rounded up to the nearest higher integer number of L after the scaling; Agreement For EDT Msg3 transmission/retransmission using single-tone, pi/4 QPSK is used always. Agreement The parameter L for cyclic repetition is identical for all 4 actual TBS candidates. - When the repetition number for the maximum TBS is equal or larger than 8, L = 4. - When the repetition number for the maximum TBS is smaller than 8, L=1. Agreement - For a retransmission of msg3 carrying early data transmission, the UE uses the same TBS as previous transmission of Msg3 scheduled by RAR. - Note: this does not exclude possibility that DCI can indicate fallback to transmit legacy Msg3; Agreement - For retransmission, if the UE selects an actual TBS smaller than the maximum TBS, the number of repetitions is down-scaled in the same way as for the initial transmission; Agreement - DCI format for scheduling retransmission of Msg3 can be used to indicate UE to transmit a legacy Msg3 to fallback to non-EDT Msg3 transmission when the indicated TBS is 88 bits; - Note: it is up to spec. editor which state to choose for EDT retransmission - Note: Whether to introduce higher layer support for this mechanism is up to RAN2. Agreement The timing for monitoring DCI is based on the number of Msg3 NPUSCH repetitions indicated in RAR/DCI