3GPP TSG RAN WG1 Meeting #94	R1-1809712

Gothenburg, Sweden, August 20 – 24, 2018	(Update to R1-1809394)

Agenda item:		7.1.7

Source:	Nokia, Nokia Shanghai Bell

Title:	On selected UE features

Document for:		Discussion and Decision

# Introduction

This document is an update to R1-1809394, with additional points added from section 6 onwards. There is also a change in feature 2-24 in section 2.2, shown with yellow highlighting.

# DL MIMO related features

## DL MIMO number of layers

The feature 2-3 ‘PDSCH MIMO layers” TSG-RAN decision was updated in RAN#80 [1] to state:

For single CC standalone NR, it is mandatory with capability signaling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. Some relaxations to this requirement may be applicable in the future (including in Rel-15). Mandatory in all cases above means mandatory with capability signaling. It is not expected that there is a signaling change resulting from this (i.e. signaling remains to be defined as {1, 2, 4, 8} in every band, including FR1 and FR2.

RAN#80 discussed also other cases like EN-DC, CA and if relaxations could be considered for these cases. Unfortunately, RAN#80 was not able to complete this discussion and RAN4 and RAN1 were tasked to continue the discussion before the September RAN#81 meeting. In our view on the bands where 4Rx is mandatory, UE should also be mandated to support 4 MIMO layers for all operating scenarios including EN-DC, CA, non-ENDC/non-CA. On all other bands, UE should be mandated to support 2 MIMO layers. We propose that the feature 2-3 ‘PDSCH MIMO layers” is further updated as follows to avoid any confusion on the applicability of 4 MIMO layer UE requirement in different scenarios:

Proposal 1: Update the feature 2-3 ‘PDSCH MIMO layers” in the RAN1 NR UE Feature as follows

| 2-3 | PDSCH MIMO layers | 1. Supported maximal number of MIMO layers | Candidate values: {1,2,4,8} FFS on the minimal layers for different band or band combination. | For single CC standalone NR, it is mandatory with capability signaling to support at least 4 MIMO layers for all operating scenarios (EN-DC, CA, non-EN-DC/non-CA) in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2 for all operating scenarios (EN-DC, CA, non-EN-DC/non-CA). Some relaxations to this requirement may be applicable in the future (including in Rel-15). Mandatory in all cases means mandatory with capability signaling. It is not expected that there is a signaling change (i.e. signaling remains to be defined as {1, 2, 4, 8} in every band and every band combination, including FR1 and FR2 in all cases. |
| --- | --- | --- | --- | --- |

## Beam reporting and CSI reporting

Regarding features 2-21 and 2-22, beam-based transmission schemes envisioned for FR2 can also be employed in FR1 deployments. As a result, the beam reporting features for beam refinement that are currently mandatory for FR2 should also be made mandatory for FR1. SRS-based beam refinement can be seen as an alternative to beam reporting, but has disadvantages such as poor cell edge performance, sensitivity to interference, and relatively poor performance in FDD. It is expected that beam reporting would be much more robust than beam selection based on SRS due to the significantly higher transmit power on the downlink. Simulation results showing the disadvantages of SRS even in TDD and especially for cell edge UEs are shown in the Annex.

| # | FG | Components | Consequence if not supported by UE | RAN WG recommendation | TSG-RAN decision | Nokia comment |
| --- | --- | --- | --- | --- | --- | --- |
| 2-20 | Beam correspondence | 1. Support Beam correspondence | Beam correspondence is not supported | [Mandatory at least for FR2] |  | [Mandatory for FR2 without capability signalling] |
| 2-21 | Periodic beam report | 1. Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot 2. Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot | No support of periodic L1-RSRP report | Note: Beam correspondence means each Tx port can be beamformed in a desirable direction but does not imply setting phase across ports Mandatory with UE capability at least for FR2 FFS: for FR1 | Mandatory with UE capability at least for FR2 | Mandatory with UE capability for FR1&FR2 |
| 2-22 | Aperiodic beam report | 1. Support report on PUSCH | No support of aperiodic L1-RSRP report | Mandatory with UE capability at least for FR2 | Mandatory with UE capability at least for FR2 | Mandatory with UE capability for FR1&FR2 |
| 2-24 | SSB/CSI-RS for beam measurement | 1. The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_1 2. The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 3. Supported density of CSI-RS | RSRP measurement is not supported | Component-1, candidate value set for MB_1 is {8, 16, 32, 64} Support MB_1 =8 is mandatory for at least for >6Ghz bands Component-2, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-3: candidate value set: {“1 only”, “3 only”, “both 1 and 3”} At least density of CSI-RS =3 is mandatory at least for FR2 |  | Support MB_1 =64 is mandatory for FR1&FR2 Support MB_2 =8 is mandatory for FR1&FR2 At least density of CSI-RS =3 is mandatory for FR1&FR2 |
| 2-38 | CSI report without PMI | Support CSI report without PMI | CSI report without PMI is not supported | FFS |  | Mandatory without UE capability |

# UL MIMO support for SUL carriers

RAN4#87 agreed that SUL carrier does not support UL MIMO. However, this is not reflected in the feature list or in the specification [2].

R4-1806546 Switching time for NR UL and NR SUL with MIMO transmissions CR- rev Cat: (Rel-15) v Source: MediaTek inc. Abstract: Proposal 1: RAN4 decide if UL-MIMO is considered in SUL scenario. Proposal 2: Support of UL-MIMO for SUL is added in feature list if RAN4 decide to have this scenario. Proposal 3: ~130us switching time for SUL with UL-MIMO case is needed. Discussion: Nokia: We are not ready to accept it. We have concerns on applying different requirements. It will cause complexity for NR specification. RAN1 spec is defined based on 0us switching time. We need to check the RAN1 view. MTK: 0us is defined assuming NR can reuse the LTE RF path. Given LTE have 1 RF path, if NR SUL supports UL-MIMO, LTE RF path cannot be reused. Also, MIMO has not been considered for SUL operation. Huawei: We discussed it in the ad-hoc. Switching time is defined assumed without considering uplink. SUL is supposed to be operated under low frequency in which UL-MIMO is challenging to be implemented. => It is agreed that UL-MIMO is not supported in SUL carrier in Rel-15 Decision: The document was Noted.

The lack of UL MIMO support would seem to be more naturally covered by RAN1, and the restriction is included in the UL MIMO features as follows:

| 2-14 | Codebook based PUSCH MIMO transmission | 1. Supported codebook based PUSCH MIMO with maximal number of supported layers 2. Supported max number of SRS resource per set (SRS set use is configured as for codebook). | Component-1: Candidate value: {no codebook based MIMO, 1, 2, 4}. For SUL carrier only value “no codebook based MIMO” can be used. Component-2 Candidate value: {1, 2}. For SUL carrier only value 1 can be used |
| --- | --- | --- | --- |
| 2-15 | non-codebook based PUSCH transmission | 1. Maximal number of supported layers (non-codebook transmission scheme): | Component-1 candidate values: {“No non-codebook based MIMO”, 1, 2, 4}. For SUL carrier only value “No non-codebook based MIMO” can be used. |

The MIMO features 2-13 “PUSCH codebook coherency subset”, and 2-54a “Simultaneous SRS Tx” could also include the same restriction, but it may be somewhat redundant.

# UL control features

Feature 4-19, SR/HARQ-ACK/CSI multiplexing once per slot should be a basic feedback feature. 4-24 PUCCH spatial relation indication info should be mandatory for FR2 beam based systems, while it could be optional for FR1.

| # | FG | Note | RAN WG recommendation | TSG RAN decision | Nokia proposal |
| --- | --- | --- | --- | --- | --- |
| 4-19 | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or piggybacked on a PUSCH) | RAN1 needs to clarify this feature | [Optional with capability signaling] |  | Mandatory with UE capability |
| 4-24 | PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource |  |  | Optional or mandatory? | Optional with UE capability for FR1, mandatory with capability for FR2 |

# BWP features

As discussed in[3], the features 6-2 and 6-3, seems to be the next implementation delta from the mandatory feature 6-1. And we think that it should be clarified what scenarios should be supported with this optional feature. To our understanding, feature 6-2 supports 2 dedicated BWPs plus initial active BWP, i.e. switching DCI-based switching between 3 BWPs total is supported by feature 6-2 (see RAN2 configuration Option 1 in Section 2). It should be clarified that numerology of initial active BWP should be the same as numerology of 2 UE-specific RRC configured DL BWPs for feature 6-2 and similarly for features 6-3. Feature 6-4 maintains the capability for switching between BWPs of different SCS.

Proposal: Feature 6-2/6-3 (Type A/B BWP adaptation with same numerology): Clarify that the initial active BWP (configured by MIB, SIB1 and/or dedicated RRC) should be of the same numerology as numerology of UE-specific RRC configured DL BWPs.

| # | FG | Components | Note |
| --- | --- | --- | --- |
| 6-2 | Type A BWP adaptation with same numerology | 1) Up to 2 UE-specific RRC configured DL BWPs per carrier 2) Up to 2 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the initial DL BWP and SSB for Pcell[/PScell] and BW of the UE-specific RRC configured BWP includes SSB for Scell if there is SSB on Scell |  |
| 6-3 | Type B BWP adaptation with same numerology | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the initial DL BWP and SSB for Pcell[/PScell] and BW of the UE-specific RRC configured BWP includes SSB for Scell if there is SSB on Scell |  |
| 6-4 | BWP adaptation with different numerologies | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) More than one numerologies for the UE-specific RRC configured BWPs per carrier 5) Same numerology between DL and UL per cell except for SUL at a given time 6) BW of a UE-specific RRC configured BWP includes BW of the initial DL BWP and SSB for Pcell[/PScell] and BW of the UE-specific RRC configured BWP includes SSB for Scell if there is SSB on Scell |  |

# TCI state and RS configuration capabilities

The following additional clarifications are suggested to TCI state and RS configuration capabilities:

| # | FG | Components | Note | RAN WG recommendation | Nokia comment |
| --- | --- | --- | --- | --- | --- |
| 2-4 | TCI states for PDSCH | 1. Support number of active TCI states per BWP per CC , including control and data 2. maximum number of configured TCI states per CC for PDSCH |  | Component-1: Candidate value set: {1, 2, 4, 8 } Component-2: candidate value set: {4, 8, 16, 32, 64} FFS: mandatory values for component-2 | Component-2: FR1: at least 8 is mandatory FR2: at least 64 is mandatory |
| 2-31 | Beam failure recovery | 1. Maximal number of CSI-RS resources across all CCs for UE to monitor PDCCH quality 2. Maximal number of different SSBs across all CCs for UE to monitor PDCCH quality 3. Maximal number of different CSI-RS and/or SSB resources across all CCs for new beam identifications. 4. COREST recoveryControlResourceSetId if configured | Component 4) is in addition to component 1) of 3-1 | Component-1 candidate value set: {from 1 to 64} Component-2 candidate: {from 1 to 64} Component-3: Candidate value set is: {from 1 to 256} [UE is mandate to support at least 64.] | Component-1: FR1: at least 8 is mandatory FR2: at least 64 is mandatory Component-2: FR1: at least 8 is mandatory FR2: at least 64 is mandatory |
| 2-51 | TRS (CSI-RS for tracking) | 1. Support TRS BW 2. TRS burst length (X), 3. Max # of TRS resource sets (per CC) UE is able to track simultaneously 4. Max # of TRS resource sets configured to UE per CC 5. Max # of TRS resource sets configured to UE across CCs |  | Component-1: candidate values set: {BWP, min(52,BWP), both} Component-2: candidate values {1,2} Component-3: Candidate value set: {1 to 8} Component-4: Candidate value set: {1 to 64} Component-5: Candidate value set: {1 to 128} | Component-4: FR1: at least 8 is mandatory FR2: at least 64 is mandatory Component-5: FR1: at least 8*#CCs is mandatory FR2: at least 64*#CCs is mandatory |
| 2-53 | SRS resources | 1. Maximum number of aperiodic SRS resources (configured to UE) per BWP 2. Maximum number of aperiodic SRS resources (configured to UE) per BWP per slot 3. Maximum number of periodic SRS resources (configured to UE) per BWP 4. Maximum number of periodic SRS resources (configured to UE) per BWP per slot 5. Maximum number of semi-persistent SRS resources (configured to UE) per BWP 6. Maximum number of semi-persistent SRS resources (configured to UE) per BWP per slot 7. Maximum number of SRS port per resource |  | Component-1: candidate value: {from 1 , 2, 4, 8, 16} Component-2 candidate value: {1,2,3,4,5,6} Component-3: candidate value: {from 1 , 2, 4, 8, 16} Component-4 candidate value: {1,2,3,4,5, 6} Component-5: candidate value: {from 1 , 2, 4, 8, 16} } Component-6 candidate value: {1, 2,3,4,5, 6} Component-7 candidate values: {1, 2, 4} Support SP-SRS is mandatory with capability | Support SP-SRS is mandatory with capability, with the capability bit always set to 1 |

Feature group 3-4 leaves some room for interpretation, and as it is mandatory, it should be included in feature 3-1 rather than overriding a sub-component of a feature in 3-1.

| 3-1 | Basic DL control channel | 1) One UE-specific configured CORESET per BWP per cell in addition to CORESET0 - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols for FR1 - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSSs, CORESET resource allocation of 6RB bit-map and duration 1-3 OFDM symbols for FR2 - For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 1-2 OFDM symbols for FR2 - REG-bundle sizes of 2/3 RBs or 6 RBs - Interleaved and non-interleaved CCE-to-REG mapping - Precoder-granularity of REG-bundle size - PDCCH DMRS scrambling determination - Up to 64 TCI states for a CORESET configuration 2) CSS and USS configurations for unicast PDCCH transmission per BWP per cell - PDCCH aggregation levels 1, 2, 4, 8, 16 - For type 1 with dedicated RRC configuration, type 3, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 without dedicated RRC configuration and for type 0, 0A, and 2, the monitoring occasion can be any OFDM symbol(s) of a slot 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table 5) Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot per scheduled CC for FDD 6) Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot per scheduled CC for TDD | Mandatory without capability signaling | Mandatory without capability signaling |
| --- | --- | --- | --- | --- |

| # | FG | Components | Note | RAN WG recommendation | TSG RAN Decision | Nokia comment |
| --- | --- | --- | --- | --- | --- | --- |
| 3-4 | More than one TCI state configurations per CORESET |  | UE is only required to track one active TCI state [per CORESET] | Optional with capability signaling | Mandatory with capability signaling which shall be set to ‘1’ | This feature can be removed, and included to 3-1 |

# References

[1]	RP-181483	RAN1 NR UE feature list, NTT DOCOMO

[2] draft RAN4#87 meeting report

[3] R1-1808953	On remaining details of BWPs and CA, Nokia, Nokia Shanghai Bell

# Annex: Loss from non-ideal channel estimation with SRS-based beam selection

To highlight some of the disadvantages of relying only on SRS for beam selection, this Annex shows an example set of simulation results showing the performance loss from using SRS-based beam selection with realistic channel estimation relative to ideal channel estimation. The simulated scenario is the NR-UMa scenario at 2GHz with a 500m inter-site-distance, TDD, full buffer traffic, and 10 UEs per sector on average. At the base station was an 8-column dual polarized array configured for 64 controllable elements leveraging a simple beam-based transmission methodology. The UE total transmit power was 23dBm, and single-antenna SRS. The transmission scheme is simply selecting the best beam for a UE where data transmission was either in an SU-MIMO mode or MU-MIMO mode. In this example simulation, the best beam for a UE was selected every 10 msec. Note the sizeable losses in cell edge performance with a simple beam-based transmission schemes. These results point out the disadvantages in leveraging SRS for beam management, namely poor cell edge performance.

Figure 1: Performance Loss Uniform Panel array with four sub-panels, 256 total antenna elements