3GPP TSG RAN WG1 Meeting #94                               R1-1809718

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	CATT

Title:	Self-evaluation on User Plane latency for LTE

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

- Introduction

At RAN#75 meeting, new Study Item on Self Evaluation towards IMT-2020 submission was approved [1]. Self-Evaluation will provide the performance towards all the ITU-R IMT-2020 requirements as defined in Report ITU-R M.2410 [2]:

The minimum requirements for user plane latency are:

–	4 ms for eMBB

–	1 ms for URLLC

assuming unloaded conditions (i.e. a single user) for small IP packets (e.g. 0 byte payload + IP header), for both downlink and uplink

In this contribution, preliminary evaluation results of LTE user plane latency are provided. The evaluation is applied to both TDD and FDD.

- User plane latency evaluation

  - User plane latency components

Based on the definition and evaluation methodology provided in Report ITU-R M.2412 [2], the procedure and components for LTE user plane latency are listed in Table 1. Also notations are provided for those components.

Table 1: Procedure and components for LTE user plane latency

| Step | Component | Notations |
| --- | --- | --- |
| 1 | Processing delay for preparing data at Tx side | tTx,data |
| 2 | Transmission alignment delay for data | tTA |
| 3 | TTI for data packet transmission | tdata_duration |
| 4 HARQ | - HARQ timing: receiving data + preparing PUCCH (ACK/NACK) or PDCCH | tHARQtiming |
|  | - Transmission alignment delay for PUCCH (ACK/NACK) or PDCCH | tTA_HARQ |
|  | - PUCCH (ACK/NACK) or PDCCH duration time for transmission | tHARQ_duration |
|  | - HARQ timing: receiving PUCCH (ACK/NACK) or PDCCH + preparing data | tHARQ_timing |
|  | - Frame alignment delay for retr data | tTA |
|  | - Transmission alignment delay for retransmission data | tdata_duration |
|  | HARQ RTT time | tHARQ_RTT = tHARQtiming + tFA_HARQ + tHARQ_duration + tHARQtiming + tFA + tdata_duration |
| 5 | TTI for (retransmission) data packet transmission | tRx_data |

With the one shot transmission latency and HARQ RTT time calculated in Table 1, user plane latency can be obtained for transmission with target BLER p:

t = tTx,data+ tFA + tdata_duration + tRx_data +p * tHARQ_RTT             (1)

  - Assumptions

Some assumptions for UP latency evaluation are clarified as follows:

1. Packet arrival

Assume that the packet arrives at any timing of any OFDM symbol in a radio frame with uniform probability, i.e. it may arrive at starting or not and frame alignment time for FDD is 0.5 symbol length if next symbol is valid for transmission and otherwise 0.5 symbol length + the number of symbols for waiting.

Proposal 1: Packet arrival at any timing of any OFDM symbol in a radio frame with uniform probability is proposed.

2. Processing time

BS processing time is assumed as 1 TTI for preparing or receiving data. UE processing time is assumed longer than BS processing time, as 1.5 TTI for preparing or receiving data.

3. HARQ timing

N+4 HARQ timing is considered, i.e. HARQ delay (delay between data is received and HARQ is prepared) is equal to 3 TTI.

4. PDCCH/PUCCH occasion

For simplification, it is assumed PDCCH and PUCCH occasion occur at every OFDM symbol. In this case, the PDCCH and PUCCH are assumed to be able to be transmitted at every OFDM symbol. However, the transmission cannot be across the slot boundary.

Furthermore, 1 symbol PDCCH and 1 symbol PUCCH are considered in the evaluation.

Proposal 2: Assume PDCCH and PUCCH occasion occur at every OFDM symbol and 1 symbol PDCCH and 1 symbol PUCCH are considered.

5. Transmission alignment delay consideration

To calculate transmission delay, the following assumptions are considered:

- Transmission cannot be across the slot boundary;

- PDCCH monitoring occasion occurs.

6. UP latency calculation method

It is complicated to summarize the UP latency since there are different start symbol occasions. One straightforward method is to count the latency from any possible data arrival case considering PDCCH monitoring occasion and PDSCH non-slot length case by case. Under those, average value can be reached in a certain UL/DL configuration and one length of non-slot. Also it is easy to pick up the best case from those cases. The evaluation in this contribution provides both results.

Proposal 3: Two options are proposed for UP latency calculation:

Under FDD or a certain TDD UL/DL configuration, a certain length of non-slot:

Option-1 Average method: It is average value considering all data arrival cases.

Option-2 Best case method: The instant case which can achieve the minimum UP latency among all the cases.

7. Target HARQ BLER

Different target BLER p for the first transmission are considered with p = 0 and 0.1, corresponding to first transmission reliability = 100% and 90%.

8. PDSCH/PUSCH non-slot length

For FDD, 2/3/7-os non-slot are evaluated. For TDD, 7-os non-slot TTI is evaluated.

- UP latency results

In this section, user plane latency evaluation results for LTE under 15kHz SCS are provided. FDD and TDD mode (including two TDD configurations: DU and DSUUD) are considered. Both average method and best case method are applied.

  - LTE FDD

For FDD, the DL and UL user plane latency results are given in Table 2.

Table 3: DL and UL UP latency results for LTE FDD

| non-slot duration | Error probability | Average method |  | Best case method |  |
| --- | --- | --- | --- | --- | --- |
|  |  | DL | UL | DL | UL |
| 2 OS non-slot | p=0 | 0.59 | 0.59 | 0.50 | 0.50 |
|  | p=0.1 | 0.69 | 0.69 | 0.60 | 0.60 |
| 3 OS non-slot | p=0 | 0.86 | 0.86 | 0.75 | 0.75 |
|  | p=0.1 | 1.02 | 1.02 | 0.90 | 0.90 |
| 7 OS non-slot | p=0 | 2.02 | 2.02 | 1.75 | 1.75 |
|  | p=0.1 | 2.41 | 2.41 | 2.12 | 2.12 |

Observation 1: For LTE FDD,

- By both average and best case method, 2/3/7-os non-slot can fulfill user plane latency requirement of IMT-2020 for eMBB;

- By both average and best case method, 2-os non-slot can fulfill user plane latency requirement of IMT-2020 for URLLC;

  - LTE TDD

For LTE TDD, DSUUD configuration is evaluated. For the “S” slot in UL/DL configuration 1 with DSUUD structure, slot format with 11 DL symbols, 1 GP symbol and 2 UL symbols are evaluated. The DL and UL user plane latency results are given in Table 3.

Table 3: DL and UL UP latency results for LTE TDD (DSUUD)

| non-slot duration | Error probability | Average method |  | Best case method |  |
| --- | --- | --- | --- | --- | --- |
|  |  | DL | UL | DL | UL |
| 7 OS non-slot | p=0 | 2.59 | 3.18 | 2.12 | 1.75 |
|  | p=0.1 | 3.05 | 3.66 | 3.00 | 2.12 |

Observation 2: For LTE TDD (DSUUD), 7-os non-slot can fulfill user plane latency requirement of IMT-2020 for eMBB by both average and best case method.

- Conclusions

In this contribution, user plane latency evaluation procedure is discussed and we have the following proposal:

Proposal 1: Packet arrival at any timing of any OFDM symbol in a radio frame with uniform probability is proposed.

Proposal 2: Assume PDCCH and PUCCH occasion occur at every OFDM symbol and 1 symbol PDCCH and 1 symbol PUCCH are considered.

Proposal 3: Two options are proposed for UP latency calculation:

Under FDD or a certain TDD UL/DL configuration, a certain length of non-slot:

Option-1 Average method: It is average value considering all data arrival cases.

Option-2 Best case method: The instant case which can achieve the minimum UP latency among all the cases.

Preliminary evaluation results of LTE user plane latency are provided. The evaluation is applied to FDD and TDD. And both average and best case method are applied. The following observations are made:

Observation 1: For LTE FDD,

- By both average and best case method, 2/3/7-os non-slot can fulfill user plane latency requirement of IMT-2020 for eMBB;

- By both average and best case method, 2-os non-slot can fulfill user plane latency requirement of IMT-2020 for URLLC;

Observation 2: For LTE TDD (DSUUD), 7-os non-slot can fulfill user plane latency requirement of IMT-2020 for eMBB by both average and best case method.

- References

- RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.