3GPP TSG RAN WG1 Meeting RAN1#94	 R1-18xxxxx

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:	7.2.2.4.2

Source:	Charter Communications

Title:	Feature lead summary #1 of initial access and mobility

Document for:	Discussion

Introduction

This paper summarizes open issues on initial access and mobility in RAN1#94.

- The issues for broadcast and system information transmissions are discussed in Section 2 first, followed by random access in Section 3 and mobility management in Section 4.

- The proposals generated by the feature lead are appended at the end of each subsection.

  - If agreed, it is expected that the feature lead will help preparing corresponding text proposals.

Open issues for broadcast transmissions

Multiplexing of SS Block/PBCH and other channels/signals

Description:

In response to the FFS point on other channels and signals transmitted together as part of the signal that contains at least SS/PBCH block burst set transmission, several companies have expressed their views, largely focused on RMSI and OSI as candidates. A majority of the sources in favor of including RMSI also proposed that FDM of RMSI and SS/PBCH block be supported.

Agreement:

- NR-U should have a signal that contains at least SS/PBCH block burst set transmission

  - FFS: Other channels and signals transmitted together as part of the signal

- The design of this signal should consider the following characteristics specific to unlicensed band operation

  - There are no gaps within the time span the signal is transmitted at least within a beam

    - FFS: Whether any gaps are needed for beam switching and, if needed, their duration

  - The occupied channel bandwidth is satisfied (although this may not be a requirement)

  - Strive to minimize the channel occupancy time of the signal

  - Characteristics that may facilitate fast channel access

Alternatives:

- Alt-1: At least RMSI and corresponding CORESET are one of the other channels transmitted together with SS/PBCH burst set

- Alt-2: RMSI and corresponding CORESET are not transmitted together with SS/PBCH burst set

| Company(s) | View/position |
| --- | --- |
| Huawei, HiSilicon | Alt-1 |
| Samsung | Alt-1 as a baseline, and consider whether Alt-2 is needed to guarantee RMSI’s coverage. |
| Sony | Alt-1 |
| Ericsson | Alt-1 |
| Charter | Alt-1 |
| AT&T | Alt-1 |
| Qualcomm | Alt-1 |
| Xiaomi | Alt-1 |
| Nokia | Alt-1 |
| ITL | Alt-1 |
| Spreadtrum | Support both Alt-1 and Alt-2. For SSB/CORESET multiplexing pattern 2/3, Alt-1 is supported. For SSB/CORESET multiplexing pattern 1, both Alt-1 and Alt-2 can be supported by reusing the current tables for RMSI PDCCH monitoring occasions and configuring the group offset to zero or non-zero. |
| OPPO | Alt-1 |
| InterDigital | Alt-1 |
| Ericsson | Alt-1 |
| LG Electronics | Alt-1 at least within SMTC |

Proposal:

- At least RMSI and corresponding CORESET are one of the other channels transmitted together with SS/PBCH burst set

  - FFS: How they are transmitted together

    - Alt-1: RMSI can be FDM with SS/PBCH block

    - Alt-2: RMSI can only be TDM with SS/PBCH block

- FFS: Additional channels and signals that are a part of the signal, e.g., paging, OSI

Impact of LBT on SS/PBCH Block positions

Description:

Due to unsuccessful LBT, one or more SS/PBCH Blocks may have to be dropped at certain time instances. This was identified in the previous meeting as "Develop techniques to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure.”

How to continue with the SS/PBCH Block burst set transmission in this case is a key question – options include either shift the entire burst set in time, or truncate the start of the burst set and and cyclically wrap the dropped indices around to the end of the burst set transmission.

Alternatives:

- Alt-1: Shift entire burst set in time to the next  transmission instance

- Alt-2: Truncate the start of the burst set and and cyclically wrap the dropped SSB indices around to the end of the burst set transmission

- Alt-3: Network to flexibly schedule SSB index and indicate the timing information

| Company(s) | View/position |
| --- | --- |
| Samsung | Alt-2 |
| Ericsson | Alt-2? |
| Qualcomm | Alt-2 |
| ITL | Alt-2 |
| LGE | Both of Alt-1 and Alt-2 |
| Intel | Alt-3 |
| Spreadtrum | In our understanding, there are several successive burst sets within a window, e.g. 5ms window, and the possible SSBs time locations are pre-defined within the window. Under this assumption, Alt-1 and Alt-2 are similar, and both mean that there is only one LBT for a transmission of L SSBs. Since Alt-1 and Alt-2 both guarantee that the L SSBs are transmitted if a TXOP is achieved, both Alt-1 and Alt-2 can be fine for us. Under this assumption, the different b/w Alt-1 and Alt-2 could be the LBT occasion relative to the burst set. For Alt-1, the LBT happens just before the burst set. For Alt-2, the LBT happens within the burst set. We think the number of LBT occasions in Alt-1 is enough, so we prefer Alt-1 slightly. In addition, since the definitions of the burst set and the window are not clear, we propose to clarify the definitions of the burst set and the window. |
| OPPO | Both Alt-1 and Alt-2 are fine. For Alt-2, the offset between the cyclically wrapped SSB and the dropped SSB should be indicated to UE. |
| InterDigital | Alt-2. (Alt-3 may also be considered) |
| Ericsson | In our view it is too early to agree on the exact scheme. Perhaps a first step is to agree that the SSBs/DRS can be shifted in time due to LBT failure, with FFS on the exact details of how the UE shall (1) obtain cell timing information, and (2) how RLM/RRM/RACH procedures are affected Proposal: Define a mechanism for the UE to derive the timing offset with which an SS/PBCH block has been shifted relative to its configured time position FFS: Details of how the UE obtains cell timing information for a shifted SS/PBCH block, including signaling details FFS: Impact on SS/PBCH block to PRACH resource mapping FFS: Impact on RRM/RLM procedures |

Proposal:

- Truncate the start of the SS/PBCH block burst set upon LBT failure and and cyclically wrap the dropped indices around to the end of the burst set transmission

  - FFS: Details of cyclic wrapping

  - FFS: Mechanism to indicate cyclic shift to UEs

  - FFS: Impact on UE PRACH resource mapping

Note: Several companies discussed the applicability of Cat-2 and Cat-4 LBT for SS/PBCH block transmissions, this issue is better addressed in AI 7.2.2.4.1

Open issues for random access

LBT impact on preamble power ramping and retransmission counter

Description:

If LBT fails prior to preamble transmission, many companies propose that equating the dropped transmission to an unsuccessfully received preamble at the gNB can lead to undesirable UE behavior in terms of unnecessary power ramping and transmission counter incrementing.

Alternatives:

Power ramping is not performed due to preamble transmissions that are dropped due to LBT failure

- Alt-1: Yes

- Alt-2: No

| Company(s) | View/position |
| --- | --- |
| Samsung | Need further study between Alt-1 and Alt-2. |
| Ericsson | Alt-1? |
| ZTE | Alt-1 |
| Charter Communications | Alt-1 |
| InterDigital | Alt-1 |
| Intel | Alt-1 |
| Huawei, HiSilicon | Alt-1 |
| Sony | Alt-1 |
| Spreadtrum | Alt-1 |
| OPPO | Alt-1 |
| Ericsson | Alt-1 |
| LG Electronics | Alt-1 |

Proposal:

  - If preamble transmissions are dropped due to LBT failure, then

    - Preamble power ramping is not performed

    - Preamble transmission counter is not incremented

Maximum RAR window size

Description:

Another issue to combat reduced transmission opportunities for Msg 2 is the RAR reception window at the UE. Many companies suggest that the current maximum window size of 10 ms should be extended.

Alternatives:

The maximum RAR window size is extended to beyond 10 ms

- Alt-1: Yes

- Alt-2: No

| Company(s) | View/position |
| --- | --- |
| Sony | Alt-1 |
| Ericsson | Alt-1 |
| Qualcomm | Alt-1 |
| ZTE | Alt-1 |
| Huawei, HiSilicon | Alt-1 |
| Nokia, NSB | Alt-1 |
| Charter Communications | Alt-1 |
| InterDigital | Alt-1 |
| Intel | Alt-1 |
| Samsung | Alt-1 |
| Spreadtrum | Alt-1 |
| OPPO | Alt-1 |
| Ericsson | Alt-1 |
| LG Electronics | Alt-1 |

Proposal:

    - The maximum RAR window size is extended to increase robustness to DL LBT failure

      - FFS: Value of maximum window size

Enhancements in RACH resources

Description:

LBT can also impact the number of  PRACH transmission opportunities. Several companies have put forward proposals on densifying PRACH transmission resources in either frequency or time domain. The enhancements below need not be mutually exclusive.

Alternatives:

- Alt-1: Define multiple PRACH resources in frequency domain

- Alt-2: Support dynamic scheduling of PRACH resources

- Alt-3: Configure multiple RACH occasions per SSB (may already be supported in Rel-15)

- Alt-4: Increase number of PRACH transmission opportunities

| Company(s) | View/position |
| --- | --- |
| Samsung | Alt-3 |
| Ericsson | Our view is that legacy NR PRACH configuration is flexible enough already to suit NR-U operation. Both Alt-1 and Alt-3 are supported already, and are beneficial for NR-U. The PRACH configuration tables include configurations with a large number of PRACH opportunities; we do not need to redesign. |
| Qualcomm | Alt-4 |
| Intel | Alt-1 |
| Huawei, HiSilicon | Alt-2 |
| Motorola Mobility, Lenovo | Alt-1 |
| Spreadtrum | Alt-1 |
| OPPO | Alt-3 |
| InterDigital | Alt-3. (Alt-4 may also be considered) |
| LG Electronics | At least Alt-4 with the clarification that Alt-4 implies increasing the number of PRACH transmission opportunities in time domain |

Proposal: Continue  in

Introduction of 2-step RA

Description:

In the previous meeting, a two-step RA procedure was deemed to potentially have benefit for channel access. Several companies have reiterated the benefits of introducing a two-step procedure wherein a Msg A (Msg 1 + Msg3) is sent in the first step and Msg B (RAR+UL grant) is received from the gNB.

Alternatives:

- Alt-1: Support for two-step RA is introduced in NR-U

- Alt-2: Only four-step RA is supported in NR-U

| Company(s) | View/position |
| --- | --- |
| Samsung | Alt-1 |
| Ericsson | Alt-1 |
| Qualcomm | Alt-1 |
| Huawei, HiSilicon | Alt-1 |
| Intel | Alt-1 |
| MediaTek | Alt-1 |
| Sony | Alt-1 |
| InterDigital | Alt-1 (support both 2-step and 4-step RACH) |
| Charter Communications | Alt-1 |
| ITL | Alt-1 |
| Nokia, NSB | Alt-2? |
| Spreadtrum | Alt-1 |
| OPPO | Alt-1 |
| LG Electronics | Alt-2 as baseline (additionally, Alt-1 if feasibility study in terms of resource efficiency and spec efforts is done and revealed as reasonable) |

Proposal:

    - Two-step RA procedure is supported in NR-U

      - FFS: Detailed design of Msg A and Msg B

      - FFS: If four-step RA is baseline/fallback method

Open issues for RRM and RLM

RSSI reporting

In LTE-LAA, RSSI reporting together with channel occupancy (percentage of time that RSSI was above a threshold) is supported. Such reports are not present in Rel-15 NR, which led to several companies suggesting that NR-U should support RSSI reporting.

Alternatives:

- Alt-1a: Support RSSI measurement reporting

- Alt-1b: Support RSSI measurement and channel occupancy reporting

- Alt-2: RSSI reporting is not introduced

| Company(s) | View/position |
| --- | --- |
| Samsung | Alt-1b |
| Sony | Alt-1b |
| Qualcomm | Alt-1b |
| ZTE | Alt-1b |
| Intel | Alt-1b |
| Spreadtrum | Alt-1b |
| Ericsson | Support RSSI measurement reporting |
| LG Electronics | Alt-1b |

Proposal:

    - RSSI measurement reporting is supported

      - FFS: Definition of RSSI

      - FFS: Channel occupancy reporting

RLM in-sync evaluation

Description:

The location of RLM L1 samples used for in-sync evaluation is critical due to the possibility of gNB LBT failure during scheduled RLM-RS transmission. One option is to at least consider samples coinciding with the SMTC for IS evaluation. Furthermore, several companies propose that an explicit indication be provided if RLM-RS transmissions did not occur due to LBT failure. Below options are not mutually exclusive.

Alternatives:

- Alt-1a: At least RLM-RS within SMTC-U (to be defined for NR-U) are utilized for in-sync evaluations

- Alt-1b: Explicit indication is provided by gNB if RLM-RS transmissions did not occur due to LBT failure

- Alt-1c: The measurement results of RLM-RS can be taken as valid if the RLM-RS is transmitted within a COT

| Company(s) | View/position |
| --- | --- |
| Nokia, NSB | Alt-1a |
| Ericsson | Alt-1a |
| Qualcomm | Alt-1a |
| Vivo | Alt-1b |
| Sony | Alt-1b |
| InterDigital | Alt-1b |
| Spreadtrum | Alt-1b |
| OPPO | Alt-1b or Alt-1c |
| LG Electronics | Alt-1a, We don’t think Alt-1b is applied only for RLM-RS, also applied for DL signal/channels for measurement including RRM measurement. |

Proposal: Continue discussion in RAN1#94

Agreements in RAN1#93 on initial access and mobility

Agreement:

- The following modifications to initial access procedures are beneficial

- Modifications to initial access procedures considering limitations on access to the channel based on LBT

  - Develop techniques to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure

- Enhancement to 4-step RACH

  - Mechanisms to handle reduced Msg 1/2/3/4 transmission opportunities due to LBT failure

- 2-step RACH potentially has benefit for channel access

- Potential modifications to RLM/RRM procedures due to reduced transmission opportunities for DL signals and channels due to LBT failure should be identified and studied.

- Modifications to paging procedures due to reduced transmission opportunities for paging due to LBT failure are beneficial and should be identified and studied.

Agreement:

- NR-U should have a signal that contains at least SS/PBCH block burst set transmission

  - FFS: Other channels and signals transmitted together as part of the signal

- The design of this signal should consider the following characteristics specific to unlicensed band operation

  - There are no gaps within the time span the signal is transmitted at least within a beam

    - FFS: Whether any gaps are needed for beam switching and, if needed, their duration

  - The occupied channel bandwidth is satisfied (although this may not be a requirement)

  - Strive to minimize the channel occupancy time of the signal

  - Characteristics that may facilitate fast channel access

Agreements in RAN2

In RAN2 AH1807 meeting [4], the following agreement was reached for NR-U RA procedure:

Agreement:

- Both CBRA and CFRA are supported on NR-U SpCell and CFRA is supported on NR-U SCells.

- At the first stage, RAR can be transmitted via SpCell

- Assume we Use a predefined HARQ process ID for RAR

- References

- R1-1808336		Considerations on initial access and mobility for NR unlicensed operations	Sony

- R1-1808322		 on initial access and mobility for NR-U	ZTE

- R1-1808275	On initial access procedure in NR-U	MediaTek Inc.

- R1-1808238		 on initial access and mobility procedure for NR-U	vivo

- R1-1808062		Initial access in NR unlicensed	Huawei, HiSilicon

- R1-1808508		Initial access and mobility for NR unlicensed operation	LG Electronics

- R1-1808396		Initial Access and Mobiity for NR Unlicensed Operations	CATT

- R1-1809064		NR-U design for initial access and mobility	AT&T

- R1-1808686		Enhancements to initial access and mobility for NR-unlicensed	Intel Corporation

- R1-1808769		Initial access and mobility procedure for NR-U	Samsung

- R1-1808900		Initial access procedure on NR-U	Guangdong OPPO Mobile Telecom

- R1-1808920		 on initial access for NR-U	Potevio

- R1-1808951		On Initial Access and Mobility for NR-U	Nokia, Nokia Shanghai Bell

- R1-1809315		Diversity in RACH transmissions	Motorola Mobility, Lenovo

- R1-1809326		 on initial access and mobility for NR-U operation	WILUS Inc.

- R1-1809090		On Initial Access and Mobility for NR-U	InterDigital, Inc.

- R1-1809205		On initial access, RRM, mobility and RLM	Ericsson

- R1-1809185		Enhancement of configured grant for NR unlicensed band	ITRI

- R1-1809179		SI Transmission in NR-U	Charter Communications

- R1-1809402		 on initial access in NR-U	KT Corp.

- R1-1809219		Initial access in NR Unlicensed	Xiaomi Technology

- R1-1809480		Initial access and mobility for NR-U	Qualcomm Incorporated

- R1-1809473		Initial access and mobility for NR-U	ITL

- R1-1809492		Initial Access and Diversity in RACH transmissions	Motorola Mobility, Lenovo