3GPP TSG RAN WG1 #94	R1-1809769

Augest 20th  – Augest 24th, 2018

Gothenburg, Sweden

Agenda item:	7.1.3.1

Source: 	Qualcomm Incorporated

Title: 	Summary of GC-PDCCH carrying SFI

Document for:	Discussion and Decision

# Background

This paper summarizes some proposals for GC-PDCCH carrying SFI maintenance for Rel.15 NR.

# Proposals

On SFI field length computation, Samsung proposed In 38.213 Section 11.1.1, changelog2(maxSFIindex) to   log2(maxSFIindex)  when compute the SFI field length. Wilus also pointed out the issue when the maxSFIindex=0 (there is single entry in UE-specific SFI table). Then an more accurate way to compute the length of the field is max⁡(log2(maxSFIindex+1),1).

Proposal: In 38.213 Section 11.1.1, change log2(maxSFIindex) to   max⁡(log2maxSFIindex+1,1) when compute the SFI field length.

- For SFI monitoring skipped by Scell deactivation, DCM proposes to treat this the same as if the SFI is configured but not detected and all RRC configured transmissions/receptions are cancelled. This seems to be the natural interpretation. Not sure if spec changes are needed though.

- Proposal: For SFI monitoring skipped by Scell deactivation, treat this the same as if the SFI is configured but not detected and all RRC configured transmissions/receptions are cancelled.

- On search space configuration of DCI format 2_0, current 38.331 says

dci-Format2-0							SEQUENCE {

nrofCandidates-SFI						SEQUENCE {

aggregationLevel1						ENUMERATED {n1, n2}											OPTIONAL,	-- Need R

aggregationLevel2						ENUMERATED {n1, n2}											OPTIONAL,	-- Need R

aggregationLevel4						ENUMERATED {n1, n2}											OPTIONAL,	-- Need R

aggregationLevel8						ENUMERATED {n1, n2}											OPTIONAL,	-- Need R

aggregationLevel16						ENUMERATED {n1, n2}											OPTIONAL	-- Need R

},

...

- }

- The RAN1 agreement is only support one AL. The 331 is confusing and seems to allow multiple. Need to send LS to RAN2 to suggest change keyword SEQUENCE to CHOICE for nrofCandidates-SFI.

- Proposal: Send LS to RAN2 to suggest change keyword SEQUENCE to CHOICE for nrofCandidates-SFI.

- On RRC configured UL transmission conflicting with DCI granted DL, ZTE proposes:

- When the RRC configured UL transmission which starts before N2 conflicts with DCI scheduled PDSCH/CSI-RS, the gNB can cancel the conflicting part of DCI scheduled PDSCH/CSI-RS transmission and transmit PDSCH/CSI-RS in the remaining part by rate matching.

- When the RRC configured DL reception which starts before N2 conflicts with DCI scheduled PUSCH/SRS, UE can cancel the conflicting part of DCI scheduled PUSCH/SRS transmission and transmit this PUSCH/SRS in the remaining part by rate matching.

- This is DL DCI cancelles UL RRC transmission case. If the DL reception is after N2, the DL can be received. If the DL reception starts before N2, since gNB scheduler is fully aware of the UE capability, it might be more straight-forward to treat this as an error case, instead of introducing another rate matching behaviour.

- Proposal: For DCI granted DL reception or UL transmission conflicting with RRC configured UL transmission and DL reception, do not introduce special rate matching for the DCI granted DL reception and UL transmission. The DCI granted DL reception and UL transmission after N2 will be received/transmitted as granted. The DCI granted DL reception and UL transmission within N2 will be treated as error case

- For RRM measurement vs SFI, Vivo proposed:

- In case intra-frequency neighbor cell SSB based RRM measurement occasion collides with serving cell UL/flexible symbols, following UE behaviors are defined

  - UE cancels RRM measurements when the UE detects a dynamic-SFI indicating at least one of symbols where SSB is transmitted as uplink or flexible, or when UE detects DCI format 0_0/0_1/1_1/2_3 indicating PUSCH/PUCCH/SRS/PRACH transmission in at least one of symbols where SSB is transmitted.

  - When DCI format 2_0 monitoring is not configured and uplink transmission is not scheduled in any symbols of the SSB to be measured, SSB based RRM can still be performed.

- Recommend to discuss this in RRM session.

- On period of semi-static TDD UL/DL configuration, there is a RAN2 LS asking if there is major issue if introducing 3ms or 4ms as new periods. Among the companies provide input in RAN1, E///, Nokia and CATT support while LGE against.

- Proposal: Further discussion if 3ms or 4ms period is needed for semi-static TDD UL/UL configuration, consider RAN2 already agree to support.

- LGE proposes to add a monitoring duration control when configuring the search space of SFI. Recommend to discuss this with general search space configuration, as the SFI search space configuration now is aligned with other search spaces. It may be hard to introduce something special for this search space only.

- Proposal: Further discuss if search space for dynamic SFI can configure monitoring duration

- LGE also proposes for group common DCI, it is assumed that contents are beam agnostic in Rel-15. It is not clear how to make GC-PDCCH not beam agnostic, consider there is no beam index or TCI field in the GC-DCI. Further discussion needed.

- Proposal: Further discuss if there is a need to make the contents of GC-DCI beam agnostic in Rel-15.