3GPP TSG RAN WG1 Meeting #94                                                         	                      R1-1809786

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	ZTE

Title:	Offline summary of transmitter side signal processing schemes for NOMA

Agenda Item:	7.2.1.1

Document for: 	Discussion and Decision

# Introduction

In RAN1#93, it has been agreed that

- Detailed transmission schemes particularly MA signature design per scheme will be captured in TR. Performance and complexity comparisons and observation/conclusion should at least be made scheme-wise.

- Transmitter side data processing for NOMA can be based on one or more of the following aspects

  - UE -specific bit-level scrambling

  - UE -specific bit-level interleaving

  - UE -specific symbol-level spreading

    - Can be with NR legacy modulation or modified modulation

  - UE -specific symbol-level scrambling

  - UE -specific symbol-level interleaving, with symbol-level zero padding

  - UE -specific power assignment

  - UE-specific sparse RE mapping

  - Cell-specific MA signature

  - Multi-branch/MA signature transmission (irrespective of rank) per UE

The intention of this summary is to capture all the solutions on each Tx side design aspect, based on the contributions submitted to RAN1#92, RAN1#94, RAN1#93 and RAN1#94.

# Candidate MA signature designs per transmitter design aspect for NOMA

### Bit level processing

The bit-level processing for NOMA achieves the user separation by randomizing the signals from other users and thus making multi-user interference as random as possible [4][8][11][13][15][16][36][57]. The code rate is comparatively low to get the benefit from the maximum coding gain to overcome the inter-user interference. There are two ways of randomization, i.e. scrambling or interleaving.

#### UE-specific bit-level scrambling:

Bit-level scrambling based NOMA schemes such as LCRS [46] utilize the same transmitter processing procedure as NR PUSCH including channel encoding, rate matching, bit-level scrambling and then modulator, as shown in Figure 1. The main operational difference from current NR PUSCH is that the physical resources mapped to the generated signals are to be shared by multiple users.

Bit level scrambling function defined in TS 38.211 Section 6.3.1.1 [58] can be the MA signature as it is defined in a UE-specific manner.

Figure 1 Transmitter procedure for bit-level processing (modules with dash blocks are optional).

#### UE-specific bit-level interleaving:

Bit-level interleaving based NOMA schemes such as IDMA [51][36] and IGMA [47] share the same transmitter processing at bit-level as in Figure 1. UE-specific interleaving pattern can be the MA signature. For example, the block interleaver has been applied to NR LDPC and NR CCE interleaving process can be reused and changed to UE-specific, i.e. the starting position of reading could be cyclic shifted differently for different UEs as shown in Figure 2 [47][55].

Figure 2 UE specific block interleaver design

### Symbol level processing

#### UE-specific symbol-level spreading with NR legacy modulation.

Symbol-level spreading based NOMA schemes use low cross-correlation or low density spreading sequences as the MA signature, in order to reduce the inter-user interferences at symbol-level[1][2][3][9][10][14][17][18][49][58]. The transmitter processing procedure can be found in Figure 3.

Figure 3 Transmitter procedure for symbol-level spreading with NR legacy modulation

Various designs of symbol-level spreading sequences are proposed for NOMA.

- WBE sequences

In Welch bound equality (WBE) based spreading schemes such as WSMA [22] and RSMA [56], the design metric, for the signature vectors is the total squared cross-correlation Tc≜i,j|siHsj|2. The lower bound on the total squared cross-correlation of any set of K vectors of length N, is K2/N≤Tc. The WBE sequences are designed to meet the bound on the total squared cross-correlations of the vector set with equality BWelch≜K2/N. Some examples of sequence generation method for the construction of WBE spreading sequences can be found in Annex X.1 and Annex X.2.

- Complex-valued sequences with quantized elements

Elements of spreading sequences are complex values which follow the quantized constellations are adopted for MUSA [19]. Example of QPSK and 9-QAM constellations are illustrated in Figure 4. The total available numbers of sequences are 4N and 9N respectively where N is the spreading factor. Usually a subset of sequences should be identified in order to make the allocation and detection of sequences manageable. The selection of sequences should be based on the cross-correlation property, e.g. joint design of quantized constellation and Welch-bound can be considered. Example sets of such sequences can be found in Annex X.3.

(a) 9-QAM                                              (b) QPSK

Figure 4 Constellation of complex-valued spreading sequence.

- ETF/Grassmannian sequences

A stricter Welch-bound equality is to minimize the maximum value of the cross-correlations among any two sequences, which is known as Equiangular Tight Frames (ETF) [22] or Grassmannian sequences [44].

  - Grassmannian Sequence

    - Each complex spreading sequence of this sequence set is generated by Grassmannian line packing problem. Let the Grassmannian sequence set defined by G=s(1)…s(K)=s1(1)…s1(K)⋮⋱⋮sN(1)…sN(K), G ⊂CN×K, where N is the spreading factor and K is the superposition factor. Then, the sequence design problem can be posed in terms of maximizing the minimum chordal distance between sequence pairs: minGmax1≤k<j≤K1-sk*∙sj2, where sk* is the conjugate sequence of sk.

    - Spreading sequence set: G=s(1)…s(K)

  - M-QAM quantized Grassmannian Sequence

    - Each complex coefficient of this sequence (which is generated by the Grassmannian sequence) is quantized by M-QAM constellations. Then, the M-QAM quantized Grassmannian sequence set is defined by G=s(1)…s(K)=s1(1)…s1(K)⋮⋱⋮sN(1)…sN(K), G ⊂CN×K, where N is the spreading factor and K is the superposition factor.

    - Complex coefficient: si(j)=c, i=1≤i≤N, i=1≤j≤K, c∈the set of M-QAM constellations

    - Spreading sequence set: G=s(1)…s(K)

It should be noticed that the existence of an ETF/Grassmannian is not guaranteed for arbitrary combination of N and K. Some example of Grassmannian sequences can be found in Annex X.4.

- GWBE sequences

Generalized welch-bound equality (GWBE) sequences are used as the MA signature for the optimization of cross-correlation under unequal received powers, in the scheme of UGMA [55]. For unequal received powers among different users, the cross-correlations becomes Rx=SHPS, where P=diag{P1,...,PK} is a diagonal matrix whose diagonal elements are received powers of K users. Then, optimal sequences satisfy

minskHsk=1 ∀kRx=SHPSF2=i=1Kj=1KPiPjsiHsj2,

where Pj is the received power of user j. In this case, sequences meeting the equality in generalized welch-bound  i=1Kj=1KPiPjsiHsj2≥k=1KPk2N are optimal, which are called as GWBE sequences. For any given number of users K, spreading factor N and powers {P1,…,PK}, the corresponding GWBE sequences can be generated by the algorithm in Annex X.5.

It should be noted that the elements of GWBE sequences are irregular complex values, which may increase the complexity for hardware implementation. For this end, both the real and imaginary parts of GWBE sequences can be quantized into discrete values, e.g., -1,0,1 or -1,1 or -2, -1, 1, 2 etc. Besides, for equal received power, GWBE sequences reduce to WBE sequences.

- QPSK-based sequence

QPSK-based sequence applied in NOCA [51] which is same as DMRS sequence generation, ru,vn=exp⁡(jφnπ4), 0≤n≤N-1, where φn is given in Annex X.6 for spreading factor equals to 6 and 12 for each root u. The cross-correlations are 0 among sequences with the same root and different cyclic shifts, and 1/N among different root sequences.

- Sparse spreading pattern

Another way to alleviate the inter-user interferences is by using sparse code spreading, where the elements of spreading sequences are picked from [0,1].

- Equal weighted such as SCMA [20]: the number of zeros is equal to that of ones. For example, the sparse patterns of sparse level 50% over the spreading block of size 4 are

1100,0011,1010,0101,1001,0110.

- Unequal weighted such as PDMA [43]: the numbers of zeros and ones are flexible. For example assuming K = 6 and N = 4, an unequal weighted sparse pattern matrix  can be

More unequal weighted pattern matrix can be found in the Annex X.7.

- MUI-qualified sequences

Another sequence generation method is to use the Multi-User Interference (MUI) parameter as a criterion [49]. Details can be found in Annex X.8.

#### UE-specific symbol-level spreading with modified modulation:

Figure 5 Transmitter procedure for symbol-level spreading with modified modulation

Joint spreading and modulation is proposed for SCMA [39]. The symbol-level dependency is attributed to the flexibility to optimize the constellations across multiple symbols. By adjusting the labelling of the input binary bits to the constellation points for each resource element, a NoMA transmitter can optimize the overall distances (Euclidean/product).

The M-bit to N-symbol mapping of multi-dimensional modulation can be represented by a N×2M table in which each column represents the symbol sequence in term of an index of the input bit stream. The same mapping function can also be presented by a formula expressing the relation between the input bit stream b and the output symbol sequence x. For example, the formula of 8-point table is

x=23-12j0-120j(1-2b).

More bits-to-symbols mappings with different input bit length can be found in Annex X.9.

For further user separation, more choices of the joint mapping sequences for a given bit stream (equivalently more M-bit to N-symbol mapping matrices) can be generated by casting some N-by-N transform matrix. For instance of N = 2, this UE-specific 2-by-2 transform matrix can be one of

1001, 100-1, 100j, 100-j.

It can be found below that the multi-dimensional modulation is equivalent to multi-branch with branch-specific symbol-level spreading, without changing NR legacy modulation. This corresponds to the type of multi-branch transmission illustrated in Figure 9 (b) Actually the above formula of 8-point table consists of two part, i.e. 1-2b denotes BPSK modulation and the matrix 23-12j0-120j represents a linear superposition. The detailed illustration of the equivalence can be found in Annex X-10.

#### UE-specific symbol-level scrambling:

Figure 6 Transmitter procedure for hybrid symbol-level spreading and scrambling

Schems like RSMA [56] use hybrid short code spreading and long code scrambling as the MA signatures. The use of long scrambling sequence is helpful in PAPR reduction. Besides, the use of different scrambling sequences across adjacent cells can reduce the inter-cell interference. Both features are desirable to enhance link budget and system capacity.

The generation of scrambling sequences can be UE-group and/or cell specific. The sequences used for scrambling code can down select from Gold sequences, Zadoff-Chu sequences, or a combination of the two, according to 3GPP TS 38.211 [59].

It should be noticed that the effect of hybrid spreading and scrambling may be as same as symbol-level spreading where each modulated symbol is spread with different spreading sequence [51].

#### UE-specific symbol-level interleaving with symbol-level zero padding:

Figure 7 Transmitter procedure for hybrid symbol-level spreading and scrambling

In the proposed IGMA design [47], the symbol level operation is the grid mapping process which can be applied with the robustness to inter-cell interference and reduction of detection complexity. More specifically, it consists of zero padding and symbol level interleaving process, where in the end the sparse symbol-to-RE mapping can be achieved. The sparsity through grid mapping can be configurable. An exemplary interleaving with zero padding for sparse mapping is given below.

Figure 8 Illustration of symbol-level interleaving with zero padding

### UE-specific sparse RE mapping

Sparse repetition mapping comes up with the optimal tradeoff between the coding gain and multi-user interference mitigation capability by adjusting the size of mapping block, and the UE-specific sparsity levels and patterns.

It should be noticed that sparse mapping can be done by ether RE mapping or designing spreading sequences as shown in Section 2.2.1 [40][41][46][50].

### Multi-branch transmission per UE

Multi-branch transmission per user can be considered in order to achieve high per-user spectral efficiency [39][40][47][55][56].

This Multi-branch processing could be a common feature to all the above-mentioned design aspects, i.e. multi-branch transmission can be operated before FEC, at bit level or at symbol level, as shown in Figure 9. UE-specific MA signature may be replaced by branch-specific MA signature if multi-branch transmission is applied, and these branch-specific MA signatures could be either orthogonal, non-orthogonal or share the same MA signature.

- Multi-branch transmission before FEC

- Multi-branch transmission at bit-level

- Multi-branch transmission at symbol-level

Figure 9 Different operation modes for multi-branch transmission

### UE/layer-specific power assignment

Power assignment per user/layer can be considered. This could be a common feature to all the above-mentioned design aspects. In fact, some of the designs, such as GWBE sequences [55] and multi-branch transmission [40][56], take into account the effect of power assignment in the design of MA signatures.

# Detailed descriptions of NOMA schemes

Note: strive to use the MA signatures listed in Section 2 to describe the scheme details with additional information associated with each scheme. Whether it is captured in the main text or the appendix of TR is up to the editor.

### Symbol level processing based

#### Scheme A: MUSA, based on complex-valued sequences with quantized elements

MA signatures are described in Section 2.2.1 (2).

In the case of DFT-s-OFDM waveform, time-domain spreading is used. In the case of multi-branch transmission, Figure 9 (c) is considered.

#### Scheme B: …

For example, MA signatures are described in Section 2.2.1 (y).

Additional description of the scheme in certain scenarios/operations….

…

### Bit level processing based

#### Scheme H:…

For example, MA signatures are described in Section  2.1.1.

Additional description of the scheme in certain scenarios/operations.

#### Scheme I:…

For example, MA signatures are described in Section 2.1.2.

Additional description of the scheme in certain scenarios/operations….

…

### Hybrid bit and symbol-level processing based

#### Scheme X

For example, MA signatures are described in Section  2.1.x and Section 2.2.1 (y).

Additional description of the scheme in certain scenarios/operations.

…

# Proposal

Proposal 1:

- Capture Section 2, 3 and Annex in TR38.812.

  - Companies are encouraged to check the exact content of Section 2.

  - Detailed description of each scheme in Section 3 should be provided by email discussion before Aug. 31.

# References

- R1-1803615	Key processing modules at transmitter side for NOMA	ZTE, Sanechips

- R1-1803663	Discussion on the design of SCMA	Huawei, HiSilicon

- R1-1803770	PDMA transmitter side signal processing	CATT

- R1-1803850	Discussion on NOMA transmitters	vivo

- R1-1803953	Considerations on NOMA schemes	Google, Inc.

- R1-1804107	Considerations on Rel-15 NoMA transmitter design	CMCC

- R1-1804116	Discussion on NoMA Schemes for NR	NEC

- R1-1804396	Transmitter side signal processing schemes for NOMA	Samsung

- R1-1804462	Considerations on NOMA transmitter	Nokia, Nokia Shanghai Bell

- R1-1804573	Transmitter side signal processing schemes for NCMA	LG Electronics

- R1-1804601	Consideration on the transmitter side signal processing for NOMA	Sony

- R1-1804667	On Transnmitter side processing for NOMA	AT&T

- R1-1804743	NOMA transmission scheme: LCRS	Intel Corporation

- R1-1804823	Transmitter side signaling processing schemes for NOMA	Qualcomm Incorporated

- R1-1804859	Implementation Aspects of Low Code Rate NOMA Schemes 	InterDigital, Inc.

- R1-1804941	Tx side processing for UL NOMA	NICT

- R1-1805003	Signature design for NoMA	Ericsson

- R1-1805065	Transmitter design for uplink NOMA	NTT DOCOMO, INC.

- R1-1805840	Key processing modules at transmitter side for NOMA	ZTE

- R1-1805907	Discussion on the design of SCMA	Huawei, HiSilicon

- R1-1806079	Discussion on NOMA transmitters	vivo

- R1-1806241	Signature design for NoMA	Ericsson

- R1-1806305	NOMA transmitter side signal processing	CATT

- R1-1806372	Considerations on Rel-15 NoMA Transmitter Design	CMCC

- R1-1806469	Discussion on NoMA Schemes for NR	NEC

- R1-1806531	NOMA transmission scheme: LCRS	Intel Corporation

- R1-1806565	Consideration on the transmitter side signal processing for NOMA	Sony

- R1-1806635	Transmitter side signal processing schemes for NCMA	LG Electronics

- R1-1806751	Transmitter side signal processing schemes for NOMA	Samsung

- R1-1806876	TX side processing for UL NOMA	NICT

- R1-1806930	Considerations on NOMA transmitter	Nokia, Nokia Shanghai Bell

- R1-1806979	On Transmitter Side signal Processing for NOMA	AT&T

- R1-1807021	Implementation Aspects of Low Code Rate NOMA Schemes 	InterDigital, Inc.

- R1-1807073	Transmitter design of UGMA	NTT DOCOMO, INC.

- R1-1807167	Discussion on NOMA schemes	Google Inc.

- R1-1807223	Design Considerations for NOMA Transmitter 	Convida Wireless LLC

- R1-1807290	On the NOMA schemes	KT Corp.

- R1-1807377	Transmitter side signaling processing schemes for NOMA	Qualcomm Incorporated

- R1-1808049	Discussion on the design of SCMA	Huawei, HiSilicon

- R1-1808151	Transmitter side designs for NOMA	ZTE

- R1-1808230	Discussion on NOMA transmitters	vivo

- R1-1808331	Consideration on the transmitter side signal processing for NOMA	Sony

- R1-1808386	NOMA transmitter side signal processing	CATT

- R1-1808499	Transmitter side signal processing schemes for NCMA	LG Electronics

- R1-1808563	Discussion on NOMA Schemes for NR	NEC

- R1-1808677	NOMA transmission scheme	Intel Corporation

- R1-1808760	Transmitter side signal processing schemes for NOMA	Samsung

- R1-1808834	Discussion on NoMA transmitter schemes	CMCC

- R1-1808948	New Coding Scheme for NOMA with Traffic Control	Sequans Communications

- R1-1808955	Discussion on general symbol-to-resource mapping for NR-NOMA	Fraunhofer HHI, Fraunhofer IIS

- R1-1808968	Considerations on NOMA Transmitter	Nokia, Nokia Shanghai Bell

- R1-1808975	TX schemes for NoMA	Ericsson

- R1-1809056	On Transmitter Side signal Processing for NOMA	AT&T

- R1-1809078	Discussion on Transmit Timing Offset for NOMA Evaluation 	InterDigital, Inc.

- R1-1809148	Transmitter design for uplink NOMA	NTT DOCOMO, INC.

- R1-1809434	Transmitter side signaling processing schemes for NOMA	Qualcomm Incorporated

- R1-1804169 CMA-OFDM with NR LDPC code for Uplink NOMA	HUGHES

- R1-1802069 Signature generation and structure of LSSA 	ETRI

- 3GPP TS38.211, Physical channels and modulation (Release 15)

## Annex X: MA signature deisgns for NOMA schemes

X.1 Generation method for the construction of WBE spreading sequences [17]

For a user k, let bk be the transmitted symbol that modulates a unit norm SS vector sk. The Additive White Gaussian Noise (AWGN) signal model may be given as y=Sb+z, where z is the zero-mean AWGN vector with a covariance matrix I, i.e., an Identity matrix. the overall SS matrix with a CW in each of its columns is S, the transmit symbol vector is b.  The transmit power of each user is set to unity, so the power control problem is not addressed here. A unit norm temporal receive filter fk, such as a Matched Filter (MF) or a linear Minimum Mean Squared Error (MMSE) filter, may be employed by the receiver to obtain an estimate bk for the transmitted symbol bk. The post processed SINR γk of a user k is given as

γk=fkHsk2j=1,j≠kKfkHsj2+fkHz2=fkH(skskH)fkfkH(j=1,j≠kKsjsjH+I)fkfk=sk1traceSST2-1+νk,

where trace∙ is the trace operator, νk is the noise component in the SINR γk. The trace∙ term in the denominator is the TSC, which also contains the desired unit signal power. So an additional unity term arises in the denominator. If the post processed noise is white, i.e., the noise power of each νk is the same, then the TSC can directly be used as a PI.

From the center part of the SINR equation, let Rk=(j=1,j≠kKsjsjH+I), which is the correlation matrix of the interference plus noise. It can be identified that minimizing the denominator (or equivalently maximizing γk) is a well known Rayleigh-Quotient problem. From this, the Eigen vector corresponding to the minimum Eigen value of Rk may be considered as CW for UE k, if it is assumed that fk is matched to sk.

The fixed-point iterations start from the users choosing a random CW. In a given sequential user order, say k=1,2,…K, each user updates its SS sk by solving the Eigen value problem while other SS, sj, j≠k, are kept fixed, i.e. After user k, the next user updates it’s CW in the same way by assuming the other CWs to be fixed. The iterations progress up to the final user in the order, such that in each iteration there are K updates, one for each CW in S. After the final update in the given iteration, the first user in the order restarts the updates. This repeats until convergence.

Again, from the center part of the SINR equation, the solution to fk can also be identified as the well known Generalized Eigen Value Problem (GEVP), i.e., finding a common Eigen value for the matrix pair (I,Rk). The solution to which is the linear MMSE vector given as Rk-1skskHRk-2sk1/2, in its normalized form. Sequential iterations as mentioned before can be used, except that instead of solving the Eigen value problem, the normalized linear MMSE expression is used during updates. For this SINR maximization problem (or TSC minimization), the obtained solution to S from both the MMSE IA iterations and the Eigen vector IA iterations is the same fixed-point.

A Kronecker product based approach may be employed to obtained (or construct) higher dimensional WBE SS, i.e., higher N values, from lower dimensional WBE SS.

X.2 WBE based on modified Chirp sequence [56]

Assume the spreading factor is K and the number of distinct spreading codes is N. Then the n-th spreading code can be denoted by

Sn≜sn1  sn2      ⋯    snK ;     1≤n≤N,  N>K≥2.    			 (1)

One example of closed-form construction would be

snk≜1Kexpjπk+n2Nwk;       1≤k≤K,  1≤n≤N          	      (2)

where w(k) is a perfect sequence of period K, that is

k=1Kwkw*k+l=Kδl.	           					(3)

It can be shown that the spreading code generated above is a WBE set, which achieves the WB on sum squared correlations for arbitrary K and N satisfying N>K≥2.

X.3 Examples of complex-valued sequences with quantized elements sequences [1]

Table X.3-1 Example of MUSA sequences with SF = 2, pool size = 6 (before normalization).

| No. | c1 | c2 |
| --- | --- | --- |
| 1 | 1 | 1 |
| 2 | 1 | -1 |
| 3 | 1 | j |
| 4 | 1 | -j |
| 5 | 1 | 0 |
| 6 | 0 | 1 |

Table X.3-2 MUSA sequences for SF = 3:

Table X.3-3 Example of MUSA sequence with SF = 4, pool size = 64 (before normalization).

| No. | c1 | c2 | c3 | c4 | No. | c1 | c2 | c3 | c4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 33 | 1 | 1 | 1 | -j |
| 2 | 1 | 1 | -1 | -1 | 34 | 1 | 1 | -1 | j |
| 3 | 1 | -1 | 1 | -1 | 35 | 1 | -1 | 1 | j |
| 4 | 1 | -1 | -1 | 1 | 36 | 1 | -1 | -1 | -j |
| 5 | 1 | 1 | -j | j | 37 | 1 | 1 | -j | 1 |
| 6 | 1 | 1 | j | -j | 38 | 1 | 1 | j | -1 |
| 7 | 1 | -1 | -j | -j | 39 | 1 | -1 | -j | -1 |
| 8 | 1 | -1 | j | j | 40 | 1 | -1 | j | 1 |
| 9 | 1 | -j | 1 | j | 41 | 1 | -j | 1 | 1 |
| 10 | 1 | -j | -1 | -j | 42 | 1 | -j | -1 | -1 |
| 11 | 1 | j | 1 | -j | 43 | 1 | j | 1 | -1 |
| 12 | 1 | j | -1 | j | 44 | 1 | j | -1 | 1 |
| 13 | 1 | -j | -j | -1 | 45 | 1 | -j | -j | j |
| 14 | 1 | -j | j | 1 | 46 | 1 | -j | j | -j |
| 15 | 1 | j | -j | 1 | 47 | 1 | j | -j | -j |
| 16 | 1 | j | j | -1 | 48 | 1 | j | j | j |
| 17 | 1 | 1 | 1 | -1 | 49 | 1 | 1 | 1 | j |
| 18 | 1 | 1 | -1 | 1 | 50 | 1 | 1 | -1 | -j |
| 19 | 1 | -1 | 1 | 1 | 51 | 1 | -1 | 1 | -j |
| 20 | 1 | -1 | -1 | -1 | 52 | 1 | -1 | -1 | j |
| 21 | 1 | 1 | -j | -j | 53 | 1 | 1 | -j | -1 |
| 22 | 1 | 1 | j | j | 54 | 1 | 1 | j | 1 |
| 23 | 1 | -1 | -j | j | 55 | 1 | -1 | -j | 1 |
| 24 | 1 | -1 | j | -j | 56 | 1 | -1 | j | -1 |
| 25 | 1 | -j | 1 | -j | 57 | 1 | -j | 1 | -1 |
| 26 | 1 | -j | -1 | j | 58 | 1 | -j | -1 | 1 |
| 27 | 1 | j | 1 | j | 59 | 1 | j | 1 | 1 |
| 28 | 1 | j | -1 | -j | 60 | 1 | j | -1 | -1 |
| 29 | 1 | -j | -j | 1 | 61 | 1 | -j | -j | -j |
| 30 | 1 | -j | j | -1 | 62 | 1 | -j | j | j |
| 31 | 1 | j | -j | -1 | 63 | 1 | j | -j | j |
| 32 | 1 | j | j | 1 | 64 | 1 | j | j | -j |

Table X.3-4 Example of BPSK or {+1/-1} sequence with SF = 6, pool size = 16 (before normalization).

| No. | c1 | c2 | c3 | c4 | c5 | c6 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 1 | 1 | 1 | 1 | -1 | -1 |
| 3 | 1 | 1 | 1 | -1 | 1 | -1 |
| 4 | 1 | 1 | 1 | -1 | -1 | 1 |
| 5 | 1 | 1 | -1 | 1 | 1 | -1 |
| 6 | 1 | 1 | -1 | 1 | -1 | 1 |
| 7 | 1 | 1 | -1 | -1 | 1 | 1 |
| 8 | 1 | 1 | -1 | -1 | -1 | -1 |
| 9 | 1 | -1 | 1 | 1 | 1 | -1 |
| 10 | 1 | -1 | 1 | 1 | -1 | 1 |
| 11 | 1 | -1 | 1 | -1 | 1 | 1 |
| 12 | 1 | -1 | 1 | -1 | -1 | -1 |
| 13 | 1 | -1 | -1 | 1 | 1 | 1 |
| 14 | 1 | -1 | -1 | 1 | -1 | -1 |
| 15 | 1 | -1 | -1 | -1 | 1 | -1 |
| 16 | 1 | -1 | -1 | -1 | -1 | 1 |

X.4 Examples of Grassmannian sequences [10]

Examples of 64QAM-quantized Grassmannian Sequence based spreading codebook for minimizing the MUI are represented as follows:

Table X.4-1 64QAM-quantized Grassmannian Sequence based codebook for Spreading Factor: N = 2

| # of codewords (Max. # of users: K) | Examples of spreading codebook c1⋯cK |
| --- | --- |
| 2 | 5+5i5+5i5+5i-5-5i*Pno,2,2 |
| 4 | -3+5i-3+7i3+7i-3+1i-1-3i-7-3i7-3i5+3i*Pno,2,4 |
| 6 | -3+1i-7-3i7-3i-5+3i-1-5i-5+3i-1-7i-5-5i-1-7i-7+1i-1-1i3-3i*Pno,2,6 |
| 8 | -3-7i-7-1i-1-3i1-5i-3-1i-1-5i-1+7i-5-7i-3-1i-7+1i1-7i3+1i-3-7i-1+5i-5+1i-1-7i*Pno,2,8 |

NOTE: Pno,N,K is (K×K) normalized matrix for the power constraints, Pno,N,K=Pno,1   0      0   Pno,2   0      0      0      0      0      0      0      0      ⋱      0      0   Pno,K. Here, Pno,k=1ck×N, for k=1, …, K.

Table X.4-2. 64QAM-quantized Grassmannian Sequence based codebook for Spreading Factor: N = 4

| # of codewords (Max. # of users: K) | Examples of spreading codebook c1⋯cK |
| --- | --- |
| 4 | 5+5i5+5i5+5i-5-5i5+5i5+5i5+5i-5-5i5+5i5+5i5+5i-5-5i-5-5i-5-5i-5-5i5+5i*Pno,4,4 |
| 6 | -7+3i-3+1i-5+1i-1-5i3+1i-1-7i1-7i-7+3i-1-7i-1+1i-3+1i3+5i-1-3i-1+7i-3+1i-5+5i-7+5i-3+1i-3-7i5+7i3+3i5-7i-1+3i1+3i*Pno,4,6 |
| 8 | -5-3i-7+1i1-5i1+3i-1-3i-7-3i-7+7i5-1i-1-3i-5-3i7-3i3+1i-5-3i-7+7i-1+5i-5+5i-7-3i1-7i1-3i1-7i-1+3i7+1i-5+7i-3+5i-5+7i-3-5i1+1i-7+7i-5+3i-1-1i7-5i3+1i*Pno,4,8 |

NOTE: Pno,N,K is (K×K) normalized matrix for the power constraints, Pno,N,K=Pno,1   0      0   Pno,2   0      0      0      0      0      0      0      0      ⋱      0      0   Pno,K. Here, Pno,k=1ck×N, for k=1, …, K.

X.5 Algorithms of constructing GWBE sequences [55]

Table X-5 Algorithms of constructing GWBE sequences S  for any spreading factor N, number of users K, received powers P1,…,PK

1: Find the set of oversized users K satisfying Pk>i=1KPi∙signPi>PkN-i=1KsignPi≥Pk for k∈K

2: Construct a matrix Q∈CK-K×K-K with diagonal elements Pi¦i∉K and eigenvalues i∉KPiN-K1N-KT,0K-N×1TT with “Generalized Chan-Li” or “Generalized Bendel-Mickey” algorithms in [15]

3: Decompose Q=UVUH, where U is the matrix of eigenvectors

4: Denote the eigenvectors in U corresponding to the non-zero eigenvalues as U∈CK-K×K-K and the non-zero eigenvalues as Λ=i∉KPiN-KIN-K

5: Construct sequences S=ΛUP-12, where P=diag{Pi¦i∈K}

6: Construct S=CorthIK0K×K-K0N-K×KS, where Corth∈CN×N is any orthogonal matrix satisfying CorthCorthH=IN.

X.6 Examples of QPSK based sequences [9]

| Table X.6-1: for SF=6 0; -1; -3; 3; -3; 3; -3. 1; -1; 3; -1; 1; 1; 1. 2; 3; -1; -3; -3; 1; 3. 3; 3; -1; -1; 1; -1; -1. 4; -1; -1; -3; 1; -3; -1. 5; 1; 3; -3; -1; -3; 3. 6; -3; 3; -1; -1; 1; -3. 7; -1; -3; -3; 1; 3; 3. 8; 3; -1; -1; 3; 1; 3. 9; 3; -3; 3; 1; -1; 1. 10; -3; 1; -3; -3; -3; -3. 11; -3; -3; -3; 1; -3; -3. 12; 3; -3; 1; -1; -3; -3. 13; 3; -3; 3; -1; -1; -3. 14; 3; -1; 1; 3; 3; 1. 15; -1; 1; -1; -3; 1; 1. 16; -3; -1; -3; -1; 3; 3. 17; 1; -1; 3; -3; 3; 3. 18; 1; 3; 1; 1; -3; 3. 19; -1; -3; -1; -1; 3; -3. 20; 3; -1; -3; -1; -1; -3. 21; 3; 1; 3; -3; -3; 1. 22; 1; 3; -1; -1; 1; -1. 23; -3; 1; -3; 3; 3; 3. 24; 1; 3; -3; 3; -3; 3. 25; -1; -1; 1; -3; 1; -1. 26; 1; -3; -1; -1; 3; 1. 27; -3; -1; -1; 3; 1; 1. 28; -1; 3; -3; -3; -3; 3. 29; 3; 1; -1; 1; 3; 1. | Table X.6-2: for SF=12. 0; -1; 1; 3; -3; 3; 3; 1; 1; 3; 1; -3; 3. 1; 1; 1; 3; 3; 3; -1; 1; -3; -3; 1; -3; 3. 2; 1; 1; -3; -3; -3; -1; -3; -3; 1; -3; 1; -1. 3; -1; 1; 1; 1; 1; -1; -3; -3; 1; -3; 3; -1. 4; -1; 3; 1; -1; 1; -1; -3; -1; 1; -1; 1; 3. 5; 1; -3; 3; -1; -1; 1; 1; -1; -1; 3; -3; 1. 6; -1; 3; -3; -3; -3; 3; 1; -1; 3; 3; -3; 1. 7; -3; -1; -1; -1; 1; -3; 3; -1; 1; -3; 3; 1. 8; 1; -3; 3; 1; -1; -1; -1; 1; 1; 3; -1; 1. 9; 1; -3; -1; 3; 3; -1; -3; 1; 1; 1; 1; 1. 10; -1; 3; -1; 1; 1; -3; -3; -1; -3; -3; 3; -1. 11; 3; 1; -1; -1; 3; 3; -3; 1; 3; 1; 3; 3. 12; 1; -3; 1; 1; -3; 1; 1; 1; -3; -3; -3; 1. 13; 3; 3; -3; 3; -3; 1; 1; 3; -1; -3; 3; 3. 14; -3; 1; -1; -3; -1; 3; 1; 3; 3; 3; -1; 1. 15; 3; -1; 1; -3; -1; -1; 1; 1; 3; 1; -1; -3. 16; 1; 3; 1; -1; 1; 3; 3; 3; -1; -1; 3; -1. 17; -3; 1; 1; 3; -3; 3; -3; -3; 3; 1; 3; -1. 18; -3; 3; 1; 1; -3; 1; -3; -3; -1; -1; 1; -3. 19; -1; 3; 1; 3; 1; -1; -1; 3; -3; -1; -3; -1. 20; -1; -3; 1; 1; 1; 1; 3; 1; -1; 1; -3; -1. 21; -1; 3; -1; 1; -3; -3; -3; -3; -3; 1; -1; -3. 22; 1; 1; -3; -3; -3; -3; -1; 3; -3; 1; -3; 3. 23; 1; 1; -1; -3; -1; -3; 1; -1; 1; 3; -1; 1. 24; 1; 1; 3; 1; 3; 3; -1; 1; -1; -3; -3; 1. 25; 1; -3; 3; 3; 1; 3; 3; 1; -3; -1; -1; 3. 26; 1; 3; -3; -3; 3; -3; 1; -1; -1; 3; -1; -3. 27; -3; -1; -3; -1; -3; 3; 1; -1; 1; 3; -3; -3. 28; -1; 3; -3; 3; -1; 3; 3; -3; 3; 3; -1; -1. 29; 3; -3; -3; -1; -1; -3; -1; 3; -3; 3; 1; -1. |
| --- | --- |

X.7 Examples of unequal weighted sparse spreading pattern [9]

Table X.7: Examples of different sparse spreading pattern

| OF | Type | Sparse spreading pattern |
| --- | --- | --- |
| 150% | Type 1 |  |
|  | Type 2 |  |
| 200% | Type 1 |  |
|  | Type 2 |  |
| 300% | Type 1 |  |
|  | Type 2 |  |

X.8 MUI based sequence generation method [49]

MUI is calculated using Equation 1. Where M represents the number of code words in a pool, vi is a single code word that MUI parameter is evaluated toward. N is the length of the codeword.

MUIi=N ∙Mj=0,j≠iM-1vi,vj                                                        Eq. (X.1)

To create a spreading code, it is first required to set the length of the spreading code. For an N-length code, we can create N orthogonal sequences. This will span the space of the spreading code. We will generate the code using complex Hadamard codes in normalized de-phased form. The construction equation is showed as Eq. (X.2) below. Examples for orthogonal complex Hadamard codes can be found in the appendix.

FN=ejk2πN                                                                 Eq. (X.2)

After we create the orthogonal base, we start the creation of the non-orthogonal set.

First, we construct the hereby defined hyperplanes – we choose pairs of orthogonal vectors from the base set FN. Each pair of orthogonal vectors defines a hyperplane. Therefore, for a set of N vectors we can define K≡N2 orthogonal hyperplanes, as seen in Eq. (X.3).

K=N2=N!2!N-2!=N2∙(N-1)                                            Eq. (X.3)

Second, we construct non-orthogonal vectors in each hyperplane. To do so, we need to decide how many non-orthogonal vectors we would like to create. Considering K hyperplanes, N orthogonal sequences and M required sequence pool size, the number of non-orthogonal vectors per hyperplanes L is calculated by Eq. (X.4). Looking at Eq. (X.4) it is seen that there is a flexibility at the size of the sequence pool that is dependent on L.

L=M-NK                                                                    Eq. (X.4)

To generate the new sequences, we will generate linear combination in each hyperplane by setting minimum cross correlation between the new vector and its building hyperplanes. This is called the constraint of least projection which is defined by minA,B.

Let us see an example of such an operation. Assume two orthogonal vectors A,B defined as in Eq. (X.5). It is possible to see that using α and β we create a linear combination to generate the new spreading sequence. To generate α and β we use θs≔π*s2(L+1) for s={1,…,L} so that: αs=cos⁡(θs) and βs=sinθs.

A=a1+a2+…+aN 
B=b1+b2+…+bN
Vnon-orthogonals=αs∙A+i∙βs∙B                                         Eq. (X.5)

Last, we assign the MUI quality figure for each sequence by using Eq. (X.1). We will also look at the standard deviation as a comparison tool. An example sequence can be viewd in table X.8-1.

Table X.8-1 - N=4, L=2 proposed sequence pool

| a1 | a2 | a3 | a4 |
| --- | --- | --- | --- |
| 1 | 1 | 1 | 1 |
| 1 | 0+1i | -1 | -0-1i |
| 1 | -1 | 1 | -1 |
| 1 | -0-1i | -1 | 0+1i |
| 1.366 | 0.866+0.5i | 0.366 | 0.866-0.5i |
| 1.366 | 0.5+0.866i | -0.366 | 0.5-0.866i |
| 1.366 | 0.366 | 1.366 | 0.366 |
| 1.366 | -0.366 | 1.366 | -0.366 |
| 1.366 | 0.866-0.5i | 0.366 | 0.866+0.5i |
| 1.366 | 0.5-0.866i | -0.366 | 0.5+0.866i |
| 1.366 | -0.5+0.866i | -0.366 | -0.5-0.866i |
| 1.366 | -0.866+0.5i | 0.366 | -0.866-0.5i |
| 1.366 | -0+0.366i | -1.366 | -0-0.366i |
| 1.366 | -0-0.366i | -1.366 | -0+0.366i |
| 1.366 | -0.866-0.5i | 0.366 | -0.866+0.5i |
| 1.366 | -0.5-0.866i | -0.366 | -0.5+0.866i |

X.9 Examples of bit-to-symbol mapping pattern for symbol-level spreading with modified modulation [39]

Table X.9-1. Mapping function for the 8-point modulated symbol sequence of length 2

| Sequence index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Corresponding bit sequence | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| Output symbol sequence | -a8+jb8-a8+jb8 | a8+jb8a8+jb8 | -a8-jb8-a8+jb8 | a8-jb8a8+jb8 | -a8+jb8-a8-jb8 | a8+jb8a8-jb8 | -a8-jb8-a8-jb8 | a8-jb8a8-jb8 |

with a8=0.5774 and b8=0.8165.

Table X.9-2. Mapping function for the 16-point modulated symbol sequence of length 2

| Sequence index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Corresponding bit sequence | 0000 | 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 |
| Output Symbol sequence | 3+3j1+j | -1+3j3+j | 1+3j-3+j | -3+3j-1+j | 3-j1+3j | -1-j3+3j | 1-j-3+3j | -3-j-1+3j |
| Sequence index | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| Corresponding bit sequence | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
| Output Symbol sequence | 3+j1-3j | -1+j3-3j | 1+j-3-3j | -3+j-1-3j | 3-3j1-j | -1-3j3-j | 1-3j-3-j | -3-3j-1-j |

Table X.9-3. Mapping function for the 64-point modulated symbol sequence of length 2

| Sequence index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Corresponding bit sequence | 000 000 | 000 001 | 000 010 | 000 011 | 000 100 | 000 101 | 000 110 | 000 111 |
| Output symbol sequence | 1+j1+j | 1+3j1+3j | 1-j1+j | 1-3j1+3j | 3+j3+j | 3+3j3+3j | 3-j3+j | 3-3j3+3j |
| Sequence index | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| Corresponding bit sequence | 001 000 | 001 001 | 001 010 | 001 011 | 001 100 | 001 101 | 001 110 | 001 111 |
| Output symbol sequence | -1+j1+j | -1+3j1+3j | -1-j1+j | -1-3j1+3j | -3+j3+j | -3+3j3+3j | -3-j3+j | -3-3j3+3j |
| Sequence index | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| Corresponding bit sequence | 010 000 | 010 001 | 010 010 | 010 011 | 010 100 | 010 101 | 010 110 | 010 111 |
| Output symbol sequence | 1+j1-j | 1+3j1-3j | 1-j1-j | 1-3j1-3j | 3+j3-j | 3+3j3-3j | 3-j3-j | 3-3j3-3j |
| Sequence index | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 |
| Corresponding bit sequence | 011 000 | 011 001 | 011 010 | 011 011 | 011 100 | 011 101 | 011 110 | 011 111 |
| Output symbol sequence | -1+j1-j | -1+3j1-3j | -1-j1-j | -1-3j1-3j | -3+j3-j | -3+3j3-3j | -3-j3-j | -3-3j3-3j |
| Sequence index | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 |
| Corresponding bit sequence | 100 000 | 100 001 | 100 010 | 100 011 | 100 100 | 100 101 | 100 110 | 100 111 |
| Output Symbol sequence | 1+j-1+j | 1+3j-1+3j | 1-j-1+j | 1-3j-1+3j | 3+j-3+j | 3+3j-3+3j | 3-j-3+j | 3-3j-3+3j |
| Sequence index | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 |
| Corresponding bit sequence | 101 000 | 101 001 | 101 010 | 101 011 | 101 100 | 101 101 | 101 110 | 101 111 |
| Output Symbol sequence | -1+j-1+j | -1+3j-1+3j | -1-j-1+j | -1-3j-1+3j | -3+j-3+j | -3+3j-3+3j | -3-j-3+j | -3-3j-3+3j |
| Sequence index | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 |
| Corresponding bit sequence | 110 000 | 110 001 | 110 010 | 110 011 | 110 100 | 110 101 | 110 110 | 110 111 |
| Output Symbol sequence | 1+j-1-j | 1+3j-1-3j | 1-j-1-j | 1-3j-1-3j | 3+j-3-j | 3+3j-3-3j | 3-j-3-j | 3-3j-3-3j |
| Sequence index | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 |
| Corresponding bit sequence | 111 000 | 111 001 | 111 010 | 111 011 | 111 100 | 111 101 | 111 110 | 111 111 |
| Output Symbol sequence | -1+j-1-j | -1+3j-1-3j | -1-j-1-j | -1-3j-1-3j | -3+j-3-j | -3+3j-3-3j | -3-j-3-j | -3-3j-3-3j |

Table X.9-4. Three mapping functions with 4-point low PAPR modulated symbol of length 4 
(no sparsity is applied, only used when transform precoding is enabled in coverage limited case)

| Sequence index | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Corresponding bit sequence | 00 | 01 | 10 | 11 |
| Output symbol sequence | 1111 | 1-j-1j | -1j1-j | -1-1-1-1 |
| Sequence index | 1 | 2 | 3 | 4 |
| Corresponding bit sequence | 00 | 01 | 10 | 11 |
| Output symbol sequence | 1111 | 1j-1-j | -1-j1j | -1-1-1-1 |
| Sequence index | 1 | 2 | 3 | 4 |
| Corresponding bit sequence | 00 | 01 | 10 | 11 |
| Output symbol sequence | 1j-1-j | 1-j-1j | -1j1-j | -1-j1j |

X.10 Implementation of multi-dimensional modulation by multi-branch transmission

Figure X.10-1 8point bit-to-symbol mapping is realized by three layer symbol-level spreading with BPSK.

Figure X.10-2 16point bit-to-symbol mapping is realized by two layer symbol-level spreading with QPSK.