3GPP TSG RAN WG1 Meeting #94	R1-1808076

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:	7.2.7.2

Source:	Huawei, HiSilicon

Title:	Considerations and evaluation results for IMT-2020 for high mobility

Document for:	Discussion and decision

Introduction

In this contribution, the considerations on self evaluation and evaluation results for mobility is presented.

Evaluation method

The general evaluation method and procedure for mobility evaluation is defined in Report ITU-R M.2412 [1]. This procedure includes system level simulation (SLS) part and link level simulation (LLS) part. In RAN1#92bis meeting, the detailed evaluation method and parameters for these two parts are discussed [2].

For SLS part, the pre-processing SINR is used. The pre-processing SINR is defined on an Rx antenna port with respect to a Tx antenna port. The details of calculating pre-processing SINR is refer to Section 2.1.1 in [2].

For LLS part, the antenna pattern options are proposed, and inter-port distance is specified. There are three options for the use of antenna port pattern [2]. 0dBi omni-directional gain is assumed for Tx and Rx antenna ports is used in this contribution.

The evaluation method in [2] is used to derive the evaluation results in this contribution.

Mean ZoD spread of system level channel model

In [2], there is a remaining issue on mean value of ZoD spread. According to ITU-R guideline, the ZoD spread in link level channel model should be scaled to the mean value in system level channel model. It is agreed in [2] that,

Use the 50%-tile point of CDF of the mean value of ZoD spread of all UEs based on UE dropping in system level simulation. The mean value of ZoD spread of a UE is calculated according to the formula given in Table 1 and Table 2 for Dense Urban and Rural test environment, respectively, where d2D and hUT of a UE is determined by the UE dropping.

Table 1 Mean value calculation for UMa ZoD spread

|  | Frequency | Parameters | UMa_x |  |
| --- | --- | --- | --- | --- |
|  |  |  | LOS | NLOS |
| UMa_A | 0.5 GHz≤ fc ≤6 GHz | lgZSD | max[-0.5, -2.1(d2D/1000) -0.01 (hUT - 1.5)+0.75] | max[-0.5, -2.1(d2D/1000) -0.01(hUT - 1.5)+0.9] |
|  | 6 GHz< fc ≤ 100 GHz | lgZSD | max[-0.5, -2.1(d2D/1000) -0.01 (hUT - 1.5)+0.75] | max[-0.5, -2.1(d2D/1000)-0.01(hUT - 1.5)+0.9] |
| UMa_B | 0.5 GHz≤ fc ≤ 100 GHz | lgZSD | max[-0.5, -2.1(d2D/1000)- 0.01 (hUT -1.5)+0.75] | max[-0.5, -2.1(d2D/1000)-0.01(hUT - 1.5)+0.9] |

Table 2 Mean value calculation for RMa ZoD spread

|  | Frequency | Parameters | RMa_x |  |
| --- | --- | --- | --- | --- |
|  |  |  | LOS | NLOS |
| RMa_A RMa_B | RMa_A：0.5 GHz≤ fc ≤ 6 GHz, RMa_B：0.5 GHz≤ fc ≤ 7 GHz | lgZSD | max(-1, -0.17*( d2D /1000)-0.01*( hUT -1.5)+0.22) | max(-1, -0.19*( d2D /1000) - 0.01*( hUT -1.5)+0.28) |

Detailed method of deriving mean value of ZoD spread for LOS and NLOS

In link level simulation, LOS and NLOS channel will be evaluated separately. Therefore, the mean value of ZoD spread should be derived for LOS and NLOS, separately. Based on the above agreement, the detailed derivation is as follows:

- In the UE drop in system level simulation, determine “LOS” UE and “NLOS” UE according to LOS probability from system level channel model (“LOS UE” means the channel state is LOS for UE to its serving TRxP; “NLOS UE” means the channel state is NLOS for UE to its serving TRxP)

- Assume there are N “LOS” UEs, and M “NLOS” UEs; N+M=570 for dense urban and rural test environment. Calculate the value of lgZSD for “LOS” UE and “NLOS” UE according to “LOS” and “NLOS” column in Table 1 or Table 2, respectively.

- Plot CDF of lgZSD for “LOS” UEs, based on the N samples, and find the 50%-tile point as the mean value of ZoD spread in case of LOS. Similarly, plot CDF of lgZSD for “NLOS” UEs, based on the M samples, and find the 50%-tile point as the mean value of ZoD spread in case of NLOS.

Proposal 1: Use the detailed method shown in Section 3.1 to derive the mean value of ZoD spread for LOS and NLOS.

The results of mean value of ZoD spread

According to the above-mentioned method, the mean value of ZoD in degree is shown is Table 3 (see gray grids), and the CDF figures are provided in Appendix 1.

Table 3 Mean value of ZoD spread for Dense Urban and Rural – eMBB test environment

| Parameters | Dense Urban-eMBB |  |  |  | Rural-eMBB |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Config A (4 GHz) |  | Config B (30 GHz) |  | Config A/B (700 MHz/4 GHz) |  |
| Link-level Channel model | LOS: CDL/TDL-v | NLOS: CDL/TDL-iii | LOS: CDL/TDL-v | NLOS: CDL/TDL-iii | LOS: CDL/TDL-v | NLOS: CDL/TDL-iii |
| ZoD angular spreads scaling parameter (degree) | 3.3 | 4.6 | TBD from 50%-tile point of CDF of ZoD spread | TBD from 50%-tile point of CDF of ZoD spread | 1.25 | 1.44 |

Evaluation results

SINR distribution

In this section, the evaluation results for mobility is provided. In Figure 1, the pre-processing SINR CDFs for eMBB test environment are provided. The assumptions are provided in Appendix 2.

- Rural – eMBB (700 MHz)                                        (b) Rural – eMBB (4 GHz)

(c) Dense Urban – eMBB (4 GHz)                               (d) Indoor Hotspot – eMBB (4 GHz)

Figure 1 UL SINR distribution for eMBB test environments

Based on the above figures, the 50%-tile point of the CDF for different test environments are listed in Table 4.

Table 4 The 50%-tile point of SINR CDF for different test environments

| Test environment | Evaluation configuration | UE mobility | 50%-tile point of SINR CDF (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Channel model A | Channel model B |
| Rural – eMBB | Config. A (700 MHz) | 120 km/h | 10.21 | 10.14 |
|  |  | 500 km/h | 9.67 | 9.65 |
| Rural - eMBB | Config. B (4 GHz) | 120 km/h | 4.66 | 4.50 |
|  |  | 500 km/h | 2.90 | 2.72 |
| Dense Urban – eMBB | Config. A (4 GHz) | 30 km/h | 5.52 | 5.32 |
| Indoor Hotspot – eMBB (12 TRxP) | Config. A (4 GHz) | 10 km/h | 3.90 | 3.95 |
| Indoor Hotspot – eMBB (36 TRxP) | Config. A (4 GHz) | 10 km/h | -0.21 | -0.07 |

Link performance

In this section, the uplink link level evaluation results for mobility is provided, and the results of NR for different test environments are listed in Table 5, the results of LTE are listed in Table 6.

Table 5 The uplink link level evaluation results for different test environments for NR

| Test environment | ITU requirement (bit/s/Hz) | Evaluation configuration | Channel Model | 50%-tile point of SINR CDF (dB) | Uplink SE (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | FDD |  | TDD |  |
|  |  |  |  |  | NLOS | LOS | NLOS | LOS |
| Indoor Hotspot –eMBB (12 TRxP) | 1.5 | Config. A (4 GHz) | Channel model A | 3.90 | 1.75 | 2.05 | 1.59 | 1.94 |
|  |  |  | Channel model B | 3.95 | 1.75 | 2.07 | 1.60 | 1.95 |
| Dense Urban – eMBB | 1.12 | Config. A (4 GHz) | Channel model A | 5.52 | 1.92 | 2.22 | 1.82 | 2.17 |
|  |  |  | Channel model B | 5.32 | 1.89 | 2.19 | 1.79 | 2.06 |
| Rural –eMBB (120 km/h) | 0.8 | Config. A (700 MHz) | Channel model A | 10.21 | 2.32 | 2.90 | 2.10 | 2.63 |
|  |  |  | Channel model B | 10.14 | 2.31 | 2.90 | 2.09 | 2.63 |
|  |  | Config. B (4 GHz) | Channel model A | 4.66 | 1.30 | 1.74 | 1.18 | 1.57 |
|  |  |  | Channel model B | 4.50 | 1.28 | 1.68 | 1.16 | 1.52 |
| Rural –eMBB (500 km/h) | 0.45 | Config. A (700 MHz) | Channel model A | 9.67 | 2.07 | 2.64 | 1.88 | 2.39 |
|  |  |  | Channel model B | 9.65 | 2.07 | 2.64 | 1.87 | 2.39 |
|  |  | Config. B (4 GHz) | Channel model A | 2.90 | 0.92 | 1.33 | 0.84 | 1.22 |
|  |  |  | Channel model B | 2.72 | 0.91 | 1.33 | 0.83 | 1.22 |

Observation 1: NR can fulfill the mobility requirements of IMT-2020 in all test environments.

Table 6 The uplink link level evaluation results for different test environments for LTE

| Test environment | ITU requirement (bit/s/Hz) | Evaluation configuration | Channel Model | 50%-tile point of SINR CDF (dB) | Uplink SE (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | FDD |  | TDD |  |
|  |  |  |  |  | NLOS | LOS | NLOS | LOS |
| Rural –eMBB (120 km/h) | 0.8 | Config. A (700 MHz) | Channel model A | 10.21 | 2.20 | 2.79 | 1.96 | 2.49 |
|  |  |  | Channel model B | 10.14 | 2.19 | 2.79 | 1.94 | 2.49 |
| Rural –eMBB (500 km/h) | 0.45 | Config. A (700 MHz) | Channel model A | 9.67 | 1.94 | 2.59 | 1.73 | 2.31 |
|  |  |  | Channel model B | 9.65 | 1.93 | 2.59 | 1.72 | 2.31 |

Observation 2: LTE can fulfill the mobility requirements of IMT-2020 in Rural–eMBB test environment.

Summary

In this document, the consideration and evaluation results on mobility is provided. The following proposals are made.

Proposal 1: Use the detailed method shown in Section 3.1 to derive the mean value of ZoD spread for LOS and NLOS.

Observation 1: NR can fulfill the mobility requirements of IMT-2020 in all test environments.

Observation 2: LTE can fulfill the mobility requirements of IMT-2020 in Rural–eMBB test environment.

- Reference

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- R1-1805643, “Summary of discussion on mobility evaluation method and parameters”, Huawei, 3GPP TSG RAN WG1#92bis, April 2018.

- Appendix 1 	CDF for ZoD spread for LOS and NLOS

The CDF of mean value of ZoD spread for LOS and NLOS for Rural and Dense Urban test environment are plotted from Figure A1-1 to Figure A1-3, respectively.

Figure A1-1 Mean value of ZoD (degree) for Rural (4 GHz)

Figure A1-2 Mean value of ZoD (degree) for Rural (700 MHz)

Figure A1-3 Mean value of ZoD (degree) for Dense Urban (4 GHz)

- Appendix 2 	Simulation assumption of SLS part for mobility

The simulation assumption of system level part for mobility evaluation is listed in Table A2-1.

Table A2-1. Simulation assumptions of SLS

|  | Indoor Hotspot – eMBB | Dense urban - eMBB | Rural - eMBB |
| --- | --- | --- | --- |
| Evaluation configuration | Configuration A, Configuration B | Configuration A | Configuration A, Configuration B |
| Carrier frequency for evaluation | 4 GHz | 4 GHz | Configuration A :700 MHz Configuration B : 4 GHz |
| Multiple access | OFDMA | OFDMA | OFDMA |
| Duplexing | FDD, TDD | FDD, TDD | FDD, TDD |
| Transmission scheme | UL SIMO | UL SIMO | UL SIMO |
| BS antenna height | 3m | 25 m | 35 m |
| Total transmit power per TRxP | 21 dBm for 10 MHz bandwidth | 41 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | - | 20% high loss, 80% low loss (applies to Channel model B) | 100% low loss (applies to Channel model B) |
| Inter-site distance | 20 m | 200 m | 1732 m |
| Number of antenna elements per TRxP | 32 Tx/Rx, (M,N,P,Mg,Ng) = (4,4,2,1,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization | 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 32 Tx/Rx, (M,N,P,Mg,Ng) = (8,2,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) |
| Number of UE antenna elements | 1Tx/Rx, (M,N,P,Mg,Ng) = (1,1,1,1,1) | 1Tx/Rx, (M,N,P,Mg,Ng) = (1,1,1,1,1) | 1Tx/Rx, (M,N,P,Mg,Ng) = (1,1,1,1,1) |
| Device deployment | 100% indoor Randomly and uniformly distributed over the area | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area |
| UE speeds of interest | 10 km/h | 30 km/h; | 120 km/h;500km/h; |
| Traffic model | Full buffer | Full buffer | Full buffer |
| Simulation bandwidth | For FDD: 10 MHz For TDD: 20 MHz | For FDD: 10 MHz For TDD: 20 MHz | For FDD: 10 MHz For TDD: 20 MHz |
| UE density | 10 UEs per TRxP | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height | 1.5 m | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) | 1.5 m |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B | Alt. 1: Channel model A Alt. 2: Channel model B | Alt. 1: Channel model A Alt. 2: Channel model B |
| TRxP number per site | 1 or 3 | 3 | 3 |
| Mechanic tilt | For 1 TRxP per site:180° in GCS (pointing to the ground) For 3 TRxPs per site: 110° | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | 90° in LCS | 105° in LCS | 100° in LCS |
| Handover margin (dB) | 1 | 1 | 1 |
| TRxP boresight | For 1 TRxP per site::- For 3 TRxP per site: 30 / 150 / 270 degrees | 30 / 150 / 270 degrees | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | No wrapping around | Geographical distance based wrapping | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=0m | d2D_min=10m | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 | Model-2 in TR36.873 |
| Power control parameters | = 0.6, P0 = -60 dBm | = 0.9, P0 = -86 dBm | For 700 MHz := 0.8, P0 = -76 dBm For 4 GHz := 0.6, P0 = -60 dBm |
| Numerology | For FDD: One slot with 15 kHz SCS For TDD: One slot with 30 kHz SCS | For FDD: One slot with 15 kHz SCS For TDD: One slot with 30 kHz SCS | For 700 MHz: - 120 km/h: one slot with 15 kHz SCS - 500 km/h: one slot with 30 kHz SCS For 4 GHz: - 120 km/h: one slot with 30 kHz SCS - 500 km/h: one slot with 60 kHz SCS |
| Scheduling | PF | PF | PF |
| Receiver | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| Pre-processing SINR calculation | Aligned with Section 2.1.1 in R1-1805643 | Aligned with Section 2.1.1 in R1-1805643 | Aligned with Section 2.1.1 in R1-1805643 |

- Appendix 3 	Simulation assumption of LLS part for mobility

The simulation assumption of link level part for mobility evaluation is listed in Table A3-1.

Table A3-1. Simulation assumptions of LLS

|  | Indoor Hotspot – eMBB | Dense urban - eMBB | Rural – eMBB |  |
| --- | --- | --- | --- | --- |
| Carrier frequency for evaluation | 4 GHz | 4 GHz | Configuration A :700 MHz Configuration B : 4 GHz (for NR only) |  |
| RIT | NR | NR | NR | LTE |
| Waveform | CP-OFDM | CP-OFDM | CP-OFDM | DFT-S-OFDM |
| Duplexing | FDD, TDD | FDD, TDD | FDD, TDD | FDD, TDD |
| TDD frame structure | DDDSU | DDDSU | DDDSU | DSUDD |
| Evaluated service profiles | Full buffer best effort | Full buffer best effort | Full buffer best effort | Full buffer best effort |
| Simulation bandwidth | 10 MHz | 10 MHz | 10 MHz | 10 MHz |
| Number of users in simulation | 1 | 1 | 1 | 1 |
| Link-level Channel model | NLOS: CDL-i LOS: CDL-iv | NLOS: CDL-iii LOS: CDL-v | NLOS: CDL-iii LOS: CDL-v | NLOS: CDL-iii LOS: CDL-v |
| UE speed | 10 km/h | 30 km/h | 120 km/h, 500 km/h | 120 km/h, 500 km/h |
| Subcarrier spacing | For FDD: 15 kHz For TDD: 30 kHz | For FDD: 15 kHz For TDD: 30 kHz | For 700 MHz: - 120 km/h: 15 kHz - 500 km/h: 30 kHz For 4 GHz: - 120 km/h: 30 kHz - 500 km/h: 60 kHz | 15 kHz |
| Symbols number per slot | 14 | 14 | 14 | 14 |
| Antenna configuration at TRxP | 8R, (4,4,2,1,1; 1,4) | 8R, (8,4,2,1,1; 1,4) | 4R, (8,2,2,1,1; 1,2) | 4R, (8,2,2,1,1; 1,2) |
| Antenna configuration at UE | 1T, (1,1,1,1,1; 1,1) | 1T, (1,1,1,1,1; 1,1) | 1T, (1,1,1,1,1; 1,1) | 1T, (1,1,1,1,1; 1,1) |
| TXRU pattern at TRxP | Option 1: 0dBi Omni-directional | Option 1: 0dBi Omni-directional | Option 1: 0dBi Omni-directional | Option 1: 0dBi Omni-directional |
| TXRU pattern at UE | Option 1: 0dBi Omni-directional | Option 1: 0dBi Omni-directional | Option 1: 0dBi Omni-directional | Option 1: 0dBi Omni-directional |
| Transmission mode | SIMO | SIMO | SIMO | SIMO |
| Transmission rank | Rank 1 | Rank 1 | Rank 1 | Rank 1 |
| UL precoder | - | - | - | - |
| TRxP receiver type | MMSE-IRC | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| Channel estimation | LMMSE | LMMSE | LMMSE | LMMSE |
| Number of subcarriers per PRB | 12 | 12 | 12 | 12 |
| Data allocation | 14 symbol slots, with 12 RB allocated | 14 symbol slots, with 12 RB allocated | 14 symbol slots, with 12 RB allocated | 14 symbol slots, with 12 RB allocated |
| Channel coding scheme | LDPC | LDPC | LDPC | Turbo |
| Link adaptation | Yes | Yes | Yes | Yes |
| HARQ | Max 4 HARQ transmissions | Max 4 HARQ transmissions | Max 4 HARQ transmissions | Max 4 HARQ transmissions |
| DMRS configuration | 2 symbol DMRS (front loaded and one additional) with configuration type 2, no FDM with data and full power utilization in DMRS symbols | 2 symbol DMRS (front loaded and one additional) with configuration type 2, no FDM with data and full power utilization in DMRS symbols | - For 4GHz 500km/h: 4 symbol DMRS (front loaded and 3 additional) with configuration type 2, no FDM with data and full power utilization in DMRS symbols - Others: 2 symbol DMRS (front loaded and one additional) with configuration type 2, no FDM with data and full power utilization in DMRS symbols | 2 symbol DMRS |
| Other overhead | - SRS: 2 symbols per 5 slots. For TDD, the 2 symbols are the 2 uplink symbols in S sub-frame - PUCCH :2 RB in 10MHz bandwidth | - SRS: 2 symbols per 5 slots. For TDD, the 2 symbols are the 2 uplink symbols in S sub-frame - PUCCH :2 RB in 10MHz bandwidth | - SRS: 2 symbols per 5 slots. For TDD, the 2 symbols are the 2 uplink symbols in S sub-frame - PUCCH :2 RB in 10MHz bandwidth | - SRS: FDD: 1 symbols per 5 slots. TDD: 2 symbols per 5 slots in the S sub-frame - PUCCH :2 RB in 10MHz bandwidth |