3GPP TSG RAN WG1 Meeting #94                                                                              R1-1808428

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	CATT

Title:	Self evaluation on mobility

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

- Introduction

At RAN#75 meeting, new Study Item on Self Evaluation towards IMT-2020 submission was approved [1]. Self-Evaluation will provide the performance towards all the ITU-R IMT-2020 requirements as defined in Report ITU-R M.2410 [2]. High-level assessment methods for these evaluation characteristics are given in § 6 of Report ITU-R M.2412 [3]. Mobility for  eMBB (Indoor Hotspot-eMBB, Dense Urban-eMBB, and Rural-eMBB) need to be evaluated with both system-level simulation (SLS) and link-level simulation (LLS). A median SINR provided by SLS will be used in link level simulation to achieve final evaluation results [3].

In this contribution, we provide the complete mobility evaluation results for eMBB scenario, inculding UL pre-processing SINR CDF and preliminary self-evaluation results for mobility.

- Pre-processing SINR CDF and median SINR

At Ran1#92bis meeting, it is agreed that for SU-MIMO pre-processing SINR as defined in [4] is calculated in SLS. In this section, we provided the preliminary evaluation results of UL preprocessing SINR in eMBB scenarios.

The preliminary UL median pre-processing SINR results for eMBB scenarios are listed in Table 1. System level simulation assumptions are listed in Appendix 1.

Figure 1: UL pre-processing SINR distribution for Rural-eMBB Config A with channel model A and B

Figure 2: UL pre-processing SINR distribution for Rural-eMBB Config B with channel model A and B

Figure 3: UL pre-processing SINR distribution for Dense Urban-eMBB Config A with channel model A and B

Figure 4: UL pre-processing SINR distribution for InH-eMBB 1TxRP/site Config A with channel model A and B

Figure 5: UL pre-processing SINR distribution for InH-eMBB 3TxRP/sites Config A with channel model A and B

The preliminary UL median pre-processing SINR results for eMBB scenarios are listed in Table 1.

Table 1. UL median pre-processing SINR

| Test environment | Configuration | Channel Model | 50%-tile UL pre-processing SINR |
| --- | --- | --- | --- |
| Rural-eMBB | Config A | Model A | 8.96 |
|  |  | Model B | 8.87 |
|  | Config B | Model A | 8.02 |
|  |  | Model B | 7.76 |
| Dense Urban -eMBB | Config A | Model A | 12.57 |
|  |  | Model B | 12.88 |
| InH-eMBB | 1TRP/site Config A | Model A | 4.15 |
|  |  | Model B | 4.25 |
| InH-eMBB | 3TxRPs/site Config A | Model A | 4.14 |
|  |  | Model B | 4.21 |

- Link level simulation

- Evaluation results

Based on the pre-processing SINR provided in the Table 1, the complete self-evaluation results for mobility are provided in Table 4. Besides ITU defined link level parameters of Mobility, some additional link level assumption parameters applied in this preliminary evaluation can be found in Table 5a-Table 5c.

Table 4. Mobility traffic channel link data rates

| Test environment | Configuration | Channel Model | Median UL SINR[dB] | ITU Requirement [bps/Hz] | Spectrum efficiency of NLOS | Spectrum efficiency of LOS | Fulfil the requirements |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Dense Urban -eMBB | Config A 4GHz | Model A | 12.57 | 1.12 | 2.9612 | 3.0580 | Y |
|  |  | Model B | 12.88 |  | 3.0382 | 3.1720 | Y |
| InH-eMBB Config A 4GHz | 1TRP/site | Model A | 4.15 | 1.5 | 1.7306 | 1.6526 | Y |
|  |  | Model B | 4.25 |  | 1.7516 | 1.6751 | Y |
| Rural-eMBB (120km/h) | Config A 700MHz | Model A | 8.96 | 0.8 | 2.1742 | 2.2153 | Y |
|  |  | Model B | 8.87 |  | 2.1416 | 2.1940 | Y |
|  | Config B 4GHz | Model A | 8.02 |  | 1.7589 | 1.8037 | Y |
|  |  | Model B | 7.76 |  | 1.7016 | 1.7879 | Y |
| Rural-eMBB (500km/h) | Config A 700MHz | Model A | 8.96 | 0.45 | 1.9932 | 2.1308 | Y |
|  |  | Model B | 8.87 |  | 1.9717 | 2.1146 | Y |
|  | Config B 4GHz | Model A | 8.02 |  | 0.9176 | 0.9963 | Y |
|  |  | Model B | 7.76 |  | 0.8894 | 0.9659 | Y |

Table 5a. The additional link level simulation assumption for Indoor-hotspot-eMBB test environment

| Parameters | Reference value |
| --- | --- |
| Carrier frequency | 4GHz |
| Waveform | CP-OFDM |
| Simulation bandwidth | 10MHz |
| Link-level Channel model | CDL-i |
|  | CDL-iv |
| UE speed | 10 km/h |
| Subcarrier spacing | 15kHz |
| Symbols number per slot | 14 |
| Antenna configuration at TRxP | LOS: 8R NLOS:16R |
| Antenna configuration at UE | 1T |
| Transmission rank | Rank1 |
| TRxP receiver type | MMSE |
| Channel estimation | Real |
| Number of subcarriers per PRB | 12 |
| Data allocation | 14 symbol slots, with 10RB |
| Channel coding scheme | LDPC |
| Link adaptation | No link adaptation |
| HARQ | Max 4 HARQ tansmissions |
| DMRS configuration | 2 symbol DMRS (front loaded and one additional) with configuration type 1, no FDM with data and full power utilization in DMRS symbols |
| Other overhead | SRS：2 symbols per 10 slots, PUCCH：(3 PRB, 14 symbols) with 1 slot and (1 PRB, 2 symbols) with 9 slots |

Table 5b. The additional link level simulation assumption for Dense Urban-eMBB test environment

| Parameters | Reference value |
| --- | --- |
| Carrier frequency | 4GHz |
| Waveform | CP-OFDM |
| Simulation bandwidth | 10MHz |
| Link-level Channel model | CDL-iii |
|  | CDL-v |
| UE speed | 30km/h |
| Subcarrier spacing | 15kHz |
| Symbols number per slot | 14 |
| Antenna configuration at TRxP | 8R |
| Antenna configuration at UE | 1T |
| Transmission rank | Rank1 |
| TRxP receiver type | MMSE |
| Channel estimation | Real |
| Number of subcarriers per PRB | 12 |
| Data allocation | 14 symbol slots, with 10RB |
| Channel coding scheme | LDPC |
| Link adaptation | No link adaptation |
| HARQ | Max 4 HARQ tansmissions |
| DMRS configuration | 2 symbol DMRS (front loaded and one additional) with configuration type 1, no FDM with data and full power utilization in DMRS symbols |
| Other overhead | SRS：2 symbols per 10 slots, PUCCH：(3 PRB, 14 symbols) with 1 slot and (1 PRB, 2 symbols) with 9 slots |

Table 5c. The additional link level simulation assumption for Rural-eMBB test environment

| Parameters | Value （120km/h） | Value （500km/h） |
| --- | --- | --- |
| Carrier frequency | 700 MHz，4GHz | 700 MHz，4GHz |
| Waveform | CP-OFDM | CP-OFDM |
| Simulation bandwidth | 10 MHz | 10 MHz |
| Link-level Channel model | CDL-iii | CDL-iii |
|  | CDL-v | CDL-v |
| UE speed | 120 km/h | 500 km/h |
| Subcarrier spacing | 700MHz: 15 kHz 4GHz: 30kHz | 30 kHz |
| Symbols number per slot | 14 | 14 |
| Antenna configuration at TRxP | 8R | 8R |
| Antenna configuration at UE | 1T | 1T |
| Transmission rank | 1 | 1 |
| TRxP receiver type | MMSE | MMSE |
| Channel estimation | Real | Real |
| Number of subcarriers per PRB | 12 | 12 |
| Data allocation | 14 symbol slots, with 10 RB allocated | 14 symbol slots, with 10 RB allocated |
| Channel coding scheme | LDPC | LDPC |
| Link adaptation | No link adaptation | No link adaptation |
| HARQ | Max 4 HARQ tansmissions | Max 4 HARQ tansmissions |
| DMRS configuration | 2 symbol DMRS (front loaded and one additional) with configuration type 1, no FDM with data and full power utilization in DMRS symbols | 700MHz： 2 symbol DMRS (front loaded and one additional) with configuration type 1, no FDM with data and full power utilization in DMRS symbols 4GHz： 4 symbol DMRS (front loaded and 3 additional) with configuration type 1, no FDM with data and full power utilization in DMRS symbols |
| Other overhead | SRS：2 symbols per 10 slots, PUCCH：(3 PRB, 14 symbols) with 1 slot and (1 PRB, 2 symbols) with 9 slots | SRS：2 symbols per 10 slots, PUCCH：(3 PRB, 14 symbols) with 1 slot and (1 PRB, 2 symbols) with 9 slots |

According to above self-evaluation results, the achieved normalized traffic channel data rate in Indoor Hotspot-eMBB scenario, Dense Urban-eMBB scenario and Rural-eMBB scenario are higher than the normalized traffic data rate for both LOS and NLOS. Thus, the IMT-2020 mobility requirements in these eMBB test environments can be fulfilled.

Observation: The achieved normalized traffic channel data rate in Indoor Hotspot-eMBB scenario, Dense Urban-eMBB scenario and Rural-eMBB scenario are higher than the required spectrum efficiency values. Hence, the IMT-20202 mobility requirements are fulfilled.

- Conclusion

Based on the pre-processing SINR provided in the Table 2, the achieved normalized traffic data rate in Indoor Hotspot-eMBB scenario, Dense Urban-eMBB scenario and Rural-eMBB scenario can all fulfill the IMT-2020 mobility requirements for both LOS and NLOS.

Observation: The achieved normalized traffic channel data rate in Indoor Hotspot-eMBB scenario, Dense Urban-eMBB scenario and Rural-eMBB scenario are higher than the required spectrum efficiency values. Hence, the IMT-20202 mobility requirements are fulfilled.

- References

- RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- R1-1805643, “Summary of discussion on mobility evaluation method and parameters”, Huawei

- RP-180524, “Summary of calibration results for IMT-2020 self evaluation”, Huawei

Appendix 1: Simulation assumption of SLS part for mobility

The simulation assumption of system level part for mobility evaluation is listed in Table A1-1.

Table A1-1. Simulation assumptions in SLS

|  | Indoor Hotspot – eMBB | Dense urban - eMBB | Rural - eMBB |
| --- | --- | --- | --- |
| Evaluation configuration | Configuration A | Configuration A | Configuration A, B |
| Carrier frequency for evaluation | 4 GHz | 4 GHz | Configuration A :700 MHz Configuration B : 4 GHz |
| Multiple access | OFDMA | OFDMA | OFDMA |
| Duplexing | FDD | FDD | FDD |
| Transmission scheme | SU-MIMO with rank 1 | SU-MIMO with rank 1 | SU-MIMO with rank 1 |
| BS antenna height | 3m | 25 m | 35 m |
| Total transmit power per TRxP | 21 dBm for 10 MHz bandwidth | 41 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | - | 20% high loss, 80% low loss (applies to Channel model B) | 100% low loss (applies to Channel model B) |
| Inter-site distance | 20 m | 200 m | 1732 m |
| Number of antenna elements per TRxP | Config A: 32Tx/Rx, (M,N,P,Mg,Ng) = (4,4,2,1,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization | 128Tx/Rx, (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | Config A: 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1); Config B:128Tx/Rx, (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 32TXRU, (Mp,Np,P,Mg,Ng) = (4,4,2,1,1) (1-to-1 mapping) | 4TXRU, (Mp,Np,P,Mg,Ng) = (2,1,2,1,1) | Config A: 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) Config B: 16TXRU, (Mp,Np,P,Mg,Ng) = (1,8,2,1,1) |
| Number of UE antenna elements | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization | Config A: 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) Config B: 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization |
| Number of TXRU per UE | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) | Config A: 2Tx/Rx, (Mp,Np,P,Mg,Ng)= (1,1,2,1,1) (1-to-1 mapping) Config B: 4Tx/Rx, (Mp,Np,P,Mg,Ng)= (1,2,2,1,1), (1-to-1 mapping) |
| Device deployment | 100% indoor Randomly and uniformly distributed over the area | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area |
| UE speeds of interest | 3 km/h | Indoor users: 3km/h Outdoor users (in-car): 30 km/h | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; |
| Traffic model | Full buffer | Full buffer | Full buffer |
| Simulation bandwidth | 10 MHz | 10 MHz | 10 MHz |
| UE density | 10 UEs per TRxP | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height | 1.5 m | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) | 1.5 m |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B | Alt. 1: Channel model A Alt. 2: Channel model B | Alt. 1: Channel model A Alt. 2: Channel model B |
| TRxP number per site | 1 or 3 | 3 | 3 |
| Mechanic tilt | For 1 TRxP per site:180° in GCS (pointing to the ground) For 3 TRxPs per site: 110° | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | 90° in LCS | beam based: TRxP analog beam steering (in LCS): Azimuth angle φi = [-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16] Zenith angle θj = [5*pi/8, 7*pi/8] | 100° in LCS |
| Handover margin (dB) | 1 | 1 | 1 |
| TRxP boresight | For 1 TRxP per site::- For 3 TRxP per site: 30 / 150 / 270 degrees | 30 / 150 / 270 degrees | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | No wrapping around | Geographical distance based wrapping | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=0m | d2D_min=10m | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 | Model-2 in TR36.873 |
| Power control parameters | α= 0.8, P0 = -76 dBm | α = 0.8, P0 = -80 dBm | α = 0.8, P0 = -80 dBm |
| Numerology | One slot with 15 kHz SCS | One slot with 15 kHz SCS | One slot with 15 kHz SCS |
| Scheduling | PF | PF | PF |
| Receiver | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| Pre-processing SINR calculation | Aligned with Section 2.1.1 in R1-1805643 | Aligned with Section 2.1.1 in R1-1805643 | Aligned with Section 2.1.1 in R1-1805643 |