3GPP TSG RAN WG1 Meeting #94                                                                              R1-1808429

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	CATT

Title:	Self evaluation on Reliability

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

- Introduction

At RAN#75 meeting, new Study Item on Self Evaluation towards IMT-2020 submission was approved [1]. Self-Evaluation will provide the performance towards all the ITU-R IMT-2020 requirements as defined in Report ITU-R M.2410 [2]. High-level assessment methods for these evaluation characteristics are given in § 6 of Report ITU-R M.2412 [3]. Reliability for URLLC need to be evaluated with both system-level simulation (SLS) and link-level simulation (LLS). A 5th-percentile SINR provided by SLS will be used in link level simulation to achieve final evaluation results [3].

In this contribution, we provide initial reliability evaluation results, including UL and DL pre-processing SINR CDF, and preliminary evaluation results for reliability.

- Pre-processing SINR

At Ran1#93 meeting, it was agreed that for SU-MIMO pre-processing SINR as defined in [4] is calculated in SLS. In this section, we provide the preliminary evaluation results of UL and DL preprocessing SINR for URLLC scenario. The 5%-tile point pre-processing SINR values are provided in Table 1. Pre-processing SINR CDFs are shown in Figure 1 and Figure 2. The general evaluation SLS assumptions are listed in Appendix I.

Table 1: 5%-tile point pre-processing SINR

| Scenario | Configuration | Channel Model | 5%-tile pre-processing SINR (dB) |
| --- | --- | --- | --- |
| URLLC UL | Config A <α = 0.8, P0 = -90 dBm> | Model A | -4.01 |
|  | Config A <α = 0.8, P0 = -84 dBm> | Model B | 0.39 |
|  | Config B <α = 0.6, P0 = -60 dBm> | Model A | -0.04 |
|  | Config B <α = 0.9, P0 = -86 dBm> | Model B | 1.25 |
| URLLC DL | Config A | Model A | -3.47 |
|  |  | Model B | -3.46 |
|  | Config B | Model A | -2.55 |
|  |  | Model B | -2.46 |

| Figure 1: UL pre-processing SINR CDF | Figure 2: DL pre-processing SINR CDF |
| --- | --- |

- Link level simulation

At Ran1#93 meeting, analytical calculation methods for total reliability were agreed in [5]. In order to obtain final reliability, probabilities (denoted as p0~p4) as listed in the following table need to be evaluated. p1 and p2 are obtained by running PDCCH and PDSCH/PUSCH simulations respectively.  p0, p3 and p4 are acquired by PUCCH simulations. For a better understanding of these probabilities, notations are also provided in Table 2.

Table 2: Successful probability notations for various events

| Successful Probability | Description | Notations |
| --- | --- | --- |
| p0 | Successful probability of SR detection | P(SR detected ¦ UE sent SR) |
| p1 | Successful probability of PDCCH transmission | P(PDCCH correctly received) |
| p2 | Successful probability of PDSCH/PUSCH transmission | P(PDSCH/PUSCH correctly received) |
| p3 | Successful probability of PUCCH NACK detection | P(NACK or DTX detected ¦ UE sent NACK) = 1 - P(ACK detected ¦ UE sent NACK) |
| p4 | Successful probability of PUCCH DTX detection | P(NACK or DTX detected ¦ UE sent DTX) = 1 - P(ACK detected ¦ UE sent DTX) |

According to [5], Alternative 1 without soft combination and Alternative 2 with soft combination gain can be applied to LLS. In the following section, LLS assumptions and results based on Alternative 1 (without soft combination gain) on PDCCH, PDSCH, PUSCH and PUCCH are provided and then total reliability results are obtained.

- PDCCH

Table 3: PDCCH simulation assumptions

| Numerology | 30kHz SCS |
| --- | --- |
| Waveform | CP-OFDM |
| Aggregation level | 16 |
| PDCCH symbol | 2 symbols |
| Payload size | 34 bits |
| Carrier frequency | Config B: 700MHz |
| Antenna config | Config B: 2T2R |
| Antenna pattern | 0dB omni-directional gain |
| channel model | TDL-C, 363ns |
| Speed | 3km/h |
| Channel estimation | Practical |

PDCCH BLER results are given in figure 3. The successful probability of PDCCH transmission p1 can be calculated according to PDCCH BLER.

Figure3  PDCCH simulation results for ConfigB

- PDSCH

PDSCH LLS assumptions are listed in Table 4. Since TBS is set to be 256 bits, the allocation bandwidth for PDSCH is derived based on the TBS determination procedure as defined in Section 5.1.3 of TS38.214 [6].  1 symbol DMRS overhead is accounted for TBS calculation. The MCS table for URRLC agreed at Ran1 #93 meeting is used.

Table 4: PDSCH simulation assumptions

| Numerology | 30kHz SCS |
| --- | --- |
| Waveform | CP-OFDM |
| Scheduling | Slot-based scheduling |
| PDSCH length | 4 symbols (1xDMRS+3xData) 7 symbols (1xDMRS+6xData) |
| Carrier frequency | Config A: 4GHz Config B: 700MHz |
| Antenna config | Config A: 8T4R Config B: 2T2R |
| Antenna pattern | 0dB omni-directional gain |
| Transmission scheme | Spatial multiplexing |
| channel model | TDL-C, 363ns |
| Speed | 3km/h |
| Channel estimation | Practical |
| Data and RB allocation | TBS = 256 bits : 4-os, MCS3, 58RB 4-os, MCS5, 37RB 7-os, MCS1, 46RB 7-os, MCS3, 29RB |
| Transmission scheme | No HARQ |
| Overhead | 1 symbol DMRS accounted |

PDSCH BLER results are given in figure 4. The successful probability of PDSCH transmission p2 can be calculated according to PDSCH BLER.

| Figure 4: PDSCH simulation results for Config A | Figure 5: PDSCH simulation results for Config B |
| --- | --- |

- PUSCH

PUSCH LLS assumptions are listed in Table 5.

Table5 PUSCH simulation assumptions

| Numerology | 30kHz SCS |
| --- | --- |
| Waveform | CP-OFDM |
| Scheduling | Slot-based scheduling |
| PUSCH length | 4 symbols (1xDMRS+3xData) |
| Carrier frequency | Config A: 4GHz Config B: 700MHz |
| Antenna config | Config A: 1T8R Config B: 1T4R |
| Antenna pattern | 0dB omni-directional gain |
| channel model | TDL-C, 363ns |
| Speed | 3km/h |
| Channel estimation | Practical |
| Data and RB allocation | TBS = 256 bits : Config A: 4-os, MCS3, 61RB 4-os, MCS4, 50RB Config B: 4-os, MCS2, 78RB |
| Transmission scheme | No HARQ |
| Overhead | 1 symbol DMRS accounted |

According to the configuration parameters above, the BLER curves are given as follows, which are used to calculate the successful probability of PUSCH transmission p2 in Table 8. and Table 9.

| Figure 6: PUSCH simulation results for Config A | Figure 7: PUSCH simulation results for Config B |
| --- | --- |

- PUCCH

PUCCCH LLS assumptions are listed in Table 6. For latency reduction, 1 symbol PUCCH format 0 with 1 bit UCI is evaluated.

Table 6: PUCCH simulation assumptions

| Numerology | 30kHz SCS |
| --- | --- |
| Waveform | CP-OFDM |
| Carrier frequency | Config A: 4GHz Config B: 700MHz |
| Antenna config | Config A: 1T8R Config B: 1T4R |
| Antenna pattern | 0dB omni-directional gain |
| channel model | TDL-C, 363ns |
| Speed | 3km/h |
| PUCCH activation detection | Enabled |
| PUCCH format | 1 symbol PUCCH format 0 with 1 bit UCI |
| PUCCH RB allocation | 1RB |

In figure8, PUCCH simulation results are given in terms of  NACK to ACK probability and SR missing probability.

Figure 8: PUCCH simulation results

For PUCCH simulation, p0 (successful probability of SR detection), p3 (successful probability of PUCCH NACK detection) and p4 (successful probability of PUCCH DTX detection) as listed in Table 2 need to be evaluated. Due to the conduction of PUCCH activation detection, the DTX detection successful probability p4 is always 99%. p0 and p3 can be calculated according to NACK to ACK probability and SR missing probability respectively.

- Total reliability analysis

In this section, total reliability for 5%-tile pre-processing SINR point is calculated according to [5]. Considering possible situations when there is no time left for retransmissions, evaluation should be done at least for one transmission tempt firstly, that indicate whether NR capability can fulfill the requirement when there is only one transmission for an emerged packet arrival.

Proposal 1: At least one transmission tempt within 1ms latency shall be evaluated for Reliability.

- DL reliability

For the DL transmission scheme as shown in figure 9, DL total reliability (without soft combination) can be expressed as

(1)

Figure 9: DL data transmission

For one transmission tempt, i.e. K=1, if slot aggregation scheme is conducted with M = 2, non-slot length = 4 symbols, total reliability results at 5%-tile SINR point are given in Table 7.

Table 7: One transmission tempt reliability with M = 2 for Config B

| Configuration | Channel model | 5%-tile SINR | 1-p1 | 1-p2 (4-os, MCS3, 58RB) | 1-p2 (4-os, MCS5, 37RB) | Total reliability (4-os, MCS3, 58RB) | Total reliability (4-os, MCS5, 37RB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Config B | Model A | -2.55 | 0.71*10-5 | <1.25*10-5 | <1.67*10-5 | >0.9999928998 | >0.9999928997 |
|  | Model B | -2.46 | 0.71*10-5 | <1.25*10-5 | <1.67*10-5 | >0.9999928998 | >0.9999928997 |

Observation 1:  DL One transmission tempt with M = 2 can fulfill the 10-5 reliability requirement of IMT-2020.

It is noted that with high M, e.g. M = 3 would further improve the reliability.

- UL reliability

For the UL grant free transmission scheme as shown in figure 10, UL total reliability (without soft combination) can be expressed as

(2)

Figure 10: UL data transmission without grant

For one transmission tempt, i.e. K=1, if slot aggregation scheme is conducted with M = 2, total reliability results at 5%-tile SINR point are given in Table 8.

Table 8: One transmission tempt reliability with M = 2 for Config A

| Configuration | Channel model | 5%-tile SINR | 1 - p2 (4-os, MCS3, 61RB) | 1 - p2 (4-os, MCS4, 50RB) | Total reliability (4-os, MCS3, 61RB) | Total reliability (4-os, MCS4, 50RB) |
| --- | --- | --- | --- | --- | --- | --- |
| Config A | Model A | -4.01 | < 1.00*10-4 | < 3.00*10-4 | > 0.99999999 | > 0.99999991 |
|  | Model B | 0.39 | < 1.00*10-4 | < 3.00*10-4 | > 0.99999999 | > 0.99999991 |

Table 9: One transmission tempt reliability with M = 2 for Config B

| Configuration | Channel model | 5%-tile SINR | 1 - p2 (4-os, MCS2, 78RB) | Total reliability |
| --- | --- | --- | --- | --- |
| Config B | Model A | -0.04 | < 4.00*10-4 | > 0.99999984 |
|  | Model B | 1.25 | < 4.00*10-4 | > 0.99999984 |

Observation 2: UL One transmission tempt with M = 2 can fulfill the 10-5 reliability requirement of IMT-2020.

It is noted that with high M, e.g. M = 3 would further improve the reliability.

- UP latency analysis

Referring to NR UP latency results in [7], UP latency for slot-based DL and UL transmission without HARQ for UE capability #2 under 30kHz SCS are summarized in Table 10 and Table 11.

Table 10: UP latency results for DL slot-based transmission without HARQ for UE capability #2, 30kHz SCS

| UP latency calculation method | FDD | TDD (DDSUU) | TDD (DU) |
| --- | --- | --- | --- |
| Average method | 0.93 ms | 1.23 ms | 1.18 ms |
| Best case method | 0.68 ms | 0.68 ms | 0.68 ms |

Table 11: UP latency results for UL slot-based transmission without HARQ for UE capability #2, 30kHz SCS

| UP latency calculation method | FDD | TDD (DDSUU) | TDD (DU) |
| --- | --- | --- | --- |
| Average method | 0.93 ms | 1.53 ms | 1.18 ms |
| Best case method | 0.68 ms | 0.68 ms | 0.68 ms |

Observation 3: For FDD, DL and UL slot-based transmission without HARQ for UE capability #2 under 30kHz SCS can fulfill 1ms latency limitation by average method and best case method. For TDD, 1 ms requirement can be fulfilled by best case method.

- Conclusion

In this contribution, reliability evaluation is discussed and the following proposal is provided:

Proposal 1: At least one transmission tempt within 1ms latency shall be evaluated for Reliability.

The following observations are made:

Observation 1:  DL one transmission tempt with M = 2 can fulfill the 10-5 reliability requirement of IMT-2020.

Observation 2: UL one transmission tempt with M = 2 can fulfill the 10-5 reliability requirement of IMT-2020.

Observation 3: For FDD, DL and UL slot-based transmission without HARQ for UE capability #2 under 30kHz SCS can fulfill 1ms latency limitation by average method and best case method. For TDD, 1 ms requirement can be fulfilled by best case method.

- References

- RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- R1-1805643, “Summary of discussion on mobility evaluation method and parameters”, Huawei

- R1-1807760, “Summary on offline discussion on URLLC evaluation method and parameters for IMT-2020 self-evaluation”, Huawei

- R1-1805796, “draft CR to 38.214 capturing the RAN1#92bis meeting agreements”, April. 2018.

- R1-1808425, “Self evaluation on User Plane latency for NR”, CATT

Appendix I

Simulation assumption of SLS part for reliability

|  | Urban Macro - URLLC |
| --- | --- |
| Evaluation configuration | Configuration A, Configuration B |
| Carrier frequency for evaluation | Configuration A : 4 GHz Configuration B :700 MHz |
| Multiple access (DL/UL) | OFDMA |
| Duplexing | FDD |
| Transmission scheme | SU-MIMO with rank 1 |
| BS antenna height | 25 m |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm |
| Percentage of high loss and low loss building type | 100% low loss (applies to Channel model B) |
| Inter-site distance | 500m |
| Number of antenna elements per TRxP | Config A: 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1) Config B: 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1) (dH,dV) = (0.5, 0.8)λ, +45°, -45° polarization |
| Number of TXRU per TRxP | Config A: 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) Config B: 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) |
| Number of UE antenna elements | Config A: 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1) Config B: 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) (dH,dV) = (0.5, N/A)λ0°,90° polarization |
| Number of TXRU per UE | Config A: 4TXRU (1-to-1 mapping) Config B: 2TXRU (1-to-1 mapping) |
| Device deployment | 80% outdoor ,20% indoor Randomly and uniformly distributed over the area |
| UE speeds of interest | 3 km/h for indoor and 30 km/h for outdoor |
| Traffic model | Full buffer |
| Simulation bandwidth | 10 MHz |
| UE density | 10 UEs per TRxP |
| UE antenna height | 1.5 m |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B |
| TRxP number per site | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | 99° in LCS |
| Handover margin (dB) | 0 |
| TRxP boresight | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 |
| Numerology | one slot with 15 kHz SCS |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Pre-processing SINR calculation | Aligned with Section 2.1.1 in R1-1805643 |