3GPP TSG RAN WG1 94 Meeting	R1-1808541

Gothenburg, Sweden, Aug 20th – 24th, 2018

Source:	Polaran

Title:	A comparison of polar and LDPC codes for URLLC

Agenda Item:	7.2.1.5

Document for:	Information

# Introduction

CRC Aided (CA) polar codes are selected for eMBB control channels; however, there is no consensus on URLLC channel coding scheme which requires more reliable transmission and shorter latency. Design constraints for URLLC are described in [1] as the following:

- Information packet length is 32 bytes.

- User plane latency is at most 1 ms.

- Error-floor-free BLER as low as 10-5with or without HARQ support during 1 ms period.

In this contribution, a comparison of polar and LDPC codes under these design constraints is given. Simulations are performed with QPSK modulation, AWGN channel and MIMO configuration with one transmit and two receive antennas.

# Discussion

### Polar Code

Code Construction: CA polar codes with 24 bit CRC are used. The maximum polar code block length is Nmax. The code construction of the length-Nmax polar code is performed according to the construction method defined in [2]. For smaller length-N polar codes (eg. N=Nmax2) the least reliable N bit indices of length-Nmax reliability indicators are chosen as introduced in [3]. CRC attachment (with 24C polynomial), block segmentation, sub-block interleaving and rate matching (puncturing and repetition) methods in [3] are applied. Different from [3], code segmentation is used when the number of coded bits (E) is greater than Nmax. For the number of information bits A={104, 256, 408, 1032}, Nmax is fixed to 8192. For A=1672, two different Nmax=16384 and Nmax=8192 values are utilized for comparison.

Decoding: Polar decoder utilizes successive cancellation list (SCL) algorithm with adaptive list sizes, 1, 2, 4, 8, 16 and Lmax =32. If the list-L decoder has a bit error, then list-2L decoder becomes operational and the list size increases recursively until L= Lmax.

### LDPC Code

LDPC code simulation results are taken from [4] as benchmark performance.

Table 1: Polar code and LDPC code simulation parameters

| Number of Information Bits (A) | Target Rate (R) | #PRBs Allocated | #REs Allocated | Number of Coded Bits (E) | LDPC (with 16 bits CRC) | Polar (with 24 bits CRC) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Encoding Rate | Polar Code Block Length (N) | Number of Code Segments | Encoding Rate | Rate Matching Method |
| 104 | 1/4 | 3 | 216 | 432 | 0.2778 | 512 | 1 | 0.2963 | Puncturing |
|  | 1/6 | 5 | 360 | 720 | 0.1667 | 1024 | 1 | 0.1778 | Puncturing |
|  | 1/12 | 9 | 648 | 1296 | 0.0926 | 1024 | 1 | 0.0987 | Repetition |
| 256 | 1/3 | 6 | 432 | 864 | 0.3148 | 1024 | 1 | 0.3240 | Puncturing |
|  | 1/6 | 11 | 792 | 1584 | 0.1717 | 2048 | 1 | 0.1768 | Puncturing |
|  | 1/12 | 22 | 1584 | 3168 | 0.0859 | 2048 | 1 | 0.0884 | Repetition |
| 408 | 1/3 | 9 | 648 | 1296 | 0.3272 | 2048 | 1 | 0.3333 | Puncturing |
|  | 1/6 | 17 | 1224 | 2448 | 0.1732 | 4096 | 1 | 0.1765 | Puncturing |
|  | 1/12 | 34 | 2448 | 4896 | 0.0866 | 4096 | 1 | 0.0882 | Repetition |
| 1032 | 1/3 | 22 | 1584 | 3168 | 0.3308 | 4096 | 1 | 0.3333 | Puncturing |
|  | 1/6 | 43 | 3096 | 6192 | 0.1693 | 8192 | 1 | 0.1705 | Puncturing |
|  | 1/12 | 86 | 6192 | 12384 | 0.0846 | 8192 | 1 | 0.0853 | Repetition |
| 1672 | 1/3 | 35 | 2520 | 5040 | 0.3349 | 8192 | 1 | 0.3365 | Puncturing |
|  | 1/6 | 70 | 5040 | 10080 | 0.1675 | 16384 | 1 | 0.1682 | Puncturing |
|  | 1/12 | 140 | 10080 | 20160 | 0.0837 | 16384 | 1 | 0.0841 | Repetition |
| 1672 | 1/3 | 35 | 2520 | 5040 | 0.3349 | 4096 | 2 | 0.3413 | Puncturing |
|  | 1/6 | 70 | 5040 | 10080 | 0.1675 | 8192 | 2 | 0.1706 | Puncturing |
|  | 1/12 | 140 | 10080 | 20160 | 0.0837 | 8192 | 2 | 0.0853 | Repetition |

# Simulation Results

Figures in this chapter are the results of the simulations that are performed according to the parameters given in Table 1. The LDPC codes proposed in [4] are used for comparison with the polar codes with adaptive list size. The list size of the polar code is indicated under the simulation curves.

BLER versus SNR performance results of polar and LDPC codes with 104 information bits are depicted in Figure 1. As it can be observed from Figure 1, the polar code with list size 8 performs more or less the same with the LDPC code for all given code rates.

Figure 1 LDPC vs Polar (List-1, 2, 4, 8, 16, 32) for A = 104, AWGN channel

BLER versus SNR performance of polar and LDPC codes with 256 block size are depicted Figure 2 which shows that the polar code with list size 8 has slightly better performance compared to the LDPC code for all given code rates.

Figure 2 LDPC vs Polar (List-1, 2, 4, 8, 16, 32) for A = 256, AWGN channel

BLER versus SNR performance of polar and LDPC codes with 408 information bits are depicted in Figure 3 which shows that the polar code with list-16 has comparable performance with the LDPC code for all cases.

Figure 3 LDPC vs Polar (List-1, 2, 4, 8, 16, 32) for A = 408, AWGN channel

BLER versus SNR performance of polar and LDPC codes with 1032 information bits are depicted in Figure 4 which shows that the polar code with list-16 has almost the same performance with the LDPC code, for all given code rates, when the target BLER is smaller than 10-5.

Figure 4 LDPC vs Polar (List-1, 2, 4, 8, 16, 32) for A = 1032, AWGN channel

BLER versus SNR performance of polar and LDPC codes with 1672 information bits are depicted in Figure 5 which shows that the polar code with list-32 has the same performance with the LDPC at 1/6 and 1/12 code rates when BLER is smaller than 10-5.

Figure 5 LDPC vs Polar (List-1, 2, 4, 8, 16, 32) for A = 1672, AWGN channel

BLER versus SNR performance of polar and LDPC codes with 1672 information bits are depicted in Figure 6. If the polar code is transmitted in two code segments, the LDPC code outperforms the polar code for all cases at 10-5 BLER.

Figure 6 LDPC vs Polar (List-1, 2, 4, 8, 16, 32) for A = 1672, AWGN channel

Sufficient list size for polar SCL decoder in order have comparable performance with the LDPC code at BLER 10-5 is depicted in Table 2 which shows that segmented polar codes with 1672 information bits are outperformed by benchmark LDPC codes. However, if the segmentation is not utilized, the performance loss of polar codes can be recovered.

|  |  | Number of Information Bits (A) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 104 | 256 | 408 | 1032 | 1672 | 1672* |
| Code Rate (R) | 1/3 | 4t | 8 | 16 | 16 | >32 | >32 |
|  | 1/6 | 8 | 8 | 16 | 16 | 32 | >32 |
|  | 1/12 | 8 | 8 | 4 | 8 | 32 | >32 |

t Target information code rate is 1/4.

* Number of segments is 2.

Table 2 Sufficient list size of polar code to outperform LDPC code at BLER 10-5

Observation 1: The polar codes with list-32 SCL algorithm without segmentation perform slightly better than LDPC codes at code rates, 1/12 and 1/6.

Observation 2: The polar codes become inferior to LDPC codes under segmentation.

Observation 3: Although the benchmark LDPC codes experience noticeable performance degradation at target URLLC BLERs, the simulated polar codes do not experience such degradation.

# Conclusions

In this contribution performance of the CA polar codes under SCL decoding algorithm are evaluated. Simulation results show that proposed polar codes with list-32 has comparable performance with the LDPC codes  which is proposed for URLLC scenarios. Thus, polar code is a promising candidate for URLLC channel coding.

Observation 1: The polar codes with list-32 SCL algorithm without segmentation perform slightly better than LDPC codes at code rates, 1/12 and 1/6.

Observation 2: The polar codes become inferior to LDPC codes under segmentation.

Observation 3: Although the benchmark LPDC codes experience noticeable performance degradation at target URLLC BLERs, the simulated polar codes do not experience such degradation.

Proposal 1: CRC optimization should be carried out for each case depending on different number of coded bits.

Proposal 2: Proposed polar codes should be selected for URLLC.

# References

  - TR 38.913, Study on Scenarios and Requirements for: Study on New Radio Access Technology.

[2]     R1-1700088, Summary of Polar Code Design for Control Channels, Huawei  HiSilicon, 3GPP TSG RAN WG1 Ad-Hoc Meeting, Spokane, USA, January 16th - 20th 2017.

[3]      TS 38.212 v15.2.0, NR; Multiplexing and channel coding, June 2018.

[4]      R1-1806006, Channel Coding Techniques for URLLC, Ericsson, 3GPP TSG RAN WG1 93 Meeting, Busan, Korea, May 21st - 25th 2018.