3GPP TSG RAN WG1 Meeting #94								R1-1809063 Gothenburg, Sweden, Aug 20 – 24, 2018

Agenda Item:	7.2.2.4.1

Source:	AT&T

Title:	Enhancements for NR-U Channel Access Procedures

Document for:	Discussion/Approval

# Introduction

A study item has been approved for the feasibility study of using NR in unlicensed band in RAN#75 [1]. Following the success of LAA in unlicensed band, there is a lot of interest to develop features and functionality that allow NR to operate in unlicensed bands. There are many features of NR such as self-contained sub-frames, mini slots, flexible numerology that made NR a better candidate for operating in unlicensed band as compared to LTE. However, in order to ensure that we have a robust design of NR that is not only efficient but also forward compatible, we must start by putting forth requirements and enhancements that will ensure the design of NR in unlicensed bands achieves these goals. Progress has been made in RAN1#92 and 93 and the following conclusions have been reached.

Agreement:

- LTE-LAA channel access mechanism is adopted as baseline for 5GHz

  - Further enhancements not precluded

- LTE-LAA channel access mechanism is adopted as starting point of the design for 6GHz

  - Further enhancements not precluded

- For 5GHz band, a no-LBT option is beneficial for NR-U, such as for supporting fast A/N feedback, and is permitted per regulation.

  - Restrictions/conditions on when no-LBT option can be used will be further identified, e.g., in consideration of fair coexistence.

- No-LBT option can be applied to 6GHz band if allowed by regulation

  - Restrictions/conditions on when no-LBT option can be used will be further identified, if fair coexistence criterion is defined for 6GHz band

Note: Channel access mechanisms need to comply with regulations and may therefore need to be adapted for particular frequency ranges.

- Agreement:

- Benefits of using a signal that facilitates its detection with low complexity can be investigated including all/part of the following scenarios/use cases:

  - UE power saving

  - Improved coexistence

  - Spatial reuse at least within the same operator network

  - Serving cell transmission burst acquisition

  - FFS: further usage scenarios

In this document, we discuss channel access procedures that should be studied to enable NR to operate and coexist in unlicensed bands.

# Channel Access Procedures

By design NR is better suited for unlicensed channel access. Flexible frame structure – mini-slots and higher subcarrier spacing allow higher granularity of access thereby increasing channel access opportunities. Layer 1 carrier aggregation can help access large channel bandwidths and beamforming can enable higher spectral reuse.

## Beam Based Access

NR systems, especially for mmWave spectrum, will have a large number of antenna elements which could be used for analog, digital or hybrid beamforming. With Time Division Duplex (TDD) transmission, every transmit beam has a corresponding receive beam with identical characteristics. Using this property, a transceiver can tell if other users are active on some beams but not on other beams. This allows the transceiver to use the inactive beams for its transmissions, thus increasing channel reuse efficiency without causing interference.

When using transmit beamforming, a node may have to reduce its transmit power to meet EIRP limits. This, however, does not mean that the node will have to yield to other nodes transmitting from farther than what it can reach. That’s because under current rules the node is allowed to increase its energy detection threshold Level (TL) by the same amount as the reduction in the transmit power (up to 10 dB). As a result, its sensing distance would remain the same as its coverage.

The ED Threshold Level is given by:

For PH ≤ 13 dBm:			TL = -75 dBm/MHz

For 13 dBm < PH < 23 dBm: 	TL = -85 dBm/MHz + (23 dBm - PH)

For PH ≥ 23 dBm:			TL = -85 dBm/MHz

Where PH is the transmit power of the node. For example, a node with omni-directional antennas with 6 dBi gain and 23 dBm transmit power must use TL = -72 dBm for a 20 MHz channel. On the other hand, a node with beamforming antennas with 16 dBi gain and 13 dBm transmit power can use TL = -62 dBm.

Transmission and reception in narrow beams allows for higher spatial reuse, increasing spectrum reuse efficiency. Contrary to popular belief, beam based access does not worsen hidden / exposed node problems. Rather it reduces these problems as it shrinks the region where these problems occur as illustrated below. Triangles represent transmitting nodes and rectangles represent receiving nodes.

Beam based access

No contention

Omnidirectional access

Contention

Beam based access

No hidden node problem

Omnidirectional access

Hidden node problem

Beam based access

No exposed node problem

Omnidirectional access

Exposed node problem

Further discussion on beam based channel access and beam management can be found in [2] and [3].

Proposal 1: Beam based channel access should be studied for improving channel reuse and reducing hidden / exposed node problems.

## Fast Channel Access

Some NR transmission bursts are quite short. Examples include HARQ response and SS Block transmission. A 4 symbol SS Block with 120 kHz SCS takes only 36 us which is shorter than a defer period (43 us). Such transmissions may not cause much interference. Therefore, channel access without LBT could be considered.

Proposal 2: For time-sensitive transmissions that are short in duration, channel access without LBT should be studied.

## Closed-Loop License Assisted Access

As part of the LBT procedure the devices perform spectrum sensing also known as Clear Channel Assessment (CCA), where multiple time/frequency slots are measured with respect to a configured energy detection (ED) threshold. While LBT performed independently at a transmitting node can be used to avoid collisions of transmissions at a target receiver, the performance may suffer from so-called “hidden node problems” if the interfering transmitting nodes are outside the sensing range of the transmitting node as shown in Figure 1.

Figure 1. Hidden Node Problem

However, joint operation of licensed (NR-L) and unlicensed (NR-U) carriers under a License Assisted Access (LAA) framework can be used to help overcome the challenges of hidden nodes and associated latency incurred by LBT procedures on unlicensed carriers. Closed-Loop LBT is one potential enhancement to basic LBT procedures which can enable simultaneous carrier sensing at the gNB and UE, and then based on LBT feedback provided to the transmitter avoid missed LBT detection at the transmitter and unwanted transmission collisions at the receiver. In addition the LBT feedback may be used to adapt various LBT parameters over time such as sensing threshold, duration, priority, etc.

An example of Closed-Loop LBT is shown in Figure 2 below. The gNB1 serving UE1 sends a trigger on the licensed (NR-L) carrier which indicates to the UE to perform sensing on the NR-U carrier simultaneously with the gNB. After carrier sensing is performed, UE1 provides feedback of the CCA result on the NR-L carrier and the network can determine whether to schedule (DL/UL) transmissions on the NR-U carrier based on the LBT feedback in conjunction with the result of the gNB’s own sensing.

Figure 2. Example Closed-Loop LBT Scenario

Observation 1: By utilizing the licensed carrier, LBT procedure can become more robust with lower latency/overhead than techniques which can only utilize unlicensed spectrum.

Closed-Loop LBT can additionally be extended to enable synchronous LBT across multiple UEs. In Figure 3, the gNB1 sends LBT triggers to both UE1 and UE2 which aligns their clear channel assessment (CCA) periods. When both UEs complete the sensing they send LBT feedback messages on the NR-L carrier providing the sensing result. This enables the gNB to determine which of the UEs should be scheduled based on whether the channel is clear on both ends of the gNB/UE links. In case multiple UEs indicate clear channel status, the gNB may schedule them simultaneously for example with (DL or UL) multi-user MIMO transmissions, increasing the spectral efficiency of the NR-U carrier.

Figure 3. Multi-User Closed-Loop LBT Scenario

Observation 2: In case multiple UEs indicate clear channel status, the gNB may schedule them simultaneously for example with multi-user MIMO transmissions, increasing the spectral efficiency of the NR-U carrier.

In addition to achieving synchronizing LBT between the transmitter and receivers on the same cell, Closed Loop LBT can be utilized to achieve LBT synchronization across cells. This is beneficial when nearby cells are deployed by the same operator and in this case the overhead of LBT can be reduced or eliminated through spectrum reuse (e.g. reuse-1) for transmissions from the same operator, with minimal backhaul coordination. Figure 3 gives an example of a multi-cell closed-loop LBT scenario.

Figure 4. Multi-Cell Closed-Loop LBT Scenario

Since the reuse-1 transmissions are from nodes of the same operator the interference can be managed using CSI measurements and reports and is expected to be significantly less of a factor than interference in the case of transmissions from nodes which are not part of the same network.

Observation 3: Closed-Loop LBT can be used to enable functionality for NR-U which maximizes frequency reuse, especially when operating under light load or sparse deployment.

Proposal 3: The NR-U SI should study Closed-Loop LBT techniques which utilize licensed spectrum signaling and UE sensing feedback for avoiding hidden/exposed node problems and to enable efficient spectrum utilization through multi-user MIMO and multi-cell reuse-1 transmissions.

## Preamble Design

Preamble detection is used in Wi-Fi to coexist among Wi-Fi nodes. Preamble improves coexistence among Wi-Fi nodes because it can be detected more reliably than energy due to coding gain. Preamble based channel access enables higher spatial reuse by sharing information about other nodes’ channel use. Preamble detection consumes less power at the UE than energy detection. These and potentially other benefits warrant consideration of preamble based channel access for NR-U. Preamble design should at least enable inter-operator NR-U coexistence.

Observation 4: Preamble based channel access should be considered for at least inter-operator NR-U coexistence

The preamble could be based on transmission of SS Blocks, CSI-RS or both which can either be TDM’ed or FDM’ed with symbols transmitted aperiodically, for example at the start of a data burst. Additional information to consider including in the preamble along with NAV is communication direction, beam pair information, etc.

Proposal 4: Preamble should be considered to improve NR channel access without affecting coexistence with legacy technologies

# Summary

In this contribution, we discussed channel access enhancements needed for NR operation in unlicensed bands. We made the following proposals:

Proposal 1: Beam based channel access should be studied for improving channel reuse and reducing hidden / exposed node problems.

Proposal 2: For time-sensitive transmissions that are short in duration, channel access without LBT should be studied.

Proposal 3: The NR-U SI should study Closed-Loop LBT techniques which utilize licensed spectrum signaling and UE sensing feedback for avoiding hidden/exposed node problems and to enable efficient spectrum utilization through multi-user MIMO and multi-cell reuse-1 transmissions.

Proposal 4: Preamble based channel access should be considered for NR-U

# References

[1] RP-170828, New SID on NR-based Access to Unlicensed Spectrum, Qualcomm, 3GPP TSG RAN Meeting #75, March 2017

[2] R1-1804666, Physical Layer Enhancements for NR Unlicensed, AT&T, 3GPP TSG RAN Meeting #92bis, April 2018

[3] R1-1806990, Beam Measurement for NR Unlicensed, AT&T, 3GPP TSG RAN Meeting #93, May 2018

[4] R1-1804665, Closed Loop LBT for License Assisted NR, AT&T, 3GPP TSG RAN Meeting #92bis, April 2018