3GPP TSG RAN WG1 Meeting #94                           	                  R1-1809368

Gothenburg, Sweden, August 20- 24, 2018

Agenda Item:	6.1.7

Source:	Huawei, HiSilicon

Title:	Views on LTE Rel-15 UE features

Document for:	Discussion and decision

# Introduction

GSMA Intelligence issued a report “Global Mobile Trends 2017” [1] shows that 4G will dominate in volume terms for at least the next 10 years although 5G continues to occupy thought space as the next big thing in mobile. It was forecasted that 4G will account for two thirds of the global mobile user by 2025. Therefore, LTE will continue to play an import role in the mobile network market, and it is worth to investigate Rel-15 UE features carefully.

Among the Rel-15 UE features, sTTI with benefits of short latency and high throughput is a compelling feature. This contribution focuses to provide views on sTTI UE feature.

# Discussion on sTTI UE feature

## sTTI is key solution for low latency

Low network latency opens new business opportunities, such as mobile AR, drone, telemedicine, cloud VR, robot, etc. It also promotes new business model as the centralized business model for example as illustrated in the top right figure in Figure 1.

There is a test case showing that the user experience of real-time gaming is improved significantly by using sTTI as shown on the left in Figure 1. There is also evidence showing that users are willing to pay for low game latency by which it is easier to win.

Figure 1: sTTI is key solution to reduce radio latency

## sTTI improves user experienced throughput

sTTI enables a packet to be transmitted in a short time, so that user experienced throughput (UPT) is expected to be increased. Simulation results illustrated in Figure 2 demonstrate the mean UPT gains introduced by sTTI.

Figure 2: sTTI increases user experienced throughput

In addition, targeting the lowest delays is also essential for the 5G requirements on latency [2], which are in reach of LTE. For eMBB cases, the minimum requirement for user plane latency is 4 ms, which can only be achieved by sTTI operation. For URLLC cases, the minimum requirement for user plane latency is 1 ms, which can only be met by the sTTI length combination {2, 2}.

Therefore, we propose the sTTI length combination {2, 2} as least for band 3, band 8, and band 20 is mandatory, and to mandate the basic sTTI operation including sDCI monitoring, transmission of SPUCCH in formats 1 and 4, transmission of subslot/slot based PUSCH on a single antenna port, and decoding of CRS based subslot/slot based PDSCH. The UE feature list for sTTI is updated as appended in the end of this contribution based on R1-1807458.

Proposal 1: Mandate the sTTI length combination {2, 2} as least for band 3, band 8, and band 20, as well as the basic sTTI operation as updated as in the appendix.

# References

“Global Mobile Trends 2017,” GSMA Intelligence, September 2017.

TR 38.913, “Study on scenarios and requirements for Next Generation Access Technologies”.

| Features | # | Feature group | Components | Prerequisite feature groups (listed in this sheet only) | Need for gNB to know whether the feature is supported by the UE (what happens if gNB does not know?) | Consequences if the feature is not supported by the UE | Need of FDD/TDD differentiation | RAN5 implication | Note | Responsible WG | RAN WG recommendation | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5. Shortened TTI and PT | 5-1 | Short processing time operation | 1) Configuration of short processing time 2) Support of n+3 processing time between UL DCI and PUSCH transmission 3) Support of n+3 processing time between PDSCH and DL HARQ feedback transmission in PUCCH. * UE capability is reported per band combination. For each band combination the maximum number of supported CCs for short processing time are reported. The reported number of carriers apply to all the FS-type(s) supported. The supported FS-type(s) are also reported per band combination.. * The maximum number of CSI processes for aperiodic CSI is also reported for each band combination |  | yes | Network cannot utilize short processing time | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-2 | EPDCCH with SPT | Support of EPDCCH and short processing time on different serving cells. | 5-1 | yes | Network cannot utilize EPDCCH and SPT on different carriers | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-3 | sTTI operation | 1) sTTI length combination (2, 2) 2) sDCI monitoring in PDCCH and CRS based SPDCCH 3) Transmission of SPUCCH in formats 1 and 4 4) Transmission of subslot/slot based PUSCH on a single antenna port 5) Decoding of CRS based subslot/slot based PDSCH * different sTTI capabilities for different UL/DL sTTI length combinations and for each band of the reported band combinations. * UE capability is reported on skipping subframe processing to start processing slot/subslot PDSCH/PUSCH * UE capability is reported on the maximum number of blind decodes in USS in one subframe for CCs configured with sTTI operation, including both legacy DCI formats and sDCI * UE capability is reported on the supported processing timeline sets for subslot operation |  | yes | Network cannot schedule sTTI length combination (2,2) data transmission | No need |  |  | RAN1 | 1) is mandatory at least for band 3, band 8, and band 20. 2), 3), 4), 5) are mandatory without capability signaling |  |
|  | 5-4 | sTTI operation with length combination (2, 7) | sTTI length combination (2, 7) | 5-3 | yes | Network cannot schedule sTTI length combination (2,7) data transmission | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-5 | sTTI operation with length combination (7, 7) | sTTI length combination (7, 7) | 5-3 | yes | Network cannot schedule sTTI length combination (7,7) data transmission | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-6 | DMRS based SPDCCH | Support of sDCI monitoring in DMRS based SPDCCH | 5-3 | yes | Network cannot utilize DMRS based SPDCCH | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-7 | SPDCCH monitoring on SPDCCH resource sets with different RS types | Support of monitoring sDCI in SPDCCH resource sets with different RS types in a slot/subslot | 5-3 and 5-4 | yes | Network cannot utilize SPDCCH resource sets with different RS types | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-8 | TM8 for slot/subslot PDSCH | 1) configuration of TM8 for slot PDSCH 2) Decoding of TM8 based slot PDSCH. | 5-3 | yes | Network cannot utilize TM8 based slot PDSCH | Only for TDD |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-9 | TM9 and TM10 for slot/subslot PDSCH | 1) configuration of TM9/TM10 for slot/subslot PDSCH 2) Decoding of TM9-10 based slot/subslot PDSCH. | 5-3 | yes | Network cannot utilize TM9/10 based subslot/slot PDSCH | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-10 | Tx Diversity for TM9/10 based slot/subslot PDSCH | 1) configuration of TM9/TM10 for slot/subslot PDSCH 2) Decoding of Tx diversity based subslot/slot PDSCH | 5-3 | yes | Network cannot utilize Tx diversity fallback for TM9 or TM10 | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-11 | SPUCCH format 3 | Support of transmission of SPUCCH in format 3 | 5-3 | yes | Network cannot utilize SPUCCH format 3 | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-12 | Tx Diversity for SPUCCH | SPUCCH format 1/1a/1b/3 transmission with Tx Diversity | 5-3 | yes | Network cannot utilize Tx diversity for SPUCCH format 1/1a/1b/3 | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-13 | TM2 for slot/subslot PUSCH | 1) configuration of TM2 for slot/subslot PUSCH 2) Transmission of subslot/slot based PUSCH * UE capability is reported on supported maximum number of layers for slot/subslot PUSCH | 5-3 | yes | Network cannot utilize UL TM2 for slot/subslot PUSCH | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-14 | Simultaneous reception of slot/subslot and subframe PDSCH | Support of simultaneous reception of subframe and slot/subslot PDSCH. | 5-3 | yes | Network cannot utilize simultanenous reception of subframe and slot / subslot PDSCH | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-15 | Simultaneous transmission of PUSCH of different lengths across carriers | Support of simultaneously transmission of PUSCH with different lengths across different carriers * capability reported per band/band combination | 5-3 | yes | Network cannot utilize simultanenous transmission of subslot/slot PUSCH of different lengths in different carriers across carriers | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-16 | A-CSI for sTTI | Support of aperiodic CSI feedback for sTTI * UE capability is reported on the maximum number of CSI processes supported on a CC within a band with slot/subslot PDSCH TM10 for a bandwidth class with a single CC * UE capability is reported on the maximum number of CSI processes supported on a single CC and on all CCs within a band with PDSCH TM10 for a bandwidth class with multiple CCs * UE capability is reported on the maximum number of CSI processes to be updated across CCs | 5-3 | yes | Network cannot utilize CSI on subslot/slot PUSCH | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-17 | EPDCCH with sTTI | Support of EPDCCH and sTTI on different serving cells. | 5-3 | yes | Network cannot utilize EPDCCH and sTTI on different carrier | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-18 | SRS triggers on sDCI | Support of SRS triggers in DCI formats 7.x for FS2 | 5-3 | yes | Network cannot utilize SRS triggers in DCI 7.x | Only for TDD |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-19 | SPS for sTTI | 1) Configuration of SPS with sTTI 2) Monitoring of sDCI for SPS activation and deactivation, 3) Decoding of SPS subslot/slot based PDSCH 4) Transmission of SPS slot/subslot based PUSCH | 5-3 | yes | Network cannot utilize SPS for sTTI | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-20 | L1 based SPDCCH reuse | 1) Configuration of L1 based SPDCCH reuse 2) Rate-matching of subslot/slot based PDSCH around SPDCCH resources according to configured L1 based SPDCCH reuse method | 5-3 | yes | Network cannot utilize L1 indication of rate matching method of subslot/slot based PDSCH around SPDCCH resources | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-21 | CS for SPS | 1) Configuration of cyclic shift in SPS for 1ms TTI UL SPS operation. 2) Transmission of DMRS for SPS PUSCH using the configured cyclic shift | SPS support for 1ms TTI | yes | SPS using different configured CS values not possible | No need |  |  | RAN1 | Optional with capability signaling |  |