3GPP TSG RAN1 Meeting #94                                                                   R1-1809416

Gothenburg, Sweden, 20th - 24th August 2018

Source: 	Thales

Title:	[NR-NTN] Link and system level performance evaluation for NR-NTN

TDOC Type:	Discussion

Agenda Item:	7.3

Document for:	Information

Release:	Re-16

Specification:       38.811

Study item:	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

A first Study Item “Study on NR to support non-terrestrial networks” was approved during the #80 meeting in La Jolla, with the following achievements

- Definition of deployment scenarios and related system parameters

- Adaptation of the 3GPP channel models for non-terrestrial networks

- Identification of any key impact areas on the New Radio

A second Study Item “Study on solutions for NR to support non-terrestrial networks” was approved during the same meeting, where solutions will be investigated on the key issues identified in [1].

This contribution proposes performance evaluation parameters for this new Study Item.

# References

[1] 3GPP TR 38.811 “Study on NR to support non-terrestrial networks”

[2] ETSI 302 307-1 v1.4.1 “Second generation framing structure, channel coding and modulation systems for Broadcasting, Interactive Services, News Gathering and other broadband satellite applications; Part 1 (DVB-S2)”, July 2014

[3] DVB Document A171-2, “Digital Video Broadcasting (DVB) Implementation guidelines for the second generation system for Broadcasting, Interactive Services, News Gathering and other broadband satellite applications; Part 2 - S2 Extensions (DVB-S2X)”, March 2015

[4] TR 38.803 “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects” v14.2.0

[5] 3GPP TR 38.802 “Study on New Radio (NR) Access Technology; Physical Layer Aspects“

[6] 3GPP R1-1802551 “UE antenna assumption and beam modeling for NTN”

# Discussion

Section 7 of [1] highlights the design constraints specific to non-terrestrial networks compared to cellular systems and the potential features of NR that may require modifications. From a RAN1 perspective, the table below summarizes these constraints and features, and identifies the required simulation type (link- or system level).

Table 1: Non-terrestrial design constraints and potential NR feature modifications from a RAN1 perspective

| Non-terrestrial design constraint | NR features that may require modifications | Simulation type needed |
| --- | --- | --- |
| Specific propagation channel (high Doppler, satellite impairments in case of a transparent satellite) | Reference signals Preamble sequence SCS | Link-level - study of new pilot arrangement/SCS if needed - related to initial access |
| Power limited link budget | Very low SNR MCS may be required | Link-level - study of new MCS if needed |
| Large differential delay due to large cell size | Initial access procedure | Link-level - related to initial access System-level |
| Large propagation delay | Initial access procedure HARQ procedure Power control command ACM | Link-level - related to initial access - channel coding (to target low BLER) System-level |
| Specific spectrum | Carrier numbering/pairing Carrier bandwidth Carrier aggregation method | System-level |

While RAN3 will provide reference scenarios, performance evaluation parameters need to be agreed at RAN1 level for both link-level and system-level simulations.

As reference scenarios are under discussion at RAN3 level, two scenarios issued from [1] will be considered

- A scenario with transparent GEO satellite

- A scenario with a LEO constellation at 600km altitude

The first scenario corresponds to the largest propagation delay while the second corresponds to the highest Doppler values that will be considered in this Study Item.

# UE characteristics

For performance simulations, it is proposed to adopt the RF characteristics of UE described in Table 2. These characteristics are derived from [1] and [6].

Table 2: RF characteristics of UE

|  | Fixed or mounted on moving platforms | Handheld for < 6GHz | Handheld for > 6GHz |
| --- | --- | --- | --- |
| Transmit Power | 2 W (33 dBm) | 200 mW (23 dBm) | 200 mW (23 dBm) |
| Antenna type | 60 cm equivalent aperture diameter (circular polarisation) | Co-phased array – Dual linear polarization as described in Table 1 of [6] | Co-phased array – Dual linear polarization as described in Table 1 of [6] |
| Polarisation | Circular | Linear | Linear |
| Number of antenna elements | 1 | 4 | 32 |
| Number of antenna pannels | N/A | 1 | 2 |
| Antenna panel orientation (bearing angle, downtilt) | N/A | (0°, 0°) | Panel 1: (0°, 0°) Panel 2: (180°, 0°) |
| Antenna panel configuration | N/A | (M, N, P) = (1, 2, 2) dH=0.5λ Polarization: 0°, 90° | M, N, P) = (2, 4, 2) dH, dV=0.5, 0.5λ Polarization: 0°, 90° |
| TXRU to element mapping | N/A | One TXRU per element | One TXRU per panel per polarization |
| Antenna element gain pattern | Very directional | Omni-directional | See Table 2 of [6] |
| Antenna element gain | Tx: 43.2 dBi Rx: 39.7 dB | Tx and Rx: 0 dBi | Tx and Rx: 5 dBi |
| Noise figure | 1.2 dB | 7 dB | 10 dB |

# Link-level simulation assumptions

The metrics for link-level simulations given in Section 7.1.1 of [5] are considered. Other metrics are not precluded (e.g. required satellite power per useful transmitted bit).

## Simulation assumption for evaluation of new pilot arrangement/SCS/MCS (if needed)

Table 3: Simulation assumptions for evaluation of new pilot arrangement/SCS/MCS

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 2/20 GHz |
| Duplex | FDD/TDD |
| Waveform | OFDM (Downlink) OFDM /SC-FDMA (Uplink) |
| System Bandwidth | 30 MHz for < 6GHz 400 MHz for > 6GHz |
| TTI length | 1 ms as baseline |
| Subcarrier spacing | 15kHz, 30kHz, 60kHz for f0 < 6GHz 60kHz, 120kHz f0 > 6GHz Other SCS may also be considered if needed |
| Guard time interval | Interval of normal CP as baseline |
| FFT size | e.g. 1024 for 15kHz subcarrier spacing |
| Data transmission bandwidth | FFS |
| Antenna configuration | 1T1R as baseline |
| MIMO mode | N/A |
| Rank per UE | N/A |
| MCS | QPSK 1/3 1/2 2/3 16-APSK 1/2 2/3 Other MCS for very low SNR may also be considered if needed |
| Control Overhead | Zero |
| Channel estimation * | Realistic (Pilot pattern to be given) |
| Channel Model ** | TDL from [1] |
| Phase noise model (only for >6GHz) | As proposed in [2][3][4] depending on the considered UE type, see Annex A.1 for typical values |
| Satellite power amplifier model (transparent scenarios only) | As proposed in [2] for GEO, see Annex A.2 FFS for LEO |

## Simulation assumption for evaluation related to initial access

Target requirements defined in Section 7.1.5 of [5] are considered as baseline, with an additional frequency offset from the satellite.

Table 4: Simulation assumptions for synchronization signals/channels

| Assumptions | Value |
| --- | --- |
| Carrier Frequency | 2GHz , 20GHz |
| Channel Model | TDL from [1] (TBC) |
| Subcarrier Spacing(s) | As in Table 2 |
| SNR range | > -6dB |
| UE speed | 3 km/h 500 km/h (mandatory) 1000km/h (optional in LEO, mandatory in GEO) |
| Search window | As in [5], Table A.1.5-1 |
| Frequency Offset | As in [5], Table A.1.5-1 |
| Doppler from satellite | 0ppm 12ppm 24ppm (TBC) |
| Number of interfering TRPs | 0 TRP (TBC) |

## Simulation assumption for evaluation of channel coding

Simulation assumptions for eMBB from [5], Table A.1.3-1, are considered (or a subset to be defined).

# System-level simulation assumptions

In the following, the coordinate system is the one described in Section 6.3 of [1] (“Earth Centred Earth fixed”).

## LEO constellation at 600km

For some performance evaluations (e.g. HARQ performance evaluation), it is expected that the simulation of a single satellite may be sufficient.

Table 5: Simulation assumptions for LEO constellation at 600km

| Parameter | Values |
| --- | --- |
| Carrier frequency | 2GHz and 20GHz |
| System Bandwidth | 30 MHz for f0 < 6GHz 400 MHz for f0 > 6GHz |
| Satellite orbit | 600 km altitude on polar plane |
| Number of satellites | 1 or to be provided by each proponent |
| Reference satellite initial position | (6971,0,0) Other values are not precluded |
| Beam layout | Regular, as illustrated in Figure 1 |
| Number of beams per satellite | To be provided by each proponent |
| Beam size | 200km |
| Satellite antenna pattern | As described in Section 6.4.1 of [1] |
| Satellite Tx power | FFS |
| Channel Bandwidth | 5/10/30MHz for f0< 6GHz 100/400MHz for f0>6GHz |
| UE orientation | FFS |
| Handover margin (for calibration) | 0dB |
| UE distribution | 100% outdoor Uniform dropping, number of users per beam FFS |
| UE attachment | Geometric |
| Traffic model | FFS |
| Traffic load | FFS |
| Fast fading channel | Fast fading channel is not modelled |

Figure 1: Example of regular beam layout (The centre of Beam 1 lies on the intersection between the earth surface and the axis satellite - earth centre)

As illustration, Table 4 gives the beam centre positions calculated from a given satellite position (6971,0,0) and the beam layout from Figure 1, assuming that the straight line from centre of beam 7 to the centre of beam 4 is parallel to the satellite direction. The distance between the centre of two neighboring beams is equal to 200km.

Table 6: Example of satellite and beam positions

| Satellite | (6971,0, 0) |
| --- | --- |
| Beam 1 | (6371,0,0) |
| Beam 2 | (6351,-433,-250) |
| Beam 3 | (6351,-433,250) |
| Beam 4 | (6351,0,500) |
| Beam 5 | (6351,433,250) |
| Beam 6 | (6351,433,-250) |
| Beam 7 | (6351,0,-500) |

## Transparent GEO satellite

Table 7: Simulation assumptions for transparent GEO satellite

| Parameter | Values |
| --- | --- |
| Carrier frequency | 2GHz and 20GHz |
| System Bandwidth | 30 MHz for f0 < 6GHz 400 MHz for f0 > 6GHz |
| Satellite orbit | Geostationary orbit |
| Number of satellites | 1 |
| Satellite position | (42157,0,0) |
| Beam layout | Regular, as illustrated in Figure 1 |
| Number of beams per satellite | To be provided by each proponent |
| Beam size | 500km |
| Satellite antenna pattern parameter | As described in Section 6.4.1 of [1] |
| Satellite Tx power | FFS |
| Channel Bandwidth | 5/10/30MHz for f0< 6GHz 100/400MHz for f0>6GHz |
| UE orientation | FFS |
| Handover margin (for calibration) | 0dB |
| UE distribution | 100% outdoor Uniform dropping, number of users per beam FFS |
| UE attachment | Geometric |
| Traffic model | FFS |
| Traffic load | FFS |
| Fast fading channel | Fast fading channel is not modelled |

# Annex A.1: Phase noise models in Ka band from [2][3]

Table 8: Example of typical phase noise mask in Ka-band (from 3])

| Offset (Hz) | SSB (dBc/Hz) |  |  |
| --- | --- | --- | --- |
|  | Uplink | Satellite | Downlink |
| 10 | -42 | -33 | -42 |
| 100 | -72 | -62 | -72 |
| 1K | -82 | -80 | -82 |
| 10K | -92 | -90 | -92 |
| 100K | -102 | -95 | -102 |
| 1M | -112 | -106 | -112 |
| 10M | -122 | -116 | -122 |
| 50M | -124 | -118 | -124 |

# Annex A.2: Power amplifier model from [2]

Figure 2: Power amplifier model from [2]