3GPP TSG RAN WG1 Meeting #94	R1-1809470

Gothenburg, Sweden, 20th– 24st August 2018

Agenda Item:	7.2.8

Source:			CEWiT

Title:			On BWP configuration for efficient spectrum utilization

Document for: 	Discussion and Decision

# Introduction

In 3GPP TSG RAN#80 under section 9.1.12 [1], proposals related to NR spectrum utilization efficiency enhancements for Rel.16  were discussed. A summary of email discussion was presented which considers different BWP enhancements including switching and adaptation enhancements. This document discusses a potential enhancement to the current BWP configuration in Rel.15 which provides better spectrum utilization.

# BWP Configuration

Consider a BWP whose size is to be determined as per the UE capability. Defining the numerology of the BWP as BWPμ , the maximum number of resource blocks (RBs) in the wideband carrier is considered as NRB using the numerology BWPμ. Now the RBs are respectively indexed as  0 to (NRB-1). Now a BWP configuration is defined using the parameters BWPμ ,  BWPbw and BWPloc.   A BWP is defined from a UE’s perspective, hence there can be multiple BWPs sharing the same resource.  This is represented in Fig.1

WB-CC

BWP3

BWP1

BWP2

Fig.1   Multiple BWPs configured in a WB-CC

Now the BWP configuration parameters can be summarized as

- BWPμ    – 	SCS of the BWP

- BWPbw – 	Size of the BWP in terms of BWPμ , with a resolution on 1 RB.

- BWPloc – 	Offset of the starting RB of the BWP with respect to zeroth RB (CRB0) of the WB-CC, in         terms of the number of RBs with respect to BWPμ .

BWP

CRB0

NRB

BWPbw

BWPloc

Fig.2   BWP configuration

# BWP Scheduling and Resource Wastage

Scheduling of the WB-CC is done by the base station, which indicates the scheduling decision to the user equipment (UE). Scheduling is done in terms of Resource Block Group (RBG), which is a collection of contiguous RBs, in the frequency domain. RBGbwp is defined as the number of RBs in an RBG for a given BWP. After scheduling, the scheduled RBGs are indicated to the UE using a RBG bitmap or using a start and length indicator value (SLIV). In type0 based resource allocation, a bitmap of size the number of RBGs is used for frequency allocation signaling.

Wastage due to offset

1RBG = 4RBs

BWP2

BWP1

Fig.3a Wastage of resources  because of misalignment due to offset

BWP1

BWP2

Wastage due to BWP size

Fig.3b Wastage of resources because of misalignment due to the size of BWP

Consider two overlapping BWPs as shown in Fig.3a and Fig.3b. With Type0 signaling of frequency allocation for BWP1, the RBGs corresponding to the edges of BWP2 can be signaled only with the resolution of 1 RBG. This leads to wastage of resources in BWP1, when BWP2 is not starting from the RBG boundary as shown in Fig.3a or when BWP2 is not ending at the RBG boundary as shown in Fig.3b. This is also true with the signaling for BWP2. The magnitude of waste can be quantified based on  RBGbwp of the BWP. Tab. 1 shows wastage of resources for different  RBGbwp

With respect to  BWPbw    and BWPloc respectively.

| RBGbwp | Wastage due to misalignment in BWPbw (in RBs) | Wastage due to misalignment in BWP BWPloc (in RBs) | Maximum number of RB’s wasted |
| --- | --- | --- | --- |
| 2 | 1 | 1 | 2 |
| 4 | 1-3 | 1-3 | 6 |
| 8 | 1-7 | 1-7 | 14 |
| 16 | 1-15 | 1-15 | 30 |

Table.1 Wastage of resources for different RBG sizes

Fig. 4a discusses a scenario where two BWPs with different BWPbw  and different BWPloc without a common factor. Now with Type0 allocation, RBG bitmap for BWP1 cannot distinguish the highlighted region from the BWP2 region to be excluded from scheduling.

Fig.4a Unaligned BWPs with RBG bitmap

Fig.4b Aligned BWPs with RBG bitmap

Proposal 1: Study the BWP configurations defined in Rel.15 to identify the configurations that are inefficient in terms of spectrum utilization and possible enhancements

3. Conclusion

In this contribution, we made the following proposal:

Proposal 1: Study the BWP configurations defined in Rel.15 to identify the configurations that are inefficient in terms of spectrum utilization and possible enhancements.

4. References

- RP-181491- Draft Report of 3GPP TSG RAN meeting #80 La Jolla, USA, June 11 – 14, 2018

- RP-181293- Summary of the mail discussion on NR spectrum utilization efficiency enhancements

- 38.214 V15.2.0 (2018-07)- 5G NR Physical layer procedures for data (3GPP TS 38.214 version 15.2.0 Release 15)

- 38.213 V15.2.0 (2018-06)- 5G NR Physical layer procedures for control (3GPP TS 38.213 version 15.2.0 Release 15)