3GPP TSG RAN WG1 Meeting #94	R1-1809481

Gothenburg, Sweden

Aug. 20th – Aug. 24th, 2018

Agenda Item:	7.2.2.4.3

Source: 		Qualcomm Incorporated

Title:			Enhancements to Scheduling and HARQ operation for NR-U                                
Document for: 	Discussion/Decision

# Introduction

In RAN#77 a new SI [1] for NR based access to unlicensed spectrum was approved which was then revised in RAN#80 [2]. The objectives of the SI include the following:

- Study NR-based operation in unlicensed spectrum (RAN1, RAN2, RAN4) including

  - Physical channels inheriting the choices of duplex mode, waveform, carrier bandwidth, subcarrier spacing, frame structure, and physical layer design made as part of the NR study and avoiding unnecessary divergence with decisions made in the NR WI

    - Consider unlicensed bands below 7GHz

    - Consider similar forward compatibility principles made in the NR WI

  - Initial access, channel access. Scheduling/HARQ, and mobility including connected/inactive/idle mode operation and radio-link monitoring/failure

  - Coexistence methods within NR-based and between NR-based operation in unlicensed and LTE-based LAA and with other incumbent RATs in accordance with regulatory requirements in e.g., 5GHz, 6GHz bands

    - Coexistence methods already defined for 5GHz band in LTE-based LAA context should be assumed as the baseline for 5GHz operation. Enhancements in 5GHz over these methods should not be precluded. NR-based operation in unlicensed spectrum should not impact deployed Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier;

In RAN1 meeting #92b, an agreement was reached to study possible enhancements to HARQ operation and in RAN1#93 the following agreements were reached regarding the HARQ operation.

- Transmission of HARQ A/N for the corresponding data in the same shared COT is identified as beneficial

  - Strive to support transmitting all HARQ A/N for the corresponding data in the same shared COT, if possible, considering the current NR UE processing time required

    - Mechanisms to support this need to be identified

  - It is understood in some cases, the HARQ A/N has to be transmitted in a separate COT from the one the corresponding data was transmitted

    - Mechanisms to support this need to be identified

- Techniques to handle reduced HARQ A/N transmission opportunities for a given HARQ process due to LBT failure are identified as beneficial

  - Potential techniques include mechanisms to provide multiple and/or supplemental time and/or frequency domain transmission opportunities

- NR-U uses NR HARQ feedback mechanisms as baseline, and enhancements can be identified

- When UL HARQ feedback is transmitted on unlicensed band, NR-U considers mechanisms to support flexible triggering and multiplexing of HARQ feedback for one or more DL HARQ processes

- Scheduling multiple TTIs for PUSCH each using a separate UL grant in the same PDCCH monitoring occasion is identified as beneficial

- Scheduling multiple TTIs for PUSCH using a single UL grant is identified as beneficial and should be supported in NR-U

In this contribution, we present our views on some of the remaining aspects of HARQ operation.

# PDSCH/PUSCH Scheduling

In unlicensed operation it is quite likely that there is a long string of DL slots followed by a long string of UL slots. Such a TxOP structure avoids frequent switches between DL and UL which in some cases involves LBT gaps and can lead to potential loss of the medium. This includes switching from DL to UL for HARQ ACK feedback. Furthermore, with use of mini-slots and slots with larger SCS, such as 60KHz for sub 7 GHz the number of required HARQ processes can get quite large. The number of HARQ processes will have to be designed to consider such long streak of DL only or UL only slots. An example of this is shown in Figure 1 where with an 8ms DL portion in a TxOP, we need at least 8 HARQ processes for 15Khz SCS but with 60KHz SCS the number of required HARQ processes to fill in the 8ms DL portion of the TxOP increases to 32. Note that the larger SCS is also desirable as it provides more start opportunities for the TxOP and hence improves the medium access probability and reduces LBT pass and slot boundary mis-alignment related overhead.

Figure 1: Number of HARQ processes for NR-U with larger SCS

In general, increasing the number of HARQ processes per cell has difficult software and hardware implementation implications. It will be preferred if an alternative approach can be used that takes advantage of the existing UE NR capability.

One way to mitigate the increase in the number of HARQ processes is to allow a single grant to schedule a single TB over multiple slots. In Figure 1, for example, with multi-slot grants that span 2 slots, the number of required HARQ processes reduces by a factor of two. The TB size with the multi-slot grant should scale proportionally with the number of slots and the TB should be rate-matched over multiple slots (rather than rate-matched for one slot and repeated over multiple slots which may have link-level performance losses). Such multi-slot grants will help control the DL overhead for PDCCH as well as the UL overhead for HARQ ACK feedback. These of course apply to both PDSCH and PUSCH scheduling.

Proposal 1: Introduce multi-slot grants with the TB(s) rate matched jointly across the multiple allocated slots

NR provides flexibility in the UL grant to specify a delay between the PDCCH and PUSCH as part of the time domain resource assignment which is a key requirement for enabling efficient unlicensed operation. One may consider enhancing the number of bits for the delay field to have more scheduling flexibility.

Proposal 2: Number of bits in the UL PUSCH time domain resource assignment field are increased

In RAN1#93, it was agreed to introduce multi-TTI UL grant. In unlicensed operation, to avoid frequent DL to UL switches (which may need LBT gap each time) the gNB may just schedule the same set of UE on multiple back to back slots. Having a multi-TTI UL grant enables this to be done in an efficient manner as it reduces the number of UL grants and the number of slots for just scheduling PUSCH.

To enable more LBT opportunities for the UE, it may also be desirable for the multi-TTI grant to support mini-slot type grant at the beginning which switches to full slot grant at slot boundaries. Some examples of how the multi-TTI grant would operate with LBT and with mix of mini-slots and full slots is shown in Figure 2.

Figure 2: Examples of operation with Multi-TTI grant

The multi-TTI grant should also support CBG based retransmission. To avoid significantly increasing the DCI payload size with multi-TTI grant, compression schemes for the CBG based retransmission indication should be considered.

Proposal 3: The multi-TTI grants should include support for transmitting mini-slots in the initial portion of the transmission followed by switching to full slot at slot boundaries.

Proposal 4: The multi-TTI grants should also support CBG based retransmissions

Figure 3: Two-stage grant for scheduling PUSCH

As shown in Figure 3, due to the delay between the UL grant sent in PDCCH in DL and the PUSCH transmission in UL, a node winning the medium could still lose it prior to the UL transmission. The delay itself is necessary as the UE has to decode the PDCCH to detect a grant and then it has to prepare the packet for PUSCH transmission. In eLAA a two-stage grant was introduced for PUSCH scheduling to address this issue. As shown in Figure 3, the two-stage grant splits the PUSCH scheduling is two parts. A first grant provided the trigger to prepare the PUSCH. A second grant asks it to transmit the PUSCH. Since the UE already has the PUSCH packet prepared prior to the second grant, the delay from end of second grant to PUSCH transmission is thus reduced.

For NR PUSCH timing capability 1, the PDCCH to PUSCH delay is 10 symbols (~710us) for 15KHz SCS and 12 symbols (~430us) for 30KHz SCS. Although this gap is much smaller than what LAA had between the PDCCH and PUSCH, the delay is still very large compared to the duration needed for cat-4 LBT. For example, a node with contention window of 16, in absence of other interferers, would complete its cat-4 LBT in at most 205us which is much smaller than the PDCCH to PUSCH delay in NR. This can thus lead to consistent loss of medium to neighbouring nodes. Hence, we propose that two-stage grant should also be supported in NR-U.

Proposal 5: Two-stage grant where a first grant gives the parameters necessary for PUSCH transmission preparation and a second grant that provides the trigger for PUSCH transmission should be supported for scheduling PUSCH.

# HARQ ACK Feedback

NR supports fairly flexible HARQ operation. In both DCI Format 1-0 and DCI Format 1-1 there is a 3-bit HARQ ACK delay field (PDSCH-to-HARQ_feedback timing indicator) that can be used to control the location of the ACK. In DCI Format 1-1 the delay values come from a higher layer configured set while for DCI Format 1-0 the delay is from the set {1, 2, .., 8}.

NR supports two modes for determining the ACK codebook size – semi-static mode and dynamic mode.

- In semi-static mode the ACK codebook size is determined based on the maximum number of TBs across cells and PDCCH monitoring occasions in time that can be configured to have ACK on the same slot. This mode is more robust to missed and false DCI detections than dynamic mode but comes at the expense of more bits for ACK feedback.

- In dynamic mode the ACK codebook size is determined based on the number of actually received DCIs pointing to the same ACK slot. To make this robust to missed and false DCI detections, a counter DAI and total DAI field is introduced in the DCI.

These modes enable adapting the HARQ ACK feedback to dynamic frame structures. The fast HARQ-ACK feedback helps control the number of HARQ processes that are needed and to adapt to changing interference conditions. Additionally, depending on the time between the DL and UL switches, the feedback and following DL transmissions may not need any LBT gaps. Similar considerations apply for UL HARQ operation. Allowing grants between UL slots in TxOP provides gNB more scheduling flexibility such as for adapting the rate to changing interference conditions. We hence propose that the NR based HARQ-ACK procedures and timelines also be supported for NR-U.

Proposal 6: Switches between DL and UL for exchange of control information during data transmission should be supported. The switching should not require any LBT when the gap is within the regulatory requirements. This includes

- UCI transmission such as for HARQ ACK between DL slots carrying PDCCH/PDSCH

- DCI transmission such as for UL grants between UL slots carrying PUCCH/PUSCH

In both the semi-static and dynamic ACK feedback modes, the ACKs that are included in a PUCCH/PUSCH are determined based on the configured delay. Hence the ACK for any particular HARQ process has only one opportunity of being sent. Having a single opportunity for ACK feedback may result in not transmitting ACK in many cases due to LBT failures. For example, in Figure 4, if the LBT fails in the first LBT gap, the HARQ ACK feedback for the first two HARQ processes are never sent. To make the system more robust to LBT related failures, it is desirable to have multiple opportunities to send the HARQ ACK feedback.

Figure 4: HARQ ACK Timeline options leveraging the PDSCH to ACK delay field

In RAN1#93, it was agreed that

- NR-U uses NR HARQ feedback mechanisms as baseline, and enhancements can be identified

- Techniques to handle reduced HARQ A/N transmission opportunities for a given HARQ process due to LBT failure are identified as beneficial

  - Potential techniques include mechanisms to provide multiple and/or supplemental time and/or frequency domain transmission opportunities

- When UL HARQ feedback is transmitted on unlicensed band, NR-U considers mechanisms to support flexible triggering and multiplexing of HARQ feedback for one or more DL HARQ processes

In line with these agreements we propose specific mechanisms to provide multiple Tx opportunities for HARQ ACK feedback as well as for flexible triggering and multiplexing of HARQ ACK feedback for multiple DL HARQ processes. The first is group HARQ-ACK feedback and the second uses a configurable window size specified in the DCI. Details of these mechanisms are described in the subsections below.

## Group HARQ ACK feedback

In NR-U operation, in some cases, the gNB needs to create LBT gaps for switches from DL to UL which increase overhead and results in potential loss of access to the medium. To avoid these issues gNB may choose a frame structure that has just one switch from DL to UL. LTE eLAA adopted such a design for example. In this case we may want a larger set of delay values that are supported. For example, DCI Format 1-0 currently supports only delay of 1 to 8 which means that with a SCS of 60KHz, the maximum contiguous DL portion can be at most of the order of 2ms (8 * 0.25ms). This however doesn’t meet the objective of providing multiple opportunities to transmit the ACK.

An alternate way to solve this problem is to not include an explicit ACK delay field in the DCI. Instead have an ACK trigger that causes the UE to transmit the HARQ-ACK bits for all configured HARQ processes, i.e., have a group HARQ-ACK feedback. This is shown in Figure 5.

Figure 5: HARQ ACK Timeline with group HARQ-ACK feedback

It is desirable to support both kind of solutions in NR – one that leverages the NR PDSCH to HARQ ACK delay field as well as one that uses the one-shot group HARQ ACK feedback. Hence, we have the following proposals:

Proposal 7: Support configurable bitwidth for PDSCH-to-HARQ_feedback timing indicator from 0 to N bits

- 0 bit to handle triggered group HARQ ACK feedback

- N>3 bits to support more flexible feedback timing

Proposal 8: Introduce the following mechanisms to trigger group HARQ ACK feedback for all configured HARQ processes

- A trigger included in the DL PDSCH grant

- A trigger included in a PUSCH grant

- A separate DCI

A separate DCI could be common DCI or a UE-specific DCI. Mechanisms to control which HARQ IDs are included in the feedback could be based on a window size included in the DCI (as described below) or a bitmap which indicates which HARQ IDs to include in the feedback.

NR supports CBG based HARQ-ACK feedback. Similar CBG-based feedback support should also be provided for the group HARQ-ACK feedback for NR-U. However, compression schemes for the CBG level group HARQ-ACK feedback should also be considered due to the high overhead it entails.

Proposal 9: The group HARQ-ACK feedback should also support CBG level ACK/NACK feedback

## HARQ ACK codebook size determination enhancements

In the current NR design for HARQ ACK codebook size determination, if the supported delays are from {1,2,..,8} the ACK codebook size would be 8 x #ACK bits for one TB. Now if we increase the number of supported delays, the ACK codebook size would increase significantly. It may not be efficient to always mandate transmission of the larger sized ACK codebook.

Figure 6: Dynamic adaptation of max delay for ACK codebook size determination

To avoid increasing the number of ACK bits in every feedback a new field in the DCI can indicate the max delay to consider for the ACK feedback. As an example, in Figure 6, we show a case where the DCI can indicate the max delay to be either 4 or 8. When things function normally the delay can be set to 4 so HARQ ACK feedback for 4 HARQ processes is included in each UL ACK feedback. However, if an LBT failure occurs the gNB can set the delay to 8 HARQ processes thereby giving a second opportunity for feedback of the HARQ ACK of the first 4 processes. Note that in such a solution UE may be expected to transmit ACK on a slot different from the one indicated by the ACK delay field in the DL grant. The new field may also control such behaviour. For example, it may indicate whether the feedback should include ACK feedback for all detected DL grants that are in the window or only include the detected DL grants that point to this slot. Such a solution thus helps control the ACK overhead while at the same time enables providing multiple opportunities to transmit the ACK.

Proposal 10: Introduce a new field in the DCI that controls the max delay considered in semi-static HARQ ACK codebook size determination.

## Enhancements to interpretation of HARQ ACK delay field

Currently in NR, an ACK is transmitted corresponding to each received DCI in the slot determined based on the HARQ-ACK delay field. In NR-U however, in some scenarios we may not want an ACK to be triggered for some DL grants. For example, if the grant leads to the HARQ-ACK feedback outside the TxOP, we may want the ACK feedback to be skipped related to current grant and the ACK feedback may be explicitly triggered in subsequent TxOP. We could add an explicit trigger or reserve a delay value in the HARQ-ACK delay field to indicate whether ACK needs to be transmitted or skipped corresponding to the current PDSCH.

Proposal 11: Introduce mechanisms to disable ACK transmissions corresponding to a DL grant

# Conclusion

The proposals made in this contribution are summarized below.

Proposal 1: Introduce multi-slot grants with the TB(s) rate matched jointly across the multiple allocated slots

Proposal 2: Number of bits in the UL PUSCH time domain resource assignment field are increased

Proposal 3: The multi-TTI grants should include support for transmitting mini-slots in the initial portion of the transmission followed by switching to full slot at slot boundaries.

Proposal 4: The multi-TTI grants should also support CBG based retransmissions

Proposal 5: Two-stage grant where a first grant gives the parameters necessary for PUSCH transmission preparation and a second grant that provides the trigger for PUSCH transmission should be supported for scheduling PUSCH.

Proposal 6: Switches between DL and UL for exchange of control information during data transmission should be supported. The switching should not require any LBT when the gap is within the regulatory requirements. This includes

- UCI transmission such as for HARQ ACK between DL slots carrying PDCCH/PDSCH

- DCI transmission such as for UL grants between UL slots carrying PUCCH/PUSCH

Proposal 7: Support configurable bitwidth for PDSCH-to-HARQ_feedback timing indicator from 0 to N bits

- 0 bit to handle triggered group HARQ ACK feedback

- N>3 bits to support more flexible feedback timing

Proposal 8: Introduce the following mechanisms to trigger group HARQ ACK feedback for all configured HARQ processes

- A trigger included in the DL PDSCH grant

- A trigger included in a PUSCH grant

- A separate DCI

Proposal 9: The group HARQ-ACK feedback should also support CBG level ACK/NACK feedback

# Proposal 10: Introduce a new field in the DCI that controls the max delay considered in semi-static HARQ ACK codebook size determination.

Proposal 11: Introduce mechanisms to disable ACK transmissions corresponding to a DL grant

# References

[1] RP-172021, Revised SID on NR-based Access to Unlicensed Spectrum, Qualcomm Incorporated, RAN#77, Sapporo, Japan, September 11 - 14, 2017.

[2] RP-181339, “Revised SID on NR-based Access to Unlicensed Spectrum”, Qualcomm Incorporated, RAN#80, La Jolla, USA, June 11-14, 2018.